3GPP TSG-RAN WG2 Meeting #121bis-e	R2-2303065

# Electronic, April 17 – 26, 2023				               Revision of R2-2301563

Agenda item:	7.4.2.3

Source:	Samsung

Title:	Considerations on Cell Switch for LTM

Document for:	Discussion & Decision

Introduction

In RAN2#120 meeting, RAN2 made following agreements on the cell switch for LTM [1].

Cell switch The MAC CE agreed to carry LTM related information for cell switch is used for LTM triggering of the cell switch. LTM cell switch is supervised by a timer UE arrival in the target cell need to be indicated (somehow) Security Permanent Identities such as PCI will not be used in L1 L2 signalling, instead L1 L2 signalling will use temporary identities configured by RRC.

In this contribution, we consider some detail issues regarding LTM MAC CE following: beam information handling and L2 security protection.

Beam information for LTM MAC CE

RAN2 has agreed that L1 measurement is assumed to trigger LTM, and RAN2 has confirmed that the details will be determined by RAN1 decision. In Rel-17 feMIMO WI, inter-cell beam management (ICBM) using unified TCI framework is operated with the enhanced L1 measurement and report procedure which enables to measure/report the SSB/CSI-RS resources associated with PCIs different from the PCI of a serving cell. CSI resources to be measured are configured and list of references to SSB resources used for CSI measurement/report in a CSI-RS resource set is provided.

- CSI-SSB-ResourceSet includes the additional PCI info for CSI resource set.

- The number of additional PCI configured for CSI-RS is at most 7 (i.e. servingAadditionalPCIList-r17).

CSI-SSB-ResourceSet ::=             SEQUENCE {

csi-SSB-ResourceSetId               CSI-SSB-ResourceSetId,

csi-SSB-ResourceList                SEQUENCE (SIZE(1..maxNrofCSI-SSB-ResourcePerSet)) OF SSB-Index,

...,

[[

servingAdditionalPCIList-r17           SEQUENCE (SIZE(1..maxNrofCSI-SSB-ResourcePerSet)) OF  ServingAdditionalPCIIndex-r17    OPTIONAL  -- Need R

]]

}

ServingAdditionalPCIIndex-r17  ::=  INTEGER(0..maxNrofAdditionalPCI-r17)

CSI report is configured in ServingCellConfig and following entities are configured:

- Report type (periodic, semi-persistent for PUCCH, semi-persistent for PUSCH, aperiodic)

- Report quantity

- Etc.

We assume this L1 measurement/report procedure would be the baseline of Rel-18 LTM as well. Based on L1 measurement results reported by the UE, the NW can know which beam associated with the target cell is suitable for the UE. Therefore, when triggering the cell switch by L1/L2 signaling (e.g., MAC CE), the NW can indicate the TCI state(s) associated with the target cell to be activated upon cell switch. In this way, instead of further refinement of beam selection based on L1 measurement after L3 HO execution, the UE can directly apply the indicated TCI state(s) in LTM for a better performance.

In RAN1#111 and RAN1#112 meetings [2][3], RAN1 agreed following agreements regarding the L1 measurement and report principles for LTM.

RAN1#111 meeting Agreement - For candidate cell measurement for Rel-18 LTM, - SSB based L1-RSRP is supported for intra-frequency measurement - SSB based L1-RSRP is supported for inter-frequency measurement from RAN1 point of view - FFS: L1-SINR, CSI-RS based L1-RSRP Agreement - The beam indication of candidate cell(s) for Rel-18 LTM should be designed based on the following: - Beam indication for Rel-18 LTM is designed based on Rel-17 unified TCI framework, if both serving cell and candidate cell support Rel-17 unified TCI framework - FFS: whether/how to design mechanism for Beam indication for Rel-18 LTM when at least one from serving cell and candidate cell supports only Rel-15 TCI framework. - Note: How and whether to indicate the new serving cell(s) and timing for beam indication are separately discussed Agreement - For gNB scheduled L1 measurement report for Rel-18 LTM, report as UCI is supported - Semi-persistent report on PUSCH, and aperiodic report on PUSCH are supported - FFS: periodic and semi-persistent PUCCH - In a single report instance, report for serving cell and candidate cell(s) for intra-frequency and/or inter-frequency can be included. RAN1#112 meeting Agreement - RAN1 shares the same understanding as RAN2 on agreement: - The LTM mobility trigger information is conveyed in a MAC CE - The same MAC CE is used for the LTM triggering. Agreement - The agreement on scenario 2 (Beam indication together with cell switch command) at RAN1#111 is further clarified as the following: - Beam indication for the target cell(s) is conveyed in the MAC CE used for LTM triggering for scenario 2 Agreement - For L1-RSRP measurement RS configuration - For SSB based L1-RSRP measurement: - As a starting point, at least the following information needs to be provided to a UE, e.g. - For intra- and inter- frequency: PCI or logical ID (e.g., as being defined in R17 ICBM), time domain (e.g. SMTC or periodicity and SSB position in burst) - For inter-frequency: frequency domain location (e.g. center frequency), SCS - FFS: transmission power (for pathloss calculation) - Note: other parameters included in the configuration can be further discussed - Including above agreement into the LS - The detailed design of RRC structure is up to RAN2, and send an LS to RAN2 to request to work on the RRC structure design on the measurement configuration. - Following RAN1 understanding will be provided in the LS - RAN1 has discussed the following configuration options for L1 measurement configurations for SSB till RAN1#112: - Option 1) Configurations for L1 measurement RS is provided under ServingCellConfig for the serving cells - is useful to reuses the mechanism for Rel-17 ICBM and necessary information to support inter-frequency measurement will be added there. - Option 2) Configurations for L1 measurement RS is provided separately from ServingCellConfig for the serving cells and CellGroupConfig for the candidate cells - is useful to avoid the duplicated configurations for L1 measurement RSs, [and avoid UE to process configurations for L1 measurement RS provided under CellGroupConfig for the candidate cells] - Option 3) Configurations for L1 measurement RS is provided under CellGroupConfig for the candidate cells - can achieve the similar benefit as Option 2) by directly referring to the candidate cell configurations. - Note RAN2 has a full flexibility to design the whole RRC structure design. - RAN1 believes this is RAN2 expert region, and respectfully asks RAN2 to finalize the RRC structure design after RAN1 finalizes the discussion on RRC parameters. - It is noted that RAN1 foresees the necessity of similar discussions on TCI state pool for candidate cells and L1 measurement report configurations.

Proposal 1: MAC CE triggering LTM can indicate TCI state(s) to be activated for the target cell.

By inter-cell beam management (ICBM), a UE can receive or transmit UE dedicated channels/signals via a TRP associated with a PCI different from the PCI of a serving cell. In RAN2#119-e meeting, RAN2 has agreed that ICBM is one scenario considered for L1/L2 mobility. Thus, the cell with additional PCI configured for ICBM should be a candidate cell for LTM.

Proposal 2: The cell with additional PCI for ICBM can be configured as a candidate cell for LTM.

For a UE under ICBM, i.e., receiving or transmitting UE dedicated channels/signals via a TRP associated with a cell with additional PCI, LTM can be triggered for cell switch to a target cell, where the target cell can be the cell with additional PCI for ICBM. In this case, the NW can send the L1/L2 signaling via the dedicated channels from the cell with additional PCI. By this operation, UE first under the ICBM on the serving cell connection, but NW indicate HO this UE to the target cell by LTM based on L1 measurement/report. It means two step operation of ICBM and LTM should be supported as well as one step LTM with beam indication.

Proposal 3: For the UE under ICBM, the NW can send L1/L2 trigger signaling (i.e. LTM) via the cell with additional PCI.

RAN1 requested the RRC structure design on the measurement configuration, following options are now on the table. See the below options and pros/cons provided from RAN1.

- Option 1: Configurations for L1 measurement RS is provided using csi-MeasConfig IE under ServingCellConfig for the serving cells

  - is useful to reuses the mechanism for Rel-17 ICBM and necessary information to support inter-frequency measurement will be added there.

- Option 2: Configurations for L1 measurement RS is provided separately from ServingCellConfig for the serving cells and CellGroupConfig for the candidate cells

  - is useful to avoid the duplicated configurations for L1 measurement RSs, [and avoid UE to process configurations provided under CellGroupConfig for the candidate cells]

- Option 3) Configurations for L1 measurement RS is provided under CellGroupConfig for the candidate cells

  - can achieve the similar benefit as Option 2) by directly referring to the candidate cell configurations.

As we proposed in above, we think joint operation with ICBM and LTM could be supported so option 1 is the most simple and efficient approach to support both features i.e. reusing the L1 measurement configuration for ICBM and add further configurations to support inter frequency.

Proposal 4: Reuse the L1 measurement configuration for ICBM and add further configurations to support inter frequency (i.e. configurations for L1 measurement RS is provided using csi-MeasConfig IE under ServingCellConfig for the serving cells).

As LTM is triggered based on L1 measurement without L3 filtering, the advantage of LTM is that the UE can switch to the target cell with a better beam in a shorter interruption time, while the drawback is the ping-pong effect. In order to take the advantage of fast cell/beam switch and mitigate the ping-pong effect, one way is that the UE first applies ICBM as a transition phase and then triggers LTM if certain conditions are fulfilled. The NW can configure LTM execution conditions in RRC pre-configuration for candidate cells. The following options can be considered for UE-triggered LTM.

- Option 1: Timer-based LTM triggering

- The UE starts a timer upon receiving a TCI state switch that requests UE to use a beam from a candidate cell.

- The UE re-starts the timer upon receiving a TCI state switch that requests UE to use a beam from a different candidate cell.

- The UE stops the timer upon receiving a TCI state switch that requests UE to use a beam from the serving cell.

- At the expiry of the timer for a candidate cell, the UE executes handover to that candidate cell.

- Option 2: Counter-based LTM triggering

- UE counts the number that L1-RSRP of candidate cell is better than the serving cell.

- UE triggers the LTM when the counter reaches the configured max value.

- Option 3: NW indication based LTM triggering

- UE just operate based on NW signaling i.e. if LTM is required NW will send the LTM signaling.

- In this case, mitigating ping-pong effect is up to NW implementation.

Proposal 5: For the UE under ICBM, consider how UE initiates LTM to mitigate ping-pong effect (i.e. determine if the execution condition is required or just follow the NW indication).

Security protection for LTM MAC CE

In RAN2#120 meeting, RAN2 discussed the security issues on the LTM MAC CE and made some agreement that the LTM MAC CE will use the temporary identities configured by RRC to indicate the LTM candidate cell. In addition, there was some initial discussion if the security protection such as integrity and ciphering in MAC layer is necessary for the LTM. In legacy, PDCP is the only place to perform ciphering and integrity protection. However, we think the question to introduce the security protection for the LTM MAC CE is valid, RAN2 needs to study more the pros/cons on this issue. If the LTM MAC CE triggers the HO but not protected, the false base station (gNB) could attack the UE to indicate HO wrongly. All MAC CEs have introduced are used for some functionality for better performance in the serving cell(s) but LTM MAC CE is used to indicate the cell change. The cell change requires setup/release all configurations from the target/source cell and this is very important procedure (i.e. consist of many procedure steps). We think that the LTM MAC CE without security protection causes the severe reliability trouble for inter-cell mobility.

Ciphering and integrity protection are performed in PDCP but these security protection not consider the protection of RLC control PDU, PDCP control PDU, and MAC CE. Here, we consider how to support the security protection for (LTM) MAC CE. First, we need to consider is which part would be protected and how to protect it.

- Issue 1: Protection part of MAC PDU

- Only considers the UL MAC PDU for Rel-18 LTM MAC CE

- UL MAC PDU format: see the below Figure 1

Figure 1. UL MAC PDU format

  - MAC subPDU can include MAC CE or MAC SDU or padding.

  - MAC SDU can include RRC message/User traffic/RLC header/PDCP header/PDCP control PDU/RLC control PDU

  - Option 1: Protection for whole MAC PDU, including control PDUs

    - Integrity

- New layer 2 MAC-I is placed in front of MAC PDU with a separate LCID for this MAC sub-header

- Protection is not applied to padding to reduce the processing burden.

    - Ciphering

- Ciphering could be applied to all MAC subPDUs except for the new layer 2 MAC-I.

- Protection is not applied to padding to reduce the processing burden.

  - Option 2: Apply the protection for the MAC CEs

    - Integrity

- All MAC CEs included in one MAC PDU are protected together and the new layer 2 MAC-I field is located after MAC subPDUs for MAC CEs.

- Define a separate LCID for this MAC sub-header

    - Ciphering

- Ciphering could be applied to all MAC subPDUs for MAC CEs

  - Option 3: Apply the protection only for the LTM MAC CE

    - Integrity

- Only LTM MAC CE included in one MAC PDU are protected (i.e. LTM MAC CE could be designed as adding a new layer MAC-I field)

    - Ciphering

- Ciphering could be applied to only LTM MAC CE

- Issue 2: Security protection details

  - Which key mechanism is used: Could use symmetric key mechanism as used in PDCP

  - Which algorithm is used: Could use legacy ciphering or integrity protection algorithms as used in PDCP

  - Need to check SA3 which input for algorithm could be used.

  - Which entity has a role for L2 security protection

    - Alt 1: MAC entity requires the new functionality to check L2 security protection (integrity and ciphering for MAC CE)

    - Alt 2: MAC and PDCP needs the interaction for L2 security protection

Based on our above analysis, introducing the security protection in LTM MAC CE could be possible even RAN2 needs to study further for the details. We think it would be worth to introduce the L2 security protection using a simple and valid approach. For the first issue (Protection part of MAC PDU), we think the Option 3 is the simple way RAN2 could introduce in Rel-18 feMob WI because it has a less impact than other options. For the second issue (Security protection details) if RAN2 to determine to introduce the L2 security protection especially for the LTM MAC CE, RAN2 could discuss the detail solution in order to make L2 security protection valid considering above listed issues.

Proposal 6: The new L2 security protection for LTM MAC CE is introduced.

Proposal 7: RAN2 determine the detail solution in order to make L2 security protection valid considering following listed issues.

  - Which key mechanism is used: Could use symmetric key mechanism as used in PDCP

  - Which algorithm is used: Could use legacy ciphering or integrity protection algorithms as used in PDCP

  - Need to check SA3 which input for algorithm could be used.

  - Which entity has a role for L2 security protection

    - Alt 1: MAC entity requires the new functionality to check L2 security protection (integrity and ciphering for MAC CE)

    - Alt 2: MAC and PDCP needs the interaction for L2 security protection

Conclusion

Based on the above, RAN2 is requested to discuss and agree on the following proposals:

Proposal 1: The cell with additional PCI for ICBM can be configured as a candidate cell for LTM.

Proposal 2: For the UE under ICBM, the NW can send L1/L2 trigger signaling (i.e. LTM) via the cell with additional PCI.

Proposal 3: For the UE under ICBM, consider how UE initiates LTM to mitigate ping-pong effect (i.e. determine if the execution condition is required or just follow the NW indication).

Proposal 4: Reuse the L1 measurement configuration for ICBM and add further configurations to support inter frequency (i.e. configurations for L1 measurement RS is provided using csi-MeasConfig IE under ServingCellConfig for the serving cells).

Proposal 5: For the UE under ICBM, consider how UE initiates LTM to mitigate ping-pong effect (i.e. determine if the execution condition is required or just follow the NW indication).

Proposal 6: The new L2 security protection for LTM MAC CE is introduced.

Proposal 7: RAN2 determine the detail solution in order to make L2 security protection valid considering following listed issues.

  - Which key mechanism is used: Could use symmetric key mechanism as used in PDCP

  - Which algorithm is used: Could use legacy ciphering or integrity protection algorithms as used in PDCP

  - Need to check SA3 which input for algorithm could be used.

  - Which entity has a role for L2 security protection

    - Alt 1: MAC entity requires the new functionality to check L2 security protection (integrity and ciphering for MAC CE)

    - Alt 2: MAC and PDCP needs the interaction for L2 security protection

References

Chairman notes for RAN2#120 meeting	RAN2.

Chairman notes for RAN1#111 meeting	RAN1.

Chairman notes for RAN1#112 meeting	RAN1.