3GPP TSG-RAN WG2 #121-bis	Tdoc R2-2303068

Electronic meeting, 17th April – 26th April 2023

Agenda Item:	7.8.2

Source:	Ericsson

Title:	UAV measurement reports

Document for:	Discussion, Decision

# Introduction

As described in [1], the WID for supporting UAV in NR includes an objective on enhancing measurement reports, as follows:

1. Specify the following enhancements on measurement reports [RAN2]: - UE-triggered measurement report based on configured height thresholds - Reporting of height, location and speed in measurement report - Flight path reporting - Measurement reporting based on a configured number of cells (i.e. larger than one) fulfilling the triggering criteria simultaneously Note: Work done in LTE is a starting point for this objective. NR-specific enhancements can be considered, if needed, while overall the LTE and NR solutions should be harmonized as much as possible.

In RAN2#119e, the following agreements are made related to enhancement for measurement reports in NR:

Agreements - Use LTE principle as a baseline, introduce similar event H1 (aerial UE height become higher than threshold) and H2 (aerial UE height become lower than threshold. FFS if further NR enhancements are needed. FFS study scaling of RRM parameters (e.g. which parameters and what is the purpose/benefit of the scaling and how) FFS how to limit excessive measurements and measurement reporting FFS if user consent is needed for location reporting in CONNECTED FFS study the vertical movement and associated mobility for UAV UEs - Rel-18 NR supports reporting of UAV UE’s height, location and velocity. It is for further study what accuracy and reporting mechanisms are required and if further enhancements are needed. - As in LTE, flight path plan reporting will be introduced. Location list of waypoints (3D location information) and timestamp is adopted as the basic content of flight path report. FFS if timestamp is mandatory or optional for NR. FFS if further enhancements are needed - Introduce similar functionality to LTE (numberofTriggeringCells). FFS whether numberoftriggerbeams for NR is required or other enhancements. FFS study how to avoid sending the measurement reports mainly due to reportOnLeave.

In RAN2#119bis-e, the following agreements were made:

Agreements: - The time information reported as part of flight path plan is optional. UE includes time info, if configured by the network and available at the UE. FFS on flight path details (waypoints and what is time information). - Allow the flight path to be updated. FFS on the details. - FFS on reporting format and initial flight path reporting (i.e. what information to report and how) – next meeting - Continue to study height-depending scaling, triggering and combinations - As in LTE, as a baseline, events A3, A4 and A5 can be configured with the configured number of cells (numberofTriggeringCells).

In RAN2#121, the following agreements were made to enhance the measurement report for mobility and interference control:

Agreements:

- When event H1 or H2 triggers, the content of the measurement report is configurable by the network (i.e. it can contain UAV UEs height, location information and/or RSRP/RSRQ measurement results). FFS whether UAV UE’s height is mandatorily reported and which parameter/IE is used for height reporting.

- Joint use of height-dependent condition and RSRP/RSRQ/SINR-based condition for measurement report triggering is supported in NR Rel-18 UAV.   The combination of existing events will be used

- Height-dependent parameter scaling is not supported as a part of Rel-18 NR

- Do not extend the Number of triggering cells mechanism to apply to the inter-RAT scenario, i.e. event B1 and B2 triggering

- Do not restrict the applicability of Number of triggering cells mechanism to FR1 only. In other words, the Number of triggering cells mechanism is applicable to FR1 and FR2 (up to network configuration).

- The UE shall not ignore or bypass the Number of triggering cells mechanism, once configured.

- Do not introduce the use of a “numberOfTriggeringBeams” mechanism.

- Do not introduce an alternative mechanism to the Number of triggering cells mechanism.

- Do not introduce an additional mechanism based on Number of changed cells.

- For the purpose of interference control (i.e. for number of trigger cells), do not introduce a prohibit timer mechanism.

- Report on leave is not triggered by a cell that was not previously included in the measurement report for the number of triggering cell.

Another agreement was made for “more than a single config, each for a specific height region” as follows:

Agreements:

- Support configuring height-dependent more-than-one configurations targeting measurement and measurement reporting enhancement. UE applies corresponding configuration based on the UE height. The proposed solutions should aim at avoiding RAN4 impacts. FFS how this would be configured (i.e. different MO configurations or different parameters. FFS Exact parameters and details.

In this paper we discuss these agreements and related FFS about measurement report enhancement for UAV UE in NR.

# Support for ECC decision (22)07

## Location reporting for no-transmit zone

Besides flight path reporting, in many cases it is beneficial for the network to know that a UAV is entering or leaving a certain area. For example, when a drone is to perform a mission in a certain geographical area in a certain time window according to a plan, it can be useful for the network to know that the drone is actually entering or leaving the area so that the network can optimize the connectivity for the mission of the drones. Especially, in many scenarios, the so-called no-transmit zones (NTZ) [12] (or sometimes no-fly zones) are imposed to limit the transmissions from drones, e.g., in order not to cause too much interference to a nearby base station. In fact, in the ECC decision (22)07 on “harmonised framework on aerial UE usage in MFCN harmonised bands”, it stated that (see below) a mechanism is necessary to ensure that aerial UE respect no-transmit zones.

ECC decision (22)07 on “harmonised framework on aerial UE usage in MFCN harmonised bands” requires a mechanism to ensure that aerial UE respect no-transmit zones.

From a network perspective, it is important for the network to know when a connected drone is entering a NTZ, for the network to be prepared to maintain the connectivity with the drone (e.g., to switch to another frequency band or carrier that is allowed in the NTZ). Vice versa, knowing when the drone is leaving a NTZ enables the network to recover to a previous mode of communication with the drones.

It is beneficial for the mobile network to know when a UAV is entering or leaving a certain area or zone, e.g., a no-transmit zone.

- Support UE reports triggered when a UAV enters or leaves a special area or zone, e.g., a no-transmit zone.

# User consent on location reporting

Another FFS from RAN2#119e is whether user consent is needed for location reporting in CONNECTED. In our view, as this topic has been discussed in NTN WI and it is a general issue which involves other WGs like SA2 and SA3, RAN2 can consider the decisions from these WGs when available.

- RAN2 to work on signalling for location reporting and consider the decision from SA2 and SA3 on the topic of user consent-based location report once available.

# Controlling the amount of measurement reporting

While a triggering condition based on multiple cells reflects interference conditions while airborne, it is not a suitable measurement event configuration for mobility performance where it is crucial to report as quickly as possible since the serving cell quality may drop fast. In fact, the mobility evaluations in Rel-15 used the shortest TTT of 40ms with RSRP triggering based on single cell as the triggering based on multiple cells would be delayed.

For ensuring mobility performance, an aerial UE should be configured with single cell RSRP reporting with short TTT.

Due to the high mobility nature of airborne UEs, the number of measurements for mobility as discussed above can be large. When the number of airborne UEs in the network increases, the total volume of such measurements can negatively impact the resource utilization and network performance. This issue was discussed in Rel-15 and also during the preparation of the Rel-18 WI [7].

RSRP measurement reporting for mobility can create high load in the network. Therefore, it is beneficial to control the amount of reporting.

It is therefore important to limit the volume of UE measurement reports to a reasonable level. Here we review the existing mechanisms to control the measurement reporting. The TTT and hysteresis are used to control reporting if the value for the metric goes back and forth of the threshold value potentially causing entering/leaving of the condition. This is efficient, e.g., for terrestrial UEs which move around on cell edge to prevent ping pong effect for HO.

TTT and hysteresis control reporting amount when the value for the metric goes back and forth of the threshold value potentially causing entering/leaving of the condition.

For periodical reporting there are parameters reportInterval and reportAmount. The ReportInterval indicates the interval between periodical reports. The ReportInterval is applicable if the UE performs periodical reporting (i.e., when reportAmount exceeds 1), for triggerType event as well as for triggerType periodical. These parameters control reporting after event has triggered due to a cell. These parameters do not affect reporting when the event is triggered due to different cells.

Parameters reportInterval and reportAmount do not affect reporting when the event is triggered due to different cells.

When an aerial UE flies or takes off, an event triggers consecutively due to different cells as observed in [4]. For example, cell A, B, C and D may exceed threshold one after another and start their respective TTT follow up. Once TTT expires for A UE sends report, then it expires for B and then for C. The report sent based on B and C are very similar and difference start to show maybe around report D. Thus, it is seen beneficial to be able to skip report based on B and possibly also C.

When an aerial UE flies or takes off, an event triggers consecutively due to different cells. This may lead to redundant UE measurement reports.

Thus, we propose that RAN2 specify mechanism to control the amount of measurement reporting. The proposed way is to limit the measurement reporting by defining a prohibit timer which prevents sending the measurement reports for a configurable time after the event has been triggered and the first report has been sent. The use of prohibit timer is associated to the reporting configuration. Just as it is proposed for regular mobility events originally, this method can enable timely reporting when the event is triggered first time for the purpose of mobility control, as well as controlling the amount of measurement reporting. For mobility events, when the UE starts to fly, same event is going to be triggered by new cell consecutively. In this case, it is difficult for RRC configuring to impact the mobility behavior as it is slow. Introducing prohibit timer would be an effective way to impact the configuration. As the idea of preventing certain measurement results can result in a worry on decreasing mobility performance, we present simulation results to show that this seem not to be the case for drone UEs. The results were performed under LTE UAV simulation assumptions (TR 36.777) and was first shown in [8], but the observations and conclusions should hold true for NR as well.

We propose to introduce a prohibit timer in NR similar to the CRs RAN2 discussed for LTE in [9][10][11]

- Introduce a prohibit timer in reporting configuration for the purpose of mobility control: UE is prevented from triggering the measurement report for a configurable time after the event is first triggered and first report is sent.

### Simulation results

The main issue of the excessive measurement reporting lies in measurement reports that are being triggered close in time, as seen in figure 1a where up to 3 measurement reports are being sent within less than 500ms. In figure 1b, we introduce a prohibit timer to reduce the measurement reports triggered close in time. In this case the amount of measurement reports is reduced while not impacting performance.

Figure 1. A RSRP-trace of a drone at 50m altitude flying with 44m/s. The yellow lines denote an A3 measurement triggering without a prohibit timer and red line with a prohibit timer.

In order to compare the prohibit timer to the baseline, we introduce two measures, double-triggering and triple-triggering 3. The A3 measurement event with a threshold of 0.5dB is used. Double-triggering is defined as two measurement reports that are triggered within x ms and triple-triggering is defined as three measurement reports being triggered within y ms.

As an example, of the flexibility, a prohibit timer of 80ms and 160ms is introduced. In figure 2a, the amount of A3 single-triggering, double-triggering and triple-triggering can be seen for different heights. It is clear that the double-triggering and triple-triggering are highly reduced and in the case of a 160ms prohibit timer the triple-triggering are completely removed.

Figure 2. a) The amount of single triggering, denoted A3_1, b) amount of double-triggering denoted A3_2 and c) triple-triggering denoted by A3_3

In order to make sure that the introduced prohibit timer does not affect performance, we also measure the amount of RLFs declared, handovers initiated and handover failures which can be seen in figure 3a, 3b and 3c.

Figure 3. a) The amount of RadioLinkFailures declared, b) the amount of handover initiated and handover failures.

From figure 3 we can see that the RLFs, handover initiated and handover failures remain almost constant. The reason for this can be seen in figure 1. In cases where many measurement reports are sent consecutively in time, the interference is very large and it is unclear whether the condition reported in the measurements will remain, thus delaying some of them will not hurt the performance, because if the condition is still present, then the report will eventually surface.

# Height-dependent configuration for reporting measurements

In the email discussion [Post121][313], the following is concluded:

How to configure (different MO or different parameters)

Proposal 3: (13/15) Height-dependent more-than-one configurations is supported on parameter/field level (i.e. different fields/values within the same MO) where different values (or value ranges) of the parameter/field applies to different height or height range.

Which IEs/parameters can be modified

Proposal 1: For MO configuration parameters: at least the following will have ability to be configured with height-dependent more-than-one configurations/values, each for a specific height region: SSB-ToMeasure. Details on how to specify is FFS.

Proposal 2: For MR configuration parameters: at least the following will have ability to be configured with height-dependent more-than-one configurations/values, each for a specific height region: Event A4 threshold. Details on how to specify is FFS.

Expected UE behavior

Proposal 4: When height-dependent more-than-one configurations are provided, UE applies the new value once it moves to new height (or height range) similar to the case of RRC reconfiguration. Need Codes, field descriptions, etc. as in legacy specifications apply.

Proposal 5: If a height-specific value is not explicitly configured for certain height, whether to keep using the value that was used or consider the parameter as released (i.e. parameter/value not applicable at this height) should be looked into case by case, and can be clarified by need code, field description, or procedural text as needed.

One of the discussion points in the above email discussion is the expected UE behaviour when the applicable value changes due to the change of height. When the UE moves across a height threshold and the configuration/parameter in the new height region is not changed compared to the configuration/parameter used in the previous height region, in our view, it can be handled by the specification (e.g. by the ASN.1 structure, or by field descriptions) and no necessary UE actions are needed. If the configuration/parameter in the new region is not configured, the expected UE behaviour should be discussed case by case, e.g., depending on the parameter whether it is a valid configuration or not. If the value in the new height region is different compared to the value in the previous height region, it can also depend on the exact parameter. Leaving some changes to UE implementation is also possible, however, feasible and explicit specification is preferred to limit the varieties in UE behaviour.

When the UE enters a new height region and a height-dependent configuration parameter changes, the UE behaviour should be discussed case by case (i.e. depends on what the parameter is).

In the height dependent measurement report configuration, the selected parameters may be configured with different values corresponding to the UE height. For example, report interval, report amount and maximum number of non-serving cells in the ReportConfigNR can be configured corresponding to different heights. These should not lead to issues on updating filters or other ongoing procedures that the UE might have.

- Support height dependent configuration for at least the following parameters: report interval, report amount and maximum number of non-serving cells

# Report content on events H1 and H2

In RAN2#121, the following agreements were reached for the height event:

- When event H1 or H2 triggers, the content of the measurement report is configurable by the network (i.e. it can contain UAV UEs height, location information and/or RSRP/RSRQ measurement results). FFS whether UAV UE’s height is mandatorily reported and which parameter/IE is used for height reporting.

As confirmed in RAN2#119, the event H1 (aerial UE height become higher than threshold) and H2 (aerial UE height become lower than threshold) in LTE will be the baseline for defining similar events in NR. When it comes to the details, several issues need to be discussed by RAN2.

In our view, the height information should always be in the content of H1/H2 triggered measurement report, and whether to include CommonLocationInfo and/or RSRP/RSRQ/SINR measurement results should be configurable by the network. This configurability is important for interference control as it controls the size of the measurement report. Further, it is important to have location and height configurable such that UE can still report height even UE would not report its location.

- The height information should be mandatory in the height triggered measurement report and RAN2 to confirm the height information can be separately configured.

# Even combinations

RAN2 agreed in RAN2#121 the following:

- Joint use of height-dependent condition and RSRP/RSRQ/SINR-based condition for measurement report triggering is supported in NR Rel-18 UAV.   The combination of existing events will be used

RAN2 should discuss how to handle reportOnleave for combined events

- RAN2 should discuss how to handle reportOnleave for combined events.

# Conclusion

In the previous sections we made the following observations:

Observation 1	ECC decision (22)07 on “harmonised framework on aerial UE usage in MFCN harmonised bands” requires a mechanism to ensure that aerial UE respect no-transmit zones.

Observation 2	It is beneficial for the mobile network to know when a UAV is entering or leaving a certain area or zone, e.g., a no-transmit zone.

Observation 3	For ensuring mobility performance, an aerial UE should be configured with single cell RSRP reporting with short TTT.

Observation 4	RSRP measurement reporting for mobility can create high load in the network. Therefore, it is beneficial to control the amount of reporting.

Observation 5	TTT and hysteresis control reporting amount when the value for the metric goes back and forth of the threshold value potentially causing entering/leaving of the condition.

Observation 6	Parameters reportInterval and reportAmount do not affect reporting when the event is triggered due to different cells.

Observation 7	When an aerial UE flies or takes off, an event triggers consecutively due to different cells. This may lead to redundant UE measurement reports.

Observation 8	When the UE enters a new height region and a height-dependent configuration parameter changes, the UE behaviour should be discussed case by case (i.e. depends on what the parameter is).

Based on the discussion in the previous sections we propose the following:

Proposal 1	Support UE reports triggered when a UAV enters or leaves a special area or zone, e.g., a no-transmit zone.

Proposal 2	RAN2 to work on signalling for location reporting and consider the decision from SA2 and SA3 on the topic of user consent-based location report once available.

Proposal 3	Introduce a prohibit timer in reporting configuration for the purpose of mobility control: UE is prevented from triggering the measurement report for a configurable time after the event is first triggered and first report is sent.

Proposal 4	Support height dependent configuration for at least the following parameters: report interval, report amount and maximum number of non-serving cells

Proposal 5	The height information should be mandatory in the height triggered measurement report and RAN2 to confirm the height information can be separately configured.

Proposal 6	RAN2 should discuss how to handle reportOnleave for combined events.

# References

RP-123600, New WID on NR support for UAV (Uncrewed Aerial Vehicles), RAN#94, Dec 2021

TR 36.331, Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification

TS 38.331, NR; Radio Resource Control (RRC) protocol specification

TR 36.777, Enhanced LTE support for aerial vehicles

R2-1807257, Controlling the amount of measurement reporting, Ericsson, RAN2#102, Korea, May 2018

TR 23.754, Study on supporting Unmanned Aerial Systems (UAS) connectivity, Identification and tracking

RWS-210189, New WID on Enhanced NR Support for Aerial Vehicles, Ericsson

R2-1807257, Controlling the amount of measurement reporting, Ericsson

R2-1814762, Remaining aspects of measurement reporting, Ericsson, NTT Docomo, Nokia, Nokia Shanghai Bell

R2-1814760, Prohibit timer for measurement reporting, Ericsson, NTT Docomo, Nokia, Nokia Shanghai Bell, RAN2#103bis, Chengdu, China, 8th - 12th October 2018

R2-1814754, Introduction of capability for timer t3xx for Rel-15 Aerial W, Ericsson, RAN2#103bis, Chengdu, China, 8th - 12th October 2018

ECC report 348, Usage of aerial UE in 1.8 GHz, 2GHz and 2.6 GHz frequency bands with MHCN AAS base stations, Nov. 2022