3GPP TSG-RAN WG2 Meeting #122 electronic	R2-2303211

Online, April 17-26, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 1585 | rev | - | Current version: | 16.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on PSFCH reception for NR sidelink |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Core |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In current specification, it is stated that if an acknowledgement corresponding to the PSSCH transmission is not obtained from the physical layer, UE delivers a negative acknowledgement to the corresponding Sidelink HARQ entity for the Sidelink process. However, this does not apply to NACK-only case and should be clarified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In section 5.22.1.3.2, clarify that if NACK-only is enabled and no acknowledgement is obtained from the physical layer, UE delivers a positive acknowledgement to the SL HARQ entity for the SL process, otherwise, UE delivers a negative acknowledgement to the SL HARQ entity for the SL process. Impact analysis Impacted 5G architecture options: NR SA, NR-DC, NE-DC, (NG)EN-DC Impacted functionality PSFCH reception Inter-operability: If the network is implemented according to this CR while the UE is not, there is no inter-operability issue. If the UE is implemented according to this CR while the network is not, there is no inter-operability issue. If one UE is implemented according to this CR while the other UE is not, there is no inter-operability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE will deliver negative acknowledgement to the SL HARQ entity for the SL process when no acknowledgement is obtained from the physical layer even for NACK-only case, which is not correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.22.1.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

5.22.1.3.2	PSFCH reception

The MAC entity shall for each PSSCH transmission:

1>	if an acknowledgement corresponding to the PSSCH transmission in clause 5.22.1.3.1a is obtained from the physical layer:

2>	deliver the acknowledgement to the corresponding Sidelink HARQ entity for the Sidelink process;

1>	else:

2>	if negative-only acknowledgement was enabled in the SCI:

3>	deliver a positive acknowledgement to the corresponding Sidelink HARQ entity for the Sidelink process;

2>	else:

3>	deliver a negative acknowledgement to the corresponding Sidelink HARQ entity for the Sidelink process;

1>	if the PSSCH transmission occurs for a pair of Source Layer-2 ID and Destination Layer-2 ID corresponding to a PC5-RRC connection which has been established by upper layers:

2>	perform the HARQ-Based Sidelink RLF Detection procedure as specified in clause 5.22.1.3.3.

If sl-PUCCH-Config is configured by RRC, the MAC entity shall for a PUCCH transmission occasion:

1>	if the timeAlignmentTimer, associated with the TAG containing the Serving Cell on which the HARQ feedback is to be transmitted, is stopped or expired:

2>	not instruct the physical layer to generate acknowledgement(s) of the data in this TB.

1>	else if a MAC PDU has been obtained for a sidelink grant associated to the PUCCH transmission occasion in clause 5.22.1.3.1, the MAC entity shall:

2>	if the most recent transmission of the MAC PDU was not prioritized as specified in clause 5.22.1.3.1a:

3>	instruct the physical layer to signal a negative acknowledgement on the PUCCH according to clause 16.5 of TS 38.213 [6].

2>	else if HARQ feedback has been disabled for the MAC PDU and next retransmission(s) of the MAC PDU is not required:

3>	instruct the physical layer to signal a positive acknowledgement corresponding to the transmission on the PUCCH according to clause 16.5 of TS 38.213 [6].

2>	else if HARQ feedback has been disabled for the MAC PDU, and no sidelink grant is available for next retransmission(s) of the MAC PDU (including immediately after all PSSCH duration(s) in an sl-PeriodCG for the sidelink grant, the number of transmissions of the MAC PDU has not reached sl-MaxTransNum corresponding to the highest priority of the logical channel(s) in the MAC PDU, if configured in sl-CG-MaxTransNumList for the sidelink grant by RRC), if any:

3>	instruct the physical layer to signal a negative acknowledgement corresponding to the transmission on the PUCCH according to clause 16.5 of TS 38.213 [6].

2>	else:

3>	instruct the physical layer to signal an acknowledgement corresponding to the transmission on the PUCCH according to clause 16.5 of TS 38.213 [6]

1>	else:

2>	instruct the physical layer to signal a positive acknowledgement on the PUCCH according to clause 16.5 of TS 38.213 [6].

END OF CHANGE