3GPP TSG-RAN WG2 Meeting #121bis-e	R2-2303316

Online Meeting, April 17- 26, 2023

Agenda Item:	7.3.2 DTX/DRX mechanism

Source:	NEC

Title:	UE and gNB behaviors to support cell DTX/DRX

Document for:	Discussion, Decision

# Introduction

There was extensive discussion last RAN2 on the support of cell DTX/DRX operation. The following agreement was reached during such discussion:

Agreements

- There will be no impact to RACH, paging, and SIBs in idle/inactive for both gNB and Rel-18 and legacy UEs

- Rel-18 NES capable CONNECTED UE(s) can perform RACH and receive SIBs in non-active duration of cell DTX and/or DRX (i.e., same behavior for cell DTX and cell DRX).  No further enhancements for CBRA and CFRA will be pursued.

- Pattern configuration for cell DRX/DTX is common for Rel-18 UEs in the cell.   FFS whether we have DTX UE specific inactivity timer .  FFS on configuration signaling and stage 3.

- Confirm study item agreement that we can have separate DTX and DRX configuration.   We will focus on designing DTX/DRX for at least single configuration.  FFS whether multiple configuration of cell DTX or DRX will be supported.

This paper discusses the potential uplink and downlink transmission restrictions to support cell DRX/DTX operation following the agreements made last meeting.

# Discussion

## Dynamic Grant based UL transmission

In NR current re-transmission handling for PDSCH, when a new dynamic DL transmission is received by the UE, it should start the drx-HARQ-RTT-TimerDL in the immediate first symbol after transmitting NACK in the UL. Once drx-HARQ-RTT-TimerDL timer is expired, the UE starts drx-RetransmissionTimerDL timer in the next symbol and becomes active for this duration of this timer. As soon as UE detects a DL transmission for the corresponding HARQ process, it should stop the timer drx-RetransmissionTimerDL.

However, in the context of Cell DRX operation for NES, when there is any overlapping between started drx-HARQ-RTT-TimerUL/drx-RetransmissionTimerUL and the inactive period of Cell DRX cycle, the expected UL grant for PUSCH retransmission from the network to the UE may be delayed until the cell goes to the Cell DRX active period. In this case, the UE should adjust its PDCCH monitoring accordingly for the purpose of its power saving.

It should be noted that it does not make sense for the UE to start the drx-RetransmissionTimerUL timer if the cell prepares to go to the inactive period of the cell DRX cycle. A potential way for the UE to adopt can be to delay the start the of this timer until the cell goes to the Cell DRX active period. This may be implemented by extend the already started drx-HARQ-RTT-TimerUL timer until the cell goes to the Cell DRX active period.

Proposal-1: Discuss the possible handling by the UE for the stop drx-HARQ-RTT-TimerUL timer and the start for drx-RetransmissionTimerUL timer for UL dynamic scheduling.

## Principle for Configured Grant based UL transmission

The following options for UE transmitting on CG occasions during cell DRX inactive period was discussed during the last RAN2 meeting following the corresponding proposal in reference document [1]:

- UE to autonomously deactivate UL CGs during cell DRX non-active periods

- UE keeps the CG active, but the UE does not transmit on CG occasions overlapping with cell DRX non-active periods

- Allow the UE to transmit on CG occasions overlapping with cell DRX non-active periods

- UE is configured with one of the above options per CG, per MAC entity, or per cell DRX configuration

At first, allowing the UE to transmit on CG occasions overlapping with cell DRX non-active periods would cause the cell not really go to cell DRX/DTX, since the NES capable UE(s) may periodically send the UL via CG occasions and the network needs to provide corresponding DL acknowledgement for such transmission. Hence in this case the performance of network energy saving for the cell would degrade. This option should be excluded at this first step. With this said, making the UL CG based transmission configurable is not preferred, since this added flexibility is achieved by the sacrifice of network energy saving for the cell.

Secondly, we do not think there is a big difference for the UE to autonomously deactivate UL CGs or keeps the UL CGs active but does not transmit on UL CG occasions, since both options restrict the UE’s UL transmission on the UL CG occasions, and this autonomous deactivation and activation on UL CG seems to be UE implementation.

Proposal-2: The configuration to allow UE to perform UL CG based transmission during Cell DRX inactive period is not supported.

## Configured Grant based UL transmission

The CG based transmission includes both initial transmission and retransmission. It is questionable on how to support the ongoing CG based initial transmission and retransmission if the some of the CG occasions overlap with non-active periods of cell DRX cycle as shown in Figure-1 below.

Figure-1: Overlapping Example between Configured Grant and cell DRX

As shown within Figure-1, CG TO-1 and CG TO-4 are not overlapped with the cell DRX inactive period, but CG TO-2 and CG TO-3 are overlapped with the cell DRX inactive period.

From initial transmission perspective, it would be beneficial to limit the UE’s UL CG based initial transmission in order to achieve the maximum energy saving gain for the cell. For example, in Figure-1, the CG TO-1 based initial transmission is the transmission right before the Cell DRX inactive period, in other words, it is the last CG TO before the following Cell DRX inactive period. If the initial UL transmission at CG TO-1 is acknowledged, UE can automatically skips CG TO-2 and CG TO-3 before any new UL initial transmission. Alternatively, network can indicate to the UE for such skipping via piggybacked indication in the acknowledgement to the UE for its transmission at CG TO-1. However, if the CG TO is partially overlapped with the cell DRX inactive period (i.e., part of the CG TO falls into cell DRX active period), the initial transmission based on such CG TO may be allowed by the UE.

Proposal-3: initial transmission based on CG TO should be skipped by the UE if the UL CG TO overlaps with the cell DRX inactive period

If the initial transmission at CG TO-1 is initiated and the network cannot decode its reception for such transmission, the network may schedule a dynamic grant but this dynamic grant may not fall into the cell DRX inactive period. This means UE needs to monitor the PDCCH to get the dynamic grant for the HARQ retransmission (if the cell is in the active period of the Cell DTX cycle).

Proposal-4: Retransmission should be scheduled by the network to skip the cell DRX inactive period when CG based initial transmission is not decodable.

## Handling of CGRT and CGT

In NR-U, the CG is configured with CGRT (i.e., cgRetransmissionTimer) and CGT (i.e., configuredGrantTimer). The CGRT is coupled with autonomous retransmission supported by the UE. The NR-U CG operates with contrasting feedback behaviour unlike in NR. If the UE has not received CG-DFI before CGRT expires, the UE assumes Negative ACK (NACK) and can retransmit autonomously in the next TO on the CG. There is also another timer configuredGrantTimer (CGT), which limits the maximum number of autonomous retransmission attempts. As shown in Figure-2, as a maximum, three (CG TO based) autonomous retransmission is configured.

In Figure-2, CG TO-1 and CG TO-4 are not overlapped with the cell DRX inactive period, but CG TO-2 and CG TO-3 are overlapped with the cell DRX inactive period. As can be seen, the end of the first CGRT falls into the cell DRX inactive period. However the whole configured CGT cross over the whole cell DRX inactive period.

If the initial transmission at CG TO-1 is initiated, and if the CG Retransmission Timer (CGRT) and Configured Grant Timer (CGT) is configured, following the legacy handling, the CG TO-2 and CG TO-3 based UL transmission will be triggered if the UE has not received CG-DFI during these periods, which means the UE’s UL CG transmission span over the whole cell DRX inactive period, which cause negative effect to cell’s DTX/DRX operation, and further make the energy saving gain for the cell less attractive.

One possible handling could be to suspend UE’s CG Retransmission Timer (CGRT) and to effectively delay the UE’s autonomous CG retransmission in this case. As a corresponding handling, the Configured Grant Timer (CGT) can be stopped or suspended during cell DRX inactive period, and then new data is not supposed to transmit on the latest CG TO following the end of the cell DRX inactive period (i.e., TO-4 is not used for new data transmission).

Figure-2: Overlapping Example of CGRT/CGT and cell DRX

Proposal-5: Discuss the potential handling on CGRT and CGT during the cell DRX inactive period (e.g. suspension of the CGRT and CGT).

## SR and BSR

The configuration of SR resource is relatively static. But the operation of cell DRX/DTX can be very dynamic. Then it is possible that the configured SR resources fall into the cell DRX inactive period. In this case, allowing the UE to send SR request is not helpful for energy saving, since the network wants to go to cell DRX. On the other hand, if the network receives the SR and responds to the UE with a dynamic grant, the network needs to prepare data reception from the UE, which cause the network to stay away from cell DRX operation. For this reason, it is proposed that the SR from the NES capable UE is not allowed, even though there is SR configuration available during cell DRX inactive period for the UE. Correspondingly, the BSR can be also delayed to report until the cell transits from cell DRX inactive period to cell DRX active period.

Proposal-6: UE side’s SR transmission to enable any dynamic grant (i.e., by dynamic scheduling) is suppressed during Cell DRX inactive period.

The UE triggers SR to get the dynamic grant (to send BSR report) from the network when the cell enters cell DRX active period from cell DRX inactive period, if the SR resource is configured ahead of any available Configured Grant resources. Otherwise the SR transmission may be omitted if there is CG resource for UL data transmission.

The UE can use the dynamic grant to send the BSR report if the dynamic grant is ahead of available Configured Grant resources. Otherwise (i.e., the available Configured Grant resources is ahead of the SR resource or the dynamic grant ), the UE can send the BSR report via the Configured Grant resources.

## DL SPS transmission

Similarly to UL CG based transmissions, DL SPS based transmissions scheduling may be overlapping a Cell DTX inactive period, as shown in figure 3 below.

Figure-3: Overlapping Example between DL SPS and cell DTX

To achieve maximal NES gains, the cell should not be transmitting during Cell DTX inactive periods. Hence, SPS transmissions should be skipped during Cell DTX inactive periods and the UE should not be monitoring the PDCCH or the SPS occasions on the PDSCH.

Proposal-7: The UE does not expect to acquire DL based SPS transmission during Cell DTX inactive period.

DL retransmission, which involves HARQ processes, is discussed in the following section. From the UE point of view, at least initial transmissions should be skipped, which can be easily achieved by comparing the Cell DTX cycle with its DL SPS configuration.

Proposal-8: Initial transmission based on SPS Transmit Occasion should be skipped by the UE if the DL SPS TO overlaps with the cell DTX inactive period.

## DL retransmission

In the case of retransmission, where a HARQ NACK was sent by the UE and the expected DL retransmission would happen during Cell DTX inactive period, the network should schedule such retransmission after the cell starts becoming active again, as shown below in Figure 4.

Figure-4: Overlapping Example of DL retransmission and cell DTX

Proposal-9: Retransmission should be scheduled by the network to skip the cell DTX inactive period when SPS based initial transmission is not decodable.

From the UE point of view, HARQ timer will need to be modified, according to specific network signalling for this retransmission, or implicit rules set up by the network.

Proposal-10: Discuss UE behavior in the case of DL retransmissions overlapping with the Cell DTX inactive period.

# Conclusion and Proposal

We have the following proposals:

Proposal-1: Discuss the possible handling by the UE for the stop drx-HARQ-RTT-TimerUL timer and the start for drx-RetransmissionTimerUL timer for UL dynamic scheduling.

Proposal-2: The configuration to allow UE to perform UL CG based transmission during Cell DRX inactive period is not supported

Proposal-3: initial transmission based on CG TO should be skipped by the UE if the UL CG TO overlaps with the cell DRX inactive period

Proposal-4: Retransmission should be scheduled by the network to skip the cell DRX inactive period when CG based initial transmission is not decodable.

Proposal-5: Discuss the potential handling on CGRT and CGT during the cell DRX inactive period (e.g. suspension of the CGRT and CGT).

Proposal-6: UE side’s SR transmission to enable any dynamic grant (i.e., by dynamic scheduling) is suppressed during Cell DRX inactive period.

Proposal-7: The UE does not expect to acquire DL based SPS transmission during Cell DTX inactive period.

Proposal-8: Initial transmission based on SPS Transmit Occasion should be skipped by the UE if the DL SPS TO overlaps with the cell DTX inactive period.

Proposal-9: Retransmission should be scheduled by the network to skip the cell DTX inactive period when SPS based initial transmission is not decodable.

Proposal-10: Discuss UE behavior in the case of DL retransmissions overlapping with the Cell DTX inactive period.

# Reference

- R2-2300632	Cell DTX/DRX mechanism

- R2-2300539	Cell DTX-DRX Mechanism