3GPP TSG-RAN WG2 Meeting #121bis-e	R2-2303337

# E-Meeting, 17th – 26th April 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 4007 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on PC5 RLC channel release trigger due to SL RLF |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_Relay-Core |  |  |  |  |  |  | Date: |  |  | 2023-4-10 |
| Category: | F |  |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to current RRC spec in clause 5.8.9.3, when the L2 U2N Remote UE or the L2 U2N Remote UE detects SL RLF for a specific destination, it will perform PC5 RLC channel release of this destination, which is highlighted as below: #### 5.8.9.3 Sidelink radio link failure related actions The UE shall: 1> upon indication from sidelink RLC entity that the maximum number of retransmissions for a specific destination has been reached; or 1> upon T400 expiry for a specific destination; or 1> upon indication from MAC entity that the maximum number of consecutive HARQ DTX for a specific destination has been reached; or 1> upon integrity check failure indication from sidelink PDCP entity concerning SL-SRB2 or SL-SRB3 for a specific destination: 2> consider sidelink radio link failure to be detected for this destination; 2> release the DRBs of this destination, according to clause 5.8.9.1a.1; 2> release the SRBs of this destination, according to clause 5.8.9.1a.3; 2> release the PC5 Relay RLC channels of this destination if configured, in according to clause 5.8.9.7.1;. However, in clause 5.8.9.7.1, the current if-condition is not specified for SL RLF case and the UE cannot execute the PC5 RLC channel release due to SL RLF as a consequence. Therefore, the corresponding PC5 RLC channel release trigger due to SL RLF needs to be added in clause 5.8.9.7.1. |
| Summary of change: |  | In clause 5.8.9.7.1., add the PC5 RLC channel release trigger due to SL RLF. Impact analysis Impacted 5G architecture options: NR SA Impacted functionality NR Sidelink based L2 U2N Relay Inter-operability: If the UE implements the CR but not the network, there is no inter-operability issue. If the network implements the CR but not the UE, there is no inter-operability issue. If one UE implements the CR but not the other UE, there is no inter-operability issue. |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misaligned UE behaviour on the PC5 RLC channel release trigger due to SL RLF across different clauses in RRC spec. |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.8.9.7.1 |  |  |  |  |  |  |  |  |  |
|  |  | Y |  | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

#### 5.8.9.7	PC5 Relay RLC channel management for L2 U2N relay

##### 5.8.9.7.1	PC5 Relay RLC channel release

The UE shall:

1>	if the PC5 Relay RLC channel release was triggered after the reception of the RRCReconfigurationSidelink message; or

1>	after receiving the RRCReconfigurationCompleteSidelink message, if the PC5 Relay RLC channel release was triggered due to the configuration received within the sl-ConfigDedicatedNR:

2>	for each SL-RLC-ChannelID in sl-RLC-ChannelToReleaseList received in sl-ConfigDedicatedNR within RRCReconfiguration or for each SL-RLC-ChannelID included in the received sl-RLC-ChannelToReleaseListPC5 that is part of the current UE sidelink configuration:

3>	release the RLC entity and the corresponding logical channel associated with the SL-RLC-ChannelID;

1>	if the PC5 Relay RLC channel release was triggered for a specific destination by upper layers as specified in 5.8.9.5 or due to sidelink RLF as specified in 5.8.9.3:

2>	release the RLC entity and the corresponding logical channel associated with the SL-RLC-ChannelID of the specific destination;

END OF CHANGE