3GPP TSG-RAN WG2 Meeting #121-bis electronic							   R2-2303352

E-Meeting, April 17-26, 2023

Source: 	Xiaomi

Title:  	Discussion on MUSIM gap priorities

Agenda Item:	7.17.4

Document for:	Discussion and Decision

# Introduction

According to the LS from RAN4 in [1], RAN4 made the following agreements on the “priority for MUSIM gaps”:

- Introduction of priorities for periodic MUSIM gaps - Each periodic MUSIM gap can be assigned with a different priority - The priority level of MUSIM gap(s) shall be configured to be comparable to priority level of NW A’s Type-2 MGs - MUSIM gap and Type-2 MG cannot be configured with the same priority - The priority level of MUSIM gaps should be configured/allocated by NW A - When requesting periodic MUSIM gap(s) UE can provide an assistance information for gap priority selection - UE can optionally indicate its preferred priority for all or a subset MUSIM gaps - It is up to NW A on how to use this information

Definition of Type-2 MG: Gap(s) configured via GapConfig-r17 without preConfigInd-r17 or ncsgInd-r17.

In this contribution, we provide our understandings on the signalling design of the gap priority as requested by RAN4.

# Discussion

## MUSIM gap priority configuration

According to 38.331, the “GapConfig-r17” configuration is quoted as follows:

GapConfig-r17 ::= SEQUENCE { measGapId-r17 MeasGapId-r17, gapType-r17 ENUMERATED {perUE, perFR1, perFR2}, gapOffset-r17 INTEGER (0..159), mgl-r17 ENUMERATED {ms1, ms1dot5, ms2, ms3, ms3dot5, ms4, ms5, ms5dot5, ms6, ms10, ms20}, mgrp-r17 ENUMERATED {ms20, ms40, ms80, ms160}, mgta-r17 ENUMERATED {ms0, ms0dot25, ms0dot5, ms0dot75}, refServCellIndicator-r17 ENUMERATED {pCell, pSCell, mcg-FR2} OPTIONAL, -- Cond NEDCorNRDC refFR2-ServCellAsyncCA-r17 ServCellIndex OPTIONAL, -- Cond AsyncCA preConfigInd-r17 ENUMERATED {true} OPTIONAL, -- Need R ncsgInd-r17 ENUMERATED {true} OPTIONAL, -- Need R gapAssociationPRS-r17 ENUMERATED {true} OPTIONAL, -- Need R gapSharing-r17 MeasGapSharingScheme OPTIONAL, -- Need R gapPriority-r17 GapPriority-r17 OPTIONAL, -- Need R ... }

Since RAN4 agreed that “the priority level of MUSIM gap(s) shall be configured to be comparable to priority level of NW A’s Type-2 MGs”, the priority of MUSIM gap(s) should reuse “GapPriority-r17”, as quoted below:

Text proposal for MUSIM gap priority:

MUSIM-GapConfig-r17 ::= SEQUENCE { musim-GapToReleaseList-r17 SEQUENCE (SIZE (1..3)) OF MUSIM-GapId-r17 OPTIONAL, -- Need N musim-GapToAddModList-r17 SEQUENCE (SIZE (1..3)) OF MUSIM-Gap-r17 OPTIONAL, -- Need N musim-AperiodicGap-r17 MUSIM-GapInfo-r17 OPTIONAL, -- Need N ..., [[ musim-GapPriorityToAddModList-r18 SEQUENCE (SIZE (1..3)) OF MUSIM-GapPriority-r18 OPTIONAL, -- Need N musim-GapPriorityToReleaseList-r18 SEQUENCE (SIZE (1..3)) OF MUSIM-GapId-r17 OPTIONAL -- Need N ]] } MUSIM-Gap-r17 ::= SEQUENCE { musim-GapId-r17 MUSIM-GapId-r17, musim-GapInfo-r17 MUSIM-GapInfo-r17 } MUSIM-GapPriority-r18 ::= SEQUENCE { musim-GapId-r18 MUSIM-GapId-r17, musim-GapPriority-r18 GapPriority-r17 }

Proposal 1: GapPriority-r17 is configured as the MUSIM gap priority of the periodic MUSIM gap.

## MUSIM gap priority request

According to 38.331, the Rel-17 MUSIM UE can provide its preferred MUSIM gap via UAI. According to the RAN4 LS, the UE can “optionally indicate its preferred priority for all or a subset MUSIM gaps”. Then we can add a new list containing the assistance information for MUSIM gap priority preference in UAC. The “musim-GapIndex-r18” is the entry index of musim-GapPreferenceList-r17.

Text proposal for assistance information for gap priority selection:

UEAssistanceInformation-v1700-IEs ::= SEQUENCE { ul-GapFR2-Preference-r17 UL-GapFR2-Preference-r17 OPTIONAL, musim-Assistance-r17 MUSIM-Assistance-r17 OPTIONAL, overheatingAssistance-r17 OverheatingAssistance-r17 OPTIONAL, maxBW-PreferenceFR2-2-r17 MaxBW-PreferenceFR2-2-r17 OPTIONAL, maxMIMO-LayerPreferenceFR2-2-r17 MaxMIMO-LayerPreferenceFR2-2-r17 OPTIONAL, minSchedulingOffsetPreferenceExt-r17 MinSchedulingOffsetPreferenceExt-r17 OPTIONAL, rlm-MeasRelaxationState-r17 BOOLEAN OPTIONAL, bfd-MeasRelaxationState-r17 BIT STRING (SIZE (1..maxNrofServingCells)) OPTIONAL, nonSDT-DataIndication-r17 SEQUENCE { resumeCause-r17 ResumeCause OPTIONAL } OPTIONAL, scg-DeactivationPreference-r17 ENUMERATED { scgDeactivationPreferred, noPreference } OPTIONAL, uplinkData-r17 ENUMERATED { true } OPTIONAL, rrm-MeasRelaxationFulfilment-r17 BOOLEAN OPTIONAL, propagationDelayDifference-r17 PropagationDelayDifference-r17 OPTIONAL, nonCriticalExtension UEAssistanceInformation-v18xy-IEs OPTIONAL } MUSIM-Assistance-r17 ::= SEQUENCE { musim-PreferredRRC-State-r17 ENUMERATED {idle, inactive, outOfConnected} OPTIONAL, musim-GapPreferenceList-r17 MUSIM-GapPreferenceList-r17 OPTIONAL } MUSIM-GapPreferenceList-r17 ::= SEQUENCE (SIZE (1..4)) OF MUSIM-GapInfo-r17 UEAssistanceInformation-v18xy-IEs ::= SEQUENCE { musim-GapPriorityPreferenceList-r18 MUSIM-GapPriorityPreferenceList-r18 OPTIONAL, nonCriticalExtension SEQUENCE {} OPTIONAL } MUSIM-GapPriorityPreferenceList-r18 ::= SEQUENCE (SIZE (1..4)) OF MUSIM-GapPriorityPreference-r18 MUSIM-GapPriorityPreference-r18-r18 ::= SEQUENCE { musim-GapIndex-r18 INTEGER (0..3), musim-GapPriority-r18 GapPriority-r17 }

Proposal 2: GapPriority-r17 is be reported for the MUSIM gap priority of the UE-preferred MUSIM gap.

According to the analysis given above, we consider the text proposals for the signalling design of the MUSIM gap priorities can be considered as the baseline for further discussion.

Proposal 3: The text proposals for the signalling design of the MUSIM gap priority configuration and reporting are considered as the baseline.

# Conclusions

According to the analysis given above, we have the following proposals:

Proposal 1: GapPriority-r17 is configured as the MUSIM gap priority of the periodic MUSIM gap.

Proposal 2: GapPriority-r17 is be reported for the MUSIM gap priority of the UE-preferred MUSIM gap.

Proposal 3: The text proposals for the signalling design of the MUSIM gap priority configuration and reporting are considered as the baseline.

# Reference

[1] R4-2303249, LSin, from RAN4, “LS on priority for MUSIM gaps”