3GPP TSG-RAN WG2 #121bis-e	R2-2303472

Electronic meeting, Apr 17th – 26th, 2023

Agenda Item:	6.1.3.1

Source:	Ericsson

Title:	Discussion on RAN1 LS R1-2302144

Document for:	Discussion, Decision

# 1	Introduction

We discuss aspects related to RAN1 LS R1-2302144.

# 2	Discussion

In the LS, RAN1 observed the inconsistency between 38.214 and 38.331 specifications. The following two types of multi-PUSCH scheduling are supported:

- Type 1: Rel-16 multi-PUSCH scheduling, where a row of the TDRA table can indicate 2 to 8 contiguous PUSCHs

- Type 2: Rel-17 multi-PUSCH scheduling, where a row of the TDRA table can indicate 2 to 8 non-contiguous PUSCHs

According to 38.214 specification, UE determines K2 from k2-r16 (rather than extendedK2-r17) for Type 1 while UE determines K2 from extendedK2-r17 for Type 2. However, according to 38.331 specification, if the size of puschAllocationList is higher than 1, the field extendedK2(n) corresponding to k2 of the n-th PUSCH, n>1, is mandatory present, which implies UE shall determine K2 from extendedK2-r17 regardless of Type 1 or Type 2.

To resolve this inconsistency issue, the following TP for 38.331 specification is provided as RAN1’s recommendation.

| Conditional Presence | Explanation |
| --- | --- |
| MultiPUSCH | In case size of puschAllocationList is higher than 1, the field extendedK2(n) corresponding to k2 of the n-th PUSCH, n>1, is mandatory present for all n if any two PUSCHs are non-contiguous. Otherwise, it is optionally present, Need S. |

ACTION: 	RAN1 respectfully asks RAN2 to take the above information into account and capture it in TS 38.331.

RAN1 has suggested changes for extendedK2-r17 to make the field not mandatory in case there is any two PUSCHs are non-contiguous.

RAN1 has suggested changes for extendedK2-r17 to make the field not mandatory in case there is any two PUSCHs are non-contiguous

RAN1’s intension is understandable. However, the RAN1 suggested changes are incomplete.

As shown in the below

| Conditional Presence | Explanation |
| --- | --- |
| MultiPUSCH | In case size of puschAllocationList is higher than 1, the field extendedK2(n) corresponding to k2 of the n-th PUSCH, n>1, is mandatory present for all n if any two PUSCHs are non-contiguous. Otherwise, it is optionally present, Need S. |

With the suggested changes, the “otherwise” condition would be different compared to the original texts.

In the original texts, the “otherwise” condition means that the first PUSCH, i.e., n==1, in that case the field is optionally present. The field description of extendedK2 has specified UE actions in case the field is absent.

PUSCH-TimeDomainResourceAllocationList field descriptions

extendedK2 Corresponds to L1 parameter 'K2' (see TS 38.214 [19], clause 6.1.2.1) configurable per PUSCH allocation. Only values {0..32} are applicable for PUSCH SCS of 120 kHz. When the field is absent for the first PUSCH if multiple PUSCH are configured per PDCCH, or when the field is absent and only one PUSCH is configured per PDCCH, the UE applies the value 1 when PUSCH SCS is 15/30 kHz; the value 2 when PUSCH SCS is 60 kHz, the value 3 when PUSCH SCS is 120 kHz, the value 11 when PUSCH SCS is 480 kHz, and the value 21 when PUSCH SCS is 960 kHz.

In the current texts, the field description of extendedK2 has only specified UE actions in case the field is absent and n==1.

In the updated texts, the “otherwise” condition would be extended to also cover the case where all PUSCHs are contiguous.

With RAN1 suggested changes, the “otherwise” condition in the field description has changed, i.e., the case where all PUSCHs are contiguous are also covered.

Due to extension of the “otherwise” condition, further changes would be necessary. Based on this observation, we think the changes indicated in the RAN1 LS are incomplete.

Given the changes recommended in the RAN1 LS R1-2302144 are incomplete, RAN2 discuss further changes.

In our view, RAN2 needs to further discuss the additional changes. The UE actions are missing when all PUSCHs are contiguous, the field extendedK2(n) corresponding to k2 of the n-th PUSCH, n>1 is absent. In this case, further changes to the field description of extendedK2 would be required to specify the missing UE actions.

Further changes to the field description of extendedK2 are needed to specify UE actions when all PUSCHs are contiguous, the field extendedK2(n) corresponding to k2 of the n-th PUSCH, n>1 is absent.

Therefore, we make the below proposal:

In addition to changes indicated in the RAN1 LS R1-2302144, include the below texts in the field description of extendedK2

  - If multiple contiguous PUSCH are configured per PDCCH, when the field extendedK2(n) corresponding to k2 of the n-th PUSCH, n>1 is absent, the UE applies k2 of the first PUSCH plus n-1.

PUSCH-TimeDomainResourceAllocationList field descriptions

extendedK2 Corresponds to L1 parameter 'K2' (see TS 38.214 [19], clause 6.1.2.1) configurable per PUSCH allocation. Only values {0..32} are applicable for PUSCH SCS of 120 kHz. When the field is absent for the first PUSCH if multiple PUSCH are configured per PDCCH, or when the field is absent and only one PUSCH is configured per PDCCH, the UE applies the value 1 when PUSCH SCS is 15/30 kHz; the value 2 when PUSCH SCS is 60 kHz, the value 3 when PUSCH SCS is 120 kHz, the value 11 when PUSCH SCS is 480 kHz, and the value 21 when PUSCH SCS is 960 kHz. If multiple contiguous PUSCH are configured per PDCCH, when the field extendedK2(n) corresponding to k2 of the n-th PUSCH, n>1 is absent, the UE applies k2 of the first PUSCH plus n-1.

We have also proposed corresponding CR.

Adopt the CR R2-2303557.

# 3 Conclusion

In the previous sections we made the following observations:

Observation 1	RAN1 has suggested changes for extendedK2-r17 to make the field not mandatory in case there is any two PUSCHs are non-contiguous

Observation 2	In the current texts, the field description of extendedK2 has only specified UE actions in case the field is absent and n==1.

Observation 3	With RAN1 suggested changes, the “otherwise” condition in the field description has changed, i.e., the case where all PUSCHs are contiguous are also covered.

Observation 4	Further changes to the field description of extendedK2 are needed to specify UE actions when all PUSCHs are contiguous, the field extendedK2(n) corresponding to k2 of the n-th PUSCH, n>1 is absent.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Given the changes recommended in the RAN1 LS R1-2302144 are incomplete, RAN2 discuss further changes.

Proposal 2	In addition to changes indicated in the RAN1 LS R1-2302144, include the below texts in the field description of extendedK2

a.	If multiple contiguous PUSCH are configured per PDCCH, when the field extendedK2(n) corresponding to k2 of the n-th PUSCH, n>1 is absent, the UE applies k2 of the first PUSCH plus n-1.

Proposal 3	Adopt the CR R2-2303557.

# 4 References

- R2-2303557  Correction to RRC for 71 GHz on multi-PUSCH, Ericsson

# Annex A

#### –	PUSCH-TimeDomainResourceAllocationList

The IE PUSCH-TimeDomainResourceAllocation is used to configure a time domain relation between PDCCH and PUSCH. PUSCH-TimeDomainResourceAllocationList contains one or more of such PUSCH-TimeDomainResourceAllocations. The network indicates in the UL grant which of the configured time domain allocations the UE shall apply for that UL grant. The UE determines the bit width of the DCI field based on the number of entries in the PUSCH-TimeDomainResourceAllocationList. Value 0 in the DCI field refers to the first element in this list, value 1 in the DCI field refers to the second element in this list, and so on.

PUSCH-TimeDomainResourceAllocation information element

-- ASN1START

-- TAG-PUSCH-TIMEDOMAINRESOURCEALLOCATIONLIST-START

PUSCH-TimeDomainResourceAllocationList ::=  SEQUENCE (SIZE(1..maxNrofUL-Allocations)) OF PUSCH-TimeDomainResourceAllocation

PUSCH-TimeDomainResourceAllocation ::=  SEQUENCE {

k2                                      INTEGER(0..32)                                  OPTIONAL,   -- Need S

mappingType                             ENUMERATED {typeA, typeB},

startSymbolAndLength                    INTEGER (0..127)

}

PUSCH-TimeDomainResourceAllocationList-r16 ::=  SEQUENCE (SIZE(1..maxNrofUL-Allocations-r16)) OF PUSCH-TimeDomainResourceAllocation-r16

PUSCH-TimeDomainResourceAllocation-r16 ::=  SEQUENCE {

k2-r16                                     INTEGER(0..32)          OPTIONAL,   -- Need S

puschAllocationList-r16                    SEQUENCE (SIZE(1..maxNrofMultiplePUSCHs-r16)) OF PUSCH-Allocation-r16,

...

}

PUSCH-Allocation-r16 ::=  SEQUENCE {

mappingType-r16                           ENUMERATED {typeA, typeB}                     OPTIONAL,   -- Cond NotFormat01-02-Or-TypeA

startSymbolAndLength-r16                  INTEGER (0..127)                              OPTIONAL,   -- Cond NotFormat01-02-Or-TypeA

startSymbol-r16                           INTEGER (0..13)                               OPTIONAL,   -- Cond RepTypeB

length-r16                                INTEGER (1..14)                               OPTIONAL,   -- Cond RepTypeB

numberOfRepetitions-r16                   ENUMERATED {n1, n2, n3, n4, n7, n8, n12, n16} OPTIONAL,   -- Cond Format01-02

...,

[[

numberOfRepetitionsExt-r17                ENUMERATED {n1, n2, n3, n4, n7, n8, n12, n16, n20, n24, n28, n32, spare4, spare3, spare2,

spare1}                           OPTIONAL,   -- Cond Format01-02-For-TypeA

numberOfSlotsTBoMS-r17                   ENUMERATED {n1, n2, n4, n8, spare4, spare3, spare2, spare1}   OPTIONAL,   -- Need R

extendedK2-r17                            INTEGER (0..128)                              OPTIONAL    -- Cond MultiPUSCH

]]

}

-- TAG-PUSCH-TIMEDOMAINRESOURCEALLOCATIONLIST-STOP

-- ASN1STOP

PUSCH-TimeDomainResourceAllocationList field descriptions

extendedK2 Corresponds to L1 parameter 'K2' (see TS 38.214 [19], clause 6.1.2.1) configurable per PUSCH allocation. Only values {0..32} are applicable for PUSCH SCS of 120 kHz. When the field is absent for the first PUSCH if multiple PUSCH are configured per PDCCH, or when the field is absent and only one PUSCH is configured per PDCCH, the UE applies the value 1 when PUSCH SCS is 15/30 kHz; the value 2 when PUSCH SCS is 60 kHz, the value 3 when PUSCH SCS is 120 kHz, the value 11 when PUSCH SCS is 480 kHz, and the value 21 when PUSCH SCS is 960 kHz. If multiple contiguous PUSCH are configured per PDCCH, when the field extendedK2(n) corresponding to k2 of the n-th PUSCH, n>1 is absent, the UE applies k2 of the first PUSCH plus n-1.

k2 Corresponds to L1 parameter 'K2' (see TS 38.214 [19], clause 6.1.2.1). When the field is absent the UE applies the value 1 when PUSCH SCS is 15/30 kHz; the value 2 when PUSCH SCS is 60 kHz, and the value 3 when PUSCH SCS is 120 kHz. k2 is absent/ignored if extendedK2 is present.

length Indicates the length allocated for PUSCH for DCI format 0_1/0_2 (see TS 38.214 [19], clause 6.1.2.1).

mappingType Mapping type (see TS 38.214 [19], clause 6.1.2.1).

numberOfRepetitions Number of repetitions for DCI format 0_1/0_2 (see TS 38.214 [19], clause 6.1.2.1). When numberOfSlotsTBoMS-r17 is set to 2, 4 or 8 (i.e. TB processing over multi-slot (TBoMS) PUSCH is enabled), it indicates the number of repetitions of a single TBoMS.

numberOfRepetitionsExt Number of repetitions for DCI format 0_1/0_2 if pusch-RepTypeIndicatorDCI-0-1/pusch-RepTypeIndicatorDCI-0-2 is not set to pusch-RepTypeB (see TS 38.214 [19], clause 6.1.2.1). If this field is present, the field numberOfRepeitions-r16 is ignored for PUSCH repetition Type A.

numberOfSlotsTBoMS Number of slots allocated for TB processing over multi-slot PUSCH for DCI format 0_1/0_2. If a number of repetitions K is configured by numberOfRepetitions or numberOfRepetitionsExt, the network configures numberOfSlotsTBoMS (N) and K such that N*K ≤ 32 (see TS 38.214 [19], clause 6.1.2.1).

puschAllocationList The field puschAllocationList-r16 indicates one or multiple PUSCH continuous in time domain which share a common k2 (see TS 38.214 [19], clause 6.1.2.1). The field pusch-AllocationList-r17 configures one or multiple PUSCH that may be in consecutive or non-consecutive slots (see TS 38.214 [19], clause 6.1.2.1). The puschAllocationList-r16 only has one element in pusch-TimeDomainAllocationListDCI-0-1-r16 and in pusch-TimeDomainAllocationListDCI-0-2-r16.

startSymbol Indicates the index of start symbol for PUSCH for DCI format 0_1/0_2 (see TS 38.214 [19], clause 6.1.2.1).

startSymbolAndLength An index giving valid combinations of start symbol and length (jointly encoded) as start and length indicator (SLIV). The network configures the field so that the allocation does not cross the slot boundary. (see TS 38.214 [19], clause 6.1.2.1).

| Conditional Presence | Explanation |
| --- | --- |
| Format01-02 | In pusch-TimeDomainAllocationListForMultiPUSCH-r16, the field is absent. In pusch-TimeDomainAllocationListDCI-0-1 and in pusch-TimeDomainAllocationListDCI-0-2, the field is mandatory present. |
| Format01-02-For-TypeA | In pusch-TimeDomainAllocationListForMultiPUSCH-r16, the field is absent. In pusch-TimeDomainAllocationListDCI-0-1, the field is optionally present if pusch-RepTypeIndicatorDCI-0-1 is not set to pusch-RepTypeB, Need R. It is absent otherwise, Need R. In pusch-TimeDomainAllocationListDCI-0-2, the field is optionally present if pusch-RepTypeIndicatorDCI-0-2 is not set to pusch-RepTypeB, Need R. It is absent otherwise, Need R. |
| NotFormat01-02-Or-TypeA | In pusch-TimeDomainAllocationListForMultiPUSCH-r16, the field is mandatory present. In pusch-TimeDomainAllocationListDCI-0-1, the field is mandatory present if pusch-RepTypeIndicatorDCI-0-1 is not set to pusch-RepTypeB. It is absent otherwise, Need R. In pusch-TimeDomainAllocationListDCI-0-2, the field is mandatory present if pusch-RepTypeIndicatorDCI-0-2 is not set to pusch-RepTypeB. It is absent otherwise, Need R. |
| RepTypeB | In pusch-TimeDomainAllocationListForMultiPUSCH-r16, the field is absent. In pusch-TimeDomainAllocationListDCI-0-1, the field is mandatory present if pusch-RepTypeIndicatorDCI-0-1 is set to pusch-RepTypeB. It is absent otherwise, Need R. In pusch-TimeDomainAllocationListDCI-0-2, the field is mandatory present if pusch-RepTypeIndicatorDCI-0-2 is set to pusch-RepTypeB. It is absent otherwise, Need R. |
| MultiPUSCH | In case size of puschAllocationList is higher than 1, the field extendedK2(n) corresponding to k2 of the n-th PUSCH, n>1, is mandatory present for all n if any two PUSCHs are non-contiguous. Otherwise, it is optionally present, Need S. |