# 3GPP TSG-WG2 Meeting #121 bis                        R2-2303546

# Online, Apr 17th - Mar 26th , 2023

# Source: 			CMCC

# Title:  			Discussion on service continuity

# Agenda item:	7.9.4

# Document for: 	Discussion

Introduction

For service continuity, we achieved some progress on lossless handover and path switching for the 4 scenarios in WID with the passed RAN2 and RAN3 meeting. In this contribution, we discuss the solution of lossless handover for DL and UL i2x path switching, and update the path switching procedure flow chart with the progress.

RAN2 Agreements:

- RAN2 consider that lossless data delivery in the inter-gNB i2x cases needs to be addressed.  Solutions can be considered next meeting (including the possibility of solutions needing work from RAN3).  Solutions based on the PDCP status report mechanism are the baseline.

RAN3 Agreements:

- During direct to indirect and indirect to indirect path switch procedures, the source gNB sends a list of candidate relay UEs belonging to the same target cell in the HO REQ message.

- At least Remote UE L2 ID and a list of candidate target relay UE IDs should be included in the XnAP HANDOVER REQUEST message.

Discussion

Lossless path switching

Lossless path switching issue has been discussed in last two meetings and RAN2 identified that R17 mechanism is not feasible to support lossless delivery in DL and UL in inter-gNB service continuity cases, and lossless data delivery in the inter-gNB i2x cases needs to be addressed. RAN2 also concluded that PDCP status report mechanism can be taken into account as baseline.

In inter-gNB i2x, for downlink, source gNB transmits data packets to relay UE via uu link and relay UE deliveries those data packets to remote UE via PC5 link. For the uu link between relay UE and gNB, uu AM RLC entity with ARQ is configured for transmission reliability; correspondingly, for PC5 link between relay UE and remote UE, PC5 AM RLC entity with ARQ is responsible for data loss acknowledge and re-transmission.

Source gNB may receive acknowledge from relay UE once the uu RLC entity in relay UE side successfully receives the downlink data from source gNB, without considering the transmission condition of PC5 link. For the data packets that lost in PC5 link, only the RLC entity in remote UE reports that to relay UE with PC5 RLC status report. For the transmitting PDCP entity in source gNB, it may discard the PDCP SDU when it receives the successfully receive indication from lower layer or the discard timer expires.

However, the PDCP status report triggered in receiving side of PDCP entity, that is in remote UE for downlink, which reflects the real situation of downlink during handover procedure. When the HO is performed, target gNB may retransmit the data packets reference to the PDCP status report from the remote UE.

As shown in Figure.1, the PDCP SDUs (SN=5,6,7,8,9,10,11) are transmitted, where the PDCP SDUs (SN=8,9) are failed at the first hop from source gNB to relay UE, and PDCP SDUs (SN=11) are failed at the second hop from relay UE to remote UE. PDCP status report triggered at remote UE for DL may indicate that the PDCP SDUs (SN=8,9,11) are missing and retransmission at target gNB is needed.

Fig.1 DL data transmission during Handover

The data transmission status misalignment between receiving sides (source gNB and remote UE) impacts on lossless handover. To avoid that misalignment, an indicator from relay UE to gNB can be introduced to inform the DL data transmission condition, as the green information in the Fig.1. with the enhanced receiving side acknowledge information, the PDCP SDUs (SN=8,9,11) can be retransmitted at target gNB.

Proposal 1: Indicator from relay UE to gNB can be introduced to inform the DL data transmission condition.

Inversely, for uplink, data packets may be successfully transmitted from remote UE to relay UE at PC5 link, but fail at uu link between relay UE and source gNB. The receiving side of PDCP in source gNB can report the receiving status to remote UE, however, the data packets waiting for retransmission may be discarded in PDCP transmitting side.

As shown in Fig.2, the PDCP SDUs (SN=5,6,7,8,9,10,11) are transmitted, where the PDCP SDUs (SN=6) are failed at the first hop from remote UE to relay UE, and PDCP SDUs (SN=8,9) are failed at the second hop from relay UE to source gNB. PDCP status report triggered at receiving side of PDCP for UL may indicate that the PDCP SDUs (SN=6,8,9) are missing and retransmission at remote UE side is needed, the data packets (SN=7,10,11) can be forwarded to target gNB.

Fig.2 UL data transmission during Handover

To support lossless handover for UL, the transmitting side of PDCP can not discard the data packets (SN=8,9) that has not been confirmed by source gNB. Indicator from relay UE to remote UE can be introduced to inform the UL data transmission condition.

Proposal 2: Indicator from relay UE to remote UE can be introduced to inform the UL data transmission condition.

As concluded in last RAN2 meeting, lossless handover may have some impacts on RAN3, for example, SN status transfer and data forwarding. However, the basic solutions for lossless HO have not been confirmed. RAN3 work can be started later.

HO procedure

In RAN3 #119 meeting, it concluded that during the inter gNB handover, for direct to indirect and indirect to indirect path switch procedures, the source gNB sends a list of candidate relay UEs belonging to the same target cell in the HO Request message.

The target gNB can make decision among lots of candidate relay UE considering the uu condition, for example, RRC state, uu RSRP or payload. For candidate Relay UEs with different RRC state, the uu link establishment delay are different. So, candidate relay UEs in RRC connected can be prioritized. Moreover, only the RSRP and payload information of RRC connected UE are visible to gNB, the gNB can not compare the candidate relay UEs with different RRC state, which may impacts on the candidate relay selection if it just left to gNB implementation. From our perspective, RAN2 should discuss whether and how to restrict gNB behaviour of relay UE selection.

Proposal 3: Ask RAN2 to discuss whether and how to restrict gNB behaviour of relay UE selection.

Overall procedure for service continuity

With the progress of RAN2 and RAN3, we describe the overall procedure for the 4 scenarios in WID.

- Inter-gNB indirect-to-direct path switching

The signalling flow for U2N remote UE switch from indirect path to direct path under another gNB is shown as follows,

- Fig.3 Inter-gNB indirect-to-direct path switching

1.	The source gNB configures the UE measurement procedures and the UE reports according to the measurement configuration.

2.	The source gNB decides to handover the remote UE to one target cell, based on MeasurementReport and RRM information.

3.	The source gNB sends Handover Request message to the target gNB with necessary information to prepare the handover at the target side.

4.	Admission Control may be performed by the target gNB.

5.	The target gNB prepares the handover with L1/L2 and sends the HANDOVER REQUEST ACKNOWLEDGE to the source gNB.

6.  The source gNB triggers the Uu handover by sending an RRCReconfiguration message to the remote UE, containing the information required to access the target cell.

7.  The remote UE access to the Target gNB.

8. The source gNB sends the SN STATUS TRANSFER message to the target gNB.

9. The UE synchronises to the target cell and completes the RRC handover procedure by sending RRCReconfigurationComplete message to target gNB.

10.	Target gNB sends the UE CONTEXT RELEASE to inform the source gNB about the success of the handover.

11. The gNB sends RRCReconfiguration message to the U2N Relay UE to reconfigure the connection between the U2N Relay UE and the source gNB.

12. Either U2N Relay UE or U2N Remote UE can initiate the PC5 unicast link release (PC5-S).

Proposal 4:  It is proposed to agree the overall procedure in Fig.3 as a baseline to support inter-gNB indirect to direct path switching.

- Inter-gNB direct-to-indirect path switching

- The signalling flow for U2N remote UE switch from direct path to indirect path under another gNB is shown as follows,

- Fig.4 Inter-gNB direct-to-indirect path switching

1.	The source gNB configures the UE measurement procedures and the UE reports according to the measurement configuration.

2.	The source gNB decides to handover the remote UE to indirect path, based on MeasurementReport and RRM information.

3.	The source gNB sends a Handover Request message to the target gNB to prepare the handover at the target side. The Handover Request includes Remote UE L2 ID and a list of candidate target relay UE IDs belonging to one cell at least.

4.	Admission Control may be performed by the target gNB.

5.	The target gNB prepares the handover with L1/L2 and sends the HANDOVER REQUEST ACKNOWLEDGE to the source gNB.

6.  The source gNB triggers the Uu handover by sending an RRCReconfiguration message to the remote UE.

7~13. The remote UE access to the Target gNB via relay UE.

- the source gNB sends the SN STATUS TRANSFER message to the target gNB.

- The UE synchronises to the target cell and completes the RRC handover procedure by sending RRCReconfigurationComplete message to target gNB.

- Target gNB sends the UE CONTEXT RELEASE to inform the source gNB about the success of the handover.

Proposal 5:  It is proposed to agree the overall procedure in Fig.4 as a baseline to support inter-gNB direct to indirect path switching.

- Intra-gNB indirect-to-indirect path switching

- Fig.5 Intra-gNB indirect-to-indirect path switching

- The signalling flow for U2N remote UE switch from indirect path to indirect path under same gNB is shown as follows,

1.	The gNB configures the UE measurement procedures and the UE reports according to the measurement configuration.

2.	The gNB decides to handover the UE to another indirect path of same gNB, based on MeasurementReport and RRM information.

3.	The gNB sends RRCReconfiguration message to the U2N Remote UE. The RRCReconfiguration message includes L2 U2N Relay UE ID, PC5 Relay RLC channel configuration for relay traffic at least.

4. The The gNB sends RRC reconfiguration message to the target Relay UE, which includes at least the Remote UE's L2 ID and local ID, Uu and PC5 Relay RLC channel configuration for relay traffic and bearer mapping configuration.

5. The gNB sends RRCReconfiguration message to the source Relay UE to reconfigure the connection between the source relay UE and the gNB.

6. Remote UE establishes PC5 connection to target relay UE, if not exist.

7. Remote UE sends the RRC complete message to gNB via the target Relay UE.

8. Either U2N Relay UE or U2N Remote UE1 can initiate the PC5 unicast link release (PC5-S).

Proposal 6:  It is proposed to agree the overall procedure in Fig.5 as a baseline to support intra-gNB indirect to indirect path switching.

- Inter-gNB indirect-to-indirect path switching

- Fig.6 Inter-gNB indirect-to-indirect path switching

- The signalling flow for U2N remote UE switch from indirect path to indirect path under another gNB is shown as follows,

1.	The source gNB configures the UE measurement procedures and the UE reports according to the measurement configuration.

2.	The source gNB decides to handover the UE to indirect path of another gNB, based on MeasurementReport and RRM information.

3.	The source gNB sends a Handover Request message to the target gNB to prepare the handover at the target side. The Handover Request includes Remote UE L2 ID and a list of candidate target relay UE IDs belonging to one cell at least.

4.	Admission Control may be performed by the target gNB.

5.	The target gNB prepares the handover with L1/L2 and sends the HANDOVER REQUEST ACKNOWLEDGE to the source gNB.

6.  The source gNB triggers the Uu handover by sending an RRCReconfiguration message to the remote UE.

7.  Remote UE establishes PC5 connection to target relay UE, if not exist.

8~13. The remote UE access to the Target gNB via target relay UE.

- Source gNB sends the SN STATUS TRANSFER message to the target gNB.

- The UE synchronises to the target cell and completes the RRC handover procedure by sending RRCReconfigurationComplete message to target gNB.

- Target gNB sends the UE CONTEXT RELEASE to inform the source gNB about the success of the handover.

17. The source gNB sends RRCReconfiguration message to the U2N source Relay UE to reconfigure the connection between the source Relay UE and the source gNB.

18. Either source Relay UE or Remote UE can initiate the PC5 unicast link release (PC5-S).

Proposal 7:  It is proposed to agree the overall procedure in Fig.6 as a baseline to support inter-gNB indirect to indirect path switching.

Conclusions

According the above discussion we have following proposals:

Proposal 1: Indicator from relay UE to gNB can be introduced to inform the DL data transmission condition.

Proposal 2: Indicator from relay UE to remote UE can be introduced to inform the UL data transmission condition.

Proposal 3: Ask RAN2 to discuss whether and how to restrict gNB behaviour of relay UE selection.

Proposal 4:  It is proposed to agree the overall procedure in Fig.3 as a baseline to support inter-gNB indirect to direct path switching.

Proposal 5:  It is proposed to agree the overall procedure in Fig.4 as a baseline to support inter-gNB direct to indirect path switching.

Proposal 6:  It is proposed to agree the overall procedure in Fig.5 as a baseline to support intra-gNB indirect to indirect path switching.

Proposal 7:  It is proposed to agree the overall procedure in Fig.6 as a baseline to support inter-gNB indirect to indirect path switching.

References

- [1] RAN2 #121 e-meeting, chairman notes.

- [2] R3-226805 Summary of discussion on CB:#32_SLRelay_ServiceContinuity

- [3] TS 38.323, Packet Data Convergence Protocol (PDCP) specification.