3GPP TSG-RAN WG2 #121-bis-e	Tdoc R2-2303664

Electronic meeting, April 17 – 26, 2023

Agenda Item:	7.25.2

Source:	Ericsson

Title:	On RAN2 aspects for UL TX switching Rel-18

Document for:	Discussion, Decision

# 1	Introduction

Most of the RAN2 aspects for UL Tx switching are addressed in an e-mail discussion held for this topic -  [Post121][045][MCE] UL TX Switching. This contribution further details the particular aspect of fallback compatibility between the UL Tx switching Rel-18 feature and the Rel-16 and Rel-17 features.

# 2	Discussion

The signaling structure for UL Tx switching allows the UE to report support of both Rel-16 and Rel-17 features in a single band combination.

BandCombination-UplinkTxSwitch-r16 ::= SEQUENCE {

bandCombination-r16                 BandCombination,

bandCombination-v1540               BandCombination-v1540                      OPTIONAL,

bandCombination-v1560               BandCombination-v1560                      OPTIONAL,

bandCombination-v1570               BandCombination-v1570                      OPTIONAL,

bandCombination-v1580               BandCombination-v1580                      OPTIONAL,

bandCombination-v1590               BandCombination-v1590                      OPTIONAL,

bandCombination-v1610               BandCombination-v1610                      OPTIONAL,

supportedBandPairListNR-r16         SEQUENCE (SIZE (1..maxULTxSwitchingBandPairs)) OF ULTxSwitchingBandPair-r16,

uplinkTxSwitching-OptionSupport-r16 ENUMERATED {switchedUL, dualUL, both}      OPTIONAL,

uplinkTxSwitching-PowerBoosting-r16 ENUMERATED {supported}                     OPTIONAL,

...,

[[

-- R4 16-5 UL-MIMO coherence capability for dynamic Tx switching between 3CC 1Tx-2Tx switching

uplinkTxSwitching-PUSCH-TransCoherence-r16     ENUMERATED {nonCoherent, fullCoherent}   OPTIONAL

]]

}

BandCombination-UplinkTxSwitch-v1720 ::= SEQUENCE {

bandCombination-v1720                    BandCombination-v1720                 OPTIONAL,

uplinkTxSwitching-OptionSupport2T2T-r17  ENUMERATED {switchedUL, dualUL, both} OPTIONAL

}

| uplinkTxSwitching-OptionSupport2T2T-r17 Indicates which option is supported for dynamic UL 2Tx-2Tx switching for inter-band UL CA. switchedUL represents option 1 as specified in TS 38.214 [12], dualUL represents option 2 as specified in TS 38.214 [12], both represents both option 1 and option2 as specified in TS 38.214 [12]. The field is mandatory for inter-band UL CA cases where UE supports dynamic UL 2Tx-2Tx switching. The UE indicating support of this feature shall indicate support of at least one common switching option between uplinkTxSwitching-OptionSupport2T2T-r17 and uplinkTxSwitching-OptionSupport-r16. | BC | CY | N/A | FR1 only |
| --- | --- | --- | --- | --- |

Following this design for the Rel-18 feature implies that further discussion is needed on how to handle the coexistence of Rel-16, Rel-17 and Rel-18 UL Tx switching features.

Due to the signaling structure, RAN2 should discuss how to handle coexistence of Rel-16, Rel-17 and Rel-18 UL Tx switching features.

In the e-mail discussion ([Post121][045][MCE] UL TX Switching), two approaches were suggested:

Approach 1: the 3/4 FeatureSetUplink are reported in one row in FSC for the 3/4 UL bands involved in Rel-18 UL Tx switching;

Approach 2: the FeatureSets reported for Rel-16/17 Tx switching between 2 bands can be combined to indicate UL capabilities on the 3/4 UL bands for Rel-18 UL Tx switching;

In 38.331, the following is described for the IE FeatureSetCombination:

#### –	FeatureSetCombination

The IE FeatureSetCombination is a two-dimensional matrix of FeatureSet entries.

Each FeatureSetsPerBand contains a list of feature sets applicable to the carrier(s) of one band entry of the associated band combination. Across the associated bands, the UE shall support the combination of FeatureSets at the same position in the FeatureSetsPerBand. All FeatureSetsPerBand in one FeatureSetCombination must have the same number of entries.

The number of FeatureSetsPerBand in the FeatureSetCombination must be equal to the number of band entries in an associated band combination. The first FeatureSetPerBand applies to the first band entry of the band combination, and so on.

The highlighted sentence above states that what the UE supports is derived from FeatureSets at the same position in the FeatureSetsPerBand, i.e. a “row of FeatureSets” in FeatureSetCombination. The approach 2 suggested in the e-mail discussion is not in line with the design of UE capabilities since it would require that the UE support is derived from two different “row of FeatureSets” in the IE FeatureSetCombination.

Combining different “rows of FeatureSets” in FeatureSetCombination changes the logic of the structure of FeatureSetCombination.

It should be noted that we already decided to have a separate band combination list for UL Tx Switching feature just for the sake of indicating different capabilities for UL Tx Switching compared to the legacy band combinations. We do not think it is sustainable to have yet further special handling of the already established capability design since Rel-15. Many features can benefit and can be enhanced by having their separate design of UE capability logic, but a balance is usually needed between signaling design and complexity, thus we should strive to minimize changes as usual.

It is not sustainable to have different handling of UE capabilities in multiple aspects of UL Tx Switching.

Moreover, the benefits do not seem to justify the complexity. From signaling size point of view, approach 2 would not require the UE to report a “row of FeatureSets” for Rel-18 UL Tx switching, and thus could save a couple of rows in the FeatureSetCombination IE. However, the design of Feature Set IDs is made it so that the UE can indicate the same feature for multiple band combinations and/or per band per band combination, hence a couple of rows within the FeatureSetCombination IE should not be the main reason to introduce a different logic in the FeatureSetCombination, which can anyway have up to 128 different rows (maxFeatureSetsPerBand).

From the point of view of UE capability flexibility, if there is a need to report different flavours of the same capabilities e.g. different bandwidths for different band pairs, the UE can already use multiple “rows of FeatureSets” to indicate different capabilities on each row, which is actually the intention with the design of FeatureSetCombination. Alternatively, the UE can indicate different band combinations with different capabilities if there is need for yet further flexibility.

Current capability design is already quite flexible with FeatureSetCombination structure and efficient from signaling size point of view.

Approach 1, however, is in line with the adopted approach for Rel-17 UL Tx switching (since it does not require any new logic for FeatureSetCombination), and simply intends to have a “row of FeatureSets” in FeatureSetCombination that indicates support for Rel-18 UL Tx switching. Therefore, we think approach one should be adopted.

The 3/4 FeatureSetUplink are reported in one “row of FeatureSets” in FeatureSetCombination for the 3/4 UL bands involved in Rel-18 UL Tx switching.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	Due to the signaling structure, RAN2 should discuss how to handle coexistence of Rel-16, Rel-17 and Rel-18 UL Tx switching features.

Observation 2	Combining different “rows of FeatureSets” in FeatureSetCombination changes the logic of the structure of FeatureSetCombination.

Observation 3	It is not sustainable to have different handling of UE capabilities in multiple aspects of UL Tx Switching.

Observation 4	Current capability design is already quite flexible with FeatureSetCombination structure and efficient from signaling size point of view.

Based on the discussion in the previous sections we propose the following:

Proposal 1	The 3/4 FeatureSetUplink are reported in one “row of FeatureSets” in FeatureSetCombination for the 3/4 UL bands involved in Rel-18 UL Tx switching.