3GPP TSG-RAN WG2 Meeting #121b-e											R2-2303742

Online, 17th April– 26th April, 2023

Agenda item:       5.2

Source: 	LG Electronics

Title: 	Summary on MAC user plane corrections for NR V2X

Document for:	Discussion and Decision

Introduction

This document summarizes the contributions submitted to AI 5.2. Rapporteur’s recommendations on how to treat them are provided.

Corrections & proposals

The following corrections and proposals could be discussed together.

Table 1: Corrections & proposals

| Tdoc | Title | Company | Summary of changes/proposals | Rapp’s remark |
| --- | --- | --- | --- | --- |
| R2-2302574 | Left issue on SL CG clear during MAC-reset | OPPO | Observation 2 For SL CG, during handover, there is no concern on timing since GNSS should be used typically for NR SL. For MAC-set triggered by cases other than handover, another related case is RLF, for which current spec already command the UE to stop using type-2 CG (upon T310 start) and release type-1 CG (upon T311 start). Observation 3 For SL CG, current spec requires UE to switch to mode-2 using exceptional pool upon T304 start without clearing operation, to stop using type-2 CG upon T310 start, and to release type-1 CG upon T311 start. There seems no big motivation to deviate from the current spec for this quite late NBC change. Proposal. RAN2 not purse the change for clearing SL CG upon MAC-reset. | Rapporteur also agrees with proponet's observation. Proposal can be discussed. |
| R2-2303210 | Discussion on clear of SL CG upon MAC reset | Xiaomi | Observation 1: When T311 is running, UE is not allowed to use the configured SL grant based on current RRC specification, no need to duplicate in MAC. Observation 2: When T310 is running, if MAC reset is triggered by NW signalling, we can rely on NW implementation to release the configured SL grant. Proposal 1: RAN2 does not agree to clear configured SL grant when MAC is reset. | Related contributions (R2-2302574, R2-2303210, R2-2303915) can be discussed together. |
| R2-2303915 | Corrections on MAC reset regarding configured sidelink grant | ASUSTeK, Huawei, HiSilicon, Samsung, vivo | Reason for change: In the current specification, when a MAC entity performs MAC reset requested by RRC, the MAC entity considers the timeAlignmentTimer to be expired and performs the actions in clause 5.2, where the MAC entity clears any configured DL assignments and configured UL grants. The MAC entity should not keep using SL CG in scenarios requiring MAC reset, e.g. the serving cell changes due to handover/RRC re-establishment. To align with the handling of Uu CG, the MAC entity should also clear SL CG when performing MAC reset. Correction: ## 5.12 MAC Reset If a reset of the MAC entity is requested by upper layers, the MAC entity shall: 1> clear, if any, configured sidelink grants; | Related contributions (R2-2302574, R2-2303210, R2-2303915) can be discussed together. |
| R2-2303211 | Correction on PSFCH reception for NR sidelink | Xiaomi | Reason for change: In current specification, it is stated that if an acknowledgement corresponding to the PSSCH transmission is not obtained from the physical layer, UE delivers a negative acknowledgement to the corresponding Sidelink HARQ entity for the Sidelink process. However, this does not apply to NACK-only case and should be clarified. Correction: | No modification is required. According to the procedure below of 38.213, in case of ACK (absence of PSFCH reception) in NACK only mode, PHY reports ACK to MAC. - if the PSFCH reception occasion is associated with a SCI format 2-B or a SCI format 2-A with Cast type indicator field value of "11" - report to higher layers an ACK value if the UE determines absence of PSFCH reception for the PSFCH reception occasion; otherwise, report a NACK value to higher layers So the current MAC spec correctly specifies NACK only behavior. |

Proposal: Discuss the CRs listed in Table 1 offline, the agreed changes for MAC spec could be merged into Rapporteur’s correction CR.

Conclusions

The contributions submitted to AI 5.2 are summarized, Rapporteur’s recommendations are as follows.

Proposal: Discuss the CRs listed in Table 1 offline, the agreed changes for MAC spec could be merged into Rapporteur’s correction CR.