3GPP TSG-RAN WG2 Meeting #121-bis	R2- 2303810

Online, 17 April – 26 April, 2023                  (Re-submission of R2-2212639)

Agenda item:	7.8.4

Source: 	Huawei, HiSilicon

Title: 	Discussion on subscription-based UAV identification

Document for:	Discussion and Decision

- Introduction

In RAN meeting #94e, a new work item was approved for NR UAV [1] and one of the objectives is about the subscription-based aerial-UE identification as follows:

2. Specify the signaling to support subscription-based aerial-UE identification [RAN3/SA2 interaction/RAN2]

Note: Work done in LTE is a starting point for this objective. NR-specific enhancements can be considered, if needed, while overall the LTE and NR solutions should be harmonized as much as possible.

In this contribution, we would like to clarify the RAN2 work on the subscription-based UAV identification and some related proposals will be provided. This contribution is a revision of R2-2212639. Compared to the previous version, we analysed the RAN3 #117-bis agreements that mentioned RAN2.

- Discussion

2.1 Subscription-based UAV identification in LTE

In Rel-15 LTE UAV WI, a similar discussion was performed in RAN2 #101 meeting [2] and the following agreements were achieved:

Agreement:

1	RAN2 confirm the following work/responsibility split between WGs:

RAN2:

-	To clarify and confirm the usage of the subscription based identification and the overall stage2 behaviour of UAV identification signaling.

RAN3:

-	To define the signaling/IE for subscription based UAV identification.

SA2:

-	To define the subscription based UAV identification and the necessary signaling in interfaces between CN nodes (e.g., MME – HSS, etc.)

2	RAN2 to confirm that the subscription based information is signaled from the MME to the eNB

As usual, the subscription information definition is mainly RAN3 and SA2 work, since the signalling exchange is needed between network nodes. From RAN2 perspective, the only work is to capture it in stage2 specification.

Observation 1: In Rel-15 UAV WI, RAN2 only captured the stage2 description on subscription information and the definition was in RAN3/SA2 scope.

2.2 Subscription-based aerial-UE identification in NR

With respect to the objective for NR UAV, we think it is straightforward to follow LTE principle to save RAN2 standard effort, i.e. RAN2 to capture the subscription information for UAV in stage 2 specification.

Proposal 1: RAN2 to capture subscription information in stage 2 specification.

One more thing is about the LTE/5GC case. As specified in SA2 specification, the subscription information was introduced in 5GC and it is assumed to be exchanged between 5GC and NG-RAN. From the RAN perspective, the NG-RAN includes LTE eNB and NR gNB. In our understanding, the current WID focuses on NR systems, and it is not clear whether LTE/5GC is in the scope. From our perspective, we are ok to support the LTE/5GC case but we don't think any change is needed for LTE Uu.

Proposal 2: Subscription information can be used in LTE/5GC scenario and there is no need for LTE Uu enhancements.

- Conclusion

Observation 1: In Rel-15 UAV WI, RAN2 only captured the stage2 description on subscription information and the definition was in RAN3/SA2 scope.

Proposal 1: RAN2 to capture subscription information in stage2 specification.

Proposal 2: Subscription information can be used in LTE/5GC scenario and there is no need for LTE Uu enhancements.

- Reference

- RP-213600 “WI on NR Support for UAV”.

- R2-1804201