3GPP TSG-RAN WG2 Meeting #121bis Electronic	R2-2303862

Elbonia, 17 – 26 April 2023

Agenda item:	7.5.4.1

Source:	Nokia, Nokia Shanghai Bell

Title:	BSR enhancements for XR

WID/SID:	NR_XR_enh-core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

The following agreement was made in RAN2 #121 [1]:

New BSR tables are fixed (=specified) or semi-static (RRC-based). FFS how many BSR tables are defined.

This contribution gives more details on the BSR tables and delay information report for XR.

# 2	Discussion

## 2.1	New BS Table(s)

One way to evaluate the quantization errors of the BS tables is to define a quantization error metric representing the actual packet size versus the packet size reported to the network.

QE=reported packet size B-actual packet size(P)

In an ideal case the reported packet size would be identical to the actual packet size. Yet the packet size must be reported with either Table 6.1.3.1-1 for a 5 bit BSR or Table 6.1.3.1-2 for 8 bit BSR from TS 38.321. Innevitably, this leads to quantization errors -  we ask more from the network than what we actually need. For some total number of packets N, the total quantization error can be defined in percent as:

QEtotal=NQENB×100 %

An example of the quantization error for a 5-bit BSR table is depicted in Figure 1, where it reaches approximately QEtotal5 bit= 16.7 %. On the figure, the X axis shows the packet sizes for a truncated gaussian traffic model as per TR 38.838 with 10 Mbps mean data rate. The blue circles indicate the table entries in Table 6.1.3.1-1 for a 5 bit BSR reporting from TS 38.321. The 5 bit table corresponds to 32 possible values but due to its exponential nature most table entries are not used by the XR traffic and cropped out from Figure 1. The left hand Y axis corresponds to the black solid line plotting the probability density function (PDF) of the traffic model. The shaded regions between two blue points (bins) is the integrated probablity of a packet falling is that table range. This means that most of the XR packets fall in just two table bins. The right hand Y axis corresponds to the cumulative quantization error per bin, which  is shown with the red crosses. Each red cross is aligned to the right hand side of the bin range.

Figure 2 shows the utilization of the 8 bit BSR table (Table 6.1.3.1-2)  in TS 38.321 with the same interpretation as above. The total quantization error is QEtotal8 bit= 3.1 %.

Figure 1 – 5-bit BSR table utilization from TS 38.321

Figure 2 – 8-bit BSR table utilization from TS 38.321

It should be noted that no matter the mean data rate, these statistics hold true.

To reduce the quantization errors for XR traffic:

1)	it should be possible to report the long BSR even when only one bearer has data buffered; and

2)	new BS table(s) should be introduced

On how to define the new tables, the main solutions discussed in the previous RAN2 meeting were 1) to have a number of pre-defined tables and RRC configure the table index to indicate which one to be used, or 2) RRC configures parameters, e.g., the range for the table for the UE to calculate the table to be used.

Pre-defined tables are simple from implementation point of view, but it can only support a small number of tables, thus might not be optimal considering the variable data and frame rates for XR service.

In our previous proposals [2], we considered the generation of tables based on RRC configuration such that the gNB and the UE can adapt them to the changing traffic conditions. We emphasize that these tables are not generated on an often basis (i.e., at each BSR transmission). These proposals raised concerns within previous RAN2 meetings since companies pointed out UE’s complexity. However, it should be noted that these tables could be generated offline and stored without any online processing. Additionally, it should be mentioned that the UE’s implementation could generate all the tables offline, which is a one-time process, and use the corresponding table according to the configuration.

Concerning the type of distribution to be used to generate the table, truncated gaussian distribution has been assumed in the RAN1 simulations and might match best with the XR traffics. However, it might be complex to specify the formula. Thus, uniform distribution was proposed by most of the companies. If truncated gaussian distribution is preferred to better match the traffic distribution, one possibility could be to introduce a reference table with truncated gaussian distribution (e.g. based on 1Mbps and 60fps traffic), and rely on RRC to configure a scaling factor based on the actual data rate and traffic pattern.

An example of a reference table is presented in Table 1 below. This table is generated using the properties of a truncated gaussian distribution for XR traffic described in TR 38.838. The assumed data and video frame rate is 1 Mbps and 60 fps, respectively.

Table 1 – XR BS reference table for 8-bit long format based on truncated gaussian traffic distribution, 1 Mbps mean data rate and 60 frames per seconds. Index 255 can be left reserved as in 6.1.3.1-2

| Index | BS value | Index | BS value | Index | BS value | Index | BS value |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1042 | 64 | ≤ 1936 | 128 | ≤ 2084 | 192 | ≤ 2233 |
| 1 | ≤ 1502 | 65 | ≤ 1939 | 129 | ≤ 2087 | 193 | ≤ 2236 |
| 2 | ≤ 1555 | 66 | ≤ 1942 | 130 | ≤ 2089 | 194 | ≤ 2238 |
| 3 | ≤ 1588 | 67 | ≤ 1944 | 131 | ≤ 2091 | 195 | ≤ 2241 |
| 4 | ≤ 1613 | 68 | ≤ 1947 | 132 | ≤ 2093 | 196 | ≤ 2244 |
| 5 | ≤ 1632 | 69 | ≤ 1950 | 133 | ≤ 2095 | 197 | ≤ 2247 |
| 6 | ≤ 1649 | 70 | ≤ 1952 | 134 | ≤ 2097 | 198 | ≤ 2250 |
| 7 | ≤ 1663 | 71 | ≤ 1955 | 135 | ≤ 2099 | 199 | ≤ 2253 |
| 8 | ≤ 1676 | 72 | ≤ 1957 | 136 | ≤ 2102 | 200 | ≤ 2255 |
| 9 | ≤ 1688 | 73 | ≤ 1960 | 137 | ≤ 2104 | 201 | ≤ 2258 |
| 10 | ≤ 1698 | 74 | ≤ 1962 | 138 | ≤ 2106 | 202 | ≤ 2261 |
| 11 | ≤ 1708 | 75 | ≤ 1965 | 139 | ≤ 2108 | 203 | ≤ 2264 |
| 12 | ≤ 1717 | 76 | ≤ 1967 | 140 | ≤ 2110 | 204 | ≤ 2267 |
| 13 | ≤ 1726 | 77 | ≤ 1970 | 141 | ≤ 2112 | 205 | ≤ 2271 |
| 14 | ≤ 1734 | 78 | ≤ 1972 | 142 | ≤ 2115 | 206 | ≤ 2274 |
| 15 | ≤ 1741 | 79 | ≤ 1975 | 143 | ≤ 2117 | 207 | ≤ 2277 |
| 16 | ≤ 1748 | 80 | ≤ 1977 | 144 | ≤ 2119 | 208 | ≤ 2280 |
| 17 | ≤ 1755 | 81 | ≤ 1980 | 145 | ≤ 2121 | 209 | ≤ 2283 |
| 18 | ≤ 1761 | 82 | ≤ 1982 | 146 | ≤ 2123 | 210 | ≤ 2287 |
| 19 | ≤ 1768 | 83 | ≤ 1984 | 147 | ≤ 2126 | 211 | ≤ 2290 |
| 20 | ≤ 1774 | 84 | ≤ 1987 | 148 | ≤ 2128 | 212 | ≤ 2293 |
| 21 | ≤ 1779 | 85 | ≤ 1989 | 149 | ≤ 2130 | 213 | ≤ 2297 |
| 22 | ≤ 1785 | 86 | ≤ 1991 | 150 | ≤ 2132 | 214 | ≤ 2300 |
| 23 | ≤ 1790 | 87 | ≤ 1994 | 151 | ≤ 2134 | 215 | ≤ 2304 |
| 24 | ≤ 1795 | 88 | ≤ 1996 | 152 | ≤ 2137 | 216 | ≤ 2307 |
| 25 | ≤ 1801 | 89 | ≤ 1998 | 153 | ≤ 2139 | 217 | ≤ 2311 |
| 26 | ≤ 1805 | 90 | ≤ 2001 | 154 | ≤ 2141 | 218 | ≤ 2315 |
| 27 | ≤ 1810 | 91 | ≤ 2003 | 155 | ≤ 2143 | 219 | ≤ 2318 |
| 28 | ≤ 1815 | 92 | ≤ 2005 | 156 | ≤ 2145 | 220 | ≤ 2322 |
| 29 | ≤ 1819 | 93 | ≤ 2008 | 157 | ≤ 2148 | 221 | ≤ 2326 |
| 30 | ≤ 1824 | 94 | ≤ 2010 | 158 | ≤ 2150 | 222 | ≤ 2330 |
| 31 | ≤ 1828 | 95 | ≤ 2012 | 159 | ≤ 2152 | 223 | ≤ 2334 |
| 32 | ≤ 1832 | 96 | ≤ 2014 | 160 | ≤ 2154 | 224 | ≤ 2339 |
| 33 | ≤ 1836 | 97 | ≤ 2017 | 161 | ≤ 2157 | 225 | ≤ 2343 |
| 34 | ≤ 1840 | 98 | ≤ 2019 | 162 | ≤ 2159 | 226 | ≤ 2347 |
| 35 | ≤ 1844 | 99 | ≤ 2021 | 163 | ≤ 2161 | 227 | ≤ 2352 |
| 36 | ≤ 1848 | 100 | ≤ 2023 | 164 | ≤ 2164 | 228 | ≤ 2356 |
| 37 | ≤ 1852 | 101 | ≤ 2026 | 165 | ≤ 2166 | 229 | ≤ 2361 |
| 38 | ≤ 1856 | 102 | ≤ 2028 | 166 | ≤ 2168 | 230 | ≤ 2366 |
| 39 | ≤ 1859 | 103 | ≤ 2030 | 167 | ≤ 2171 | 231 | ≤ 2371 |
| 40 | ≤ 1863 | 104 | ≤ 2032 | 168 | ≤ 2173 | 232 | ≤ 2376 |
| 41 | ≤ 1867 | 105 | ≤ 2035 | 169 | ≤ 2175 | 233 | ≤ 2382 |
| 42 | ≤ 1870 | 106 | ≤ 2037 | 170 | ≤ 2178 | 234 | ≤ 2387 |
| 43 | ≤ 1873 | 107 | ≤ 2039 | 171 | ≤ 2180 | 235 | ≤ 2393 |
| 44 | ≤ 1877 | 108 | ≤ 2041 | 172 | ≤ 2182 | 236 | ≤ 2399 |
| 45 | ≤ 1880 | 109 | ≤ 2043 | 173 | ≤ 2185 | 237 | ≤ 2405 |
| 46 | ≤ 1883 | 110 | ≤ 2046 | 174 | ≤ 2187 | 238 | ≤ 2412 |
| 47 | ≤ 1887 | 111 | ≤ 2048 | 175 | ≤ 2189 | 239 | ≤ 2418 |
| 48 | ≤ 1890 | 112 | ≤ 2050 | 176 | ≤ 2192 | 240 | ≤ 2426 |
| 49 | ≤ 1893 | 113 | ≤ 2052 | 177 | ≤ 2194 | 241 | ≤ 2433 |
| 50 | ≤ 1896 | 114 | ≤ 2054 | 178 | ≤ 2197 | 242 | ≤ 2441 |
| 51 | ≤ 1899 | 115 | ≤ 2056 | 179 | ≤ 2199 | 243 | ≤ 2450 |
| 52 | ≤ 1902 | 116 | ≤ 2059 | 180 | ≤ 2202 | 244 | ≤ 2459 |
| 53 | ≤ 1905 | 117 | ≤ 2061 | 181 | ≤ 2204 | 245 | ≤ 2468 |
| 54 | ≤ 1908 | 118 | ≤ 2063 | 182 | ≤ 2207 | 246 | ≤ 2479 |
| 55 | ≤ 1911 | 119 | ≤ 2065 | 183 | ≤ 2209 | 247 | ≤ 2490 |
| 56 | ≤ 1914 | 120 | ≤ 2067 | 184 | ≤ 2212 | 248 | ≤ 2503 |
| 57 | ≤ 1917 | 121 | ≤ 2069 | 185 | ≤ 2214 | 249 | ≤ 2518 |
| 58 | ≤ 1920 | 122 | ≤ 2072 | 186 | ≤ 2217 | 250 | ≤ 2534 |
| 59 | ≤ 1923 | 123 | ≤ 2074 | 187 | ≤ 2220 | 251 | ≤ 2554 |
| 60 | ≤ 1926 | 124 | ≤ 2076 | 188 | ≤ 2222 | 252 | ≤ 2579 |
| 61 | ≤ 1928 | 125 | ≤ 2078 | 189 | ≤ 2225 | 253 | ≤ 2612 |
| 62 | ≤ 1931 | 126 | ≤ 2080 | 190 | ≤ 2228 | 254 | ≤ 2665 |
| 63 | ≤ 1934 | 127 | ≤ 2082 | 191 | ≤ 2230 | 255 | ≤ 3125 |

The reference table can be used to scale up to any data and frame rate by performing an adjustment with an offset signaled by the gNB:

BSValueadji=CEILINGBSValuerefi×offset_parameter,

where BSValueadji and BSValuerefi correspond to the adjusted BS value in the reference table and the reference BS value at index i (e.g., i ∈0…255 for 8-bit). The offset_parameter is the scaling factor configured to the UE, but transparent on how it is decided since it is determined by the gNB. Besides, it should be possible to use the 8 bits table even if there is only one LCG with data available for transmission to ensure finer granularity.

Proposal 1: define a new reference XR BS table in MAC based reference values for video and frame rates, and apply RRC configured scaling factor on top for different data and frame rates.

Proposal 2: the scaling factor with the new reference table could be configured per LCG; otherwise if not configured, the LCG uses legacy table.

Proposal 3: the new 8-bit BS reference table can be used even if there is only one LCG with data available for transmission.

## 2.2	Co-existence of the new table and the legacy table

Since there could also be other traffic other than the video traffic, e.g., voice, pose, RRC signaling etc., legacy tables are still to be used for those LCH/LCGs. Besides, when the remaining data of the video traffic becomes out of range of the new table, it makes sense to fallback to legacy table to provide enough information for lower range BS values.

Proposal 4: when the remaining data for the LCG configured with new table falls out of the range of the new table, it falls back to use legacy BSR table, i.e., a LCG configured to use new table can be reported in legacy MAC CE as well.

Proposal 5: the MAC CE with new BSR table and the legacy BSR table are identified with different LCIDs, thus the NW knows reported LCGs used legacy table or the new table without other explicit indication.

## 2.3	Delay Information

In the RAN2 #120 meeting, the following were agreed on delay information:

RAN2 will introduce data volume information associated with delay information (e.g., remaining time) in a MAC CE. FFS if this is extension of BSR or new format. FFS how to do that (e.g., what exactly is reported) and how to ensure this information is up-to-date e.g., considering UL scheduling delay.

This delay information, defined as “remaining time” is the result of the difference between the PDB or PSDB and the time data has been in the buffer (i.e., buffering time). Per DRB, UEs signal it to the network to enable it to learn about the UL traffic timing patterns and accordingly schedule/configure resource allocations for its UEs in coherence with latency constraints. In case a DRB includes PDUs with or without PDU set that have arrived at different times, UEs could use the longest buffering time to calculate the remaining time. The latter also applies in case PDU sets are used to transmit data.

Proposal 6: Define new BSR format to report remaining time information per LCG.

Proposal 7: the shortest remaining time of data buffered is reported as the remaining time for the LCG.

Proposal 8: both independent PDUs or PDUs conforming a PDU set are supported.

# 3	Conclusion

This contribution has reviewed possible new BSR tables for XR and has proposed the following:

Proposal 1: define a new reference XR BS table in MAC based reference values for video and frame rates, and apply RRC configured scaling factor on top for different data and frame rates.

Proposal 2: the scaling factor with the new reference table could be configured per LCG; otherwise if not configured, the LCG uses legacy table.

Proposal 3: the new 8-bit BS reference table can be used even if there is only one LCG with data available for transmission.

Proposal 4: when the remaining data for the LCG configured with new table falls out of the range of the new table, it falls back to use legacy BSR table, i.e., a LCG configured to use new table can be reported in legacy MAC CE as well.

Proposal 5: the MAC CE with new BSR table and the legacy BSR table are identified with different LCIDs, thus the NW knows reported LCGs used legacy table or the new table without other explicit indication.

Proposal 6: Define new BSR format to report remaining time information per LCG.

Proposal 7: the shortest remaining time of data buffered is reported as the remaining time for the LCG.

Proposal 8: both independent PDUs or PDUs conforming a PDU set are supported.

# Reference

[1] R2-2301902, Report from session on LTE legacy, XR, QoE and Multi-SIM, Vice Chairman (Nokia)

[2] R2-2301517, Capacity improvements, Nokia, Nokia Shanghai Bell