3GPP TSG-RAN WG2 Meeting #123                                                                  R2-2307100

Toulouse, France, August 21-25, 2023

Agenda Item	:	7.5.4.2

Title	:	Discussion on PDU discard for XR

Source	:	Google Inc.

Work Item	:	NR_XR_enh-Core

Document for	:	Discussion and Decision

# 1		Introduction

In this paper, we discuss the dedicated singling of PSI-based discard including the on-off indication and reduction of signalling overhead. We also discuss that the network may need some information from UE in order to determine how many UEs should perform PSI-based discard and which PSI they should apply.

# 2		Discussion

In RAN2 #122, RAN2 agreed that the network indicates to UE to apply PSI-based XR discard via dedicated signalling. There are some open issues including what the dedicated signalling is and which PSI should be applied in the PSI-based discard.

Since the PDCP is responsible to initiate PDU set discard, the network can use PDCP control PDU to indicate to UE to apply PSI-based discard. Moreover, the network is supposed to have the knowledge of when the congestion happens and how serious the congestion is. The network is capable of determining a PSI for UE to perform PSI-based discard.

Proposal 1: The network indicates a PSI to UE to apply PSI-based discard.

The SA2 has defined 16 PSI levels but UE may not use all of them. The gNB may need to know the PSI levels that UE will assign to PDU sets. With this information the gNB can determine a PSI for the UE to perform PSI-based discard. Otherwise, gNB may indicate a PSI that UE does not use, and as a result, no PDU sets are discarded.

UE can indicate a list of PSI levels that it will assign to PDU sets to the network. The mapping between a PSI and a PDU set or a type of PDU set can be pre-defined or configured by the network. For example, the I frame is a type of PDU sets and the P frame is another type of PDU set.

Proposal 2: UE indicates a list of PSIs that it will assign to PDU sets to the network.

In addition to PSI levels that UE will assign to PDU sets, the network may need other information in order to determine how many UEs should perform PSI-based discard and which PSI they should apply. For example, it is beneficial for gNB to know how much uplink traffic is reduced if a UE performs a PSI-based discard on a PSI level. With this information, gNB can determine whether more UEs should perform PSI-based discard or not.

In the last meeting, the one-shot indication and the on-off indication is discussed for PSI-based discard. For the one-shot approach, UE discards PDU sets in buffer immediately after receiving the dedicated signalling. The network may need to know how much data will be discarded after performing the one-shot discard. If more data should be discarded, the network may indicate to more UE to perform PSI-based discard.

For the on-off approach, UE discards PDU sets according to a PSI level. The network may need to know how much data is dropped during the on period. If more uplink traffic should be reduced, the network may request more UE to perform PSI-based discard.

Proposal 3: RAN2 to discuss what information the network needs from UE to support PSI-based discard.

In RAN2 #122, there are some FFSs regarding whether the network indication is a one-shot or also subsequent packets and how to minimize UL signalling. It is beneficial to support the on-off approach. If network congestion happens, the congestion may be last for a period of time. UE may need perform PSI-based discard until the network indicates the end of PSI-based discard to UE.

To reduce signalling overhead, the network can also indicate a time period to UE in a dynamic or semi-persistent way. At the end of the time period, UE stops the PSI-based discard autonomously.

Proposal 4: Support the on-off indication of PSI-based discard.

In PDCP, a SDU is discarded when the discard timer expires or a status report indicates that the SDU is successfully received. For XR application, SDU discard may happen frequently because if the delivery deadline of a PDU set is due, all SDUs in the PDU set are discarded. The discard of a PDU set also incur longer reordering delay than single PDU discard.

In order to reduce reordering delay, it is beneficial to indicate discarded SDUs to the receiving entity. The receiving entity uses the discard report to reduce reordering waiting time. The receiving entity can also transmit a status report to acknowledge the reception of the discard indication.

Proposal 5: PDCP transmitting entity indicates discarded SDUs to receiving entity.

# 3		Conclusions

In conclusion, we have the following proposals:

Proposal 1: The network indicates a PSI to UE to apply PSI-based discard.

Proposal 2: UE indicates a list of PSIs that it will assign to PDU sets to the network.

Proposal 3: RAN2 to discuss what information the network needs from UE to support PSI-based discard.

Proposal 4: Support the on-off indication of PSI-based discard.

Proposal 5: PDCP transmitting entity indicates discarded SDUs to receiving entity.