3GPP TSG-RAN WG2 Meeting #123	R2-2307410

# Toulouse, France, 21 - 25 Aug, 2023

Agenda Item:    7.13.7

Source:         Xiaomi

Title:         	Discussion on the “LS on potential override of logged MDT reports upon moving from SNPN to PLMN” from RAN3 (R3-232118)

Document for:	Discussion and Decision

Introduction

Previously, RAN3 sent an LS to ask about how to avoid the loss of stored logged MDT reports during the mobility between different types of networks. Details as follows: [1]

RAN3 is currently working on ensuring SON/MDT support for non-public networks.

In this work, RAN3 noticed that logged MDT reports collected in one type of network, e.g. an SNPN, are lost when UE moves to a network of another type, e.g. a Public Network, due to the UE deregistering from the SNPN network. RAN3 believes that this would lead to a loss of the stored logged MDT reports if the reports are not retrieved before moving to the network of another type.

RAN3 would like to check with RAN2 if there are any solutions to avoid the loss of stored logged MDT reports upon moving from a network of one type to another type, even upon deregistration.

In this contribution, we provide our consideration on the RAN3 LS and provide the draft LS reply in the annex.

Discussion

For the mobility between the SNPN and PLMN, as mentioned in the RAN3 LS, UE needs to deregister from the old network first. And in this case, the logged MDT configuration and the logged measurements information will be discarded as specified in TS 38.331 quoted below. RAN3 thinks it will cause the loss of stored logged MDT reports and asks RAN2 for some solutions on this.

TS 38.331

The UE shall initiate the procedure upon receiving a logged measurement configuration in another RAT. The UE shall also initiate the procedure upon power off or upon deregistration.

The UE shall:

1>	stop timer T330, if running;

1>	if stored, discard the logged measurement configuration as well as the logged measurement information, i.e. release the UE variables VarLogMeasConfig and VarLogMeasReport.

Before discussing about any solutions on this, we think RAN2 should discuss about on whether the issue is valid or not at first.

Actually, this issue also exists for the existing logged MDT report in the case of mobility between PLMNs. Currently, for the mobility between PLMNs which are not the equivalent PLMNs, UE also needs to perform the deregistration process in the previous PLMN first, and then perform the registration process in the new PLMN. This can ensure that UE correctly receives services in the new PLMN and releases resources in the previous PLMN. In such case, the logged MDT report in the previous PLMN may also be discarded when switching to the new PLMN.

For such issue in legacy PLMN scenario, there is no enhancement mechanism on this and it is up to network implementation on the logged MDT report retrieval. As specified in TS 38.413 quoted below, when AMF command NG-RAN to release the UE context in the deregistration procedure, the NAS cause can be set to “Deregister”. By this way, NG-RAN can decide whether to perform the report retrieval based on such NAS cause.

- 9.3.1.2	Cause

The purpose of the Cause IE is to indicate the reason for a particular event for the NGAP protocol.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| … | … | … | … | … |
| >>NAS Cause | M |  | ENUMERATED (Normal release, Authentication failure, Deregister, Unspecified, …, UE not in PLMN serving area) |  |
| … | … | … | … | … |

Observation 1: The loss of stored logged MDT reports also exists for the mobility between PLMNs upon deregistration, and it is left to network implementation on the logged MDT report retrieval.

Observation 2: NG-RAN can get aware of the deregistration procedure based on the UE context release cause “Deregister” and decide whether to perform report retrieval base on this.

Based on the above analysis, we think the motivation of introducing the enhancement for the logged MDT report during the mobility between SNPN and PLMN needs to be clarified, otherwise, we can also leave it to the network implementation in the NPN scenario.

Proposal 1: The motivation to have the enhancement for logged MDT report for the mobility between the SNPN and PLMN upon deregistration needs to be further clarified by RAN3. Otherwise, it can also be left to network implementation.

Proposal 2: RAN2 considers the draft LS reply to RAN3 in the annex.

Conclusions

In this contribution, we provide our analysis for RAN3 LS on the potential override of logged MDT reports upon moving from SNPN to PLMN, and have the observation and proposals as follows:

Observation 1: The loss of stored logged MDT reports also exists for the mobility between PLMNs upon deregistration, and it is left to network implementation on the logged MDT report retrieval.

Observation 2: NG-RAN can get aware of the deregistration procedure based on the UE context release cause “Deregister” and decide whether to perform report retrieval base on this.

Proposal 1: The motivation to have the enhancement for logged MDT report for the mobility between the SNPN and PLMN upon deregistration needs to be further clarified by RAN3. Otherwise, it can also be left to network implementation.

Proposal 2: RAN2 considers the draft LS reply to RAN3 in the annex.

References

R2-2304628 LS on potential override of logged MDT reports upon moving from SNPN to PLMN (R3-232118; contact: Ericsson).

Annex: Draft LS reply

3GPP TSG-RAN WG2 Meeting #123	R2-230xxxx

# Toulouse, France, 21 - 25 Aug, 2023

Title:	[Draft] Reply LS on the potential override of logged MDT reports upon moving from SNPN to PLMN

Response to:	R2-2304628(R3-232118) “LS on potential override of logged MDT reports upon moving from SNPN to PLMN”

Release:	Release 18

Work Item:	NR_ENDC_SON_MDT_enh2-Core

Source:	RAN2

To:	RAN3

Cc:

Contact Person:

Name:                   <name>

E-mail Address:   <email>

1. Overall Description:

RAN2 would like to thank RAN3 for the LS on the potential override of logged MDT reports upon moving from SNPN to PLMN.

Regarding the loss of stored logged MDT reports during the mobility between different types of networks, RAN2 discussed the issue and observed that the same issue also exists for the mobility between PLMNs upon deregistration, and it is up to network implementation on the logged MDT report retrieval.

Based on the above, RAN2 kindly asks RAN3 to clarify the motivation to have the enhancement for logged MDT during mobility between SNPN and PLMN. Otherwise, we can also leave it to network implementation and there is no Uu impacts.

2. Actions:

To RAN WG3

ACTION:  RAN2 kindly asks RAN3 to take above into account and provide the feedback on the clarification accordingly.

3. Date of Next TSG-RAN WG2 Meetings:

TSG RAN WG2 Meeting #123-bis	09 Oct –13 Oct 2023			Xiamen, CN

TSG RAN WG2 Meeting #124	13 Nov –17 Nov 2023			Chicago, US