3GPP TSG-RAN WG2 Meeting #123		R2-2307417

Toulouse, France, August 21-25, 2023

Source:	CATT

Title:	Discussion on the mechanism for providing TN coverage information

Agenda Item:	7.7.4.1.1

Document for:	Discussion and Decision

Introduction

In RAN2#122 meeting, the cell reselection enhancements in NTN-TN scenario were discussed with some leftover issues. In this contribution, these leftover issues will be further discussed, including:

- Which SIB to provide the TN coverage info.

- The size of TN coverage info list.

- How to provide the frequency information.

- Priority of TN frequency.

- Validity of TN coverage area information.

Discussion

  - Which SIB to provide the TN coverage info

In RAN2#122, the following agreement was achieved.

- TN coverage info is NOT included in SIB19. FFS if we use an existing SIB or a new one

For the FFS of using an existing SIB or a new SIB to carry the TN coverage info, some companies propose to include the TN coverage info in existing SIB(s), i.e., SIB3/4/5. However, the existing SIBs and the TN coverage info may suitable for different update frequencies and periodicities. For example, considering the TN coverage info is used for cell reselection measurement optimization, there is no hurry for UE to get the information. The TN coverage info can be broadcast with a long periodicity to reduce the power consumption and overhead of the network.

Observation 1: In order to reduce the network power consumption and overhead, the TN coverage info can be configured with longer periodicity than SIB3/4/5.

Besides, the TN coverage info is quasi-static, but the neighbor cells info of one area varies with the movement of the satellite, so it is more suitable to set different SIB type for TN coverage info and SIB3/4/5, i.e. area-specific SIB or cell-specific SIB. For example, in case of quasi-earth fixed cell, no matter which cell provides coverage for the UE, the TN coverage around the UE doesn’t change. The network can set the TN coverage info as area-specific SIB. By this way, upon serving cell changes, the UE doesn’t require to reacquire the TN coverage info. However, upon cell changes, the neighbor NTN cells changed with high probability. Hence, it is more suitable to set the SIB3/4/5 as cell-specific SIB.

Observation 2: Considering the different characteristics of the TN and NTN neighbor cells, it is more suitable to set the SIB3/4/5 as cell-specific SIB, while set the TN coverage info as area specific SIB.

Based on observation 1 and 2, it is better to introduce a new SIB to contain the TN coverage info.

Proposal 1: Introduce a new SIB e.g. SIBx to broadcast the TN coverage info.

  - Discussion on the size of TN coverage info list

The size of the TN coverage info list mainly depends on the amount of the TN coverage area, which is up to the geographic location and network deployment. Normally, the diameter of NTN cell is quite large, e.g. 100km for LEO and 3500km for GEO. Within the NTN cell, it may be useless for UE to judge when to initiate measurement on TN cells, if the location information of TN coverage is too rough. The gain of reducing the power consuming of UE performs measurement on TN cell is marginal. Hence, the size of TN coverage info list may in level of tens.

In RAN2#122, the following agreement was achieved.

- Reuse the same format of Rel-17 referenceLocation and distanceThresh for signaling the TN coverage area centre and radius

That is, 64 bits are needed to indicate the range of one TN coverage area. Considering the max SI message size is 2976 bits, 46 TN coverage areas can be included in the SIB approximately. But if the frequency information is also provided in the new SIB, some space needs to be reserved for the related information. Meanwhile, the SIB may also be extended in the future. We should take this into account.

Based on the above analyze, we can take 16 as a start point for the maximum number of the provided TN coverage area.

Proposal 2: For the maximum number of the provided TN coverage area, 16 can be taken as a start point.

  - How to provide the frequency information

In RAN2#122, the following 3 candidate options on how to indicate the frequency for TN coverage were discussed:

- Option 1: Frequency information (i.e. a list of TN frequencies) for each TN coverage area is indicated directly under each TN coverage area configuration.

- Option 2: A TN coverage area configuration is associated with a TN coverage Area ID. The frequency information for TN coverage area is indicated by adding TN coverage area IDs in SIB4 and SIB5.

- Option 3: Frequency index bitmap is indicated under each TN coverage area, where the frequency index refers to the frequency’s position in the frequency list of the current SIB4.

And the following agreement was reached:

Agreements:

- We no longer consider option 3 alone for signaling the frequency information for TN coverage area (in case option 3 should be combined with option 1). Come back in the next meeting to decide between option 2 (plus possible fixes if needed) and option 1+3.

Between option 2 and option 1+3, we have a preference on option 1+3, because it is an intuitive way to provide the frequency information for neighbour carrier, and it has less impact on legacy UE than option 2.

For option 2, if the TN coverage info is included in a new SIB, and the TN coverage area ID is indicated in the frequency list in SIB3/4/5, the network will trigger SI update notification when the TN coverage area ID in SIB3/4/5 is updated,. All the UEs including legacy UEs will reacquire the SIB3/4/5. However, the legacy UE cannot decode the TN coverage area related information in SIB3/4/5. From their perspective, SIB3/4/5 is not changed. This brings unnecessary power consumption for legacy UE. In contrast, if option 1+3 is adopted, the frequency index is indicated in the TN area list in the new SIB, update of the TN coverage area information will not affect SIB3/4/5. Hence, option 2 should be excluded.

Observation 3: Compared with Option 1+3, Option 2 has some impact on legacy UE.

Proposal 3: Option 2 should be excluded for providing the frequency information for TN coverage area.

If frequency index bitmap is used for each TN coverage area to indicate the corresponding TN frequency information, we need to discuss where the TN frequency information is referred to. Based on the previous discussion, the following options can be taken into consideration:

- In the same SIB including the TN coverage area, i.e., SIBx.

- In the current frequency list of the SIB4/5.

In legacy, NTN cell is allowed to provide TN frequencies in SIB4/5 to help legacy UE to perform measurement on TN cells. Under the enhanced cell reselection mechanism, we can’t prevent the NW providing TN frequencies in SIB4/5 as legacy. To avoid legacy UE keeping measuring on some undetectable TN frequencies, the NTN cell may only broadcast the frequencies which are most likely to be measured, e.g. the frequency which is deployed widely. For the frequencies deployed in small concentrated areas, the network can provide them in the new SIB containing the TN coverage area.

Based on the above analyze, it is logical the TN frequencies information is provided in SIB4/5 and the new SIB which contains the TN coverage area.

Proposal 4: The TN frequency information can be provided in SIB4/5 and the new SIB including the TN coverage area information.

Based on proposal 4, the index of the frequency bitmap refers to the entry of the frequency list in SIB4/5 and the new SIB. One way is to define a bitmap whose size is the sum of the maximum frequency number in SIB4/5 and the new SIB. The presence of the frequencies in SIB4/5 and the new SIB are indicated by different bit segment.

Proposal 5: Introduce a frequency index bitmap under each TN coverage area, where the frequency index refers to the entry of the frequency list in SIB4/5 and the new SIB containing the TN coverage area.

Based on the above analyze and discussion, the draft ASN.1 structure of the new SIB is given in the Annex.

Proposal 6: Take the draft ASN.1 of the new SIB in R2-2307417 as baseline.

  - Cell reselection priority for TN neighbour frequency

In legacy, the cell reselection priority is configured per frequency. The UE decides the frequencies that need to be measured and performs cell reselection based on the frequency priority. The network may decide the frequency priority considering many factors, e.g., the signalling load.

However, under the mechanism of enhanced cell reselection based on TN coverage area info, the NTN cell needs to provide the frequency information for a large amount of TN neighbour cells. That is, one TN frequency may correspond to a large amount of TN cells, and these cells may be distributed in different areas with different load status. There is no obvious difference on the load status among different TN frequencies. Hence, it is meaningless to configure priority per frequency.

Observation 4: For the TN neighbour cells of NTN cell, one TN frequency may correspond to a large amount of TN cells, and these cells are distributed in different areas. Hence, per frequency priority is not suitable for TN neighbour frequency of NTN cell.

Since the intention of introducing TN coverage is to reduce the UE power consumption for measurement, the characteristics of TN coverage can be taken into consideration. e.g., if the probability for UE to measure TN cells in one TN area is higher, the network may configure high priority for this area. Otherwise, a low priority is configured.

Figure 1 is an example of the TN coverage area, in which the distribution of TN cells within different TN coverage areas is different. For example, the TN cells in TN area 3 are not continuously distributed, while the TN cells in TN area 2 are continuously distributed, so that the UE around TN area 3 has lower probability on finding a TN cell than the UE around TN area 2. Hence, the network can configure TN area 2 with higher priority than the serving NTN cell to make the UE keep measurement on corresponding TN cells when the UE around TN area 2, but configure TN area 3 with lower priority than the serving NTN cell to reduce the measurement on corresponding TN cells even though the UE is under the TN area 3 coverage. By this way, the UE power consumption can be further reduced.

In summary, the priority is affected by the characteristics of TN area, and the TN cell/frequency within one TN should have the same priority. Hence, TN coverage area-specific priority can be introduced.

Figure 1. Distribution of cells within TN area

Proposal 7: Introduce TN area-specific cell reselection priority for TN coverage area. The TN area-specific cell reselection priority applies to all frequencies corresponding to the TN coverage area.

  - Validity of TN coverage area information

In RAN2#121-bis, the following agreement and work assumption were achieved.

Agreements online:

- The acquired TN area coverage information remains valid until the next system information update of the SIB including TN coverage info

Working assumption:

- We do not introduce new triggers making the UE reacquire the TN coverage information from SI

About the working assumption, we have a further study.

Figure 2: Illustration on the problem of following the legacy SI update mechanism in earth moving cell.

For earth-fixed case, the TN coverage under the NTN cell coverage is static, so there is no issue to apply the work assumption. But for earth-moving case, we identified some issues. In earth moving cell, from NW’s perspective, the cell coverage on ground keep changing due to the movement of satellite. For example, as shown in Figure 2, compare with T1, at T2, the TN coverage area with green colour move in and some TN areas move out. In figure 2, within the duration the UE served by this earth moving cell, the TN coverage area in coverage of the earth moving cell changed as the satellite movement, at least 4 times updated is needed in this example. If we follow the legacy system information update notification mechanism, the UE will be notified to acquire the update SI very frequently, which may bring a lot of power consumption.

Observation 5: The TN coverage areas in coverage of the earth moving cell keep changing with the movement of the earth moving cell. Following the legacy SI update mechanism, the UE will be notified to acquire the update SI frequently.

One attempt to solve the problem is to broadcast all the TN distributions under the trajectory of the satellite, e.g. broadcast all the TN distributions in figure 2. Then, the TN coverage information doesn’t need to be updated.

However, to achieve that, the NW may need to broadcast a large amount of TN coverage areas, 16 (as mentioned in proposal 2) is far from enough. If the value 16 is finally adopted as the number of TN coverage area, the network needs to reduce the location accuracy of the TN coverage area, e.g., one TN area may need to cover the size of serval cities.

From another perspective, this may bring some discrepancy between earth moving cell and quasi-earth fixed cell. The accuracy of the TN coverage area location provided by earth-moving cell will be lower than that for quasi-earth-fixed cell. For example, shown as figure 3, the maximum number of the TN coverage area is 4. For quasi-earth-fixed cell, it broadcasts TN area1/2/3/4 and area5/6/7/8 separately. But for earth-moving cell, it needs to merge some TN areas as one (e.g., merge area2/3/4 to be one large TN area), and give up some areas (e.g., area 8). In figure 3, the earth moving cell broadcasts the 4 TN areas highlighted by green. Obviously, the accuracy of the TN coverage area location is lower than quasi-earth-fixed cell.

Figure 3: TN area broadcasted by earth-moving cell and earth-fixed cell

Even the same TN area division method is used for earth-moving and quasi- earth-fixed cell e.g. they both broadcast the TN areas highlighted by green, there will be more signaling overhead for earth-moving cell.

Anyway, the power consumption objective cannot be achieved.

Observation 6: If broadcast all the TN coverage under the whole satellite trajectory, the NW needs to broadcast more TN coverage areas or reduce the location accuracy comparing with earth-fixed cell. That is not beneficial for the power consumption of UE.

Based on the above analysis, it is better to only broadcast the TN coverage area under the current coverage of the earth moving cell, and the network has to update the TN coverage area info with the satellite movement to ensure fresh TN coverage info for UE with demand.

Observation 7: It is better to only broadcast the TN coverage areas within the earth-moving cell, and the network update the in coverage TN coverage areas to ensure fresh info for UE with demand.

After excluding the solution of broadcast the TN coverage info under the trajectory of the satellite, the issue is focus on how to avoid the impact on UE due to the frequently update. In view of UE, after receiving the broadcast TN coverage data, the UE gets acknowledge of the distribution of surrounding TN cells. As long as the UE doesn’t moves away, it can be assumed the surrounding TN cells are not changed. In this case, UE does not need to read the updated TN coverage info.

Observation 8: UE doesn’t need to re-acquire the updated TN coverage data if UE doesn’t moves away, because the surrounding TN cells are not changed.

Hence, one way to avoid the impact on the UE caused by the frequently update is the NW update the TN coverage area info but doesn’t inform UE. That is, the update of TN coverage area info will neither result in system information change notifications nor in a modification of valueTag in SIB1.

Proposal 8: The update of the TN coverage area info caused by the movement of earth-moving cell will neither result in system information change notifications nor in a modification of valueTag in SIB1.

Apart from the movement of the earth-moving cell will cause the update of TN coverage information, some other reasons will also cause the update of TN coverage information, e.g. the switch on and off of some TN cells. In these conditions, the NW is not prevented from triggering the SI modification.

Proposal 9: The NW is not prevented from triggering the SI modification if the TN coverage area information is changed. No spec impact is introduced.

If proposal 8 is agreed, whether the TN coverage area information is valid is depends on whether the surrounding TN cells of the UE is changed. Some new conditions are needed for UE to judge whether the stored TN coverage data is valid. The location of the UE and/or the time when UE acquire the SI can be taken into consideration.

Proposal 10: New trigger is needed for UE to judge whether the stored TN coverage data is valid, e.g. location-based condition or time-based condition.

- Conclusion

In this document, we discuss the enhancement on cell reselection. According to the analysis in section 2, the following observation and proposals are provided:

Which SIB to provide the TN coverage info

Observation 1: In order to reduce the network power consumption and overhead, the TN coverage info can be configured with longer periodicity than SIB3/4/5.

Observation 2: Considering the different characteristics of the TN and NTN neighbor cells, it is more suitable to set the SIB3/4/5 as cell-specific SIB, while set the TN coverage info as area specific SIB.

Proposal 1: Introduce a new SIB e.g. SIBx to broadcast the TN coverage info.

Discussion on the size of TN coverage info list

Proposal 2: For the maximum number of the provided TN coverage area, 16 can be taken as a start point.

How to provide the frequency information

Observation 3: Compared with Option 1+3, Option 2 has some impact on legacy UE.

Proposal 3: Option 2 should be excluded for providing the frequency information for TN coverage area.

Proposal 4: The TN frequency information can be provided in SIB4/5 and the new SIB including the TN coverage area information.

Proposal 5: Introduce a frequency index bitmap under each TN coverage area, where the frequency index refers to the entry of the frequency list in SIB4/5 and the new SIB containing the TN coverage area.

Proposal 6: Take the draft ASN.1 of the new SIB in R2-2307417 as baseline.

Cell reselection priority for TN neighbour frequency

Observation 4: For the TN neighbour cells of NTN cell, one TN frequency may correspond to a large amount of TN cells, and these cells are distributed in different areas. Hence, per frequency priority is not suitable for TN neighbour frequency of NTN cell.

Proposal 7: Introduce TN area-specific cell reselection priority for TN coverage area. The TN area-specific cell reselection priority applies to all frequencies corresponding to the TN coverage area.

Validity of TN coverage area information

Observation 5: The TN coverage areas in coverage of the earth moving cell keep changing with the movement of the earth moving cell. Following the legacy SI update mechanism, the UE will be notified to acquire the update SI frequently.

Observation 6: If broadcast all the TN coverage under the whole satellite trajectory, the NW needs to broadcast more TN coverage areas or reduce the location accuracy comparing with earth-fixed cell. That is not beneficial for the power consumption of UE.

Observation 7: It is better to only broadcast the TN coverage areas within the earth-moving cell, and the network update the in coverage TN coverage areas to ensure fresh info for UE with demand.

Observation 8: UE doesn’t need to re-acquire the updated TN coverage data if UE doesn’t moves away, because the surrounding TN cells are not changed.

Proposal 8: The update of the TN coverage area info caused by the movement of earth-moving cell will neither result in system information change notifications nor in a modification of valueTag in SIB1.

Proposal 9: The NW is not prevented from triggering the SI modification if the TN coverage area information is changed. No spec impact is introduced.

Proposal 10: New trigger is needed for UE to judge whether the stored TN coverage data is valid, e.g. location-based condition or time-based condition.

- Reference

- 3GPP TS 38.331 Radio Resource Control (RRC) protocol specification.

- Annex

Example for the new SIB containing the TN coverage info:

SIBx information element

-- ASN1START

-- TAG-SIBxx-START

SIBx-r18 ::= SEQUENCE {

tn-Coverage-r18                          TN-CoverageList-r18                             OPTIONAL,       -- Need R

tn-NeighCarrierFreqList-r18              TN-NeighCarrierFreqList-r18                     OPTIONAL,       -- Need R

lateNonCriticalExtension                 OCTET STRING                                    OPTIONAL,

...

}

TN-CoverageList-r18 ::=                SEQUENCE (SIZE(1..maxTNCoverage-r18))  OF TN-CoverageConfig-r18

TN-CoverageConfig-r18 ::=              SEQUENCE {

referenceLocation-r18                    ReferenceLocation-r17                           OPTIONAL,       -- Need R

distanceThresh-r18                       INTEGER(0..65525)                               OPTIONAL,       -- Need R

tn-NeighFreqList-r18                     BIT STRING (SIZE (24))                          OPTIONAL,       -- Need R

...

}

TN-NeighCarrierFreqList-r18 ::=        SEQUENCE (SIZE (1..maxFreq)) OF TN-NeighCarrierFreqInfo-r18

TN-NeighCarrierFreqInfo-r18 ::=        SEQUENCE {

dl-CarrierFreq-r18                        ARFCN-ValueNR,

...

}

-- TAG-SIBxx-STOP

-- ASN1STOP

SIBx field descriptions

tn-Coverage Provide parameters of a list of TN coverage and is used in measurement relaxation on TN neighbour cells in RRC_IDLE and RRC_INACTIVE, as defined in TS 38.304 [20].

tn-NeighCarrierFreqList List of TN neighbouring carrier frequencies and frequency specific cell re-selection information.

TN-CoverageConfig field descriptions

referenceLocation Reference location of the TN coverage area and is used in measurement relaxation on TN neighbour cells in RRC_IDLE and RRC_INACTIVE, as defined in TS 38.304 [20].

distanceThresh Distance from the reference location of the TN coverage area and is used in measurement relaxation on TN neighbour cells in RRC_IDLE and RRC_INACTIVE, as defined in TS 38.304 [20]. Each step represents 50m.

tn-NeighFreqList Bitmap to indicate the related TN neighbour cells information. The first 8 bits corresponding to the carries in SIB4, the second 8 bits corresponding to the carries in SIB5, the last 8 bits corresponding to the carries in SIBxx. If the k-th bit is set to 1, the UE assume the (k mod 8)-th entry in the frequency list in corresponding SIB is present in the TN coverage area. Otherwise, the UE assume the (k mod 8)-th entry in the frequency list in corresponding SIB is not present in the TN coverage area.