3GPP TSG-RAN WG2 Meeting #123                                                                    R2-2307418

Toulouse, France, August 21-25, 2023

Source:	CATT

Title:	Discussion on unchanged PCI scenario

Agenda Item:	7.7.4.2

Document for:	Discussion and Decision

1. Introduction

In RAN2#122, the following agreements were achieved for hard satellite switch in unchanged PCI scenario.

Agreements:

- t-Service in SIB19 can also be interpreted by Rel-18 UE in Connected mode to know that a satellite change or feeder link change happens

- In hard switch unchanged PCI scenario (i.e. no handover), the UE needs to know the time the UE attempts to re-synchronize. (FFS whether a new “t-Start” / a t-gap is needed or whether t-Service can be reused (i.e. no other IE) if the gap is very short/zero).

After the online discussion, a past offline [114] was assigned to discuss the re-synchronization and backward compatibility aspects. Based on the above agreements and the discussion in past offline [114], some leftover issues need to be further discussed, including:

- The re-synchronize time;

- UE behaviour for unchanged PCI scenario

- Assistance  information provided by NW for re-sync

- The overall stage 2 procedure

2. Discussion

2.1 Discussion on the re-synchronize time

In hard satellite switch case, the RRC_CONNECTED UE will loss DL and UL sync of the serving cell at the satellite switching time. And there is a small service interruption due to the hard switching. In RAN2#122, it was agreed the UE needs to know the time the UE attempts to re-synchronize. To achieve that, the following three options were further discussed via post email discussion, but it seems no consensus reached.

- Option 1: Introduce a t-Start for the start serving time of the target satellite

- Option 2: Introduce a t-gap to indicate the duration between the serving time of the serving satellite and the target satellite

- Option 3: Reuse t-Service

By option 1 and 2, the exact start serving time of the target satellite is provided. The UE can attempt to re-sync to the serving cell via target satellite when the target satellite starts providing service. Considering the soft satellite switch case may be support in the future release, if option 1 is supported, the t-Start can also be applicable for soft satellite switch case. Between option 1 and option 2, we have a preference on option 1. Because option 1 can express a time point which is before the t-Service (for soft satellite switch case) or /after the t-Service (for hard satellite switch case) more flexible.

Proposal 1a: If the compatibility with soft satellite switch case is considered in future, Option 1 should be supported, i.e. introduce a t-Start to indicate the time the UE attempts to re-synchronize.

But if only considering hard satellite switch case, the interruption of hard satellite switch is mainly caused by RF switching, which is less than 1 ms in general. Reuse t-Service is enough.

Proposal 1b: If only the hard satellite switch is considered, Option 3 should be supported, i.e. reuse t-Service to indicate the time the UE attempts to re-synchronize.

2.2 UE behaviour for unchanged PCI scenario

Another issue needs be discussed is, whether additional indication is needed to inform UE that at t-Service/t-Start it needs to perform re-sync apart from the new parameter introduced for re-synchronize time or the indication for the unchanged PCI scenario.

Obviously, the unchanged PCI cell doesn’t prevent to send legacy HO/CHO command to UEs which support unchanged PCI scenario. We can assume that the network could be aware whether the UE supports this feature.  If the network expects the UE perform handover to other target cell or perform intra-cell handover, the network will send a legacy HO/CHO command to the UE before t-Service. If UE receives the HO command before t-Service, the UE will perform handover to the target cell immediately. If UE receives the CHO command before t-Service, the UE will perform CHO execution when one candidate cell fulfils the execution condition before t-service. The network can also send RRCRelease message to release the UE.

Observation 1: The network can send legacy HO/CHO/RRCRelease message before t-service to UE, if the network doesn’t expect the UE supporting unchanged PCI to perform re-sync to the target satellite.

Otherwise, if the network expects the connected UE supporting the feature perform re-sync to the target cell at t-service/t-start, the network will not send legacy HO/CHO/RRCRelease message to UE. The UE supporting unchanged PCI will keep connection with the serving cell until t-Service, and it should perform re-sync to the target satellite at t-service/t-start.

Observation 2: If the network doesn’t send legacy HO/CHO/RRCRelease message before t-service to UE, it means the network expects the UE supporting unchanged PCI to perform re-sync to the target satellite at t-Service/t-Start.

Hence, no additional indication is required to inform UE that at t-service/t-start it should perform re-sync. The UE which supports the feature of unchanged PCI scenario should perform re-sync to the target satellite at t-service/t-start.

Proposal 2: The connected UEs supporting unchanged PCI scenario should perform re-sync to the target satellite since t-service/t-start, if the UEs identify the serving cell is unchanged PCI scenario.

If re-sync to the target satellite is performed, during the satellite switching period, the connection between UE and the serving cell is interrupted. To avoid failure of UL transmission, the UE is expected to stop performing uplink transmission on the serving cell since the time of t-Service. And the UE can start preforming re-sync at t-Service/t-start (based on which option is finally agreed). I.e., the UE start searching the SSB of the serving cell via target satellite to obtain DL sync. After DL sync is achieved, the UE can perform UL sync, at least via random access procedure. Once the UL sync is obtained, the uplink transmission is allowed.

Proposal 3: When the UE attempts to perform re-synchronize to target satellite, the UE shall:

- stop performing uplink transmission at t-Service

- start searching the SSB of the serving cell to obtain DL sync since t-Service/ t-start

- perform RACH procedure to the serving cell via target satellite after DL sync is obtained (FFS RACH-Less)

- recover uplink transmission after complete the RACH procedure

2.3 Assistance information provided by NW for re-sync

For UL sync, before the random access procedure, the UE shall perform TA pre-compensation by computing the RTT between UE and the RP based on the UE location, the ephemeris of the target satellite and the common TA parameters. The most straight forward way for UE to obtain the ephemeris and common TA parameters is to read SIB19 after the target satellite starts providing service. However, this method will bring a long interruption time, because the UE needs to acquire MIB, SIB1 and SIB19 in order after the target satellite starts to provide service. To reduce the interruption time, the ephemeris and TA common of the target satellite can be provided to UE in advance, i.e., via the source satellite before t-Service.

It is also beneficial for obtaining DL synch if ephemeris and TA common of the target satellite is provided. Because although the SSB pattern of the serving cell is not changed, the transmission delay difference between source satellite and target satellite will affect the receiving time of the SSB. If the ephemeris and common TA parameters of the target satellite are provided in advance, the energy consumption caused by the blind search can be reduced.

Observation 3: Providing ephemeris and TA common of the target satellite in advance can reduce the re-sync time and the energy consumption of UE.

Proposal 4: The serving cell should provide the ephemeris and TA common of the target satellite before t-Service.

Since, the ephemeris and TA common parameters of the target satellite are common to all the UEs, these parameters can be broadcast in SIB19 via the source satellite. In detail, the ephemeris and TA common parameters of the target satellite can be provided in ntn-NeighCellConfigList as the information of a neighbour cell, which has same PCI and frequency with the serving cell.

Proposal 5: The serving cell can provide the ephemeris and TA common parameters of the target satellite via ntn-NeighCellConfigList in SIB19.

2.4 The overall stage 2 procedure

Based on the achieved agreements and the discussion in section 2.1 - section 2.3, the overall procedure for hard satellite switch can be given out. The following can be taken as baseline for the running CR of TS38.300.

##### 16.14.3.2.x Unchanged PCI In quasi-earth fixed cell case, for hard satellite switch in the same SSB frequency and same gNB (no key change), satellite switching without PCI changing (not requiring L3 mobility) is supported. In hard switch unchanged PCI scenario, the UE knows the time the UE attempts to re-synchronize by SI. The overall procedure for hard satellite switching scenario illustrated in Figure 16.14.3.2.x-1: Figure 16.14.3.2.x-1: Overall procedure for hard satellite switching scenario 1. During the serving time of the source satellite, the serving cell broadcasts indications indicating the UE the serving cell is unchanged PCI cell and the time the UE attempts to re-synchronize. At the stop serving time of the source satellite, the connected UE stops UL data transmission on the serving cell and keeps the configuration. 2. At the stop serving time of the source satellite, the UE attempts to perform DL sync to the serving cell via target satellite. 3. The UE triggers CBRA/CFRA to acquire the updated TA of the serving cell via target satellite. After completing the RACH procedure, UE resume the UL data transmission on the serving cell. FFS on RACH-less

Proposal 6: Take the description of unchanged PCI in R2-2307418 as baseline for the running CR of TS38.300.

3. Conclusion

In this contribution, we have discussion on PCI unchanged scenario, with the following observations and proposals:

Discussion on the re-synchronize time

Proposal 1a: If the compatibility with soft satellite switch case is considered in future, Option 1 should be supported, i.e. introduce a t-Start to indicate the time the UE attempts to re-synchronize.

Proposal 1b: If only the hard satellite switch is considered, Option 3 should be supported, i.e. reuse t-Service to indicate the time the UE attempts to re-synchronize.

UE behaviour for unchanged PCI scenario

Observation 1: The network can send legacy HO/CHO/RRCRelease message before t-service to UE, if the network doesn’t expect the UE supporting unchanged PCI to perform re-sync to the target satellite.

Observation 2: If the network doesn’t send legacy HO/CHO/RRCRelease message before t-service to UE, it means the network expects the UE supporting unchanged PCI to perform re-sync to the target satellite at t-Service/t-Start.

Proposal 2: The connected UEs supporting unchanged PCI scenario should perform re-sync to the target satellite since t-service/t-start, if the UEs identify the serving cell is unchanged PCI scenario.

Proposal 3: When the UE attempts to perform re-synchronize to target satellite, the UE shall:

- stop performing uplink transmission at t-Service

- start searching the SSB of the serving cell to obtain DL sync since t-Service/ t-start

- perform RACH procedure to the serving cell via target satellite after DL sync is obtained (FFS RACH-Less)

- recover uplink transmission after complete the RACH procedure

Assistance information provided by NW for re-sync

Observation 3: Providing ephemeris and TA common of the target satellite in advance can reduce the re-sync time and the energy consumption of UE.

Proposal 4: The serving cell should provide the ephemeris and TA common of the target satellite before t-Service.

Proposal 5: The serving cell can provide the ephemeris and TA common parameters of the target satellite via ntn-NeighCellConfigList in SIB19.

The overall stage 2 procedure

Proposal 6: Take the description of unchanged PCI in R2-2307418 as baseline for the running CR of TS38.300.

4. References

- 3GPP TS 38.331 Radio Resource Control (RRC) protocol specification.

- RAN2#122 chair note.