3GPP TSG-RAN WG2 Meeting #123	R2-2307735

Toulouse, France, August 21-25, 2023

Source: 			ZTE Corporation, Sanechips

Title: 	On measurement reporting enhancement

Agenda item:		7.8.2

Document for:		Discussion and Decision

- Introduction

In RAN2#122[1], following agreements were reached:

Agreements

- Add height-based list of SSB-ToMeasure with corresponding height ranges and hysteresis in MeasObjectNR.  FFS on the number of height ranges

- As a basic principle, if no height-specific SSB-ToMeasure is configured for a specific height region, the legacy behaviour applies.

- For UE behavior on L1 and L3 measurement, it is left to UE implementation whether to keep/discard the old samples while UE moves to a new height region with a different SSB-ToMeasure value

- New event types will be introduced on the combination of event Ax and event Hx, at least for event A4 + event H1/H2. FFS for other event Ax + event H1/H2. FFS on details, e.g. whether to include one height threshold (H1 or H2 threshold) or a height range (both H1 and H2 threshold) in the new event, how to configure height-dependent numberOfTriggeringCells, etc.    This will be applied to all height dependent MR parameters.

- Whether UE height is included when UAV specific MR is triggered is configurable by the network.

- We will use LTE UEheight.

There are still some FFS issues as highlighted above. This contribution provides considerations and proposals on the remaining FFS issues based on the agreements reached in previous meetings.

- Discussion

  - 2.1 On events combination

It was agreed in RAN2#122 that new event type will be introduced on the combination of event Ax and event Hx, at least for event A4+event H1/H2. It is still FFS whether this new event type can be applied for other event Ax + event H1/H2.

First of all, the purpose of height-dependent MR configuration and combination of events are very similar, i.e. to trigger MR with different conditions in different heights or height regions. Since RAN2 already agreed to introduce new event types by including height threshold, the height-dependent MR parameters are then naturally supported by the new event types. An unified solution is enough for both height-dependent MR configuration and combination of events.

- Proposal 1: An unified solution, i.e. new event type, is considered to implement both height-dependent MR configuration and combination of events.

One FFS issue is whether the new event is also introduced for other event Ax + event H1/H2. RAN2 already agreed new event type will be introduced at least for A4. We think this can also be applied for event A3 and A5 + event H1/H2. One reason is that if we want to support height dependent numberOfTriggeringCells for event A3/A4/A5, we need a mechanism anyway. And it is also beneficial to support height dependent A3 and A5 threshold.

- Proposal 2: To introduce new event types on the combination of event A3/A5 and event H1/H2.

On the FFS issue whether to introduce one height threshold (H1 or H2 threshold) or a height range (both H1 and H2 threshold) in the new event, we think it is more flexible for the network to be able to configure both H1 and H2 threshold, or only one of H1 and H2 threshold. In other words, H1 and H2 threshold can be defined as optional.

- Proposal 3: The new event type can include both H1 and H2 threshold, or one of H1 and H2 threshold.

Similar to height dependent SSB-ToMeasure configuration, hysteresis should be introduced to avoid ping-pong issue when applying the measurement report configuration of the new event type. In last RAN2 meeting, there is proposal to introduce some timers to address ping-pong issue. However, we think hysteresis is enough. Note that TTT is applied for the entering and leaving condition of the new events, which acts similar to the proposed timer.

- Proposal 4: To introduce hysteresis for the new event types along with H1/H2 threshold, i.e. similar to the hysteresis introduced for height-dependent SSB-ToMeasure configuration.

As for the height dependent numberOfTriggerCells, it is naturally supported if it is included in the event types. However, numberOfTriggeringCells should also be supported for legacy A3/A4/A5 event, so it is preferred to include numberOfTriggeringCells IE in EventTriggerConfig, but not in the new event.

- Proposal 5: The new introduced numberOfTriggeringCells is included in IE EventTriggerConfig, i.e. similar to LTE.

Following is an example of the definition of the new event types.

EventTriggerConfig::=                       SEQUENCE {

eventId                                     CHOICE {

<skipped part>

eventA3-new-r18                                SEQUENCE {

a3-Offset-r18                                   MeasTriggerQuantityOffset,

reportOnLeave-r18                               BOOLEAN,

hysteresis-r18                                  Hysteresis,

timeToTrigger-r18                               TimeToTrigger,

useAllowedCellList-r18                          BOOLEAN,

height-range {

h1-thresholdOffset-r18                          INTEGER (0..FFS)        OPTIONAL,

h2-thresholdOffset-r18                          INTEGER (0..FFS)        OPTIONAL,

h-hysteresis-r18                                 INTEGER (0..FFS)        OPTIONAL

}

},

eventA4-new-r18                                 SEQUENCE {

a4-Threshold-r18                                MeasTriggerQuantity,

reportOnLeave-r18                               BOOLEAN,

hysteresis-r18                                  Hysteresis,

timeToTrigger-r18                               TimeToTrigger,

useAllowedCellList-r18                          BOOLEAN

height-range {

h1-thresholdOffset-r18                          INTEGER (0..FFS)        OPTIONAL,

h2-thresholdOffset-r18                          INTEGER (0..FFS)        OPTIONAL,

h-hysteresis-r18                                 INTEGER (0..FFS)        OPTIONAL

}

},

eventA5-new-r18                                 SEQUENCE {

a5-Threshold1-r18                               MeasTriggerQuantity,

a5-Threshold2-r18                               MeasTriggerQuantity,

reportOnLeave-r18                               BOOLEAN,

hysteresis-r18                                  Hysteresis,

timeToTrigger-r18                               TimeToTrigger,

useAllowedCellList-r18                          BOOLEAN

height-range {

h1-thresholdOffset-r18                          INTEGER (0..FFS)        OPTIONAL,

h2-thresholdOffset-r18                          INTEGER (0..FFS)        OPTIONAL,

h-hysteresis-r18                                 INTEGER (0..FFS)        OPTIONAL

}

},

<skipped part>

},

<skipped part>

[[

numberOfTriggeringCells-r18                       INTEGER (2..FFS)        OPTIONAL,   -- Need R

]]

}

  - 2.2 On content of the measurement report triggered by event H1 or H2

- In NR, for each measurement report triggered by event, the UE shall set the quantities results of concerned cells in the measurement report. For UAV UE, including the RSRP/RSRQ/SINR measurement results in the measurement report shall increase the report size. And considering that more frequent measurement report may be triggered in aerial UE’s environment, it may cause larger interference. Thus, it is agreed in last RAN2 meeting that “When event H1 or H2 triggers, the content of the measurement report is configurable by the network (i.e. it can contain UAV UEs height, location information and/or RSRP/RSRQ measurement results).”

- An new indication (e.g. includeQuantities IE) can be introduced in the ReportConfigNR IE to indicate whether the RSRP/RSRQ/SINR measurement results shall be included in the measurement report triggered by event H1 or H2. The indication can be introduced by a SEQUENCE structure to separately indicate which quantities results are required in the measurement report.

- Proposal 6: An new indication (e.g. includeQuantities IE) is introduced in the ReportConfigNR to indicate whether the RSRP/RSRQ/SINR measurement result is required to be included in the measurement report triggered by event H1 or H2. The indication can be introduced by a SEQUENCE structure to separately indicate which quantities results are required in the measurement report.

- Currently in NR, the NW can configure includeCommonLocationInfo IE in ReportConfigNR to indicate whether to include the location information in the measurement report.

From TS 38.331 - 5.5.5 Measurement reporting - 5.5.5.1 General 1> if the includeCommonLocationInfo is configured in the corresponding reportConfig for this measId and detailed location information that has not been reported is available, set the content of commonLocationInfo of the locationInfo as follows: 2> include the locationTimestamp; 2> include the locationCoordinate, if available; 2> include the velocityEstimate, if available; 2> include the locationError, if available; 2> include the locationSource, if available; 2> if available, include the gnss-TOD-msec,

The parameters from CommonLocationInfo are related to location info, location time and velocity info, which could be useful for UAV UE. We think the current indication (i.e. includeCommonLocationInfo IE) can be reused for measurement report triggered by event H1 or H2. If any other parameters are required for UAV UE’s location reporting, the requirement should be firstly evaluated on SA2. And then we can consider the signaling design at RAN2.

- Proposal 7: The includeCommonLocationInfo IE in the ReportConfigNR is reused to indicate whether the location information (if available) is required to be included in the measurement report triggered by event H1 or H2.

- Proposal 8: The parameters in the CommonLocationInfo IE are reused for UAV UEs’ location reporting.

- Conclusion and proposals

- On event combination

- Proposal 1: An unified solution, i.e. new event type, is considered to implement both height-dependent MR configuration and combination of events.

- Proposal 2: To introduce new event types on the combination of event A3/A5 and event H1/H2.

- Proposal 3: The new event type can include both H1 and H2 threshold, or one of H1 and H2 threshold.

- Proposal 4: To introduce hysteresis for the new event types along with H1/H2 threshold, i.e. similar to the hysteresis introduced for height-dependent SSB-ToMeasure configuration.

- Proposal 5: The new introduced numberOfTriggeringCells is included in IE EventTriggerConfig, i.e. similar to LTE.

- On content of the measurement report triggered by event H1 or H2

- Proposal 6: An new indication (e.g. includeQuantities IE) is introduced in the ReportConfigNR to indicate whether the RSRP/RSRQ/SINR measurement result is required to be included in the measurement report triggered by event H1 or H2. The indication can be introduced by a SEQUENCE structure to separately indicate which quantities results are required in the measurement report.

- Proposal 7: The includeCommonLocationInfo IE in the ReportConfigNR is reused to indicate whether the location information (if available) is required to be included in the measurement report triggered by event H1 or H2.

- Proposal 8: The parameters in the CommonLocationInfo IE are reused for UAV UEs’ location reporting.

- Reference

- RAN2#122 chairman notes