3GPP TSG-RAN WG2 Meeting #123	R2- 2308006

Toulouse, France, 21st – 25th August, 2023

Agenda Item:	7.12.2.1

Source: 	Lenovo

Title: 	Mobility enhancements for mobile IAB-node and its connected UE

Document for:	Discussion and decision

- Introduction

A new WID for mobile IAB-node [1] has been approved to be discussed in Rel-18, and following detail objectives are identified mainly focus on RAN3 and RAN2.

- Define Procedures for migration/topology adaptation to enable IAB-node mobility, including inter-donor migration of the entire mobile IAB-node (full migration) [RAN3, RAN2] - The mobile IAB-node can connect to a stationary (intermediate) IAB-node. Optimizations specific to the scenarios, where the mobile IAB-node connects to a stationary (intermediate) IAB-node, or where it directly connects to an IAB-donor-DU are de-prioritized. - The mobility of dual-connected IAB-nodes is down-prioritized. - Enhancements for mobility of an IAB-node together with its served UEs, including aspects related to group mobility. No optimizations for the targeting of surrounding UEs. [RAN3, RAN2] Note: Solutions should avoid touching upon topics where Rel-17 discussions already occurred and where the topic was excluded from Rel-17, except for enhancements that are specific to IAB-node mobility. - Mitigation of interference due to IAB-node mobility, including the avoidance of potential reference and control signal collisions (e.g. PCI, RACH). [RAN3, RAN2]

In this contribution, mobility enhancements for mobile IAB-node and its connected UEs are discussed, including aspects related to CHO, RACH-less HO, and some other aspects.

- Discussion

Conditional handover for served UEs

Agreements from RAN2 119bis-e: 1) message withholding by the logical source IAB-DU with conditional delivery, e.g., upon on MT migration, 2) conditional execution by the UE based on, e.g., a broadcast indication such as SIB indication of service time or DCI indication of MT-migration, (includes CHO with new trigger). 3) legacy CHO (with implementation specific behaviour, e.g. using source-cell power down or target cell power up triggering the actual HO) RAN2 assumes that O1 and O3 above could work, and FFS if O2 above (new trigger etc) is needed.

For Option 1, RRC Reconfiguration messages are firstly sent to source IAB-DU of mobile IAB-node, and the source IAB-DU withholds the RRC Reconfiguration messages until some specific conditions are met and then delivers the RRC Reconfiguration messages to each connected UEs. Because similar delay RRC configuration procedure has been defined in RAN3 during R17 for reconfiguration of descendent nodes during intra-donor migration, and the Option 1 has no impacts on legacy UEs. However, for the deliver condition, MT migration may not feasible as defined in RAN3 that IAB-MT handover and IAB-DU migration is executed independently.

For Option 2, because it’s not supported by the legacy UEs, then this option can be deprioritized.

For Option 3, legacy CHO configuration can be fully reused for UEs, and source IAB-DU can power down the cells by implementation to trigger the served UEs perform CHO to target IAB-DU, which has fewest standards impacts among 3 options. While for power up the cells of target IAB-DU, it’s not recommended that it causes additional unnecessary HO/CHO on other UEs.

Proposal 1: Legacy CHO procedure can be reused by the served UEs of mobile IAB-node, and source IAB-DU powers down the cells by implementation to trigger the CHO.

RACH-less HO for served UEs

Agreements from RAN2 122: RAN2 think that to have a fast handover from UE point of view for legacy UEs it is important that the target cell is known to the UE (detected and measured). For RACH-less, if supported, there would need to be a beam indication (in RRC HO command), which seems feasible in this release from R2 perspective. R2 assumes that the network can know/select the beam, either from network impl specific knowledge or from UE measurement report (legacy report). for the UL grant and HO completion in RACH-less HO: - 1. Both type-1 configured grant and dynamic grant are supported - 2. FFS handling of supervision timer and when HO is considered successfully complete (expect to align with other WI). Send LS to RAN3 to check whether there are issues / feasibility concerns

As for the served UEs of mobile IAB-node, since source logical IAB-DU and target logical IAB-DU are co-located physically and the serving mobile IAB-node doesn’t change for UE during the whole IAB-DU migration procedure, at least the same TA is applied before and after mobile IAB-MT handover. And in general, RACH-less HO can be considered for UEs of mobile IAB and LTE RACH-less HO procedure can be regarded as a start point. Based on the agreement from last meeting, a fast handover for the served UEs of mobile IAB-node can be considered.

Proposal 2: Support RACH-less HO for the served UEs during mIAB-DU migration.

Since the RACH-less handover is discussed in other WID (NTN) parallelly and NTN makes faster progress than mIAB, then the agreements made by NTN as below can be partial reused.

Agreements from RAN2 121bis-e: - RAN2 confirms the general UE procedure for NTN RACH-less HO - receive a RACH-less HO command which can include pre-allocated grant optionally. FFS N_TA is optional. (RRC) - start timer T304 for the target cell (RRC) - perform DL and UL synchronization, and start timer T430. FFS how to perform RACH-less UL synchronization to NTN target cell. (RRC, MAC) - start time alignment timer (MAC) - monitor target cell PDCCH for dynamic grant if pre-allocated grant is not configured in RACH-less HO command (MAC, PHY) - send initial UL transmission including RRCReconfigurationComplete message using the available UL grant (RRC, MAC, PHY) - consider RACH-less HO is completed upon receiving NW confirmation. FFS how to confirm RACH-less HO is successfully completed. (RRC, MAC) - stop timer T304 for the target cell. (RRC) FFS whether to release UL grant if pre-allocated after RACH-less HO completion FFS RACH-less HO failure handling, e.g. whether UE fallback to RACH-based HO to the target cell FFS procedure for RACH-less HO combined with PCI unchanged or CHO if supported - The pre-allocated grant is provided as type-1 CG - At least for pre-allocated grant, for the confirmation of RACH-less HO completion we reuse of LTE approach, i.e., UE Contention Resolution Identity MAC CE is used but UE ignores the content of this field. FFS if anything else is needed for dynamic grant. Agreements from RAN2 122: - From RAN2 perspective synchronization among source and target cells is not an issue in NTN RACH-less HO - Release pre-allocated UL grant after RACH-less HO completion - LTE approach (of confirming the HO completion) is reused for both pre-allocated grant and dynamic grant. FFS any enhancement to the confirmation of RACH-less HO completion, e.g. the NW does not send the UE Contention Resolution Identity MAC CE, and sends PDCCH/PDSCH addressed to C-RNTI - Remove “FFS how to perform RACH-less UL synchronization to NTN target cell”, RAN2 assumes the UL sync handling in the target cell is the same in RACH-based HO and RACH-less HO, except how to acquire NTA (FFS on the spec impact , if any)

For confirming the HO completion, the LTE approach can be reused for both pre-allocated grant and dynamic grant as baseline as agreed in NTN, where NW sends the UE Contention Resolution Identity MAC CE to UE, but UE ignores the content of this field. And whether to introduce any enhancements for confirming HO completion can wait further progress from NTN.

Proposal 3: LTE approach of confirming the HO completion is reused for both pre-allocated grant and dynamic grant as baseline, where NW sends the UE Contention Resolution Identity MAC CE to UE, but UE ignores the content of this field.

As for the handling of RACH-less HO failure, as legacy, UE will initiate RRC re-establishment procedure when T304 expiries.

Proposal 4: For handle RACH-less HO failure, UE will initiate RRC re-establishment procedure when T304 expiries.

On-board UE status identification

RAN2 has agreed that RACH-less procedure may be considered for on-board RRC_CONNECTED UEs, which are to be handed over together with the mobile IAB-node. It means that the target IAB-donor-CU needs to be aware of the on-board/surrounding status of handover UE and may needs to make some optimizations for the on-board UE than the surrounding UE. Therefore, on-board/surrounding status of UE can be included in the handover request message.

Proposal 5: In case of UE handover during IAB-DU migration, to include on-board/surrounding status of UE in the handover request message.

In addition, for the on-board/surrounding status, all onboard UEs needs to perform migration together with the mobile IAB-node. While for the surrounding UEs camping on the mobile IAB-node, migration together with the mobile IAB-node is not an optimal choice. Once the surrounding UEs perform migration together with the mobile IAB-node, the measurement result of serving cell become worse gradually due to the mobility of mobile IAB-node, and the surrounding UEs may need to perform handover again within a short time.

In addition, as specified in the WID, no optimizations are for the targeting of surrounding UEs, which means all enhancements related to group mobility are only considered for the onboard UEs. In addition, we have agreed that RACH-less handover may be only considered for on-board UEs rather than surrounding UEs. Therefore, it’s necessary to differentiate onboard UEs and surrounding UEs for IAB-donor. And no matter how UE determine whether it’s in the status of onboard or surrounding, UE needs to report its onboard/surrounding status to IAB-donor once it determines the status or changes the status.

Proposal 6: UE reports its onboard/surrounding status to IAB-donor once it determines the status or changes the status.

Fixed IAB-node handover

For the mobile IAB-node, it should have no descendent IAB-nodes and only serve for UEs as defined in the WID. In RAN2 119 e-meeting, RAN2 has agreed that the method of not broadcasting “iab-Support” indication is sufficient to prevent other IAB-node from accessing mobile IAB-node. However, the “iab-Support” indication is only applied for IAB-node integration procedure, it needs to avoid that a source IAB-donor-CU to configure measurement to an IAB-node with a potential cell belonging to a mobile IAB-node, and the source IAB-donor-CU to trigger handover of the IAB-node to the mobile IAB-node cell. Therefore, the source IAB-donor-CU of a fixed IAB-node should be aware of whether the potential target cell belongs to a mobile IAB-node.

Proposal 7: Source IAB-donor-CU of a fixed IAB-node should be aware of whether the potential target cell belongs to a mobile IAB-node.

# Conclusion

This contribution aims to discuss enhancements for mobility of an IAB-node together with its served UEs. And following observations and proposals are concluded.

Proposal 1: Legacy CHO procedure can be reused by the served UEs of mobile IAB-node, and source IAB-DU powers down the cells by implementation to trigger the CHO.

Proposal 2: Support RACH-less HO for the served UEs during mIAB-DU migration.

Proposal 3: LTE approach of confirming the HO completion is reused for both pre-allocated grant and dynamic grant as baseline, where NW sends the UE Contention Resolution Identity MAC CE to UE, but UE ignores the content of this field.

Proposal 4: For handle RACH-less HO failure, UE will initiate RRC re-establishment procedure when T304 expiries.

Proposal 5: In case of UE handover during IAB-DU migration, to include on-board/surrounding status of UE in the handover request message.

Proposal 6: UE reports its onboard/surrounding status to IAB-donor once it determines the status or changes the status.

Proposal 7: Source IAB-donor-CU of a fixed IAB-node should be aware of whether the potential target cell belongs to a mobile IAB-node.

# Reference

- RP-222671. Revised WID for NR_mobile_IAB.

- Chairman notes of RAN2 119bis-e – 122 meetings.