3GPP TSG-RAN WG2 Meeting #123  	                            R2-2308159

Toulouse, France, August 21-25, 2023							                (Revision of R2-2305518)

Agenda Item:	7.7.4.2

Source: 	Sony

Title:	Signaling overhead reduction during NTN-NTN HOs

Document for:	Discussion

# Introduction

We think that the benefit of time/location based trigger can be realised only if signalling overheads are managed properly during handover. In this contribution, we observe that a major chunk of signalling takes place during HO preparation phase for a normal HO and could be a good candidate for signalling overhead reduction.

# Discussion

2.1 Signalling overhead during handovers

All UEs in an NTN cell are expected to experience a change of cell due to new satellite taking over every few seconds in a LEO constellation. Normally networks are not dimensioned to handle handover of all connected UEs in a cell and that too within a short period of time.

A significant chunk of HO or CHO signalling takes place during the Handover Preparation phase as shown with the red circle in the figure below. Handover preparation signalling takes place between network nodes without any UE involvement. Most handovers are predictable in NTN because these occur due to the movement of cells and at regular intervals. Only a very small number of handovers are expected to be due to UE mobility, as it was agreed during RAN2_119bise:

1. RAN2 confirms that at least for the moving cell case the next serving cells can be largely predicted in NTN (at least for UEs not at the cell edge) thanks to the existence of predefined satellite orbits and negligible UE’s mobility in comparison to satellite’s motion (we can further discuss at the next meeting whether this applies to idle mode UEs as well).

For CHO, currently two cells can be configured during the HO preparation phase. If more than two cells are prepared in advance, then HO preparation signalling overhead can be shifted from the time when actual handover takes place.

Figure 1: CHO signalling procedure

Observation: A significant chunk of HO signalling takes place during HO preparation phase and HOs are mainly due to cell movement. The oncoming cells are expected to support all UEs in a geographical area i.e. resource reservation is mainly to cater the traffic emanating from a geographical area and different from the scenario where resources are reserved due to unpredictable movement of UEs in Terrestrial Networks.

The resource reservation in target cells can be left to gNB implementation. It is only a matter of time that this new cell will anyway be prepared to handle the current load from a fixed location. In the absence of preconfiguration of multiple target cells, every few seconds full HO signalling needs to be exchanged for each connected UE in a cell. Majority of signalling can be reduced by configuring multiple candidate cells and also not deleting the CHO configuration after CHO has been executed (so that CHO configuration is still valid for next cell taking over without any explicit signalling).

CHO configuration is possible only after SRB2 has been setup and AS security is activated with at least one DRB. We think that handover command and radio bearer setup can be combined in the same RRC reconfiguration message to include both DRB setup and HO command.

Providing CHO configuration during Radio bearer setup can further randomize the UEs handover preparation signalling and also optimising the signalling overhead beyond HO preparation phase, hence reducing the network/signalling load especially when a large number of UEs handover to the same target satellite cell.

Proposal 1: Multiple target cells are included in the RRC reconfiguration message when AS security has been activated and during DRB setup.

# Conclusion

We propose RAN2 to discuss and agree following observation and proposals:

Observation: A significant chunk of HO signalling takes place during HO preparation phase and HOs are mainly due to cell movement. The oncoming cells are expected to support all UEs in a geographical area i.e. resource reservation is mainly to cater the traffic emanating from a geographical area and different from the scenario where resources are reserved due to unpredictable movement of UEs in Terrestrial Networks.

Proposal 1: Multiple target cells are included in the RRC reconfiguration message when AS security has been activated and during DRB setup.