3GPP TSG-RAN WG2 Meeting #123														R2-2308171

Toulouse, France, August 21-25, 2023

Agenda Item:	7.18.3

Source: 	SONY

Title:	User plane aspects for MT-SDT

Document for:	Discussion and Decision

# Introduction

In the past few meetings, RAN2 discussed the Mobile Terminated-Small Data Transmission (MT-SDT) for Rel-18 UEs in RRC_INACTIVE state, and the following agreements were reached: RAN2#120 [2], RAN2#121 [3] and RAN2#122 [4]:

Agreements at RAN2#120:

- For RAN paging, MT-SDT indication (at least one bit) is explicitly included per UE via a paging message. FFS if more information for MT-SDT are needed FFS what the indication will be called. FFS signalling details.

- Rel-18 MT-SDT after the MT-SDT paging trigger is detected, RA-SDT and CG SDT solutions/procedures specified in Rel-17 is re-used as a baseline. The detailed triggers will be discussed on case by case. FFS on resources used for access.

- UE can use non-SDT random access resources for accessing the network for an MT-SDT transfer. The UE can also use the configured grant resources and/or MO-RA resources.

- The network should be able to differentiate why the UL access was triggered, i.e., implicit or explicit indication by the UE.

- MT-SDT is data that belongs to bearers that are configured for SDT. FFS whether the configuration is MO-SDT or MT-SDT specific. The network can only trigger MT-SDT if the data belongs to those bearers.

- It is possible for the network to configure only MT-SDT without MO-SDT RA resources and/or CG-SDT. Subsequent UL/DL data belonging to SDT bearers while in INACTIVE is allowed like MO-SDT procedure. FFS stage 3 details.

- New Resume cause in RRC resume will be introduced, one code point MT-SDT indication.

Agreement at RAN2#121:

- Include a one-bit indication in paging to trigger MT-SDT. We will ensure that the CCCH message can be transmitted over CG.

- Indication is per UE. FFS on signalling.

- In case condition for paging triggered MT-SDT is not fulfilled the UE initiates RRC Resume procedure. Resume cause FFS.

- Upon receiving MT-SDT trigger, the UE shall initiate SDT procedure if the following checks are satisfied (all these same as Rel-17)

-     FFS 3a: Check for DVT (if UL data becomes available in UL)

-     3b: Check for SDT RSRP threshold

-     3c: Check for TA validation before selecting CG (if applicable)

-     3d: Check for SSB level RSRP threshold for CG resource (if applicable)

5.   When UE resumes for MT-SDT, UE resumes all RBs configured for SDT

6.  RBs configured for SDT are common for MO-SDT and MT-SDT

7.  If there is valid CG-SDT resources, the UE should use CG-SDT to transmit the response. FFS on whether we need to optimize for case when CG periodicity is too long.

8. To confirm that when SDT is initiated due to MT-SDT, UE can exchange subsequent DL/UL SDT data on the resumed RBs. This clarifies the RB behaviour of related RAN2#120 agreement.

Agreements at RAN2#121:

- Specify a RRC procedure for RRCResume for MT-SDT initiation without checking for availability of UL data (i.e., if MT-SDT is initiated first the resume cause will be set to MT-SDT).

- UE is allowed to initiate either MO-SDT based resume or non-SDT based resume at any point (before initiation RRCResumeRequest for MT-SDT) using separate procedures.

- If MT-SDT procedure is initiated, for RACH during subsequent data transfer (i.e., RACH triggered due to SR), UE uses only the non-SDT RACH resources (i.e., like legacy).

Agreement at RAN2#122:

- Allow support of only MT-SDT in a cell. A separate SIB configuration will be introduced. FFS what is put in there.

- For paging indication signalling, a new list of paging records for MT-SDT indication is optionally included in paging message using non-critical extension. Each record in this list optionally includes 1 bit MT-SDT indication. UE identity and access type are not included in paging record of this list.

- gNB may include MT-SDT indication in paging message only if UE’s I-RNTI is included in the paging message (i.e., MT-SDT is only used by RAN initiated paging).

- UE selects '0' as the Access Category when the resumption of the RRC connection is triggered by response to the MT-SDT triggering in a PAGING message.

- MT-SDT is only applicable to the legacy MT-Access use case (i.e., it is not applicable to access identities 1, 2 and 11-15).

- SRB2 can be used for MT-SDT (i.e., similar to MO-SDT).

- No additional enhancement is needed specifically for RedCap UE to monitor paging for MT-SDT.

- When RRC resume is triggered due to MT-SDT and in the case the condition for paging triggered MT-SDT is not fulfilled, the UE initiates RRC Resume procedure with Resume cause “mt-Access”.

- A separate sdt-RSRP threshold for MT-SDT can be configured, at least in the case where MO-SDT is not configured in the cell.

Agreement at RAN2#122:

- RRC explicitly indicates to MAC whether resume is trigged due to MT-SDT.

- LCH restrictions are checked for DRBs as in MO-SDT (if UL data is available during SDT procedure).  Ensure CCCH can be transmitted in CG-SDT when MT-SDT is triggered in stage 3 discussion.

- Assumption is that if the UE has UL data the UE can still check and trigger MO-SDT (it is up to UE implementation)

- RA-SDT resources are not used for MT-SDT initiation RACH.

- In case CG-SDT resources cannot be used or are not available for MT-SDT, UE uses non-SDT RACH for RA-based MT-SDT.  FFS whether new triggers are defined.

- There is no need to define new Rel-18 CG configurations specific to MT-SDT.

- When the UE is configured with both MO and MT SDT the radio bearer configuration is common for both.

- For both MO and MT-SDT, if the next CG-SDT resource is too far, then RACH resource can be selected first. This is checked at the point of initial resource selection (e.g., CG SDT selection). FFS what is too far and how this is configured. Assumption is that we will continue this discussion in SDT session.

In this contribution, we discuss some remaining issues of user plane aspects for MT-SDT procedures.

## User plane aspects for MT-SDT

In the last meeting, there was no consensus to support SPS for MT-SDT. Hence, only dynamic scheduling of the downlink data is supported for MT-SDT. For dynamic scheduling, it is beneficial to support the initial DL data delivery at message Msg4/MsgB to speed up data reception, because at this point, a UE has been identified and security info are established between UE and the network. In this case, initial DL data can be received together with Msg4/MsgB contention resolution, whilst subsequent DL/UL data can be dynamically scheduled afterwards.

Proposal 1: For MT-SDT, initial DL data can be received together with Msg4/MsgB contention resolution, whilst subsequent DL/UL data can be dynamically scheduled afterwards.

PUCCH resource for MT-SDT: In SDT, a UE can receive and transmit multiple UL and DL packets as part of the same SDT session by monitoring the PDCCH without transitioning to RRC_CONNECTED. Hence, when the UE receives DL PDSCH carrying MT-SDT, an acknowledgement feedback (HARQ-ACK carried on PUCCH) would be necessary to inform the network whether the DL SDT is successfully decoded or not. In order to handle PUCCH capacity isssues for Inactive state UEs, we think separate PUCCH resource for SDT should be configured for the UE when moving to Inactive state, that is in the RRC Release messge.

Proposal 2: Separate PUCCH resource for SDT should be configured for the UE when moving to Inactive state, that is in the RRC Release messge.

BWP for SDT: The basic assumption is that legacy initial DL BWP would be used for MT-SDT. However, the capacity of initial DL BWP (i.e., the size of CORESET 0) is very limited; therefore, it needs to be discussed in RAN2 whether the size can be extended for SDT UEs. For MBS, there is already extended size of initial BWP known as common frequency resource (CFR). We think the same CFR can also be used for SDT UEs to receive and transmit data without moving to connected state. In this case, the specification impact of CFR for SDT would be very limited as it is already supported in Rel-17 for MBS.

Proposal 3: RAN2 to discuss whether initial DL BWP for SDT can be extended, similar to common frequency resource (CFR) for MBS.

## Conclusions

In this contribution, we have discussed some remaining issues of user plane aspects for MT-SDT procedures. We have the following proposals:

Proposal 1: For MT-SDT, initial DL data can be received together with Msg4/MsgB contention resolution, whilst subsequent DL data reception can be dynamically scheduled.

Proposal 2: Separate PUCCH resource for SDT should be configured for the UE when moving to Inactive state, that is in the RRC Release messge.

Proposal 3: RAN2 to discuss whether initial DL BWP for SDT can be extended, similar to common frequency resource (CFR) for MBS.

## References

[1] RP-213583, “New WI: Mobile Terminated-Small Data Transmission (MT-SDT) for NR,” RAN#94e.

[2] Minutes RAN2#120

[3] Minutes RAN2#121

[4] Minutes RAN2#122