3GPP TSG-RAN WG2 Meeting #123	R2-2308228

Toulouse, France, 21 – 25 August 2023

Agenda item:	7.6.2.1

Source:	Nokia, Nokia Shanghai Bell

Title:	On HARQ enhancements for IoT NTN

WID/SID:	IoT_NTN_enh-Core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

At RAN2#119bis and RAN2#120 meetings, RAN2 discussed the HARQ enhancements on aspects including the signalling to support HARQ feedback enabling/disabling and HARQ mode A/B, the DRX impact for NB-IoT with single HARQ process, as well as LCP impact for eMTC NTN etc.

At RAN2#121bis meeting, the relevant issues were further discussed and most HARQ enhancements depend on RAN1 progress, therefore one LS were sent from RAN2 to RAN1[R2-2304274]. In RAN1 #113 meeting, RAN1 discussed the questions  in the LS and reply one LS on the answers to RAN2 [R1-2306182].In addition, RAN1 also sends one  LS to RAN2 on NPDCCH monitoring restriction for NB-IoT NTN[R1-2306245].  Moreover, the following issues are left to be further addressed by RAN2:

- For the UL HARQ mode signalling, whether both RRC configuration and DCI-based solution should be supported.

- Whether UL transmission using PUR can be configured with HARQ mode B

- For eMTC NTN, whether it can be left to eNB’s implementation to configure either HARQ mode A or HARQ mode B for all HARQ process (or no HARQ mode) if mpdcch-UL-HARQ-ACK-FeedbackConfig is configured

- For eMTC NTN, whether to enhance the TAR MAC CE transmission to avoid or mitigate the outdated TA and Koffset impact in Rel-18.

In this contribution, we further discuss the open issues in RAN2 based on the latest RAN1 and RAN2 agreements.

# 2	Discussions

## 2.1	Signalling for UL HARQ mode A/B and DL HARQ feedback disabling/enabling

### 2.1.1 Signalling for UL HARQ mode A/B

Only DL HARQ feedback disabling was discussed in RAN1 meetings while the signalling to support HARQ mode A and HARQ mode B in UL has not been discussed yet. For DL HARQ operation, the motivation to support DL HARQ feedback disabling is that it can avoid HARQ stalling to improve DL throughput while HARQ feedback enabling can not only achieve high reliability but also provide HARQ feedback for some important MAC CE activation and link adaptation.  Disabling HARQ process feedback for the “important” PDSCH carrying the MAC CE may impact the system operation since NW has no idea whether the MAC CE was transmitted successfully or not. In the single HARQ process case for NB-IoT, if only RRC based solution is supported to switch the HARQ feedback disabling/enabling via RRC reconfiguration message, it will result in not only significant signalling overhead and latency but also the reduced the system performance. Therefore, in RAN1 agreement, DCI-based HARQ feedback enabling/disabling is introduced on top of RRC-based solution for fast HARQ feedback enabling/disabling switch/indication for NB-IoT and eMTC CE mode B in which only small number of HARQ processes are supported.

For UL, NW may configure HARQ mode B for data transmission to avoid HARQ stalling while configure HARQ mode A for high reliability transmission. To minimize the efforts on the discussion and standardization impact, one unified solution on the signalling of HARQ mode A/B for UL and HARQ feedback enabling/disabling for DL is preferred. However, there are some differences between DL and UL for IoT NTN.

For UL, it is up to the NW implementation to schedule the UL grant with new transmission or retransmission before/after decoding previous uplink transmissions, and the UE can only follow the scheduling decision of the NW. For NB-IoT, due to LCP is not supported, no matter HARQ mode A or B is configured for a HARQ process, the transmission reliability for a LCH or MAC CE maybe same no matter the NW schedule the blind retx (as if in HARQ mode B) or the new transmission /retransmission based on the UL decoding results (as if in HARQ modeA). Additionally, the NW may not know whether there is MAC CE for transmission in this uplink HARQ process, thus can’t determine whether to schedule HARQ mode A in this HARQ process. Therefore, for NB-IoT NTN, it seems RRC-based HARQ mode A/B indication is enough.

Proposal 1: For NB-IoT NTN, support RRC-based UL HARQ mode A/B indication is sufficient.

For eMTC NTN, it was agreed in RAN2#119e that HARQ mode-based LCP is supported.

RAN2 Agreement: The solutions of LCP restriction on allowed HARQ mode in NR NTN can be reused for eMTC NTN

With HARQ mode related LCP restriction, the NW can configure the HARQ mode (e.g., HARQ mode A or HARQ mode B) to a UL HARQ process, additionally the NW could configure the preferred HARQ mode to a logical channel. In the data multiplexing, the LCH data is mapped only to a HARQ process configured with the same HARQ mode. Due to at most 2 HARQ processes are supported for eMTC CE mode B if the UE is not configured with higher layer parameter ce-PUSCH-MultiTB-Config, the RRC-based HARQ mode indication may bring some restriction on the scheduling flexibility. For example, assuming UE is configured with one HARQ process with HARQ mode B to avoid HARQ stalling and one HARQ process is configured with HARQ mode A for reliability,if new data arrived for transmission from one LCH configured with HARQ mode A but the HARQ processes configured with HARQ mode A is occupied for data transmission and the HARQ process configured with HARQ mode B is in idle,  the NW can’t schedule this idle HARQ processes for data transmission due to the unmatched HARQ mode between the HARQ processes and the LCH. In this case, DCI-based HARQ mode indication can bring more scheduling flexibility via changing the HARQ mode directly in DCI grant instead of changing the HARQ mode via RRC first.

So RAN2 can have further discussion on whether DCI-based HARQ mode indication is supported for eMTC CE mode B for uplink.

Observation 1: For eMTC CE Mode B, fast UL HARQ mode indication/switch via DCI can provide more scheduling flexibility.

Proposal 2: RAN2 to further discuss whether DCI-based UL HARQ mode indication is supported for eMTC NTN CE mode B.

### 2.1.2 Signalling for DL HARQ feedback disabling/enabling

In RAN1 meetings, there are the following agreements on RRC based(option1) and DCI based(option3) HARQ feedback disabling/enabling:

Agreement in RAN1#112bis: For Option 3 DCI indication: - Option A: when both per-HARQ process bitmap and DCI solution enabling/disabling signaling are configured - DCI-based overridden mechanism is DCI signaling to reverse the HARQ feedback enable/disable for the corresponding transmission from per-HARQ process RRC configuration - For single TB scheduled by DCI, the DCI based overridden indication is applied to one of the following options (to be down-selected): - Option A-1: only applied to semi-statically HARQ disabled processes - Option A-4: applied to both semi-statically HARQ disabled and enabled processes - FFS for multiple TBs scheduled by single DCI - Option B: DCI-based HARQ enabling/disabling direct indication in case DCI solution enabling/disabling signaling is configured and per-HARQ process bitmap signaling is not configured (i.e. no bitmap is configured) - DCI-based mechanism is DCI signaling to directly indicate the HARQ feedback enable/disable for the corresponding transmission - For single TB scheduled by DCI, DCI-based direct indication is applied to the scheduled TB FFS for multiple TBs scheduled by single DCI Agreement in RAN1#113: For the RRC configuration of DCI solution enabling/disabling of HARQ feedback for NB-IoT and LTE-MTC in CE Mode B, the RRC configuration is UE-specific.

Based on the above agreements, the DCI-based HARQ feedback indication (Option 3) can not only be configured together with RRC based HARQ feedback disabling/enabling (Option 1), but also can be configured separately. When both RRC based HARQ feedback disabling/enabling(Option1) and the DCI based HARQ feedback indication (Option 3) are configured for a UE, the DCI indication is used to keep or reverse the HARQ feedback enable/disable state for a HARQ process indicated in the RRC configuration (i.e., the bullet Option A in RAN1 agreement above). When the per HARQ process bitmap via RRC (Option1) is not configured and only DCI based HARQ feedback indication (Option3) is configured, the DCI indication is a direction indication of HARQ feedback enabling/disabling for the scheduled TB (i.e., the bullet Option B in RAN1 agreement above).

In NR NTN, RAN2 agreed the enabling/disabling HARQ feedback can be configured per DL HARQ process via UE specific RRC signalling. This signalling can be reused in IoT over NTN for RRC based HARQ feedback disabling/enabling. For DCI based HARQ feedback indication, RAN1 agreed a UE-specific RRC configuration is needed. In our understanding, it means network should indicate whether the option is supported hence UE may decode the DCI fields customized for Option3. Therefore, one new IE of RRC signalling should be defined to indicate whether DCI based HARQ feedback indication is supported in a UE-specific way.

Proposal 3: The UE specific RRC signalling on enabling/disabling HARQ feedback in NR NTN can be reused in IoT NTN, i.e., enabling/disabling HARQ feedback can be configured per DL HARQ process via bitmap in RRC messages.

Proposal 4: Introduce one new IE in RRC messages to indicate whether DCI based HARQ feedback enabling/disabling is supported by network.

## 2.2	UL HARQ mode for the transmission using PUR

At RAN2#121bis meeting, it was open whether UL HARQ mode B should be supported for PUR transmission. The argument to support HARQ mode B for PUR is that it can support blind retransmission by adjusting the start of pur-ResponseWindowTimer without delay of UE-eNB RTT.

UE specific PUR is introduced to allow one uplink transmission from RRC_IDLE using a preconfigured uplink resource without performing the random-access procedure. In our understanding, PUR is targeted for small packet transmission and the preconfigured uplink resource such as the TBS, periodicity is configured based on UE request. Hence HARQ stalling is not a critical issue.  In addition, preconfigured uplink resource is configured via RRCRelease when the UE moves from RRC Connected to RRC_IDLE. This means the network can know more channel information of the UE in RRC Connected state before configuring the PUR resource for the UE, thus it can configure a reasonable repetition number for the PUR transmission. Moreover, the eNB can also further adjust the repetition number though the PDCCH scrambled with PUR-RNTI. Therefore, blind retransmission seems not that necessary for PUR transmission which is different from blind retransmission for Msg3.  Furthermore, the network has no knowledge whether there is data transmission in PUR even if PUR resource is configured. It will cause resource waste if the network determines the blind retransmission when there is no UL data transmission using PUR.

Observation 2: The network has no knowledge whether there is data transmission using the configured PUR. It will cause resource waste if the network schedules the blind retransmissions when there is no UL data transmission using PUR.

Observation 3: For PUR, HARQ stalling is not a critical issue and blind retransmission support is not necessary since reasonable repetition number can be used for reliability.

Proposal 5:  No need to support UL HARQ Mode B for PUR transmission.

## 2.3	UL HARQ mode for PUSCH repetition early termination

The early termination function is used to early termination of PUSCH transmission in the case that repetition number is large. This function can be configured through mpdcch-UL-HARQ-ACK-FeedbackCronfig as specified in 36.331:

mpdcch-UL-HARQ-ACK-FeedbackConfig TRUE indicates E-UTRAN may send UL HARQ-ACK feedback or UL grant corresponding to a new transmission for early termination of PUSCH transmission, or positive acknowledgement of completed PUSCH transmissions as specified in TS 36.321 [6] and TS 36.212 [22]. In case of acknowledgement of RRC Connection Release, MPDCCH monitoring is terminated.

For UEs configured with mpdcch-UL-HARQ-ACK-FeedbackConfig, repetitions within a bundle are stopped if an UL HARQ-ACK feedback or an uplink grant corresponding to a new transmission of the bundle is received on PDCCH during the bundle transmission. At RAN2#121bis meeting, it was discussed but not agreed on the UL HARQ mode configurations if early termination is configured. E.g., for eMTC NTN, whether it can be left to eNB’s implementation to configure either HARQ mode A or HARQ mode B for all HARQ process (or no HARQ mode) if mpdcch-UL-HARQ-ACK-FeedbackConfig is configured. In our understanding, early termination is to terminate the unnecessary repetitions and is workable for either HARQ mode A or HARQ mode B for all HARQ process or even no HARQ mode configured at all. The reason is that it is NW’s implementation to decide whether to send UL HARQ-ACK feedback/UL grant in DL PDCCH (based on UL decoding result), in order to early terminate the PUSCH repetitions regardless of the HARQ mode associated with the repetitions to be early terminated.

In addition, 36.321 defined the DRX operation when mpdcch-UL-HARQ-ACK-FeedbackConfig is configured as below:

When a DRX cycle is configured, the Active Time includes the time while: - …… - mpdcch-UL-HARQ-ACK-FeedbackConfig is configured and repetitions within a bundle are being transmitted according to UL_REPETITION_NUMBER. If this Serving Cell is part of a non-terrestrial network, the Active Time starts after the first repetition within the bundle plus the UE-eNB RTT when repetitions within the bundle are being transmitted When DRX is configured, the MAC entity shall for each subframe: …… if mpdcch-UL-HARQ-ACK-FeedbackConfig is configured and an UL HARQ-ACK feedback has not been received on PDCCH until the last repetition of the corresponding PUSCH transmission: - start or restart the drx-ULRetransmissionTimer for the corresponding HARQ process in the subframe containing the last repetition of the corresponding PUSCH transmission; if the PDCCH indicates an UL HARQ-ACK feedback for an asynchronous UL HARQ process for a UE configured with mpdcch-UL-HARQ-ACK-FeedbackConfig: - if the lower layer had indicated scheduling of transmission of multiple TBs: - stop drx-ULRetransmissionTimer for the corresponding UL HARQ process(es). - else if the PUSCH transmission is completed: - stop drx-ULRetransmissionTimer for all UL HARQ processes

According to current specification, the DRX Active Time starts after the first repetition within the bundle plus the UE-eNB RTT when repetitions within the bundle are being transmitted. It makes sense since the gNB can only determine to early terminate the repetitions when it decodes the uplink transmission correctly. So when the PDCCH indicates an UL HARQ-ACK feedback, the drx-ULRetransmissionTimer is stopped. If an UL HARQ-ACK feedback has not been received on PDCCH until the last repetition of the corresponding PUSCH transmission which means the retransmisison of the PUSCH may be scheduled by eNB,   the configured HARQ mode for this PUSCH repetition  can be HARQ mode A or  HARQ mode B. Therefore, current DRX behavior is workable for both HARQ mode A and HARQ mode B when mpdcch-UL-HARQ-ACK-FeedbackConfig is configured.

Proposal 6: It can be left to eNB’s implementation to early terminate the PUSCH repetitions when mpdcch-UL-HARQ-ACK-FeedbackConfig is configured, regardless of the HARQ mode configured for the PUSCH repetitions.

## 2.4 DRX impacts for NB-IoT and eMTC in NTN

### 2.4.1 DRX impacts for NB-IoT with single HARQ process

In RAN2#119bis meeting, the agreements were reached to take R17 NR NTN DRX solution as baseline for IoT NTN as below:

RAN2 Agreements: - RAN2 agree to take R17 NR NTN DRX solution as baseline for IoT NTN, e.g. for HARQ process with DL HARQ feedback disabled, the UE will not start the corresponding DL .HARQ RTT timer. - RAN2 agree to take R17 NR NTN DRX solution as baseline for IoT NTN, e.g. for HARQ process in HARQ mode B, the UE will not start the corresponding UL HARQ RTT timer.

In addition, DRX impact for NB-IoT with single HARQ processes in NTN was discussed in the past RAN2 meetings. For DL, if the HARQ process is configured with HARQ feedback disabled, the final agreement was reached in RAN2#112bis through the continuous revision in several RAN2 meetings as below:

Agreements in RAN2#119bis: For NB-IoT NTN with single HARQ process when the HARQ feedback is disabled, the UE will start/restart drx-inactivity timer in the subframe containing the last repetition of the corresponding PDSCH reception (can still check whether the alternative to set the HARQ RTT timer to 0 also works) Agreements in RAN2#120: For NB-IoT NTN with single HARQ process when the HARQ feedback is disabled, the UE will start/restart drx-inactivity timer in the subframe containing the last repetition of the corresponding PDSCH reception plus 12 subframes. Agreements in RAN2#112bis: RAN2#121’s agreement is revised to “For NB-IoT NTN with single HARQ process when the HARQ feedback is disabled, the UE will start/restart drx-inactivity timer in the subframe containing the last repetition of the corresponding PDSCH reception plus 12 subframes plus deltaPDCCH” (Can further check in the NB-IoT session if anything needs to be done for legacy NB-IoT as well, as some timers don’t take deltaPDCCH into account)

For UL, it is still open on the DRX operation for NB-IoT NTN with single HARQ process configured with HARQ mode B.  RAN2 have discussions in the past RAN2 meetings and agreed to send LS to RAN1 to ask for the “processing time for starting drx-InactivityTimer “.

Agreements in RAN2#119bis: For NB-IoT NTN with single HARQ process in HARQ mode B, the UE will start/restart drx-inactivity timer in the subframe containing the last repetition of the corresponding PUSCH transmission (can still check whether other alternatives also work) Agreements in RAN2#120: RAN2 understands that something needs to be added to consider the processing time also for inactivity timer of HARQ mode B. Continue the discussion on the details in the next meeting. Agreements in RAN2#112bis: For a NB-IoT UE configured with a single HARQ process in HARQ mode B, send LS to RAN1 and ask for the “processing time for starting drx-InactivityTimer (i.e. start to monitor NPDCCH)”. (can further check the detailed wording of the question).

In the LS from RAN2 to RAN1[R2-2304274], RAN2 asks question 1a: For an UL HARQ process with HARQ mode B for NB-IoT UEs, what is the minimum time between the end of NPUSCH transmission and the start of NPDCCH monitoring for the same HARQ process?  RAN1 replied one LS to RAN2 [R1-2306182] with below agreement and answer:

Agreements in RAN1#113: For a NB-IoT UE operating with two HARQ processes, for an UL HARQ process with HARQ mode B, the minimum time between the end of NPUSCH transmission and the start of NPDCCH monitoring for the same HARQ process is 1 ms. - Note: this implies a RAN1 specification change in Rel-18. For a NB-IoT UE operating with one HARQ process, for an UL HARQ process with HARQ mode B, the minimum time between the end of NPUSCH transmission and the start of NPDCCH monitoring is 1 ms. - Note: this implies a RAN1 specification change in Rel-18.

Based on the above response from RAN1, we have the following proposal:

Proposal 7: For NB-IoT over NTN with single HARQ process configured with HARQ mode B,  the UE will start/restart drx-inactivity timer in the subframe containing the last repetition of the corresponding PUSCH transmission plus 1ms plus deltaPDCCH.

### 2.4.2 DRX impacts when either RRC based or DCI based HARQ feedback indication is configured

Following RAN1 agreement, the network can either configure the RRC based or DCI based HARQ feedback indication.

Agreement in RAN1#112 meeting: Confirm the following working assumption with the following update: Working assumption For NB-IoT NTN and eMTC NTN for CE Mode B, to configure/indicate enabling/disabling of HARQ feedback for downlink transmission: - Support Option 1 in case only per-HARQ process bitmap signaling is configured - Support Option 3 DCI direct indication of HARQ feedback enable/disable in case only DCI solution enabling/disabling signaling is configured - Support Option 3 DCI indication to override Option 1 configuration for corresponding transmission in case both per-HARQ process bitmap and DCI solution enabling/disabling signaling are configured

On the DL DRX impact, if either RRC based or DCI based HARQ feedback indication is configured, the UE behaviour can follow NR NTN DRX solution as below:

- For HARQ process with DL HARQ feedback disabled, the UE will not start the corresponding DL HARQ RTT timer

- For HARQ process with DL HARQ feedback enabled, the HARQ RTT Timer length is increased by an offset equal to UE-eNB RTT.

Proposal 8: For the case either RRC based or DCI based HARQ enabling/disabling is configured, the DRX operation should follow NR NTN solution as below:

- For HARQ process with DL HARQ feedback disabled, the UE will not start the corresponding DL HARQ RTT timer

- For HARQ process with DL HARQ feedback enabled, the HARQ RTT Timer length is increased by an offset equal to UE-eNB RTT.

### 2.4.3 DRX impacts when both RRC based and DCI based HARQ feedback indication are configured

In RAN1, there are the following agreements when both RRC- based and DCI- based HARQ feedback disabling/enabling are configured:

Agreement in RAN1#112bis： For Option 3 DCI indication: - Option A: when both per-HARQ process bitmap and DCI solution enabling/disabling signaling are configured - DCI-based overridden mechanism is DCI signaling to reverse the HARQ feedback enable/disable for the corresponding transmission from per-HARQ process RRC configuration - For single TB scheduled by DCI, the DCI based overridden indication is applied to one of the following options (to be down-selected): - Option A-1: only applied to semi-statically HARQ disabled processes - Option A-4: applied to both semi-statically HARQ disabled and enabled processes - FFS for multiple TBs scheduled by single DCI Agreement in RAN1#113: For single TB scheduled by DCI, - Working assumption 1 DCI based overridden indication is applied to both semi-statically HARQ disabled and enabled processes - For DCI based overridden indication, adopt indication by reusing/reinterpreting HARQ-ACK related field in DCI - For eMTC CE mode B, “HARQ-ACK resource offset” field in DCI format 6-1B is used for indication of maintaining/reversing the HARQ feedback enable/disable for the corresponding transmission from per-HARQ process RRC configuration and corresponding HARQ-ACK resource in case of indication of HARQ feedback enabled. - HARQ feedback disabled is reversed to enabled in case of any states other than state A in “HARQ-ACK resource offset”, otherwise is maintained as disabled. - HARQ feedback enabled is maintained in case of any states other than state A in “HARQ-ACK resource offset”, otherwise is reversed to disabled. - FFS: detailed state A, and whether this state A is different across different UEs - For NBIoT, “HARQ-ACK resource” field in DCI format N1 is used for indication of maintaining/reversing the HARQ feedback enable/disable for the corresponding transmission from per-HARQ process RRC configuration and corresponding HARQ-ACK resource in case of indication of HARQ feedback enabled. - The same DCI indication functionality as eMTC is adopted. - Working assumption 2 For Option 1 + Option 3 DCI based overridden mechanism, for a HARQ process configured as HARQ feedback disabled by per-HARQ process bitmap signaling and further reversed to HARQ feedback enabled by DCI, the NBIoT UE does not wait for an RTT+3ms (i.e., till subframe n+Kmac+3 in TS36.213 section 16.6) before monitoring NPDCCH for the same HARQ process (or monitoring any NPDCCH for the case of single HARQ process configuration). - Send an LS to RAN2 with the following contents: - RAN1 respectfully ask RAN2 for the feasibility of Working assumption 2 (taking into account potential RAN2 spec impact).

In RAN1#113 meeting, one work assumption (WA2) was agreed and RAN1 sends one LS to ask RAN2 for the feasibility of this working assumption [R1-2306245]. The agreed work assumption 2 means for the case that a HARQ process is configured as HARQ feedback disabled by per-HARQ process bitmap signalling (e.g. via RRC) and further reversed to HARQ feedback enabled by DCI, for the one shot of scheduling by the DCI, the ACK/NACK information will be reported to the network but the NB-IoT UE does not wait for an RTT+3ms before monitoring NPDCCH for the same HARQ process.

On one hand, reporting HARQ ACK/NACK information can provide a high reliability transmission since network can decide the retransmissions for the initial TB or new transmission based on the ACK/NACK feedback. On the other hand, reporting HARQ ACK/NACK information is also beneficial for link adaptation. Additionally, providing the feedback information is helpful for the NW knows the correct reception of some important information such as MAC CE.

- If HARQ feedback information is for reliability transmission, it is very possible for the NW to schedule a retransmission or new transmission after UE-eNB RTT based on the decoding result of the previous transmission via HARQ ACK/NACK reporting. This is more resource efficient solution compared with other solution on improving reliability such as robust MCS setting or blind retransmissions. If WA2 is supported, the early monitoring PDCCH without waiting UE-eNB RTT will cause the waste of power consumption as the grant may come only after the RTT.

- If HARQ feedback information is for link adaptation only and network is expected to continue scheduling UE without waiting UE-eNB RTT to avoid HARQ stalling, the UE behaviour can be the same as WA2 to monitor the PDDCH without waiting for RTT+3ms.

Observation 4: If the HARQ ACK/NACK feedback is expected to be used to achieve the transmission reliability, WA2 is not reasonable due to the waste of UE power consumption. If the HARQ ACK/NACK feedback is expected to be used for link adaption, WA2 is reasonable.

In fact, both the motivation to achieve the reliability and the motivation for link adaption should be supported for providing HARQ ACK/NACK. When there is no reliability requirement but with throughput requirement, the UE behaviour as WA2 can provide ACK/NACK for link adaptation as well as avoid HARQ stalling.  When there is no throughput requirement but with reliability requirement, the UE shall monitor the PDCCH after UE-eNB RTT which is beneficial for power consumption.

Proposal 9: RAN2 to discuss the motivation for the case when a HARQ process is configured as HARQ feedback disabled via RRC bitmap while further reversed to HARQ feedback enabled by DCI. i.e., the feedback ACK/NACK information for the DCI grant is to achieve the transmission reliability or for link adaption or both.

Proposal 10: Whether the WA2 in R1-2306245 can be confirmed should base on the motivation decided in Proposal 9 (i.e., the motivation for the case when a HARQ process is configured as HARQ feedback disabled via RRC bitmap while further reversed to HARQ feedback enabled by DCI.)

From RAN2’s point of view, the WA2 have impact on UE’s DRX behaviour regarding whether the DRX RTT timer should be started or not. Therefore, RAN2 should discuss the DRX impact based on the motivation for the case the HARQ feedback state is revered by a DCI.

Proposal 11: RAN2 to discuss the DRX impact based on the determination whether WA2 can be supported.

RAN1 WA2 is only for NB-IoT case, there is no agreement in RAN1 for eMTC CE mode B when a HARQ process is configured as HARQ feedback disabled by per-HARQ process bitmap signalling and further reversed to HARQ feedback enabled by DCI. We don’t see the motivation to support the different solution for NB-IoT and eMTC CE mode B, the aligned solution for both NB-IoT and eMTC CE mode B can simplify the standardization effort.

Observation 5: No motivation to support the different solution for NB-IoT and eMTC CE mode B, when a HARQ process is configured as HARQ feedback disabled by per-HARQ process bitmap signalling while further reversed to HARQ feedback enabled by DCI.

Proposal 12: The aligned solution for both NB-IoT and eMTC CE mode B can be supported to simplify the standardization effort, when a HARQ process is configured as HARQ feedback disabled by per-HARQ process bitmap signalling while further reversed to HARQ feedback enabled by DCI.

In addition, there is also no agreement in RAN1 for the case that a HARQ process is configured as HARQ feedback enabled by per-HARQ process bitmap signalling and further reversed to HARQ feedback disabled by DCI. Our understanding is that disabling HARQ feedback in this case is to avoid HARQ stalling, for the DRX behaviour for the one shot DCI grant with HARQ feedback disabled, the UE behaviour can follow the case of DCI-based HARQ enabling/disabling direct indication (i.e. not to start the DRX RTT timer).

### 2.4.4 DRX impacts for multi-TB scheduling

Multiple TB scheduling has been introduced for eMTC and NB-IoT with multiple TBs scheduled by one DCI, where the number of scheduled TBs is indicated by the DCI, each TB related to one separate HARQ process and the HARQ process IDs will also be implicitly indicated in the DCI. Moreover, the multiple TBs scheduled by one DCI use the same resource assignment, MCS and repetition number and the multiple TBs may be transmitted consecutively or interleaved.

When multiple TBs are scheduled, for NB-IoT, drx-InactivityTimer will be (re)started when all HARQ RTT Timers have expired. Additionally, for both NB-IoT and eMTC, (UL) HARQ RTT timers for all HARQ processes corresponding to the scheduled TBs are started in the subframe containing the last repetition of the (PUSCH transmission) PDSCH reception of the last scheduled TB.The detailed DRX behavior on multi-TB scheduling can be found in specification as below [36.321]

When DRX is configured, the MAC entity shall for each subframe: - if a HARQ RTT Timer expires in this subframe: - if the data of the corresponding HARQ process was not successfully decoded: - start the drx-RetransmissionTimer or drx-RetransmissionTimerShortTTI for the corresponding HARQ process; - if NB-IoT: - if lower layers had indicated multiple TBs were scheduled for the associated expired HARQ RTT Timer: - start or restart drx-InactivityTimer when all HARQ RTT Timers have expired; - else: - start or restart the drx-InactivityTimer. - if an UL HARQ RTT Timer expires in this subframe: - start the drx-ULRetransmissionTimer or drx-ULRetransmissionTimerShortTTI for the corresponding HARQ process. - if NB-IoT: - if lower layers had indicated multiple TBs were scheduled for the associated expired HARQ RTT Timer: - start or restart drx-InactivityTimer when all HARQ RTT Timers have expired; - else: - start or restart the drx-InactivityTimer. ……. if the PDCCH indicates a DL transmission or if a DL assignment has been configured for this subframe: - if the UE is an NB-IoT UE, a BL UE or a UE in enhanced coverage: - if lower layers have indicated scheduling of transmission of multiple TBs: - start the HARQ RTT Timers for all HARQ processes corresponding to the scheduled TBs in the subframe containing the last repetition of the PDSCH corresponding to the last scheduled TB; - else: - start the HARQ RTT Timer for the corresponding HARQ process in the subframe containing the last repetition of the corresponding PDSCH reception; …… if the PDCCH indicates a new transmission (DL, UL or SL): - except for an NB-IoT UE configured with a single DL and UL HARQ process and when PDCCH indicates the transmission is not for multiple TBs: - start or restart drx-InactivityTimer. - if the PDCCH indicates a transmission (DL, UL) for an NB-IoT UE: - if the NB-IoT UE is configured with a single DL and UL HARQ process; or - if the PDCCH indicates the transmission is for multiple TBs: - stop drx-InactivityTimer.

Observation 6: In Rel-17, when multiple TBs are scheduled for NB-IoT, the drx-InactivityTimer will be (re)started when all HARQ RTT Timers have expired.

On DL multiple TB scheduling for IoT NTN, it was discussed with the agreement to wait for RAN1’s progress as below:

Agreements in RAN2#121bis On DL multiple TB scheduling, wait for RAN1’s progress before discussing related DRX impact in RAN2.

In RAN1#113 meeting, there is the agreement for DCI-based HARQ enabling/disabling direct indication in multiple TB scheduling as below:

Agreements in RAN1#113 ​: for NB-IoT and LTE-MTC in CE Mode B, if multiple TBs is configured, for DCI-based HARQ enabling/disabling direct indication in multiple TBs scheduled by single DCI, the same indication is applied to all scheduled TBs, i.e. HARQ is enabled or disabled for all TBs.

Based on the RAN1 agreement, all the HARQ processes scheduled in the single DCI are either HARQ feedback disabled or HARQ feedback enabled.

- If all HARQ processes are indicated with HARQ feedback enabled, the HARQ RTT timer length of these HARQ processes is increased by an offset equal to UE-eNB RTT and the other DRX behaviour can be the same as the current specification.

- If all HARQ processes are indicated with HARQ feedback disabled, the UE will not start the corresponding DL HARQ RTT timer for these scheduled HARQ processes. Specifically, for NB-IoT UE, the drx-InactivityTimer will not be started/restarted due to drx-InactivityTimer is started/restarted only when all HARQ RTT Timers have expired. In this case, PDCCH monitoring can rely on the DRX on duration timer. Another way is as the DRX solution for single HARQ process, the UE shall start/restart drx-inactivity timer in the subframe containing the last repetition of the corresponding PDSCH reception plus 12 subframes plus deltaPDCCH.

Proposal 13: For DCI-based HARQ enabling/disabling direct indication in multiple TBs scheduling, if the DCI indicate HARQ feedback enabled, the DRX behaviour can be defined as below:

- The HARQ RTT timer length of these HARQ processes is increased by an offset equal to UE-gNB RTT

- Start the HARQ RTT Timers for these HARQ processes in the subframe containing the last repetition of the PDSCH corresponding to the last scheduled TB.

Proposal 14: For DCI-based HARQ enabling/disabling direct indication in multiple TBs scheduling, if the DCI indicate HARQ feedback disabled, the DRX behaviour can be defined as below:

- DL HARQ RTT timer for these scheduled HARQ processes is not started

Proposal 15: For NB-IoT, RAN2 to discuss whether the drx-inactivity timer is started/restarted in the subframe containing the last repetition of the corresponding PDSCH reception plus 12 subframes plus deltaPDCCH if the DCI indicate HARQ feedback disabled for all the HARQ processes scheduled in multi-TB scheduling.

On UL multiple TB scheduling, in RAN2 #121bis meeting, it was agreed to send LS to RAN1 to check which UL HARQ mode combination(s) are to be supported for UL multiple TB scheduling and  question 2 as below is asked in the LS from RAN2 to RAN1[R2-2304274]:

RAN2’s question 2: For UL multiple TB scheduling, which of the following HARQ mode combinations does RAN1 intend to support for eMTC and NB-IoT? - Case 1: all HARQ processes corresponding to the scheduled multiple TBs are configured with HARQ mode A - Case 2: all HARQ processes corresponding to the scheduled multiple TBs are configured with HARQ mode B - Case 3: some HARQ processes corresponding to the scheduled multiple TBs are configured with HARQ mode A and the others are configured with HARQ mode B

RAN1  replied  one LS to RAN2 [R1-2306182.] with below agreement and answers, RAN2 is expected to make the decision:

Agreement Adopt the response below to RAN2’s question 2: - Whether/how to support case 1, case 2 and/or case 3 for UL multiple TB scheduling for eMTC and NB-IoT is transparent to RAN1 and can be left to RAN2 decision.

Similar as DL multiple TB scheduling, the same UL HARQ mode for the scheduled HARQ processes (i.e., Case1 and Case 2) should be used which is aligned with  DL solution, which  can simplify the standardization effort. In other words, the DRX operation for multi-scheduling in UL with HARQ mode A/B can also adopt the similar solution as DL with HARQ feedback disabling/enabling.

Proposal 16: The same HARQ mode  should be applied to all the scheduled HARQ processes in UL multiple TB scheduling for eMTC and NB-IoT.

## 2.5 Unreliable Timing Advance Report MAC CE transmission

For eMTC NTN, whether to enhance the TAR MAC CE transmission to avoid or mitigate the outdated TA and Koffset impact in Rel-18 was briefly discussed in email discussion [AT121bis-e][103] with summary in report R2-2304254. Since the companies have divergent views (4 companies agree, 4 companies disagree and 5 companies with no strong view), the discussion is postponed to the following RAN2 meeting.

As explained in paper R2-2303644, the eNB may maintain an outdated Timing Advance information if the TAR MAC CE was not transmitted to eNB successfully, especially when the MAC CE was transmitted in UL HARQ Mode B.

The typical case is that, when the UE’s TA is increased due to the increase of UE-Satellite-eNB distance, the UE may trigger a TA report to update the latest TA to NW. However, if the TAR MAC CE was transmitted in UL HARQ mode B, the eNB may not decode the MAC PDU correctly since HARQ mode B best supports no UL retransmission and/or blind UL retransmission (where the retransmission is not based on UL decoding result). Different from data transmission where the RLC retransmission mechanism may be triggered for transmission robustness, there is no upper layer retransmission for MAC CE. Due to the unsuccessful TAR MAC CE transmission, the eNB would not know TA is updated/increased in UE hence the outdated TA is maintained in eNB. Therefore, the eNB has no way to update/increase its Koffset based on the outdated TA. The procedure can be well illustrated in below Figure1.

Figure 1 An example of UE using outdated UE Koffset due to unsuccessful TA report

Observation 7: The eNB may configure an outdated Koffset based the outdated Timing Advance, when the TAR MAC CE was transmitted in UL HARQ Mode B.

One may argue that the NW can configure a larger Koffset to accommodate possible the TA change hence the outdate Koffset is not an issue. However, we would like to emphasize the motivation to introduce the TA Report MAC CE is to configure updated Koffset (e.g., configure UE specific Koffset based on the UE reported TA), hence data transmission latency can be reduced for UE located in the cell centre. In short, configuring a larger Koffset to mitigate the issue is not a reasonable solution since NW has to configure a Koffset which can cover the largest TA of all the UEs under the cell. While this solution will kill the motivation to support TA report which implies the TA report is not useful at all.

Observation 8: The outdated Koffset issue cannot be resolved by configuring a larger Koffset since it will kill the motivation and benefit of introducing the TA report MAC CE mechanism.

Furthermore, based on current specification, we believe the outdated Koffset issue will happen frequently since the TA change is a common behaviour in NTN due to flying characteristics of satellites, no matter in earth fixed cell or earth moving cell.

Observation 9: Based on current specification, the outdated Koffset issue will happen frequently in NTN.

In the end, with the increase of UE’s TA, when the UE’s actual TA is larger than (UE's outdated Koffset + 4 - UE's UL processing delay) the UE cannot perform UL transmission at all due to not enough processing time in UE in between PDCCH grant and the corresponding PUSCH transmission. (i.e., the gap between slot n in UE’s DL timing and the scheduled UL transmission slot m in the UE’s UL timing is smaller than a threshold (e.g., the UL processing delay) after UE applied it TA adjustment.)

Observation 10: The outdated Koffset may cause PUSCH transmission failure in eMTC NTN.

Figure 2: UL transmission timing for eMTC NTN

To handle the outdated Koffset thus PUSCH transmission failure due to unreliable TAR MAC CE transmission, there may have two directions for the solutions: one way is to improve the TAR MAC CE transmission robustness to avoid outdated TA at network. The other way is to mitigate the PUSCH transmission failure impact caused by the outdated TA and Koffset. In our view, the enhancement on TAR MAC CE transmission reliability should be supported to avoid the PUSCH transmission failure as much as possible. Furthermore, if UE detected PUSCH transmission failure which was caused by inappropriate Koffset, the UE should inform NW to stop further UL grant hence NW can reconfigure the Koffset for normal PUSCH transmission.

Observation 11: The PUSCH transmission failure issue caused by unreliable TAR MAC CE transmission should be addressed for eMTC NTN.

Proposal 17: The TAR MAC CE transmission reliability should be enhanced to avoid PUSCH transmission failure caused by outdated TA.

Proposal 18: UE should report the PUSCH transmission failure to NW if it is caused by inappropriate Koffset configured by NW.

# 3	Conclusion

This document has made the following observations:

Observation 1: For eMTC CE Mode B, fast UL HARQ mode indication/switch via DCI can provide more scheduling flexibility.

Observation 2: The network has no knowledge whether there is data transmission using the configured PUR. It will cause resource waste if the network schedules the blind retransmissions when there is no UL data transmission using PUR.

Observation 3: For PUR, HARQ stalling is not a critical issue and blind retransmission support is not necessary since reasonable repetition number can be used for reliability.

Observation 4: If the HARQ ACK/NACK feedback is expected to be used to achieve the transmission reliability, WA2 is not reasonable due to the waste of UE power consumption. If the HARQ ACK/NACK feedback is expected to be used for link adaption, WA2 is reasonable.

Observation 5: No motivation to support the different solution for NB-IoT and eMTC CE mode B, when a HARQ process is configured as HARQ feedback disabled by per-HARQ process bitmap signalling while further reversed to HARQ feedback enabled by DCI.

Observation 6: In Rel-17, when multiple TBs are scheduled for NB-IoT, the drx-InactivityTimer will be (re)started when all HARQ RTT Timers have expired.

Observation 7: The eNB may configure an outdated Koffset based the outdated Timing Advance, when the TAR MAC CE was transmitted in UL HARQ Mode B.

Observation 8: The outdated Koffset issue cannot be resolved by configuring a larger Koffset since it will kill the motivation and benefit of introducing the TA report MAC CE mechanism.

Observation 9: Based on current specification, the outdated Koffset issue will happen frequently in NTN.

Observation 10: The outdated Koffset may cause PUSCH transmission failure in eMTC NTN.

Observation 11: The PUSCH transmission failure issue caused by unreliable TAR MAC CE transmission should be addressed for eMTC NTN.

And proposed the following:

Proposal 1: For NB-IoT NTN, support RRC-based UL HARQ mode A/B indication is sufficient.

Proposal 2: RAN2 to further discuss whether DCI-based UL HARQ mode indication is supported for eMTC NTN CE mode B.

Proposal 3: The UE specific RRC signalling on enabling/disabling HARQ feedback in NR NTN can be reused in IoT NTN, i.e., enabling/disabling HARQ feedback can be configured per DL HARQ process via bitmap in RRC messages.

Proposal 4: Introduce one new IE in RRC messages to indicate whether DCI based HARQ feedback enabling/disabling is supported by network.

Proposal 5:  No need to support UL HARQ Mode B for PUR transmission.

Proposal 6: It can be left to eNB’s implementation to early terminate the PUSCH repetitions when mpdcch-UL-HARQ-ACK-FeedbackConfig is configured, regardless of the HARQ mode configured for the PUSCH repetitions.

Proposal 7: For NB-IoT over NTN with single HARQ process configured with HARQ mode B,  the UE will start/restart drx-inactivity timer in the subframe containing the last repetition of the corresponding PUSCH transmission plus 1ms plus deltaPDCCH.

Proposal 8: For the case either RRC based or DCI based HARQ enabling/disabling is configured, the DRX operation should follow NR NTN solution as below:

- For HARQ process with DL HARQ feedback disabled, the UE will not start the corresponding DL HARQ RTT timer

- For HARQ process with DL HARQ feedback enabled, the HARQ RTT Timer length is increased by an offset equal to UE-eNB RTT.

Proposal 9: RAN2 to discuss the motivation for the case when a HARQ process is configured as HARQ feedback disabled via RRC bitmap while further reversed to HARQ feedback enabled by DCI. i.e., the feedback ACK/NACK information for the DCI grant is to achieve the transmission reliability or for link adaption or both.

Proposal 10: Whether the WA2 in R1-2306245 can be confirmed should base on the motivation decided in Proposal 9 (i.e., the motivation for the case when a HARQ process is configured as HARQ feedback disabled via RRC bitmap while further reversed to HARQ feedback enabled by DCI.)

Proposal 11: RAN2 to discuss the DRX impact based on the determination whether WA2 can be supported.

Proposal 12: The aligned solution for both NB-IoT and eMTC CE mode B can be supported to simplify the standardization effort, when a HARQ process is configured as HARQ feedback disabled by per-HARQ process bitmap signalling while further reversed to HARQ feedback enabled by DCI.

Proposal 13: For DCI-based HARQ enabling/disabling direct indication in multiple TBs scheduling, if the DCI indicate HARQ feedback enabled, the DRX behaviour can be defined as below:

- The HARQ RTT timer length of these HARQ processes is increased by an offset equal to UE-gNB RTT

- Start the HARQ RTT Timers for these HARQ processes in the subframe containing the last repetition of the PDSCH corresponding to the last scheduled TB.

Proposal 14: For DCI-based HARQ enabling/disabling direct indication in multiple TBs scheduling, if the DCI indicate HARQ feedback disabled, the DRX behaviour can be defined as below:

- DL HARQ RTT timer for these scheduled HARQ processes is not started

Proposal 15: For NB-IoT, RAN2 to discuss whether the drx-inactivity timer is started/restarted in the subframe containing the last repetition of the corresponding PDSCH reception plus 12 subframes plus deltaPDCCH if the DCI indicate HARQ feedback disabled for all the HARQ processes scheduled in multi-TB scheduling.

Proposal 16: The same HARQ mode  should be applied to all the scheduled HARQ processes in UL multiple TB scheduling for eMTC and NB-IoT.

Proposal 17: The TAR MAC CE transmission reliability should be enhanced to avoid PUSCH transmission failure caused by outdated TA.

Proposal 18: UE should report the PUSCH transmission failure to NW if it is caused by inappropriate Koffset configured by NW.