3GPP TSG-RAN WG2 Meeting #123	R2-2308237

Toulouse, France, 21th – 25th August, 2023

Agenda item:	7.19.3

Source:	Huawei, HiSilicon

Title:	Early identification and access restriction for eRedCap UEs

Document for:	Discussion and Decision

- Introduction

In RAN2#122 meeting and RAN1#133 meeting, there are some discussions on further reduced UE complexity in FR1. Some agreement are achieved as shown in the following. In this paper, we will further discuss the leftover issues.

RAN2#122 agreements: - RAN2 confirms there are cell(s) supporting Rel-18 eRedCap only, i.e. not supporting Rel-17 RedCap UE to camp and access. - We will discuss further next meeting. RAN1#113 agreements about early identification: - A network-configurable additional separate early indication in Msg1 for Rel-18 eRedCap UEs is supported. - When Msg1 indication for Rel-18 eRedCap UEs is configured, it is used by Rel-18 eRedCap UEs (with or without UE BB bandwidth reduction). - When Msg1 indication for Rel-18 eRedCap UEs is not configured while Msg1 indication for Rel-17 RedCap UEs is configured, Rel-18 eRedCap UEs shall share the PRACH that is configured for Rel-17 RedCap UEs. - Note: Rel-18 eRedCap UEs will be differentiated from Rel-17 RedCap UEs based on Msg3 of Rel-18 eRedCap UEs. - Additional early indication in MsgA PRACH is not supported.

- Discussion

  - Msg1 early identification

According to the agreements of RAN1#113 meeting, the network can optionally configure the separate PRACH resources for early identification of eRedCap UEs. If the separate PRACH resources are configured for eRedCap UEs, then the network can identify the eRedCap UEs when receiving the preamble. If the network does not configure the separate PRACH resources for eRedCap UEs, the eRedCap UEs can use the separate PRACH resources for Rel-17 RedCap UEs (if configured) and the network shall differentiate the eRedCap UEs and the RedCap UEs via Msg3-based early identification.

A network-configurable additional separate early indication in Msg1 for Rel-18 eRedCap UEs is supported. - When Msg1 indication for Rel-18 eRedCap UEs is configured, it is used by Rel-18 eRedCap UEs (with or without UE BB bandwidth reduction). When Msg1 indication for Rel-18 eRedCap UEs is not configured while Msg1 indication for Rel-17 RedCap UEs is configured, Rel-18 eRedCap UEs shall share the PRACH that is configured for Rel-17 RedCap UEs.

In Rel-17, Msg1-based early identification of RedCap UEs can be combined with other Rel-17 features (i.e. SDT, RAN slicing and CovEnh) and early identification of these feature combinations can be achieved by RACH partitioning. The network indicates a feature or a feature combination to be associated with a set of Random Access resources (i.e. an instance of FeatureCombinationPreambles) via FeatureCombination-r17.  If a RACH procedure is triggered by a RedCap UE for a feature combination, the RedCap UE should identify the sets of Random access resources that are applicable for the current RACH procedure among the configured sets of Random access resources using the associated FeatureCombination-r17 IEs, as specified in TS 38.321 clause 5.1.1c. And then the RedCap UE should select the set of Random access resources for the RACH procedure among the identified sets of Random access resources as specified in TS 38.321 clause 5.1.1b.

Accordingly, to support the Msg1-based early identification, the straightforward way is to add a new value “enRedCap-r18” in FeatureCombination-r17 using one spare bit and the network indicates a set of Random Access resources to be applicable for eRedCap UEs by setting “enRedCap-r18” to true.

Proposal 1a: Add a new value “enhRedCap-r18” in FeatureCombination-r17 only for 4-step RACH.

When the network does not configure the separate set of Random Access resources for eRedCap UEs, the eRedCap UEs are allowed to share the set(s) of Random Access resources for RedCap UE. However, the network cannot set both redcap-r17 and enhRedCap-r18 in one single FeatureCombination-r17 as true, since the Rel-17 RedCap UEs would ignore the associated set of Random Access resources once any of the spare bit within the featureCombination is set to true.

featureCombination Indicates which combination of features that the preambles indicated by this IE are associated with. The UE ignores a RACH resource defined by this FeatureCombinationPreambles if any feature within the featureCombination is not supported by the UE or if any of the spare fields within the featureCombination is set to true.

Observation 1: If both redcap-r17 and enhRedCap-r18 in one single FeatureCombination-r17 are set to true, the R17 RedCap UEs cannot use the Random Access resources set associated with this FeatureCombination-r17 anymore.

Proposal 1b: One FeatureCombination-r17 should not set both redCap-r17 and enhRedCap-r18 as true.

If the above proposal 2 and proposal 3 are agreed, RAN2 should discuss how to support the eRedCap UEs in sharing the set of Random Access resources for RedCap UEs when the set(s) of Random Access resources for eRedCap UEs are not configured. There are 3 options as following:

- Option 1: If the redcap-r17 in FeatureCombination-r17 associated with one set of Random Access resources is set to true, the eRedCap UEs consider the set of Random Access resources is applicable although the enhRedCap-r18 is not present. When performing the selection of Random Access resources, the eRedCap UEs should select the set of Random Access resources for RACH procedure among the sets of Random Access resources that redcap-r17 or enhRedCap-r18 in the associate FeatureCombination-r17 is set to true.

- Option 2: The eRedCap UEs select the set of Random Access resources according to TS 38.321 clause 5.1.1b and clause 5.1.1c. If none of the sets of Random Access resources are available for any feature applicable to the current RACH procedure, the eRedCap UE should consider itself as RedCap UE and perform the selection of Random Access resources procedure again.

- Option 3: If there is at least one configured set of Random Access resources that enhRedCap-r18 in the associate FeatureCombination-r17 is set to true, the eRedCap UE should perform the selection of Random Access resources procedure considering the eRedCap feature is applicable. Otherwise, if there is none of the configured sets of Random Access resources that enhRedCap-r18 in the associate FeatureCombination-r17 is set to true, the eRedCap UEs should perform the selection of Random Access resources procedure considering RedCap feature is applicable.

Proposal 2: For Msg1 early identification, RAN2 to discuss following options to support the eRedCap UEs sharing the Random Access resources set for RedCap UEs, when the Random Access resources set(s) specific for eRedCap UEs are not configured:

- Option 1: eRedCap UE considers both ‘eRedCap’ and ‘redCap’ features as applicable to its Random Access procedure.

- Option 2: eRedCap UE first considers ‘eRedCap’ feature applicable to its Random Access procedure. If none of the Random Access resources sets is available, the eRedCap UE should consider ‘redCap’ feature applicable to its Random Access procedure and perform the Random Access resources selection again.

- Option 3: eRedCap UE considers ‘eRedCap’ feature applicable to its Random Access procedure, if there is at least one Random Access resource set with enhRedCap-r18 set to true based on RRC configuration. Otherwise, eRedCap UE considers ‘redCap’ feature applicable to its Random Access procedure.

  - Msg4 PDSCH schedule

In RAN1#112bis-e, RAN1 sent RAN2 a LS [3] to request the feedback of UE behaviour in the case that when the UE with BB reduction capability detects a DCI scheduling a Msg4 PDSCH transmission with a larger bandwidth than it can receive or process. In RAN2 #122, there are also some discussions, but there is no consensus on this issue. So we will continue to discuss it. The some options discussed by RAN1 are shown as follows.

Option 1: The UE considers the contention resolution as not successful.

Option 2: The UE continues monitoring the DCI until ra-ContentionResolutionTimer expires.

Option 3: The UE behavior is up to the UE implementation.

The mentioned case occurs in the contention during CBRA between eRedCap UE and non-eRedCap UE. When NW only successfully received the Msg3 from a non-eRedCap UE, it may schedule the Msg3 based on the capability of non-eRedCap UE, with exceeding the bandwidth a eRedCap can receive or process. Therefore, receiving one Msg4 schedule exceeding the capability implies the contention failure.

We can see option 2 is captured by current spec, it is a workable solution for this case. However, option 2 will make eRedCap UE keep invalid monitoring even the UE will never receive its response. In our opinion, option 1 is a reasonable and efficient method to address this issue. When eRedCap UE receive the DCI scheduling a Msg4 which UE is not able to process, eRedCap UE shall stop the PDCCH monitoring and consider the contention resolution as not successful.

Observation 2: In legacy, PHY layer specification did not capture the “notification of a reception of a PDCCH transmission” to upper layer during RACH. Namely that the inter-layer interaction is UE implementation and follows the MAC specification.

Observation 3: RAN1 agreed to process the PDCCH but not the PDSCH, in case of Msg4 PDSCH transmission with a larger bandwidth than RedCap UE can receive or process. Therefore, the notification of a reception of a PDCCH transmission is still possible as before.

Observation 4: The case of PDSCH exceeding UE receiving/processing capability does not belong to the “inconsistent information” for DCI, as in TS 38.213 clause 10.1. Therefore, UE will not discard the DCI.

Proposal 3: As to the RAN1 LS, RAN2 agree on option 1, i.e. eRedCap UE considers the contention resolution as not successful and stop the ra-ContentionResolutionTimer, when the eRedCap UE detects a DCI scheduling a Msg4 PDSCH transmission with a larger bandwidth than it can receive or process.

  - Handover restriction

Similar to R17 RedCap UE, eRedCap cannot work well in the cell not supporting eRedCap, e.g. because of the further reduced capabilities, i.e. Rel-18 eRedCap: UE capable of 20MHz + PR1" and "Rel-18 eRedCap: UE capable of BW3/PR3 + PR1. In case of handover, the NW should ensure to handover the eRedCap UEs to the target cell which support eRedCap UE. It seems the similar R17 RedCap Xn capability exchanging method can be reused with a new indication. So we think RAN2 should send LS to RAN3 for supporting such case.

Proposal 4a: Network should ensure the target gNB supports/allows eRedcap UE, in the handover of eRedCap UE.

Proposal 4b: RAN2 sends LS to ask RAN3 to support the corresponding Xn signalling (at least sharing the eRedCap specific IFRI and cell barring indications, similar to the RedCap Broadcast Information IE in 38.423).

  - Cell Barring

It is agreed in RAN#99 meeting that the initial access procedure of eRedCap UE capable of 20MHz + PR1 is same as eRedCap UE capable of BW3/PR3 + PR1. They are designed to the same peak data rate with different implementation method. We do not see the need for NW to have different access control mechanisms for the 2 kinds of capabilities. Therefore, we propose not to introduce separate barring indications for the UEs capable of 20MHz + PR1 and BW3/PR3 + PR1.

Rel-18 eRedCap UE capable of 20MHz + PR1 and Rel-18 eRedCap UE capable of BW3/PR3 + PR1 are designed/targeted to same peak data rate, i.e., 10Mbps Note 1: Peak data rate of "Rel-18 eRedCap: UE capable of 20MHz + PR1" and "Rel-18 eRedCap: UE capable of BW3/PR3 + PR1" is same including unicast and broadcast respectively. Note 2: PRB processing capability of "Rel-18 eRedCap: UE capable of 20MHz + PR1" is not limited to "25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS" and it corresponds to PRB size corresponding to 20 MHz. Note 3: The only difference between "Rel-18 eRedCap: UE capable of 20MHz + PR1" and "Rel-18 eRedCap: UE capable of BW3/PR3 + PR1" is Note 2 and vLayers·Qm·f in order to have the same peak rate. Note 4: The initial access procedure of Rel-18 eRedCap UE capable of 20MHz + PR1 is realized by following: - Same as Rel-18 eRedCap UE capable of BW3/PR3 + PR1

Proposal 5: No need to have separate cell barring for “eRedCap UE capable of 20MHz + PR1” and “eRedCap UE capable of BW3/PR3+ PR1”.

  - Inter-node message

The network should be able to indicate whether eRedCap UEs are supported or allowed to access. Moreover, the network can indicate whether the eRedCap UEs with 1Rx branch or with 2Rx branches are allowed to access, which is similar with RedCap UEs. Therefore, to avoid unnecessary paging message when the eRedCap UEs are not allowed to camp on the cell, eRedCap UE type indication should be added into inter-node message UERadioPagingInformation.

Proposal 6: The eRedCap UE type indication should be added into the inter-node message UERadioPagingInformation.

- Conclusion

The corresponding proposals are listed as below:

Observation 1: If both redcap-r17 and enhRedCap-r18 in one single FeatureCombination-r17 are set to true, the R17 RedCap UEs cannot use the Random Access resources set associated with this FeatureCombination-r17 anymore.

Observation 2: In legacy, PHY layer specification did not capture the “notification of a reception of a PDCCH transmission” to upper layer during RACH. Namely that the inter-layer interaction is UE implementation and follows the MAC specification.

Observation 3: RAN1 agreed to process the PDCCH but not the PDSCH, in case of Msg4 PDSCH transmission with a larger bandwidth than RedCap UE can receive or process. Therefore, the notification of a reception of a PDCCH transmission is still possible as before.

Observation 4: The case of PDSCH exceeding UE receiving/processing capability does not belong to the “inconsistent information” for DCI, as in TS 38.213 clause 10.1. Therefore, UE will not discard the DCI.

Msg1 early identification

Proposal 1a: Add a new value “enhRedCap-r18” in FeatureCombination-r17 only for 4-step RACH.

Proposal 1b: One FeatureCombination-r17 should not set both redCap-r17 and enhRedCap-r18 as true.

Proposal 2: For Msg1 early identification, RAN2 to discuss following options to support the eRedCap UEs sharing the Random Access resources set for RedCap UEs, when the Random Access resources set(s) specific for eRedCap UEs are not configured:

- Option 1: eRedCap UE considers both ‘eRedCap’ and ‘redCap’ features as applicable to its Random Access procedure.

- Option 2: eRedCap UE first considers ‘eRedCap’ feature applicable to its Random Access procedure. If none of the Random Access resources sets is available, the eRedCap UE should consider ‘redCap’ feature applicable to its Random Access procedure and perform the Random Access resources selection again.

- Option 3: eRedCap UE considers ‘eRedCap’ feature applicable to its Random Access procedure, if there is at least one Random Access resource set with enhRedCap-r18 set to true based on RRC configuration. Otherwise, eRedCap UE considers ‘redCap’ feature applicable to its Random Access procedure.

Msg4 PDSCH schedule

Proposal 3: As to the RAN1 LS, RAN2 agree on option 1, i.e. eRedCap UE considers the contention resolution as not successful and stop the ra-ContentionResolutionTimer, when the eRedCap UE detects a DCI scheduling a Msg4 PDSCH transmission with a larger bandwidth than it can receive or process.

Handover restriction

Proposal 4a: Network should ensure the target gNB supports/allows eRedcap UE, in the handover of eRedCap UE.

Proposal 4b: RAN2 sends LS to ask RAN3 to support the corresponding Xn signalling (at least sharing the eRedCap specific IFRI and cell barring indications, similar to the RedCap Broadcast Information IE in 38.423).

Cell barring

Proposal 5: No need to have separate cell barring for “eRedCap UE capable of 20MHz + PR1” and “eRedCap UE capable of BW3/PR3+ PR1”.

Inter-node message

Proposal 6: The eRedCap UE type indication should be added into the inter-node message UERadioPagingInformation.

- Reference

[1] RAN2#122 agreements on Further NR RedCap UE complexity reduction.

[2] R1-2304262 LS on Msg4 PDSCH transmission to Rel-18 eRedCap UEs

[3] RP-223544 Revised WID on Enhanced support of reduced capability NR devices