3GPP TSG-RAN WG2 Meeting #123	       R2-2308247

Toulouse, France, August 21-25, 2023

Agenda Item	:	7.5.2

Title	:	On XR awareness

Source	:	Google Inc.

Document for	:	Discussion and Decision

# Introduction

In RAN#99, the Rel-18 XR WI objectives have been revised as follow [1]:

Specify the enhancements related to power saving:

-	DRX support of XR frame rates corresponding to non-integer periodicities (through at least semi-static mechanisms e.g. RRC signalling) (RAN2).

Specify the enhancements related to capacity:

-	Multiple Configured Grant (CG) PUSCH transmission occasions in a period of a single CG PUSCH configuration (RAN1, RAN2);

-	Dynamic indication of unused CG PUSCH occasion(s) based on Uplink Control Information (UCI) by the UE (RAN1, RAN2);

-	Buffer Status Report (BSR) enhancements including at least new Buffer Status Table(s) (RAN2);

-	Delay reporting of buffered data in uplink (RAN2);

-	Discard operation of PDU Sets for DL and UL (RAN2, RAN3);

Specify the enhancements for XR Awareness:

-	Signalling by CN of semi-static information per QoS flow (e.g. PDU set QoS parameters), dynamic information per PDU set (PDU Set information and Identification) and End of Data Burst indication (RAN3, RAN2);

-	Impact of identifying by UE of PDU Sets, Data bursts and PSI, as needed (RAN2);

-	Provisioning by UE of XR traffic assistance information e.g. periodicity, UL traffic arrival information (RAN2, RAN3);

-	Support signalling the congestion information from RAN to the CN in alignment with SA2 (RAN3);

In this Tdoc, we share our views on the XR traffic UE assistance information.

# XR Traffic UE Assistance Information

In RAN2#122, the following agreements have been made regarding the UL jitter information and the signalling of some UL traffic information:

UE reports to RAN the range of jitter in its UL traffic, defined in the similar way as the one for N6 jitter.

Reference time is defined in similar way as BAT (Burst Arrival Time) at UE side.

UL assistance information (burst arrival time, UL jitter, FFS on periodicity) is reported per QoS flow. Network can configure for which QoS flow UE should report assistance information.

RRC UAI framework is updated for Rel-18 to support signalling UL assistance information agreed so far for XR (Jitter, burst arrival time, FFS on periodicity).

Do not use PIN delay budget request for jitter reporting for XR services.

Reuse existing mechanisms (e.g. (Padding) BSR with BS value equal to zero) as implicit End of Data Burst (EoDB) indicator for the RAN.

On the UL, the identification of PDU sets, data bursts and PSI is left to UE implementation. This doesn’t mean UE cannot use information provided by upper layers, but RAN2 does not intend to specify how.

The UE can indicate the periodicity for each traffic flow, including UL AR traffic and UL pose/control information. This allows the network to configure the CG configurations periodicities for each traffic flow. The UE can also update the network about any changes in periodicities via RRC. The information about the UL traffic periodicities can be provided by the 5GC through the AF which can get the information from the UE application layer. However, this comes with latency and could impact the UL traffic mainly at the start of the traffic.

Proposal 1: The UE to report to RAN the periodicity of the UL traffic as part of the RRC UAI to assist the network, e.g., in the C-DRX configuration and CG configuration.

To assist the network to allocate the right amount of UL resources, the UE can also signal information about the sizes of data bursts and pose/control packets. Since data bursts have variable sizes, the UE can report statistics about packet sizes to assist the network in scheduling and in resource allocation. For example, the UE could be configured with a sliding window to measure and derive average size and variance of data bursts.

The size of the pose/control information packets is fixed and can be reported only once at the start of the traffic.

Proposal 2: The UE to signal statistics about size of Data Burst per QoS flow.

To optimize C-DRX configuration and CG configuration, the UE can also assist the network with information about the jitter. The jitter can be defined as statistics of the positive or the negative deviation compared to the ideal Data Burst arrival time according to the traffic periodicity.

Proposal 3: The jitter is defined as the statistics of the positive and the negative deviation of Data Burst arrival time compared to the ideal Data Burst arrival time.

The UE could be configured with a sliding window to measure the jitter by measuring the arrival time statistics such as average arrival time and variance. This allows the network to adjust C-DRX configuration parameters and the CG configuration parameters. The UE can also report the maximum and the minimum jitter in a specific window.

Proposal 4: The UE to report jitter statistics based on a configured sliding window.

Proposal 5: The UE to report the average, the maximum and the minimum jitter determined in a specified/configured window (e.g., multiple XR periodicities or CG periodicities).

There is no need to report UL arrival time or jitter for pose/control information.

Proposal 6: Jitter is not measured or reported for pose/control information.

The jitter measurement can be reported periodically or only when the jitter is larger than a threshold.

Proposal 7: jitter measurement can be reported:

- Option 1: periodically (e.g., multiple XR periodicities or CG periodicities).

- Option 2: when the jitter is larger than a threshold (e.g., Jitter>X ms ).

# 3		Conclusions

In conclusion, we have the following proposals:

Proposal 1: The UE to report to RAN the periodicity of the UL traffic as part of the RRC UAI to assist the network, e.g., in the C-DRX configuration and CG configuration.

Proposal 2: The UE to signal statistics about size of Data Burst per QoS flow.

Proposal 3: The jitter is defined as the statistics of the positive and the negative deviation of Data Burst arrival time compared to the ideal Data Burst arrival time.

Proposal 4: The UE to report jitter statistics based on a configured sliding window.

Proposal 5: The UE to report the average, the maximum and the minimum jitter determined in a specified/configured window (e.g., multiple XR periodicities or CG periodicities).

Proposal 6: Jitter is not measured or reported for pose/control information.

Proposal 7: jitter measurement can be reported:

- Option 1: periodically (e.g., multiple XR periodicities or CG periodicities).

- Option 2: when the jitter is larger than a threshold (e.g., Jitter>X ms ).

# 4		References

- RP-230786	‘Updated WID on XR Enhancements for NR’　 Nokia, Qualcomm