3GPP TSG-RAN WG2 Meeting #123	R2-2308438

Toulouse, France, 21st – 25th August 2023

Agenda Item:	7.4.2.1

Source:	Ericsson

Title:	Signalling approaches for LTM cell switch execution

Document for:	Discussion, Decision

# 1	Introduction

At the RAN2#121bis meeting, RAN2 received the LS [1] from RAN3 on execution approaches for inter-DU LTM, which reads as follows:

RAN3 has discussed the following two approaches to support inter-DU LTM cell switch during execution. Approach 1: the serving gNB-DU triggers the execution by transmitting LTM cell switch command to the UE and then informs the gNB-CU of the serving cell switch. Approach 2: the serving gNB-DU first requests information from target DU before triggering LTM cell switch command to the UE. RAN3 would like to get feedback from RAN2 about the above-mentioned approaches, and provide suggestion if there is any other possibility identified.

After offline discussions regarding resolution for the RAN3 LS in RAN2, as summarized in [2], no on-line agreement was reached. The proposal in the summary were according to below:

Proposal 5: [18/27] RAN2 reply RAN3 that: 1. Approach 1 is the baseline and should be supported; 2. RAN2 understands followings about Approach 2: - Firstly, it may delay the sending of the LTM cell switch command with the risk of source link failure. - Secondly, it may reduce LTM cell switch latency/interruption by enable the exchange of dynamic information in MAC CE (if any). RAN2 will further discuss whether to include such information in the MAC CE, which requires approach 2. RAN2 will keep RAN3 informed on this. Observation 1: It is better to ask RAN3 to clarify, since there are two different understandings on approach 2 “the serving gNB-DU first requests information from target DU before triggering LTM cell switch command to the UE”: - Approach 2a: The request and feedback can be done early before the source DU cell’s switch decision (e.g. during LTM preparation or early sync phase), which will NOT defer the triggering LTM cell switch command to the UE - Approach 2b: Those information (in MAC CE) has to be requested immediately before triggering LTM cell switch command (e.g. during LTM execution phase), which defers the triggering LTM cell switch command to the UE

At the RAN2#122 meeting, the topic was unfortunately not further discussed. On the other hand, there were some progress in other areas, which be useful to hopefully be able to converge on a common understanding on the signalling approaches.

Hence, this contribution proposes a way forward.

# 2	Discussion

## 2.1	Background: Approaches of LTM cell switch execution

The 38.300 running CR [5]includes the following stage-2 procedure for LTM:

Figure 1. Overall LTM procedure (from 38.300 running CR)

As recap from previous discussions, let’s again have a look at the different approaches for LTM cell switch execution. The differences of the approaches is when the serving DU receives information about LTM cell switch execution and when it retrieves any dynamic information about the target cell just in time for the execution.

In approach 1 the serving DU informs the CU only after sending the LTM cell switch command to the UE and it can therefore not retrieve any dynamic information just before the execution. In Figure 2, approach 1 for LTM cell switch execution is illustrated, and we have put it into the context of the overall LTM procedure.

Figure 2. Approach 1 for LTM cell switch execution in the context of the overall LTM procedure. The serving gNB-DU triggers the execution by transmitting LTM cell switch command to the UE and then informs the gNB-CU of the serving cell switch.

Regarding approach 2, in the RAN2 email discussion summary [2], it was suggested that there may be two understandings of approach 2:

-	Approach 2a: The retrieval of dynamic information about the target cell to be included in the LTM cell switch MAC CE is done early before the source DU cell’s switch decision, which will NOT defer the triggering LTM cell switch command to the UE

-	Approach 2b: The retrieval of dynamic information about the target cell to be included in the LTM cell switch MAC CE is done immediately before triggering LTM cell switch command, which defers the triggering LTM cell switch command to the UE

We have illustrated these two variants in Figure 3 and Figure 4 below.

Figure 3. Approach 2a for LTM cell switch execution in the context of the overall LTM procedure. Early before the LTM cell switch “decision”, the serving gNB-DU first requests information from target DU before triggering LTM cell switch command to the UE.

Figure 4. Approach 2b for LTM cell switch execution in the context of the overall LTM procedure. Immediately after the LTM cell switch “decision”, the serving gNB-DU first requests information from target DU before triggering LTM cell switch command to the UE.

Besides the timing of inter-node signalling during execution, another difference is when the candidate DU becomes aware of which UL TCI state the UE will use and can start listening for the UE.

## 2.2	Inclusion of dynamic information in the LTM cell switch command MAC CE

The selection about signalling approaches is a bit dependent on the content of the LTM cell switch command MAC CE, and in particular existence of any “dynamic” / “time-sensitive” information which may need to retrieved from the target DU. So, to enable an educated discussion, let’s recap what is the status of inclusion of any such information in the LTM cell switch command MAC CE.

At RAN1#112bis, there was some progress on the content of the LTM cell switch command MAC CE and they agreed the following:

Agreement From RAN1 point of view, at least the following information can be included in the cell switch command, which is conveyed by MAC CE - Information to identify the target cell(s) - The details including bit number are designed by RAN2 - TA related information (details up to the discussion in A.I. 9.10.2) - 1 joint or 1 pair of UL and DL unified TCI State index for the target Cell - Note: discussion on target SpCell is not precluded - Active DL and UL BWPs for the target cell - FFS: Triggering of aperiodic TRS transmitted from the target cell - FFS: Triggering the CSI acquisition of the target cell and reporting to the target cell - FFS: Triggering of aperiodic SRS transmission to the target cell - FFS: C-RNTI - FFS: the presence of each field (i.e. always present or configurable)

At RAN2#122, we made the following agreements:

Dynamic grant can be used for RACH-less LTM, for the first UL data transmission to the target cell: - - the UE monitors PDCCH for dynamic scheduling from the target cell, upon LTM cell switch. - - upon cell switch decision, R2 assumes that the source DU informs the target DU about the selected beam, so that the target DU can start scheduling dynamic UL grant. For PDCCH ordered early TA acquisition without RAR, there is no need for UE to maintain the TA timer for candidate cell (i.e. it is NW implementation to determine the TA validity), TA is given in the cell switch MAC CE (when available in the network).

After the RAN2#122 meeting, the email discussion [Post122][058][Mob18] Contents of Cell Switch MAC CE [4] covered various potential information which may be included:

- TA information: Based on RAN1 agreements above, there is a consensus of including this even if there I still discussion about how to convey this info for the different cases.

- Active BWP Id: This should be possible to be included according to RAN1 agreement above but it was subject to

- SCell activation/deactivation: Was discussed in the email discussion and it seems there is a consensus to not add it.

- CFRA resources: No clear outcome?

- UL grant: Seems not to be included according to views in the email discussion.

RAN2 since then received another RAN1 LS [3] and the below agreements are also relevant for the discussion in this contribution:

Agreement A UE can be indicated and activated a single joint TCI state or a pair of UL/DL TCI state in the cell switch command. Agreement For TCI state activation for candidate cell(s) before the cell switch command, - MAC CE is used and the details of MAC-CE for TCI state activation for LTM is up to RAN2 - Further study if PDCCH order for candidate cell(s) can be used

As summary, considering the above agreements and likely outcome of the MAC CE content discussion, the “dynamic” information which can be included in the LTM cell switch command is the following:

- TA-related information

- TCI state index(es) for target cell

- Active BWP for target cell

As it is the serving DU, which transmits the LTM cell switch command, it needs to determine the content of this MAC CE. For the above information, the values need to be determined by the candidate DU:

- TA-related information: this is determined by the candidate DU during TA acquisition and should be transmitted to the UE in the source cell, in the LTM cell switch command. This TA acquisition is performed during the UL pre-sync, which happens before any LTM cell switch decision.

- TCI state index(es) for target cell: Either a single joint TCI state or a pair of UL/DL TCI state.  The source DU must be provided with information so that the serving DU can send TCI state index of the target/candidate cell(s) to the UE even when source cell and target/candidate cell(s) belong to different DUs. In other words, the TCI state index(es) may be selected based on information from by the candidate DU. But this is also the case during the DL pre-sync. In fact, during DL pre-sync, the serving DU needs to be aware of the mapping between TCI state index(es) and SSB-ids of the candidate cells.

- Active BWP for target cell: Even if there is a BWP configuration in the LTM candidate cell configuration, the active UL/DL bandwidth parts is dynamic as it depends on traffic load. By including this into the LTM cell switch command MAC CE we avoid frequent reconfiguration of LTM candidate cells including the subsequent LTM cases. However, for inter-DU it is not clear that the source DU will be able to determine the active BWP and this may require inter-DU interaction just before the LTM cell switch (i.e. after cell switch decision but before LTM cell switch command has been transmitted). On the other hand, as we would have inter-node signalling to transfer information from the candidate DUs to the serving DU during the UL/DL pre-sync phases, the BWP information may be transferred during the UL or DL pre-sync as well and would not delay the cell switch.

Therefore, at some point in time before the LTM cell switch command is transmitted to the UE, the dynamic information needs to be obtained from the candidate DU. This point in time may be during the UL/DL pre-sync.

We observe:

There is a need for the serving DU to obtain, in advance, from the candidate DU, the following dynamic information about the target cell to be included in the LTM cell switch command:

  - TA-related information.

  - TCI state index(es) for target cell.

  - active BWP for target cell.

The inter-node signalling during the UL/DL pre-sync phase may be used to transfer dynamic information also to be included in the LTM cell switch command. In this way, dynamic / time-sensitive information can be included in the LTM cell switch command MAC CE without delaying the LTM cell switch.

## 2.3	Down-selection of approaches

So let’s take another look at the different proposed approaches for execution and see how they would work in the above observations in mind. Based on Observation 2 above, it seems that if the inter-node signalling will be in place to support the UL and DL pre-sync phases. there may not be a strong need to retrieve the at least parts of the dynamic information about the target cell after the LTM cell switch decision has been made. In fact, also the active BWP id may be retrieved during these phases.

What is left as an uncertainty is if the candidate cell gets information about LTM cell switch decision. We note that in approach 1, there could be a race condition for when the candidate DU becomes aware of the execution dependent on UE vs network delays:

- When the UE delay is shorter than the network delay: candidate DU becomes aware of execution by the UE arriving in target cell.

- When the network delay is shorter than the UE delay: candidate DU becomes aware of execution by reception of an F1AP message from the CU.

As we should aim for the fastest possible LTM execution in the UE, we think that even if there are likely cases when the candidate DU is informed about the LTM execution by the CU before the UE arrives, in approach 1 the candidate DU needs to be prepared for and listen for the UE in the target cell already after the LTM candidate cell configuration was performed (e.g. during the UL / DL pre-sync.

Also in approach 1, due to race conditions in this approach, the candidate DU typically need to be prepared for the UE arriving in target cell before the serving DU transmits the LTM cell switch command.

This preparation can even be performed during UL or DL pre-sync.

We think that the discussion about signalling approaches is rather about the inter-node signalling support for transferring the following information between the serving DU and the candidate DU and it should be up to the network implementation when to trigger the transfer of this information (e.g. during UL or DL pre-sync or just before the LTM cell switch command MAC CE is sent to the UE).

- Inter-node information element 1: Direction Serving DU  CU  Candidate DU. “Request for LTM dynamic information for a candidate cell”

- Inter-node information element 2: Direction DU  CU  Serving DU. “Response with LTM dynamic information for a candidate cell” (including e.g. TA, TCI states and/or active BWP)

- Inter-node information element 3: Direction Serving DU  CU  Candidate DU. “Indication of LTM execution to a target cell”

What differs in the approaches are when these inter-node information elements are transferred and in which degree they are sent together. For example, the inter-node information elements 1 and 3 may be sent either together or in separate messages (in any order). In some cases, these items are sent multiple times for the same candidate cell, depending on how early before execution the information is needed and as well, the serving DU may even request the information from multiple candidate DUs in parallel.

In all the identified execution approaches (1, 2a, 2b), the same type of inter-node information needs to be transferred. What sometimes differs is when, e.g. in relation to the LTM cell switch command is transmitted, these are transferred and how they are combined with each other.

We propose:

In the LS reply to RAN3, RAN2 should ask them to specify inter-node signalling support that allows the network to implement different signalling approaches for LTM execution (including as examples approach 1, approach 2a and approach 2b).

In the LS reply to RAN3, RAN2 should give input on which information needs to be transferred between the serving DU and candidate DU to support LTM execution.

RAN2 to agree so send the reply LS to RAN3 in Annex.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	There is a need for the serving DU to obtain, in advance, from the candidate DU, the following dynamic information about the target cell to be included in the LTM cell switch command:

a.	TA-related information.

b.	TCI state index(es) for target cell.

c.	active BWP for target cell.

Observation 2	The inter-node signalling during the UL/DL pre-sync phase may be used to transfer dynamic information also to be included in the LTM cell switch command. In this way, dynamic / time-sensitive information can be included in the LTM cell switch command MAC CE without delaying the LTM cell switch.

Observation 3	Also in approach 1, due to race conditions in this approach, the candidate DU typically need to be prepared for the UE arriving in target cell before the serving DU transmits the LTM cell switch command.

Observation 4	In all the identified execution approaches (1, 2a, 2b), the same type of inter-node information needs to be transferred. What sometimes differs is when, e.g. in relation to the LTM cell switch command is transmitted, these are transferred and how they are combined with each other.

Based on the discussion in the previous sections we propose the following:

Proposal 1	In the LS reply to RAN3, RAN2 should ask them to specify inter-node signalling support that allows the network to implement different signalling approaches for LTM execution (including as examples approach 1, approach 2a and approach 2b).

Proposal 2	In the LS reply to RAN3, RAN2 should give input on which information needs to be transferred between the serving DU and candidate DU to support LTM execution.

Proposal 3	RAN2 to agree so send the reply LS to RAN3 in Annex.

# Annex: Draft reply LS to RAN3

3GPP TSG-RAN WG2 Meeting #123	R2-230xxxx

Toulouse, France, 21st – 25th August 2023

Title:	[DRAFT] Reply LS on Approaches during execution for inter-DU LTM

Response to:	R2-2302458 / R3-230889

Release:	Rel-18

Work Item:	NR_mob_enh2-Core

Source:	Ericsson (to be RAN2)

To:	RAN3

Cc:	RAN1

Contact Person:

Name:	Antonino Orsino

E-mail Address:	antonino.orsino@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

RAN2 thanks RAN3 for their LS on approaches during execution for inter-DU LTM (R2-2302458 / R3-230889).

RAN2 has discussed the two execution approaches identified by RAN3:

- Approach 1: the serving gNB-DU triggers the execution by transmitting LTM cell switch command to the UE and then informs the gNB-CU of the serving cell switch. RAN2 thinks this approach can be used as a baseline, however we note that also in this approach the serving DU may need to request information from the candidate DU in advance.

- Approach 2: the serving gNB-DU first requests information from target DU before triggering LTM cell switch command to the UE. In the RAN2 discussion, it was brought up there may be two variants on this approach, depending whether the information can be provided well in advance or immediately before execution is triggered:

  - Approach 2a: The request and feedback can be done early before the source DU cell’s switch decision, which will NOT defer the triggering LTM cell switch command to the UE

  - Approach 2b: The serving gNB-DU immediately requests information before triggering LTM cell switch command, which defers the triggering LTM cell switch command to the UE

RAN2 thinks the difference between these approaches are small and it could be up to the network implementation when certain information is requested from the candidate DU(s) and when the candidate DU is informed about the execution, in relation to when the LTM cell switch command is transmitted to the UE.

The different approaches may serve as examples in a stage-2 description of how the signalling for LTM cell switch execution can be performed in order to identify necessary support in the inter-node interfaces.

RAN2 further kindly requests RAN3 to study and specify inter-node signalling that enables the network to implement different signalling approaches for LTM execution (including approach 1, approaches 2a-2b) without restrictions on the relative order between inter-node procedures and transmission of the LTM cell switch command.

As additional information to RAN3, RAN2 has identified the following types of information in the inter-node signalling to support inter-DU LTM cell switch execution:

- Inter-node information element 1: Direction Serving DU  CU  Candidate DU. “Request for LTM dynamic information for a candidate cell”

- Inter-node information element 2: Direction DU  CU  Serving DU. “Response with for LTM dynamic information for a candidate cell” (including e.g. TA, TCI states and/or active BWP)

- Inter-node information element 3: Direction Serving DU  CU  Candidate DU. “Indication of LTM execution to a target cell”.

2. Actions:

To RAN3 group.

ACTION: 	RAN2 kindly asks RAN3 to take the above information into account.

3. Date of Next TSG-RAN2 Meetings:

3GPP TSG RAN WG2#123bis	9-13 October 2023			Xiamen, China

3GPP TSG RAN WG2#124	13-17 November 2023		Chicago, US

# References

R2-2302458/R3-230889, LS on Approaches during execution for inter-DU LTM, RAN3, 3GPP TSG RAN WG2#121bis-e, Online, 17th - 26th April, 2023

R2-2304214, Summary of [AT121bis-e][018][eMob] Procedure Consolidation (Huawei), Huawei, HiSilicon, 3GPP TSG-RAN WG2 Meeting #121-bis electronic, April 17th - 26th, 2023

R2-2307020/R1-2306259, LS on beam application time, contents of cell switch command, TCI state activation and UE based TA measurement for LTM, RAN1, 3GPP TSG-RAN WG2 #123, Toulouse, France, 21st - 25th August 2023

R2-2307962, Summary of [Post122][058][Mob18] Contents of Cell Switch MAC CE, Huawei, HiSilicon, 3GPP TSG-RAN WG2 Meeting #123, Toulouse, France, August 21 – 25, 2023

R2-2306957, 38.300 running CR for introduction of NR further mobility enhancements, MediaTek Inc., vivo, 3GPP TSG-RAN WG2 Meeting #122, Incheon, Korea, May 22~ May 26, 2023

R2-2306016, Signalling approaches for LTM cell switch execution, Ericsson, 3GPP TSG-RAN WG2 #122, Incheon, South Korea, 22nd – 26th May 2023