3GPP TSG-RAN WG2 Meeting #123	R2-2308655

Toulouse, France, August 21-25, 2023

Agenda item:	7.18.2

Source:	China Telecom

Title:	Discussion on CP aspects of MT-SDT

Document for:	Discussion

- Introduction

During the RAN2#122 meeting, the control plane aspects of MT-SDT were widely discussed and achieved the following agreements [1]:

- Allow support of only MT-SDT in a cell.  A separate SIB configuration will be introduced.  FFS what is put in there.

- For paging indication signalling, a new list of paging records for MT-SDT indication is optionally included in paging message using non critical extension. Each record in this list optionally includes 1 bit MT-SDT indication. UE identity and access type are not included in paging record of this list.

- gNB may include MT-SDT indication in paging message only if UE’s I-RNTI is included in the paging message (i.e. MT-SDT is only used by RAN initiated paging).

- UE selects '0' as the Access Category when the resumption of the RRC connection is triggered by response to the MT-SDT triggering in a PAGING message

- MT-SDT is only applicable to the legacy MT-Access use case (i.e. it is not applicable to access identities 1, 2 and 11-15).

- SRB2 can be used for MT-SDT (i.e. similar to MO-SDT)

- No additional enhancement is needed specifically for RedCap UE to monitor paging for MT-SDT

- When RRC resume is triggered due to MT-SDT and in the case the condition for paging triggered MT-SDT is not fulfilled, the UE initiates RRC Resume procedure with Resume cause “mt-Access”.

- A separate sdt-RSRP threshold for MT-SDT can be configured, at least in the case where MO-SDT is not configured in the cell.

In this document, we further discuss the remaining issues on control plane common aspects for MT-SDT and provide some proposals.

- Discussion

- SIB1

Per the latest TS38.331, the sdt-ConfigCommon-r17 IE in SIB1 provides MO-SDT parameters. Considering that MT-SDT can be configured without R17 MO-SDT, a separate configuration for MT-SDT should be introduced in SIB1. To support the MT-SDT, the parameters of the R18 SDT configuration can consist the following parameters:

- RSRP threshold-r18

Given that MO-SDT and MT-SDT may have the different desirable TB size and the received power, a new RSRP threshold for MT-SDT should be introduced in sdt-ConfigCommon-r18 IE.

- sdt-DataVolumeThreshold-r18

In R18, RAN2 agreed that if the MT-SDT capable UE has UL data buffered in MAC layer in case MT-SDT paging received, it shall initiate the MO-SDT procedure. This agreement implicates that a new SDT data volume threshold for uplink data packet should be introduced in R18 SDT configuration. Otherwise, in case the UE only receives the R18 SDT configuration without sdt-DataVolumeThreshold, the UE could not evaluate whether to initiate the MO-SDT procedure. Therefore, a new sdt-DataVolumeThreshold-r18 should be introduced in R18 SDT configuration.

Proposal 1: Rel-18 mt-SDT-ConfigurCommon in SIB1 includes new RSRP threshold and sdt-DataVolumeThreshold.

- RRCRelease

In Rel-17 MO-SDT, the following information on MO-SDT is contained in RRC Release

SDT-Config-r17 ::=                  SEQUENCE {

sdt-DRB-List-r17                    SEQUENCE (SIZE (0..maxDRB)) OF DRB-Identity                         OPTIONAL,   -- Need M

sdt-SRB2-Indication-r17             ENUMERATED {allowed}                                                OPTIONAL,   -- Need R

sdt-MAC-PHY-CG-Config-r17           SetupRelease {SDT-CG-Config-r17}                                    OPTIONAL,   -- Need M

sdt-DRB-ContinueROHC-r17            ENUMERATED { cell, rna }                                            OPTIONAL    -- Need S

}

RAN2 agreed that the radio bearer configuration is common for MO and MT-SDT, but the MO-SDT and MT-SDT can be supported separately. To support configuring only MT-SDT mode, it is needed to introduce an independent configuration for MT-SDT in RRCRelease message. Compared to MO-SDT, there is no need to contain SRB2 indication in RRC Release as UE is unclear whether data or signalling arrival in case of paging message received.

Proposal 2: To support MT-SDT, it is needed to introduce independent configuration for MT-SDT in the RRCRelease message.

To support the CG-SDT resource with long periodicity, waiting duration threshold for the next available CG, such as CG-SDT-MaxDurationToNext-CG-Occasion, should be introduced in the RRCRelease message. If the duration to next available CG is larger than the threshold, the next CG is too far to transmit the SDT data.

During the offline discussion, some companies understood that this parameter is per LCH level to support the low latency traffic. From our point of view, however, SDT does not aim to support the low latency services in this release. Therefore, the current per CG configuration is sufficient to support the long CG-SDT resource.

Proposal 3: the current per CG configuration is sufficient to support the long CG-SDT resource.

- Conclusions

The paper analyses CP common aspects related to MT-SDT, and captures the following proposals:

Proposal 1: Rel-18 mt-SDT-ConfigurCommon in SIB1 includes new RSRP threshold and sdt-DataVolumeThreshold.

Proposal 2: To support MT-SDT, it is needed to introduce independent configuration for MT-SDT in the RRCRelease message.

Proposal 3: the current per CG configuration is sufficient to support the long CG-SDT resource.

- References

- Report of 3GPP TSG RAN WG2 meeting #122,  Incheon, KR

- RP-213583, New WI: Mobile Terminated-Small Data Transmission (MT-SDT) for NR