3GPP TSG-RAN WG2 Meeting #123	R2-2308724

Toulouse, France, 21th – 25th August 2023

Agenda Item:	7.9.4

Source:	ASUSTeK

Title:	Discussion on duplicate PDCP PDU discarding for Multi-path transmission Scenario 1

Document for:	Discussion and decision

1	Introduction

So far, RAN2 made the following agreements on Multi-path PDCP duplication:

Support direct bearer (bearer mapped to direct path on Uu), indirect bearer (bearer mapped to indirect path via relay UE), and MP split bearer (bearer mapped to both paths, based on the existing split bearer framework).

For a MP split bearer in scenario 1, one PDCP entity at the remote UE is configured with one direct Uu RLC channel and one indirect PC5 RLC channel.

PDCP DRB duplication is supported for the MP split bearer in Scenario 1 based on the existing framework.

In this document, we discuss an issue of duplicate PDCP PDU discarding in the existing split bearer framework when PDCP duplication is applied to Multi-path transmission.

2	Discussion

The following paragraph is quoted from TS 38.300-h00 [1]:

16.1.3	Packet Duplication

…

Figure 1: Packet Duplication (in R16)

…

When an RLC entity acknowledges the transmission of a PDCP PDU, the PDCP entity shall indicate to the other RLC entity(ies) to discard it. In addition, in case of CA duplication, when an RLC entity restricted to only SCell(s) reaches the maximum number of retransmissions for a PDCP PDU, the UE informs the gNB but does not trigger RLF.

Observation 1	In R16 packet duplication, when an RLC entity acknowledges the transmission of a PDCP PDU, the PDCP entity shall indicate to the other RLC entity(ies) to discard it.

As illustrated in Figure 2, in case of multi-path transmission, an RLC acknowledgement with respect to the transmission of a PDCP PDU over the indirect path only means the U2N relay has received the PDCP PDU. It does not imply the gNB has received the PDCP PDU.

Observation 2	In case of PDCP duplication for Multi-path transmission Scenario 1, an RLC acknowledgement with respect to the transmission of a PDCP PDU over the indirect path does not imply the gNB has received the PDCP PDU.

Relay UE

PDCP in gNB

PDCP in Remote UE

Indirect

LCH

Direct

LCH

ACK

Figure 2: Packet Duplication (in R18)

In this situation, it is not proper for the PDCP entity in the remote UE to indicate to the other RLC entity over the direct path to discard this PDCP PDU. Otherwise, the reliability of high QoS traffic would be degraded. This issue were also raised in the previous RAN2 tdocs [2][3].

Observation 3	In case of PDCP duplication for Multi-path transmission Scenario 1, indicating to the other RLC entity to discard the PDCP PDU over the direct path when the RLC entity over the indirect path acknowledges the transmission of the PDCP PDU would degrade the reliability of high QoS traffic.

To maintain the reliability of high QoS traffic, we propose:

Proposal 1		In case of PDCP duplication for Multi-path transmission Scenario 1, the PDCP entity shall not indicate to the other RLC entity over the direct path to discard the PDCP PDU when the RLC entity over the indirect path acknowledges the transmission of the PDCP PDU.

3 	Conclusion

Based on the discussion in the previous sections, we have the following observations and proposal:

Observation 1	In R16 packet duplication, when an RLC entity acknowledges the transmission of a PDCP PDU, the PDCP entity shall indicate to the other RLC entity(ies) to discard it.

Observation 2	In case of PDCP duplication for Multi-path transmission Scenario 1, an RLC acknowledgement with respect to the transmission of a PDCP PDU over the indirect path does not imply the gNB has received the PDCP PDU.

Observation 3	In case of PDCP duplication for Multi-path transmission Scenario 1, indicating to the other RLC entity to discard the PDCP PDU over the direct path when the RLC entity over the indirect path acknowledges the transmission of the PDCP PDU would degrade the reliability of high QoS traffic.

Proposal 1		In case of PDCP duplication for Multi-path transmission Scenario 1, the PDCP entity shall not indicate to the other RLC entity over the direct path to discard the PDCP PDU when the RLC entity over the indirect path acknowledges the transmission of the PDCP PDU.

4 	References

[1] TS 38.300-h00, NR and NG-RAN Overall Description; Stage 2 (Release 17)

[2] R2-2209375, Discussion on multi-path SL relay 				OPPO

[3] R2-2211414, Considerations on Multipath of Sidelink Relay 		NEC