3GPP TSG-RAN WG2 Meeting #123                                                        R2-2308729

Toulouse, France, 21st-25th, August 2023

Source:              CATT

Title:	             Consideration on Cell Selection/Re-selection on NES cells

Agenda Item:	7.3.4

Document for:	Discussion and Decision

- Introduction

In last RAN2#121and RAN2#122 meeting, cell selection/reselection mechanisms for NES cells were discussed. RAN2 agreed to introduce separate camping restrictions for NES-capable and non-NES UEs. And the following conclusions were achieved.

RAN2#121 meeting Agreements: - RAN2 confirms that non-NES UEs can access to NES cells if NES solution is backwards compatible NES cell definition => We will come back later once we have more clarity on all the solutions and whether and how NES definition will be used.

RAN2#122 meeting Agreements: We will define UE capabilities with signaling. Details are FFS and will be discussed later during the WI phase. Separate camping restrictions for NES-capable and non-NES UEs will be defined. FFS if it is a single bit or more.

In this contribution, we would like to give our further considerations on cell selection/re-selection on NES cells.

- Discussion

Considering R18 NES features are multiple, it is expected that a UE shall not be mandated to support all or none of them, but instead, capabilities should be at feature level. For the separate camping restrictions for NES-capable and non-NES UEs, one way is that the camping restrictions can be designed based on UE’s supported NES feature(s), for example, the network is able to bar the UE not supporting Cell DTX/DRX, or bar the UE not supporting enhanced CHO for NES, which is beneficial to select the suitable UEs for camping on NES cell and give more flexibility to network and UE to support the NES capability. However, there are several NES features and feature combinations for NES, resulting in more bits in SIB or parameters for cell selection/reselection that will introduce unnecessary complexity. Hence, we prefer a simple way to perform cell selection/reselection on UE level. The NES UEs are UEs that support at least one of the R18 NES features, consequently non-NES UEs are UEs that support no NES features (e.g. legacy UEs). In such case, a cell that intends to deploy any NES feature can set the cell Barred in MIB to restrict those (legacy) UEs for camping on the cell. And a UE supporting even only one NES feature (e.g. Cell DTX/DRX) but not supporting others (e.g. enhanced CHO for optimized cell switch-off procedure) will camp on the cell by enhanced cell selection/re-selection (see below). Then if the cell requires supporting enhanced CHO, which can be taken care of by NW during the NAS registration procedure: when gNB is made aware that the enhanced CHO is not supported by the UE, it will handoff the UE from that cell. Although, as mentioned above, the UE capability should be designed at feature level to provide some implementation flexibility in the industry, it should be expected that, in practice, the majority of NES-capable UEs will support all NES features, so that the handoff procedure or other procedure to move the UE away from a camping cell will be infrequent. So, above mechanism is acceptable.

Proposal 1: For separate camping restrictions for NES-capable and non-NES UEs, it is suggested to perform cell selection/reselection on UE level other than on UE’s NES feature level, and the NES capable UE is the UE supporting at least one of the R18 NES feature.

During the SI, the following possible solutions are captured in TR [2] for cell selection/reselection mechanism to prevent non-NES capable UEs from camping while allowing NES-capable UEs to camp on NES cell:

- Alt 1: Use IntraFreqExcludedCellList/InterFreqExcludedCellList.

- Alt 2: Extend CellReservedForOtherUse/CellReservedForFutureUse in the SIB1

- Alt 3: NES-capable UEs ignore the cellBarred in MIB and a new field is introduced in SIB1 for NES-capable UEs to indicate if the cell is allowed or not allowed for NES-capable UEs to camp on.

Alt1 is only applicable to cell re-selection, not cell-selection. Hence, alt1 is not a perfect solution for cell selection/reselection mechanism to prevent non-NES capable UEs from camping while allowing NES-capable UEs to camp on NES cell. Alt 1 is not preferred.

Alt2 preventing legacy UEs accessing the NES cell could be applicable to both cell selection and reselection. But as mentioned by some companies, two bits need to be introduced: 1 bit to enable NPN only NES cell to bar NES capable non-NPN UEs, 1 bit to enable future cell type to bar NES capable NPN only UEs, it also needs to differentiate NES capable non-NPN UE and NES capable NPN only UE, the complexity of this method is high.

For Alt3, it is similar as the connectivity to NTN. The cellBarred field in MIB is ignored by NES-capable UEs and a new field is introduced in SIB1 for NES-capable UEs to indicate if the cell is allowed or not allowed for NES-capable UEs to camp on. If the network wants to prevent non-NES capable UEs from camping while allowing NES-capable UEs to camp on NES cells, the network can set the cellBarred field in MIB to "barred" while set the new field in SIB1 to "not barred" for NES-capable UEs. Alt3 is simple and preferred.

In order to support the separate camping restrictions for NES-capable and non-NES UEs, our proposals are as follows:

Proposal 2: Use the cellBarred in MIB to prevent non-NES capable UEs from camping on NES cell.

Proposal 3: NES-capable UEs ignore the cellBarred in MIB and a new field is introduced in SIB1 for NES-capable UEs to indicate if the cell is allowed or not allowed for NES-capable UEs to camp on.

- Conclusion

According to the analysis in section 2, we provides below observation and proposals:

Proposal 1: For separate camping restrictions for NES-capable and non-NES UEs, it is suggested to perform cell selection/reselection on UE level other than on UE’s NES feature level, and the NES capable UE is the UE supporting at least one of the R18 NES feature.

Proposal 2: Use the cellBarred in MIB to prevent non-NES capable UEs from camping on NES cell.

Proposal 3: NES-capable UEs ignore the cellBarred in MIB and a new field is introduced in SIB1 for NES-capable UEs to indicate if the cell is allowed or not allowed for NES-capable UEs to camp on.

- References

- RP-223540, New WID: Network energy savings for NR

- TR38.864, Study on network energy savings for NR