3GPP TSG-RAN WG2 Meeting #123		R2-2308774

Toulouse, France，21st – 25th Aug, 2023

Source:	vivo

Title:	Discussion on inter-RAT measurements without gap

Agenda Item:	7.25.1

Document for:	Discussion

- Introduction

A RAN4 LS[1] is received regarding Rel-18 signalling for inter-RAT measurements without gap.

1. Overall Description: <skip> RAN4 categorized inter-RAT measurements without gaps into the three subcases below. - Case a-1: inter-RAT NR measurements without gap as there is vacant RF chains for UE measurements - Case b-1: inter-RAT LTE measurements without gap as there is vacant RF chains for UE measurements - Case b-2: inter-RAT LTE measurements without gap when LTE CRS to be measured are fully contained within UE’s active BWP Regarding Case b-1, RAN4 agreed that there is no need to introduce any new Rel-18 signalling from UE. Instead, we agree to reuse the existing NeedForGapNCSG-InfoEUTRAN in TS38.331 v17.1.0 for UE to indicate its support for Case b-1. Regarding Case b-2, RAN4 agreed a new per-UE capability to support case b-2, similar as interFrequencyMeas-NoGap-r16 in Rel-16, to indicate the UE capability supporting inter-RAT LTE measurements without gap when LTE CRS is fully contained in the active BWP. Further RAN4 agrees to introduce the effective measurement window for both Case b-1 and Case b-2. The effective measurement window is used to determine the location of scheduling and measurement restriction and it is configured with measurement window periodicity, time domain offset and window duration. RAN4 would like to respectfully ask RAN2 to take the above information into account when carrying out corresponding RRC signalling design in R18.

In this contribution, we will discuss Rel-18 signalling for inter-RAT measurements without gap.

- Discussion

As per the LS[1],  Rel-18 signalling requirement for inter-RAT measurements without gap is summarized in below table.

| Subcase | Signalling Requirement |
| --- | --- |
| Case b-1: inter-RAT LTE measurements without gap as there is vacant RF chains for UE measurements | - Reuse NeedForGapNCSG-InfoEUTRA-r17 to support Case b-1. no need to introduce any new Rel-18 signalling from UE - Introduce the effective measurement window, including: - measurement window periodicity: 40ms and 80ms, other values; - time domain offset; - window duration: 5ms, other values. |
| Case b-2: inter-RAT LTE measurements without gap when LTE CRS to be measured are fully contained within UE’s active BWP | - use a new per-UE capability to indicate the supporting of inter-RAT LTE measurements without gap when LTE CRS is fully contained in the active BWP, similar as interFrequencyMeas-NoGap-r16 in Rel-16. - introduce the effective measurement window (same as case b-1); |

2.1 Signaling design for Case b-1

NeedForGap info Reporting

In LS[1], it saids “

  - Regarding Case b-1, RAN4 agreed that there is no need to introduce any new Rel-18 signalling from UE. Instead, we agree to reuse the existing NeedForGapNCSG-InfoEUTRAN in TS38.331 v17.1.0 for UE to indicate its support for Case b-1

  - Define requirements for case when UE reports “nogap-noncsg” in NeedForGapNCSG-InfoEUTRA-r17 for indicating no-gap without interruption.”.

Regarding needForGap info reporting signalling for case b-1, we can follow RAN4 agreements to reuse NeedForGapNCSG-InfoEUTRA-r17 with “nogap-noncsg”.

- Regarding needForGap info reporting for case b-1, reuse NeedForGapNCSG-InfoEUTRA-r17 with “nogap-noncsg”.

UE capability reporting

If we reuse NeedForGapNCSG-InfoEUTRA-r17 with “nogap-noncsg” for case b-1, it’s natural to reuse per-UE capability  eutra-NeedForGapNCSG-Reporting-r17 to support Case b-1. The UE capability eutra-NeedForGapNCSG-Reporting-r17 is as below refer to TS38.306.

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| eutra-NeedForGapNCSG-Reporting-r17 Indicates whether the UE supports reporting of the NCSG and measurement gap requirement information for E-UTRA target bands in the UE response to a network configuration RRC message as specified in TS 38.331 [9]. | UE | No | No | No |

- Reuse UE capability eutra-NeedForGapNCSG-Reporting-r17 to support Case b-1.

2.2 Signaling design for Case b-2

NeedForGap info Reporting

For case b-2, the UE can perform inter-RAT LTE measurements without measurement gaps and no interruption if the LTE CRS to be measured is completely contained in the active BWP of the UE. Based on the LS[1], the UE may report “nogap-nointerrupt” in Rel-18 field for case b-2. The Rel-18 indication could be in addition to the legacy NeedForGapNCSG-InfoEUTRA-r17 information.

How does UE determine the gap requirement?  The gap requirement could change in case of different active BWPs. Considering BWP switching, a method is required to determine the needForGap information when multiple BWPs are configured. However, it is not described in the LS[1].

We suggest asking RAN4 regarding how UE determines and reports needForGap info for case b-2.

- Ask RAN4 regarding how UE determines needForGap info for case b-2

UE capability reporting

RAN4 agrees to introduce a new per-UE capability to support case b-2 similar to Rel-16 inter-frequency measurement without gap interFrequencyMeas-Nogap-r16.

We may define the UE capability as below.

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| interFrequencyMeas-NoGap-r16 Indicates whether the UE can perform inter-frequency SSB based measurements without measurement gaps if the SSB is completely contained in the active BWP of the UE as specified in TS 38.133 [5]. If this parameter is indicated for FR1 and FR2 differently, each indication corresponds to the frequency range of cells to be measured. | UE | No | No | Yes |
| interRAT-LTE-Meas-NoGapNoInterrupt-r18 Indicates whether the UE can perform inter-RAT LTE measurements without measurement gaps and no interruption if the LTE CRS to be measured is completely contained in the active BWP of the UE as specified in TS 38.133 [5]. | UE | No | FFS | FFS |

- Define per-UE capability, e,g. interRAT-LTE-Meas-NoGapNoInterrupt-r18,  to support Case b-2

2.3 Configuration of inter-RAT LTE measurements without gap.

The effective measurement window is used to determine the location of scheduling and measurement restriction and it is configured with measurement window periodicity, time domain offset and window duration.

It’s not clear under which condition for Network to configure the effective measurement window. There are several understandings:

  - Understanding#1: If the UE indicates the supporting of Case b-1 or b-2.

  - Understanding#2: if the UE has reported the NeedForGap info for Case b-1 or b-2

- Clarify the understandling with RAN4 regarding the configuration of the effective measurement window:

  - Understanding#1: If the UE indicates the supporting of Case b-1 or b-2. or

  - Understanding#2: if the UE has reported the NeedForGap info for Case b-1 or b-2

Furthermore, in case of understanding#1, Network may configure the effective measurement window for the UE supporting Case b-1 or Case b-2.  It’s decided to introduce a new UE capability for Case b-2, but not a new capability for Case b-1. The UE capability eutra-NeedForGapNCSG-Reporting-r17 supports “gap” and “nogap” cases. It’s not enough for Network to identify the Case b-1 capability. If we decide to reuse eutra-NeedForGapNCSG-Reporting-r17 for Case b-1, then a new UE capability for the effective measurement window may be needed.

We think further clarification from RAN4 is needed for signaling design.

- Clarify with RAN4 whether a new UE capability for the effective measurement window is needed

- Conclusion

Based on the discussion, we have the following proposals:

Proposal 1: Regarding needForGap info reporting for case b-1, reuse NeedForGapNCSG-InfoEUTRA-r17 with “nogap-noncsg”.

Proposal 2: Reuse UE capability eutra-NeedForGapNCSG-Reporting-r17 to support Case b-1.

Proposal 3: Ask RAN4 regarding how UE determines needForGap info for case b-2

Proposal 4:	Define per-UE capability, e,g. interRAT-LTE-Meas-NoGapNoInterrupt-r18,  to support Case b-2

Proposal 5:	Clarify the understandling with RAN4 regarding the configuration of the effective measurement window:

-	Understanding#1: If the UE indicates the supporting of Case b-1 or b-2. or

-	Understanding#2: if the UE has reported the NeedForGap info for Case b-1 or b-2

Proposal 6:	Clarify with RAN4 whether a new UE capability for the effective measurement window is needed

- References

- R2-2307041(R4-2310158) LS on inter-RAT measurements without gaps