3GPP TSG-RAN WG2 Meeting #123											   R2-2308820

Toulouse, France, 21-25 Aug, 2023

Source: 	Xiaomi

Title:  	Discussion on CHO with candidate SCG(s)

Agenda Item:	7.4.4

Document for:	Discussion and Decision

Introduction

According to the WID [1] of the Rel-18 mobility enhancements as quoted below, one objective of CHO including target MCG and candidate SCGs for CPC/CPA is included.

- To specify CHO including target MCG and candidate SCGs for CPC/CPA in NR-DC [RAN3, RAN2] - CHO including target MCG and target SCG is used as the baseline

In previous meetings, the following agreements have been reached for CHO including target MCG and candidate SCGs for CPC/CPA in NR-DC [2-6].

Agreements_RAN2#119-e

CHO configuration referring to or including CPC/CPA configuration (intended to be applicable together) can be supported.

FFS: When triggering CHO, UE perform CPC/CPA configuration to start CPC/CPA evaluation, FFS if CHO evaluation and CPC/CPA evaluation is concurrent or sequential.

Agreements_RAN2#120

Execution order: the UE doesn’t execute CPC/CPA unless CHO condition is fulfilled (regardless parallel or sequential evaluation)

Agreements_RAN2#121

RAN2 agrees to support the simultaneous evaluation of CHO and CPC in Rel-18

The UE should not need to unpack any of the nested conditionalconfiguration containers in order to measure, acc to agreement above

Agreements_RAN2#121bis-e

- For the CHO+CPC case:

When both CHO and CPC conditions are met, both CHO and CPC cell change is executed.

Baseline: The UE waits until both CHO and CPC conditions are met (always). (furthermore, it is assumed that if needed the network can provide a complementary CHO-only configuration, to avoid failures in deployments where failure would otherwise be likely to happen).

Alternative: FFS if When CHO condition is met, but CPC condition is not met, CHO execution is triggered (and somehow source SCG can be released). IF allowed in the new configuration the UE may continue evaluation of CPC/CPA conditions.

Agreements_RAN2#122

P3: The CHO execution conditions (for candidate PCells) and CPA/CPC execution conditions (for candidate PSCells) are provided based on the source MeasConfig.

P4: For CHO execution conditions, the source MN determines the execution conditions on candidate PCells, based on the source MCG MeasConfig.

P5: For CPA/CPC execution conditions, the candidate MN determines the parameters of the execution conditions for candidate PSCells (e.g. event A4 threshold).

P6: The candidate MN informs the source MN about the prepared candidate PSCells and parameters of the associated execution conditions (e.g. event A4 threshold). According to the received information from the candidate MN, the source MN generates the corresponding execution conditions based on the source MCG MeasConfig to the UE.

FFS how, if to support event A3/A5.

P8: For CHO with candidate SCGs for CPA/CPC, the RRCReconfigurtaion message in one CHO container includes one MCG configuration and one SCG configuration (i.e. similar to Rel-17 CHO with SCG configuration).

P9: The execution conditions associated with one CHO container includes both CHO execution condition(s) and CPA/CPC execution condition(s), i.e. triggering conditions on both candidate PCell and candidate PSCell.

P10: If there are multiple candidate PSCells associated with one candidate PCell, the NW can provide multiple CHO configurations for the same candidate PCell, i.e. each one contains one MCG configuration (for the same candidate PCell) and one SCG configuration (for different candidate PSCell).

P12: When the CPA/CPC execution condition is met but no CHO execution condition is met, the UE continues to evaluate both CHO and CPA/CPC execution conditions.

For CHO+CPC we only consider execution when BOTH conditions are met.

- (When the CHO execution condition is met but no CPC execution condition is met, if there is an available CHO-only or Rel-17 CHO with SCG configuration for which the CHO condition is met, the UE performs the CHO-only or Rel-17 CHO with SCG execution, and THUS the network can handle such situation by providing proper configurations).

In the 37.340 running CR [7], CHO with candidate SCG(s) is defined for CHO including target MCG and candidate SCGs for CPC/CPA in NR-DC. In this contribution, we provide our understandings on CHO with candidate SCG(s) in NR-DC.

Discussion

Preparation for CHO with candidate SCG(s)

In Rel-16 and Rel-17, CHO, CPA, intra-SN CPC, and inter-SN CPC including both MN/SN initiated are supported in the current specification. To support the configuration of CHO and MR-DC simultaneously with good throughput, objective 4 of CHO including target MCG and multiple candidate SCGs for CPC/CPA has been discussed in Rel-18. For the flexibility of CHO with CPC/CPA, CHO combined with all CPA and CPC scenarios can be considered, including CHO with candidate SCG(s) for CPA and CHO candidate SCG(s) for MN/SN initiated CPC (including both intra-SN and inter-SN CPC)

In current spec, for SN initiated intra-SN CPC, MN involvement is not needed. However, to support CHO with candidate SCG(s) for CPC, CPC without MN involvement is impossible because of the association between CHO and CPC. And MN need to know whether the CHO is configured with PSCell change, even though it is intra-SN PSCell change.

Observation 1: MN involvement is always needed to support CHO with candidate SCG(s) for CPA and CPC.

For the preparation of Rel-18 CHO with candidate SCG(s), it is nature the source MN initiates the preparation of CHO and decides the candidate PCells and the associated CHO conditions. However, whether and which candidate SCG(s) for CPA/CPC is needed or not can depends on the source MN, or the candidate MN, or the source SN, if configured. RAN2 should discuss which nodes can initiate the CPAC for CHO with candidate SCG(s). The following cases can be considered for the preparation of the R18 CHO with candidate SCG(s).

- Case 1: the MN initiates CHO and the MN initiates associated CPAC

- Case 2: the MN initiates CHO and the SN initiates associated CPAC

For Case 1, the following principle can be considered:

- The source MN can initiate the preparation of the R18 CHO with candidate SCG(s).

- The parameters of the CPAC conditions is determined by the candidate MN and according to the received information from the candidate MN, the source MN generates the corresponding execution conditions based on the source MCG MeasConfig to the UE, which has been agreed in RAN2#122[6].

- The candidate MN recommends the candidate PSCells to the candidate SN.

Proposal 1: The source MN can initiate the preparation of the Rel-18 CHO with candidate SCG(s).

Proposal 2: For Rel-18 CHO with candidate SCG(s), the candidate MN can recommend the candidate PSCells to candidate SN.

However, MN initiated CPC can only be configured for load balance with event A4. For better coverage, option 2 is needed to support CHO with candidate SCG(s) for SN initiated CPC, which can be configured with CondEvent A3/A5 for PSCell change. And the source SN can generate the A3/A5 based CPC conditions for candidate PSCells, based on the source SCG MeasConfig.

For CHO with candidate SCG(s) for SN initiated CPC, the source SN can recommend the candidate PSCells to candidate SN. A list of PSCell candidates recommended by the source SN can be transmitted in the following path: “the source SN -> the source MN -> the candidate MN -> the candidate SN”

Proposal 3: CHO with candidate SCG(s) for SN initiated CPC can be considered in Rel-18. CondEventA3/A5 can be configured for the CPC conditions on candidate PSCells in CHO with candidate SCG(s) and the source SN generates the A3/A5 based conditions based on the source SCG MeasConfig.

Proposal 4: For Rel-18 CHO with candidate SCG(s), the candidate PSCells can also be recommended by the source SN.

Failure handling for CHO with candidate SCG(s)

For failure handling in Rel-18 CHO with candidate SCG(s), some remaining issues can be discussed:

1) Whether the legacy CHO recovery mechanism applies to the configuration for CHO with candidate SCG(s).

In the legacy procedure, the configuration for Rel-16 CHO or Rel-17 CHO with target SCG can be applied for failure recovery. Similar to Rel-17 CHO with target SCG, the candidate cell configuration of Rel-18 CHO with candidate SCGs also includes one MCG configuration and one SCG configuration, which also can be applied for CHO recovery.

To support multiple candidate PSCells associated with one candidate PCell in Rel-18, RAN2 has agreed the network can provide multiple CHO configurations for the same candidate PCell. If more than one CHO with candidate SCGs are configured associated with the selected candidate PCell, UE can select one CHO with candidate SCG configuration for CHO recovery based on UE implementation.

Proposal 5: Rel-18 CHO with candidate SCG(s) can be triggered for CHO recovery. If multiple conditional configurations are configured for the selected PCell, it is up to UE implementation to select one configuration for CHO recovery.

2) Whether to stop conditional reconfiguration evaluation for CHO with Candidate SCG(s) upon initiating SCG failure information procedure.

In the current spec, UE only stops conditional reconfiguration evaluation for CPC upon initiating SCG failure information procedure. For Rel-18 CHO with candidate SCG(s), network can reconfigure or remove candidate SCGs and associated configurations when SCG failure to avoid unnecessary evaluation. Hence, it is not needed UE autonomously stops the evaluation for CHO with candidate SCG(s) for SCG failure. UE can continue conditional reconfiguration evaluation for CHO with Candidate SCG(s) upon initiating SCG failure information procedure and no spec change is needed for the issue.

Proposal 6: UE can continue conditional reconfiguration evaluation for CHO with candidate SCG(s) upon initiating SCG failure information procedure and no spec change is needed for Rel-18 CHO with candidate SCG(s) in the procedure of SCG failure information.

Conclusions

According to the analysis given above, we have the following observations and proposals:

Preparation for CHO with candidate SCG(s)

Observation 1: MN involvement is always needed to support CHO with candidate SCG(s) for CPA and CPC.

Proposal 1: The source MN can initiate the preparation of the Rel-18 CHO with candidate SCG(s).

Proposal 2: For Rel-18 CHO with candidate SCG(s), the candidate MN can recommend the candidate PSCells to candidate SN.

Proposal 3: CHO with candidate SCG(s) for SN initiated CPC can be considered in Rel-18. CondEventA3/A5 can be configured for the CPC conditions on candidate PSCells in CHO with candidate SCG(s) and the source SN generates the A3/A5 based conditions based on the source SCG MeasConfig.

Proposal 4: For Rel-18 CHO with candidate SCG(s), the candidate PSCells can also be recommended by the source SN.

Failure handling for CHO with candidate SCG(s)

Proposal 5: Rel-18 CHO with candidate SCG(s) can be triggered for CHO recovery. If multiple conditional configurations are configured for the selected PCell, it is up to UE implementation to select one configuration for CHO recovery.

Proposal 6: UE can continue conditional reconfiguration evaluation for CHO with candidate SCG(s) upon initiating SCG failure information procedure and no spec change is needed for Rel-18 CHO with candidate SCG(s) in the procedure of SCG failure information.

Reference

[1] RP-231475, MediaTek Inc., “Revised WID on Further NR mobility enhancements”

[2] R2-2209301, “Report of 3GPP TSG RAN WG2 meeting #119-e”

[3] R2-2300002, “Report of 3GPP TSG RAN WG2 meeting #120”

[4] R2-2302401, “Report of 3GPP TSG RAN WG2 meeting #121”

[5] R2-2306553, “Report of 3GPP TSG RAN WG2 meeting #121bis-e”

[6] R2-xxxxxxx, “Report of 3GPP TSG RAN WG2 meeting #122”

[7] R2-2306952, “37.340 running CR for introduction of NR further mobility enhancements”