3GPP TSG-RAN WG2 Meeting #122	R2-2308888

Incheon, South Korea, 22nd -26th of May 2023

Agenda item:	7.4.2.1

Source:	Nokia, Nokia Shanghai Bell

Title:	Further details on TA Acquisition and Maintenance in LTM

WID/SID:	NR_Mob_enh2-Core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

The R2#122 agreements in [1] and R2#121bis in [2] are provided in the following tables:

RAN2#122 For early TA acquisition for candidate Cells - For PDCCH ordered early TA acquisition without RAR, there is no need for UE to maintain the TA timer for candidate cell (i.e. it is NW implementation to determine the TA validity), TA is given in the cell switch MAC CE (when available in the network). - RAN2 doesn’t see a need for a solution with RAR in for Rel-18. - Observation: Without RAR (without UE maintaining TA), the UE will need to do RACH for link recovery and/or conditional (if supported), which is acceptable in Rel-18 - The UE determines to trigger RACH-less cell switch in MAC layer, if the LTM cell switch MAC CE provides the TA value (no RAR is assumed). - We send LS to R1

R2#121bis - With the assumption that the UE will skip RACH in the target cell if a TA value is given in the cell switch command: It is FFS if the following TA values can be given to the UE: - Value 0, - Value indicating that the UE shall apply the TA of one source cell. - In RACH-based LTM, the target cell is aware of the UE’s arrival based on the reception of preamble in CFRA and on the reception of Msg3/MsgA in CBRA, like the legacy HO. - From RAN2 perspective, to enable shared preamble resource among multiple UEs, it is beneficial that the information that identifies the allocated CFRA resource (i.e., SS/PBCH index, RACH occasion, and Random Access Preamble index) can be indicated in the PDCCH order (as legacy intra-cell PDCCH order). - RRC RACH configuration for early TA acquisition (e.g., including whether RAR needs to be received) is specific per target cell and is signalled separately (separate IEs) from the candidate cell configuration (the part that need to be applied at cell switch).

Additionally, RAN1#113 has made the following relevant agreements [3]:

Agreement Confirm the following Working Assumption, and sent LS to RAN4 to clarify the feasibility of supporting this mechanism Working Assumption From RAN 1 perspective, UE-based TA measurement (UE derives TA based on Rx timing difference between current serving cell and candidate cell as well as TA value for the current serving cell) is supported. - Corresponding UE capability is to be introduced to support UE-based TA measurement - For a UE reports support of this capability, configuration of UE-based TA measurement is supported - FFS: other impacts on RAN1 spec For PDCCH ordered RACH mechanism in R18 LTM, when reception of RAR is configured, - the UE stores(remembers/maintains/handles) a TA for at least one candidate cell - storing(remembering/maintaining/handling) corresponding TAs for more than one candidate cell is up to UE capability - detailed number of candidate cell is up to UE capability Agreement From RAN 1 perspective, without performing PDCCH-ordered RACH for candidate cell(s), RACH-less mechanism can be supported by indicating TA value of target cell as TA=0 or keeping the same value as source cell in cell switch command. - Note 1: this doesn’t mean to preclude TA values other than 0 and the same value as source cell in cell switch command for PDCCH-ordered RACH when RAR is not configured for the PDCCH order. - Note 2: The feasibility and signalling can be further concluded by RAN2

Stage-2 description of LTM has been drafted and included in [4]. In this contribution, the early UL synch for the candidate target cell is analysed and the ways to perform TA acquisition and maintain the TA are presented for discussion and decision.

# 2		Handling of TA Acquisition in LTM

In the Running CR for TS 38.300 [4] the following message exchange has been captured:

Figure x. Signaling procedure for LTM The procedure for LTM is as follows. 1. The UE sends a MeasurementReport message to the gNB. The gNB decides to use LTM and initiates candidate cell(s) preparation. 2. The gNB transmits an RRCReconfiguration message to the UE including the LTM candidate cell configurations of one or multiple candidate cells. 3. The UE stores the LTM candidate cell configurations and transmits a RRCReconfigurationComplete message to the gNB. 4a. The UE [may] performs DL synchronization with candidate cell(s) before receiving the cell switch command. Editor’s note: DL synchronization for candidate cell(s) before cell switch command is supported, at least based on SSB. FFS on the necessity, requirements and details of the DL synchronization mechanism. 4b. The UE [may] performs early TA acquisition with candidate cell(s) requested by the network before receiving the cell switch command. This is done via CFRA triggered by a PDCCH order from the source cell, following which the UE sends preamble towards the indicated candidate cell. In order to minimize the data interruption of the source cell due to CFRA towards the candidate cell(s), the UE doesn’t receive RAR for the purpose of TA value acquisition and the TA value of the candidate cell is indicated in the cell switch command. The UE doesn’t maintain the TA timer for the candidate cell and relies on network implementation to guarantee the TA validity. Editor’s note: FFS whether RAR-based solution is supported in Rel-18. 5. The UE performs L1 measurements on the configured candidate cell(s), and transmits lower-layer measurement reports to the gNB. L1 measurement should be performed as long as apply the RRC reconfiguration in step 2. Editor’s note: FFS whether the lower-layer measurement reports are carried on L1 or MAC. Editor’s note: The order of DL/UL sync (step 4a/4b) and L1 measurement (step 5) is not defined and subject to change. 6. The gNB decides to execute cell switch to a target cell, and transmits a MAC CE triggering cell switch by including the candidate configuration index of the target cell. The UE switches to the target cell and applies the configuration indicated by candidate configuration index.. Editor’s note: FFS how beam indication is done. 7. The UE performs the random access procedure towards the target cell, if UE does not have valid TA of the target cell. 8. The UE completes the LTM cell switch procedure [by sending RRCReconfigurationComplete message to target cell]. If the UE has performed a RA procedure in step 7 the UE considers that LTM execution is successfully completed when the random access procedure is successfully completed. For RACH-less LTM, the UE considers that LTM execution is successfully completed when the UE determines that the network has successfully received its first UL data. Editor notes: R2 assumes RRCReconfigurationComplete message is always sent at each LTM execution. The steps 4-8 can be performed multiple times for subsequent LTM cell switch using the LTM candidate cell configuration(s) provided in step 2.

## 2.1	TA Acquisition by the UE

RAN1 and RAN2 have agreed to support RACH-less handover for LTM. Additionally, it has been assumed that enhancements for LTM before cell switch, e.g., pre-synchronization, TA handling can be considered as well. In this section, we elaborate on the details of three distinctive cases for RACH-less LTM:

- Case 1: RACH-less LTM where the Timing Advance (TA) of the target cell is known by the network in advance. In this case, the UE can skip random access and apply the TA value for the target cell.

- Case 2: RACH-less LTM where the TA of the target cell is unknown and needs to be acquired using early TA acquisition before the cell switch.

- Case 3: RACH-less LTM where the TA of the target cell is a UE-based TA measurement.

### 2.1.1	Case 1: Timing Advance is known by the network

In many cases, the TA of the target cell is known by the network. Following Rel. 14 LTE RACH-less paradigm, the network can indicate to the UE whether the TA of the target cell is the same as that of source cell of handover (in case the source and target cells are co-located) or equal to 0.

Proposal 1: If the TA of the target cell is the same as that of the source cell or equal to 0, it can be configured in the RRC Reconfiguration or communicated to the UE in the cell switch command.

The way the UE considers the TA of the target cell if it is the same to that of any serving cell. The network can share the TA value or the this can be done using the TAG-ID.

Proposal 2: Cell switch command can contain TA value of the target cell to enable using RACHless access to the target cell if the TA is known. Solutions using TAG-ID to indicate which timing advance value to use during the cell switch for the UE are FFS.

For the prepared target cells with a known TA, the serving DU should avoid triggering the TA acquisition procedure. As such, the serving DU should be informed if a prepared target cell in a different DU has a known TA. For instance, when preparing a target cell, the CU can inform the serving DU whether the prepared target cell has a known TA and may provide the TA value such that serving DU can provide it to the UE in the MAC CE triggering the cell switch.

Proposal 3: From RAN2 point of view it is beneficial if the serving DU is aware if a prepared target cell has a known TA. Signalling is to be defined by RAN3.

In the RAN2#119bis, it has been agreed to support LTM for CA scenarios wherein candidate target PCell/SCell for LTM can be the current SCell/PCell. In these cases, the TA is known, and the UE can apply the TA value that was acquired for a current SCell/PCell before the cell switch as TA for the target PCell/SCell. The handling of timeAlignmentTimer (TAT) which controls how long the MAC entity considers the TA value to be valid is FFS.

Proposal 4: In CA, the UE can apply the timing advance that is acquired for a current SCell/PCell before the cell switch as TA for the target PCell/SCell. The handling of TAT is FFS.

### 2.1.2	Case 2: Early acquisition of TA before Cell Switch

In case the TA of the target cell is not known, the UE can acquire the TA of the target cell before the cell switch such that it can perform later RACH-less LTM. As captured in the running CR TS 38.300 [4], The UE [may] performs early TA acquisition with candidate cell(s) requested by the network before receiving the cell switch command. This is done via CFRA triggered by a PDCCH order from the source cell, following which the UE sends preamble towards the indicated candidate cell. In order to minimize the data interruption of the source cell due to CFRA towards the candidate cell(s), the UE doesn’t receive RAR for the purpose of TA value acquisition and the TA value of the candidate cell is indicated in the cell switch command. The UE doesn’t maintain the TA timer for the candidate cell and relies on network implementation to guarantee the TA validity.

According to the above statement, the serving DU may decide, e.g., based on L1 measurements of the UE the target cell to which it should trigger the early TA acquisition.

Currently, the components contributing to the interruption time caused by UL synchronization in CFRA based RA procedure are summarized as follows:

- Time for the UE to acquire the RACH occasion.

- Time to send PRACH preamble.

- Time to decode and obtain the TA from the cell switch command.

The following solutions are possible according to the latest RAN1 and RAN2 agreements.

The TA value of candidate cell is indicated in cell switch command.

  - Steps:

    - UE is served by the source cell while waiting for the RACH occasion.

    - UE switches from the serving cell to send PRACH Preamble.

    - After sending the PRACH preamble, the UE resumes the data transmission/reception to/from the source cell while waiting for the cell switch command.

    - UE receives the cell switch command (MAC CE) including the TA.

  - Drawbacks:

    - New procedure (requiring the introduction of a new MAC CE (in the list of the already available MAC CEs) is needed to be introduced

    - UE is not aware of the TA and the network needs to maintain its validity, which using the current framework (e.g. with TAT only) may be not possible.

Observation 1: Since the UE does not have the TA value, maintenance of the TA validity in the network needs supported.

### 2.1.3 Case 3: Timing advance is estimated by the UE

RAN1 has agreed as a working assumption to support the timing advance estimation at the UE side. The timing advance estimation by the UE simplifies the early timing advance acquisition procedure. However, UE-based TA measurement needs to be supported as a capability from the UE. Additionally, in case this procedure is supported, the network needs to make sure that this procedure is working reliably. RAN4 has already received an LS for this and will indicate whether this procedure can be supported reliably with related performance requirements.

Proposal 5: RAN2 to discuss how can the UE-based TA measurement can be supported after RAN4 agrees to support requirements for proper operation of this procedure.

## 2.2	Open issues

2.2.1 Validity Check of Acquired TA

The acquired TA value is applied by the UE after receiving the LTM cell switch command from the Source gNB-DU. However, given that there is some time between the reception of the TA value by the Source gNB-DU and cell switch command being issue to the UE, it is useful that the UE or the network check if the acquired TA value can be still useful to be used for LTM execution.

Observation 2: The source gNB-DU needs to check and maintain the validity of the acquired TAs for the candidate target cells.

To perform validity check, there are multiple options, and some options require RAN3 to specify signalling procedures.

Option 1: The Candidate gNB-DU shares the time duration of the TA validity timer with the Source gNB-DU, which runs the timer to check when the TA validity expires.

Option 2: Candidate gNB-DU does the validity check and indicates to source gNB-DU when TA value expires. Upon receiving the indication from the Candidate gNB-DU, the Source gNB-DU can re-trigger the TA acquisition via PDCCH order.

Option 3: The Source gNB-DU re-trigger the TA acquisition by checking if the received measurements deviate significantly from the ones when the last TA value has been acquired.

As can be seen, option 3 has the least signalling impact, whereas option 1 and option 2 have RAN3 signalling impact which may not be desired at this stage of the work item. Based on it, we propose the following:

Proposal 6:  The Source gNB-DU may re-trigger the TA acquisition by checking if the received measurements deviate significantly from the ones when the last TA value has been acquired. The decision to re-trigger the TA acquisition is captured by the specification to ensure fast cell switch.

2.2.2 Forwarding of Acquired TAs

The acquired TAs for a set of prepared target cells can be useful after the UE performs an LTM switch. In LTM dynamic switching, the LTM configurations of prepared target cells are kept at the UE and at the network side. As such, keeping the acquired TAs at the network side after the LTM switch would be useful to save the signalling overhead associated with TA acquisition. This can be achieved by having the source DU sharing with the target DU the acquired TA values of the prepared target cells for subsequent LTM switches.

Proposal 7: Source DU shares with the target DU(s) (via CU) the acquired TA values of prepared target cells for subsequent LTM switches.

2.2.3 Known TA value.

There are cases in which the TA value of a given cell is known beforehand (e.g., TA value = 0 in small cell deployment cases or TA value is same as serving cell as part of TAG configuration). This aspect could be taken into consideration already at the preparation phase of LTM.

For the prepared target cells with a known TA value, the Source gNB-DU should avoid triggering the TA acquisition procedure. To do so, the Source gNB-DU should be informed beforehand if a prepared target cell in a different Candidate gNB-DU has a known TA value. For instance, when preparing a target cell for LTM at a Candidate gNB-DU, the gNB-CU can inform the Source gNB-DU whether the prepared target cell has a known TA and may provide the TA value, such that source gNB-DU can provide it to the UE in the MAC CE triggering the cell switch.

Proposal 8: gNB-CU informs the Source gNB-DU if a prepared target cell has a known TA as part of LTM configuration in the preparation phase.

2.3 Assignment of Contention-Free Random Access Preambles to the DU

To our understanding the configuration of early TA acquisition resources to the UE is separate from the cell change resources, as agreed in RAN2#121bis.

However, if dedicated resources is used per UE for TA acquisition, the prepared candidate gNB-DU(s) cell(s) needs to reserve CFRA preambles for each UE and for a long-time duration if dynamic switching is enabled. This reduces the network flexibility in assigning CFRA preambles to different UEs, especially in scenarios where many UEs are configured with LTM.

Observation 3: In scenarios where many UEs are configured with LTM, reserving unique CFRA preamble for each UE will limit the network flexibility in assigning CFRA preambles.

From RAN2 perspective, it is already assumed that the candidate gNB-DU cell can assign a pool of CFRA preambles that can be used by the UEs of any source cell. This is highlighted in bold text below:

RAN2#121bis-e agreement: From RAN2 perspective, to enable shared preamble resource among multiple UEs, it is beneficial that the information that identifies the allocated CFRA resource (i.e., SS/PBCH index, RACH occasion, and Random Access Preamble index) can be indicated in the PDCCH order (as legacy intra-cell PDCCH order).

Although shared preamble resource among multiple UEs is already assumed, we prefer to make an explicit agreement on it.

Proposal 9: Target DU may assign either 1) a set of CFRA preambles per UE (as in legacy) or 2) a pool of CFRA resources for TA acquisition or RACH-based LTM that can be used by all UEs served by different cells.

However, there is no restriction that the CFRA resource cannot be shared among multiple UEs being served by different source gNB-DUs. This results in the problem that different source gNB-DUs can decide to use the same CFRA resource for different UEs which may result in PRACH collision. In other words, CFRA becomes Contention Based Random Access (CBRA) which should be avoided.

Observation 4: In case of shared CFRA resources, different source gNB-DUs may decide to use the same CFRA resource for different UEs which may result in PRACH collision.

To avoid the PRACH collision, it is important that the sharing of CFRA resources among UEs being served by cells in different source gNB-Dus is restricted. In other words, it is preferable that the CFRA resources are only shared by UEs which are served by cells in same gNB-DU. In this way, by implementation, a gNB-DU can only allocate one CFRA resource to a UE at a given time and avoid PRACH collision without the need of extra coordination between multiple DUs. The latter results in extra signalling overhead and handover latency.

Proposal 10: The sharing of CFRA resources among UEs being served by cells in different source DU is not allowed. In other words, only sharing of CFRA resources among UEs being served by cells in same source DU is allowed.

Given that it may have a RAN3 impact, we propose to send an LS to RAN3.

Proposal 11: Send an LS to RAN3 indicating the above proposal as agreement.

# 3	Conclusion

This document has made the following observations and proposals:

Observation 1: Since the UE does not have the TA value, maintenance of the TA validity in the network needs supported.

Observation 2: The source gNB-DU needs to check and maintain the validity of the acquired TAs for the candidate target cells.

Observation 3: In scenarios where many UEs are configured with LTM, reserving unique CFRA preamble for each UE will limit the network flexibility in assigning CFRA preambles.

Observation 4: In case of shared CFRA resources, different source gNB-DUs may decide to use the same CFRA resource for different UEs which may result in PRACH collision.

And the following proposals:

Proposal 1: If the TA of the target cell is the same as that of the source cell or equal to 0, it can be configured in the RRC Reconfiguration or communicated to the UE in the cell switch command.

Proposal 2: Cell switch command can contain TA value of the target cell to enable using RACHless access to the target cell if the TA is known. Solutions using TAG-ID to indicate which timing advance value to use during the cell switch for the UE are FFS.

Proposal 3: From RAN2 point of view it is beneficial if the serving DU is aware if a prepared target cell has a known TA. Signalling is to be defined by RAN3.

Proposal 4: In CA, the UE can apply the timing advance that is acquired for a current SCell/PCell before the cell switch as TA for the target PCell/SCell. The handling of TAT is FFS.

Proposal 5: RAN2 to discuss how can the UE-based TA measurement can be supported after RAN4 agrees to support requirements for proper operation of this procedure.

Proposal 6:  The Source gNB-DU may re-trigger the TA acquisition by checking if the received measurements deviate significantly from the ones when the last TA value has been acquired. The decision to re-trigger the TA acquisition is captured by the specification to ensure fast cell switch.

Proposal 7: Source DU shares with the target DU(s) (via CU) the acquired TA values of prepared target cells for subsequent LTM switches.

Proposal 8: gNB-CU informs the Source gNB-DU if a prepared target cell has a known TA as part of LTM configuration in the preparation phase.

Proposal 9: Target DU may assign either 1) a set of CFRA preambles per UE (as in legacy) or 2) a pool of CFRA resources for TA acquisition or RACH-based LTM that can be used by all UEs served by different cells.

Proposal 10: The sharing of CFRA resources among UEs being served by cells in different source DU is not allowed. In other words, only sharing of CFRA resources among UEs being served by cells in same source DU is allowed.

Proposal 11: Send an LS to RAN3 indicating the above proposal as agreement.

# References

[1] 	R2_122_ChairNotes EOM.docx

[2] R2_121bis-e Chair Notes EOM.docx

[3] R1_113 Chair Notes EOM.docx

[4] R2-2306957 Running Stage-2 CR