3GPP TSG-RAN WG2 Meeting #123	R2-2308963

Toulouse, France, August 21-25, 2023

Source: 	Session Chair (InterDigital)

Title:	Report from Session on NES, UAV, Rel-15-17 UP, Rel-17 Small Data, IIoT/URLLC, and RACH partitioning

Email discussions:

[AT123][300] Organizational Diana – NES, UAV, UP R15-17 UP/SDT

Scope:

    - Share plans for the meetings and list of ongoing email discussions for the sessions related to Rel-17 URLLC/IIoT, Small data, RA Partitioning, R15-16 UP, Rel-18 UAV and NES

    - Share meetings notes and agreements for review and endorsement

[AT123][301][MT-SDT] 38.331 Running CR (ZTE)

Scope: Running CR

Intended outcome: CR to be agreed

Deadline:  Aug. 26th, 2023

[AT123][302][MT-SDT] 38.321 Running CR (Huawei)

Scope: Running CR

Intended outcome: CR to be agreed

Deadline:  Aug. 26th, 2023

[AT123][304][R17 UP] DRX command (Asustek)

Scope : Review update to R2-2308711 DRX Command MAC CE

Outcome: agree to CR

Deadline: Thursday morning

[AT123][305][R17 UP] SDT triggering (Vivo)

Scope : Review update to R2-2308922 Correction on SDT Triggering Conditions

Outcome: agree to CR

Deadline: Thursday morning

[AT123][306][R17 UP] BFI counter (Nokia)

Scope : Review update to R2-2308925 Correction on BFI_COUNTER at SCG activation

Outcome: agree to CR

Deadline: Thursday morning

[AT123][307][R17 UP] eIAB (Ericsson)

Scope : Review update to CR  R2-2308433

Outcome: agree to CR

Deadline: Thursday morning

[AT123][308][R18 URLLC] LS to SA2 (Nokia)

Scope : Review and agree to LS to SA2

Outcome: agree to LS by email

Deadline: Friday

Post email discussions

[POST123][309][R18 URLLC] Running 38.331 (Ericsson)

Scope : Discuss stage-3 signaling details

Review running CR

Outcome: CR to be submitted to next meeting

Deadline: long

[POST123][301][MT-SDT] CR to 38.331 (ZTE)

Scope : Agree in principle with running CR

Outcome: CR to be submitted to plenary

Deadline: short

[POST123][302][MT-SDT] CR to 38.331 (Huawei)

Scope : Agree in principle with running CR

Outcome: CR to be submitted to plenary

Deadline: short

[POST123][303][MT-SDT] CR to 38.306 (Intel)

Scope : review running CR

Outcome: CR to be submitted to next meeting

Deadline: long

[POST123][310][UAV] Running CR 38.300 (Nokia)

Scope : Review running CR

Outcome: CR to be endorsed

Deadline: two weeks

[POST123][311][UAV] Running CR 38.331 (Qualcomm)

Scope : Review running CR

Outcome: CR to be submitted to next meeting

Deadline: long

[POST123][312][NES] Running CR 38.331 (Huawei)

Scope : Review running CR and discuss issue configuration per serving cell or MAC entity

Outcome: CR to be submitted to next meeting

Deadline: long

[POST123][313][NES] Running CR 38.300 (Ericsson)

Scope : Review running CR

Outcome: CR to be endorsed

Deadline: two weeks

[POST123][314][NES] Running CR 38.321 (InterDigital)

Scope : Review running CR

Outcome: CR to be submitted to next meeting

Deadline: long

[POST123][315][NES] Running CR 38.304 (InterDigital)

Scope : Review running CR

Outcome: CR to be submitted to next meeting

Deadline: long

# 1	Opening of the meeting

# 5	NR Rel-15 and Rel-16

Essential corrections only.

Tdoc Limitation: 8 tdocs in total for all sub agenda items.

In case a correction need to be reflected in both NR TS and LTE TS, the corrections should be submitted under one single AI (so the NR and LTE correction can be treatee together), the sub-AIs below this

## 5.1	Common

Includes the following WIs and input that doesn’t fit elsewhere.

(NR_newRAT-Core; leading WG: RAN1; REL-15; started: Mar. 17; closed: Jun. 19: WID: RP-191971)

(NR_IAB-Core; leading WG: RAN2; REL-16; started: Dec 18; target Aug 20; WID: RP-200840)

(NR_unlic-Core; leading WG: RAN1; REL-16; started: Dec 18; Closed June 20; WID: RP-192926).

(NR_IIOT-Core; leading WG: RAN2; REL-16; started: Mar 19; Completed: Jun 20; WID: RP-200797)

(NR_UE_pow_sav-Core; leading WG: RAN1; REL-16; started: Mar 19; Completed Jun 20; WID: RP-200494).

(NR_2step_RACH-Core; leading WG: RAN1; REL-16; started: Dec 18; Completed: June 20; WID: RP-200085).

(SRVCC_NR_to_UMTS-Core; leading WG: RAN2; REL-16; started: Dec 18; Completed; Mar 20; WID: RP-190713)

(RACS-RAN-Core, leading WG: RAN2; REL-16; started: Mar 19; completed: Jun 20; WID: RP-191088)

(NG_RAN_PRN-Core; leading WG: RAN3; REL-16; started: Mar 19; completed: June 20; WID: RP-200122)

(NR_eMIMO-Core, leading WG: RAN1; REL-16; started: Jun 18; target; Aug 20; WID: RP-200474;)

(NR_CLI_RIM; leading WG: RAN1; REL-16; started: Dec 18; Completed: Jun 20; WID: RP-191997;)

(NR_L1enh_URLLC-Core, leading WG: RAN1; REL-16; Completed: June 20; WID: RP-191584)

(LTE_NR_DC_CA_enh-Core; leading WG: RAN2; REL-16; started: Jun 18; Target Aug 20; WI RP-200791)

(NR_Mob_enh-Core; leading WG: RAN2; REL-16; started: Jun 18; Completed June 20; WID: RP-192277).

(NR_HST, NR_RRM_enh-Core, NR_RF_FR1, NR_RF_FR2_req_enh, NR_n66_BW, LTE_NR_B41_Bn41_PC29dBm-Core, NR_CSIRS_L3meas,)

(NR TEI16).

LTE mob enh corrections that are common with NR mobility enhancements should be submitted to this AI.

### 5.1.2	User Plane corrections

User Plane corrections will be handled in the User Plane break out session

#### 5.1.2.1	MAC

R2-2308660	Clarification on the trigger of MAC events	Huawei, HiSilicon	CR	Rel-15	38.321	15.13.0	1647	-	F	NR_newRAT-Core

-	Samsung thinks that there is no issue and once a MAC CE is triggered the event is cancelled and this is clearly captured.   Qualcomm shares same view as Samsung.  All companies should know what the procedure is and it has been there since Rel-15.

-	LG agrees with the intention but this has been there for a long time and there is no misunderstanding.

=>	Common understanding in RAN2 is that “has not been triggered” actually means “there is no triggered event”

=>	the CR is not pursued

R2-2308661	Clarification on the trigger of MAC events (Rel-16)	Huawei, HiSilicon	CR	Rel-16	38.321	16.12.0	1648	-	F	NR_newRAT-Core

R2-2308662	Clarification on the trigger of MAC events (Rel-16)	Huawei, HiSilicon	CR	Rel-17	38.321	17.5.0	1649	-	F	NR_newRAT-Core	Late

R2-2308669	Clarification on the trigger of MAC events (Rel-17)	Huawei, HiSilicon	CR	Rel-17	38.321	17.5.0	1650	-	F	NR_newRAT-Core

#### 5.1.2.2	RLC PDCP SDAP BAP

#### 5.1.2.3	Other

User plane related corrections that should be handled in User plane break out session.

# 6	NR Rel-17

## 6.1	Common

(NR_MG_enh-Core; leading WG: RAN4; REL-17; WID: RP-211591)

(NR_UDC_enh-Core; leading WG: RAN2; REL-17; WID: RP-211203)

(NG_RAN_PRN_enh-Core; leading WG: RAN3; REL-17; WID: RP-202363)

(NR_IAB_enh-Core; leading WG: RAN2; REL-17; WID: RP-211548)

(NR_UE_pow_sav_enh-Core; leading WG: RAN2; REL-17; WID: RP-212632)

(LTE_NR_DC_enh2-Core; leading WG: RAN2; REL-17; WID: RP-201040)

(LTE_NR_MUSIM-Core; leading WG: RAN2; REL-17; WID: RP-212610)

(NR_Slice -Core; leading WG: RAN2; REL-17; WID: RP-212534)

(NR_QoE-Core; leading WG: RAN3; REL-17; WID: RP-211406)

(NR_ext_to_71GHz-Core; leading WG: RAN1; REL-17; WID: RP-212637)

(NR_cov_enh-Core; leading WG: RAN1; REL-17; WID: RP-211566): non-RACH-indication parts

(NR_redcap-Core; leading WG: RAN1; REL-17; WID: RP-211574)

(NR_feMIMO-Core; leading WG: RAN1; REL-17; WID: RP-212535)

(NR_SmallData_INACTIVE-Core, leading WG: RAN2; REL-17; WID: RP-212594)

(NR_IIOT_URLLC_enh-Core; leading WG: RAN2; REL-17; WID: RP-210854)

(NR_MBS-Core; leading WG: RAN2; REL-17; WID: RP-201038)

PRACH partitioning items

NR TEI17: Corrections are accepted. New TEI17 tech proposal requirements: a) authored by an operator (and preferably co-signed by more), AND: b) resolves a concrete problem in the market for this operator (no new vendor initiated enhancements).

Includes Rel-17 Work Items without specific R2 Agenda Item, e.g. RAN1 and RAN4 led items, SA2 and CT1 led items (was previously “Rel-17 Other”)

Includes aspects that does not fit under the more specific AIs, e.g. multi-WI aspects.

Tdoc Limitation: 10 tdocs

### 6.1.2	User Plane corrections

User Plane Related aspects will be handled in the User Plane break out session. (exception: TEI new proposals if any).

R2-2307958	Correction on the use of the term ID in IAB MAC CEs	Samsung, Nokia, Nokia Shanghai Bell, Ericsson	CR	Rel-17	38.321	17.5.0	1643	-	F	NR_IAB_enh-Core

=>	The CR is agreed

R2-2307983	Clarification on use of SRI in IAB MAC CEs	Samsung	CR	Rel-17	38.321	17.5.0	1644	-	F	NR_IAB_enh-Core

-	LG asks why we are discussing this here instead of main session if this is related to NCR.  Samsung indicates that the issue was raised in NCR but this is an issue for IAB.

-	Qualcomm thinks this make sense

-	Samsung explain that this is how we use the term and without this we would not know which resources set it refers to.

=>	The CR is agreed

R2-2308433	Correction on number of restricted beams for eIAB	Ericsson, Samsung, Nokia, Nokia Shanghai Bell	CR	Rel-17	38.321	17.5.0	1625	1	F	NR_IAB_enh-Core	R2-2306005

-	Huawei thinks that the network will not configure more than 8 beams, if 8 is the maximum the rightmost 3 bits should be the one you use.

-	Samsung thinks that this impacts the MT as well not just the network.   Samsung also explains that we usually specify this for the MAC CE.

-	Huawei asks if we should then also clarify Number of associated IAB-MT beams IDi

-	Ericsson explains that this is different and it is a mapping

-	LG thinks that we need to follow the convention which is to specify that the 5bits are reserved.

=>	The CR is revised in R2-2309196

R2-2309196	Correction on number of restricted beams for eIAB	Ericsson, Samsung, Nokia, Nokia Shanghai Bell	CR	Rel-17	38.321	17.5.0	1625	1	F	NR_IAB_enh-Core

=>	The CR is agreed (by email in offline discussion 307)

R2-2308499	Correction to PDCCH monitoring	Google Inc.	CR	Rel-17	38.321	17.5.0	1646	-	F	NR_SmallData_INACTIVE-Core

-	Lenovo thinks that this may not be needed as PDCCH monitoring is well specified in connected. Vivo shares the same view as Lenovo.  The UE behaviour is the same as legacy UE and it captured very well in the L1 specs.

-	Google explains that the MAC specifies that the C-RNTI is monitored until SDT is terminated.

-	Apple also thinks that it is clear.  Ericsson as well.

=>	The CR is not pursued

R2-2308711	DRX Command MAC CE for MBS	ASUSTeK	CR	Rel-17	38.321	17.5.0	1653	-	F	NR_MBS-Core

-	Nokia and Samsung agree with the intention.  Ericsson thinks that this is already clear in RAN1 specification so may not be needed.

-	LG thinks that if G-CS-RNTI and SPS association is clear in RAN1 specs.

-	Huawei thinks we could have a simplified version from Nokia where we refer to corresponding .

=>	Move to email discussion and update the wording as per Nokia suggestion.

=>	The CR is revised in R2-2309217

R2-2309217	DRX Command MAC CE for MBS	ASUSTeK	CR	Rel-17	38.321	17.5.0	1653	-	F	NR_MBS-Core

=>	The CR is agreed

R2-2308906	Correction on TCI-state for RedCap UE	ZTE Corporation, Sanechips	CR	Rel-17	38.321	17.5.0	1655	-	F	NR_redcap-Core

-	ZTE thinks that we need to also include the initial bandwidth part for redcap

-	Qualcomm agrees with the intention but wonders if this should be first discussed and agreed in RAN1.

-	Samsung indicates that they also found a similar issue for non-redcap case.

-	Huawei thinks that this changes UE behaviour.   Ericsson also agrees and we may also need a UE capability to be able to handle this chage.

=>	The CR is postponed

R2-2308922	Correction on SDT Triggering Conditions	vivo	CR	Rel-17	38.321	17.5.0	1656	-	F	NR_SmallData_INACTIVE-Core

-	Samsung agrees with the intention but thinks that we can simplify the change by merging the second and third if.

=>	Update the CR with the simplified wording and review over email discussion

=>	The CR is revised in R2-2309210

R2-2309210	Correction on SDT Triggering Conditions	vivo	CR	Rel-17	38.321	17.5.0	1656	-	F	NR_SmallData_INACTIVE-Core

=>	The CR is agreed (by email)

R2-2308924	Correction on HARQ buffer flush at SCG deactivation	Nokia, Apple, Mediatek, Qualcomm, Nokia Shanghai Bell	CR	Rel-17	38.321	17.5.0	1657	-	F	LTE_NR_DC_enh2-Core

-	Nokia suggests to do this as a Rel-18 CR with a magic sentence.

=>	The CR will be agreed as a Rel-18 CR with a magic sentence

=>	The CR is postponed until Rel-18 version of spec is ready

R2-2308925	Correction on BFI_COUNTER at SCG activation	Nokia, Nokia Shanghai Bell	CR	Rel-17	38.321	17.5.0	1658	-	F	LTE_NR_DC_enh2-Core

-	Mediatek if timing alignment is expired we should always trigger legacy RACH, so we should swap the order.  Nokia and Qualcomm agrees

-	LG thinks that we don’t need the changes as we can differentiate if we have BFR or timing alignment expiry.

=>	Offline to discuss need and update the wording as per Mediatek comment (if needed).

=>	The CR is postponed

# 7	Rel-18

## 7.3	Network energy savings for NR

(Netw_Energy_NR -Core; leading WG: RAN1; REL-18; WID: RP-223540)

Time budget: 1 TU

Tdoc Limitation: 4 tdocs

### 7.3.1	Organizational

LS, workplan, email discussion etc

Expected inputs: running CRs for the following: 38.300 [Ericsson], 38.331 [Huawei], 38.321 [InterDigital]

Expected inputs after more agreements are made: 38.304 [Apple] (if needed), 38.306 [Vivo]

R2-2307017	Reply LS on Cell DTX/DRX activation/deactivation (R1-2306246; contact: Huawei, Intel)	RAN1	LS in	Rel-18	Netw_Energy_NR-Core	To:RAN2	Cc:RAN3

=>	Noted

R2-2307053	Reply LS on 3GPP work on Energy Efficiency (S1-231805; contact: Nokia)	SA1	LS in	Rel-18	EE5GPLUS_Ph2	To:SA5	Cc:SA, RAN, CT, SA1, SA2, SA3, SA4, SA6, RAN1, RAN2, RAN3, RAN4, CT3, CT4

=>	Noted

R2-2307063	Reply LS on the enhancements to restricting paging in a limited area (S2-2307984; contact: Qualcomm)	SA2	LS in	Rel-18	Netw_Energy_NR-Core	To:RAN3	Cc:RAN2

=>	Noted

R2-2307073	LS on 3GPP work on Energy Efficiency (S4-231111; contact: Qualcomm)	SA4	LS in	Rel-18	EE5GPLUS_Ph2	To:SA5, SA, RAN, CT	Cc:SA1, SA2, SA3, SA6, RAN1, RAN2, RAN3, RAN4, CT1, CT3, CT4

=>	Noted

R2-2307528	Running 38.331 CR - Introduction of Network energy savings for NR	Huawei, HiSilicon	draftCR	Rel-18	38.331	17.5.0	Netw_Energy_NR-Core

=>	The CR is endorsed

R2-2308044	Work plan for NR network energy savings	Huawei, HiSilicon	discussion	Rel-18	Netw_Energy_NR-Core

=>	Noted

R2-2308393	Running CR to 38.321 for Network Energy Savings	InterDigital	draftCR	Rel-18	38.321	17.5.0	Netw_Energy_NR-Core

-	Interdigital indicates that we have to add PUSCH transmission during non-active time for dynamic grant

-	LG thinks that we can have the behaviour in one section rather than spread over the different sections.   InterDigital indicates that it is possible but there are some cases like SR that can conflict.

-	Nokia thinks that it is better to put in individual sections where the functions are.   Samsung agrees with LG as we try to make specification simple.

=>	Discuss offline on the best structure and if it is possible to simplify

R2-2308394	TS 38.321 open issues for NES	InterDigital	discussion	Rel-18	Netw_Energy_NR-Core

=>	Noted

R2-2308492	Running CR to 38300 for Network energy savings	Ericsson	CR	Rel-18	38.300	17.5.0	0689	1	B	Netw_Energy_NR-Core	R2-2306966

=>	The CR is endorsed

### 7.3.2	DTX/DRX mechanism

Configuring Cell DTX/DRX per cell

R2-2307527	Outcome of [Post122][307][NES] DTX/DRX – alignment, single/multiple configurations, parameter values (Huawei)	Huawei, HiSilicon	discussion	Rel-18	Netw_Energy_NR-Core

Proposal 2: RAN2 to discuss whether Cell DTX/DRX configuration and activation status is per MAC entity or per Serving Cell and further spec implications of the decision.

R2-2308794	Further aspects on cell DTX/DRX	Ericsson	discussion

Proposal 7	Cell DTX/DRX can be configured for SCells and each SCell may have its own DTX/DRX configuration.

R2-2307150	Discussion on Cell DTX/DRX configuration and operation	Xiaomi	discussion	Rel-18

Proposal 8a: RAN2 is kindly asked to discuss how to handle the cell DTX configuration in CA case and dual DRX case.

- Option 1: per UE to configure cell DTX, FFS for cell DRX. The cell DTX configuration is common for more than one serving cells.

- Option 2: per cell to configure cell DTX/DRX.

Discussion per serving cell or per MAC entity

-	Huawei believes that it is simpler for MAC entity and this is simpler and it is similar to C-DRX.  Xiaomi and Oppo agrees with Huawei and it is simpler for the UE side.  LG and CMCC agrees.

-	Nokia indicates that we agreed with serving cell in the study item

-	CATT explains that to support multi-layer deployment it should be per serving cell.

-	Vivo thinks that we should separate the configuration and activation status.   Configuration should be per MAC entity and activation per serving cell.

-	Samsung and ZTE thinks per cell.

-	Apple also we should reconsider and configure per MAC entity.   The implementation will be on top of C-DRX so if it is different it will make it more complicated.  Qualcomm, Interdigital, Fraunhofer and Mediatek agrees with Apple.

-	Lenovo indicates that we should ensure that the feature should be deployable and it should be per cell.  Vodafone agrees.

-	Cewit thinks that it should be cell and we need to consider the different load.

-	NEC thinks that it should be serving cell and indicates that in RAN1 it was agreed that the activation/deactivation is per serving cell.

-	Vodafone and BT would like to ensure that we consider deployments and understand all the issues

C-DRX/Cell DTX alignment
R2-2307527	Outcome of [Post122][307][NES] DTX/DRX – alignment, single/multiple configurations, parameter values (Huawei)	Huawei, HiSilicon	discussion	Rel-18	Netw_Energy_NR-Core

Proposal 9: RAN2 will reuse the start timer formula of the onDurationTimer from UE C-DRX (including SlotOffset) to specify the start of cellDTX-onDurationTimer (and cellDRX-onDurationTimer) in 38.321. (27/27)

Proposal 1: The gNB ensures that there is at least partial overlapping between UE C-DRX on-duration and cell DTX/DRX on-duration. (16/27)

R2-2308794	Further aspects on cell DTX/DRX	Ericsson	discussion

Proposal 4	A functional coexistence of UE C-DRX and Cell DTX can be ensured by the NW through the appropriate UE C-DRX and Cell DTX configurations. No specification impact.

R2-2307147	Joint Operation between UE C-DRX and Cell DTX	NEC	discussion	NR_SL_enh2

Proposal-6: When Cell DRX is configured together with Cell DTX, it must be fully aligned with Cell DTX (i.e. exactly the same periodicity, offset and on-duration) for one serving cell.

Discussion

-	Ericsson thinks that the alignment should be up to network configuration.   Nokia agrees.

-	NEC thinks that the configuration needs to be fully aligned.   Nokia thinks that there may be some problems with full align.

-	Qualcomm is concerned with leaving it up to network implementation.   Ericsson suggests that if needed we can capture something in stage 2 but no need to capture in 331.  Apple thinks that concerns from Qualcomm would only be cause by very bad network implementation.

-	CATT indicates that a full alignment would requires all UEs to have aligned C-DRX, however leaving everything under control it would require the  network to have to send configuration to all UEs.   We can look at optimizations on aligning.

-	LG thinks that some aligned is beneficial.  The onduration can be aligned but with inactivity timer this is more difficult.

-	Vodafone thinks that we should put some guidance in stage 2 that alignment is beneficial.

-	Fujitsu agrees with partial alignment and it can be up to network implementation.

-	Nokia thinks that it is better to describe it from UE point of view.   Mediatek disagrees.

-	Qualcomm thinks that we should capture that the periodicity of cell DTX/DRX is an integer multiple of C-DRX periodicity.  LG has the same concern as Qualcomm.  Lenovo thinks that UE should not be required to verify.   Mediatek thinks that having something in stage 3 like Qualcomm suggests would be beneficial.

-	Samsung thinks that the network can handle everything.

that the periodicity of cell DTX/DRX is an integer multiple of C-DRX periodicity

Additional C-DRX cycle

R2-2307527	Outcome of [Post122][307][NES] DTX/DRX – alignment, single/multiple configurations, parameter values (Huawei)	Huawei, HiSilicon	discussion	Rel-18	Netw_Energy_NR-Core

Proposal 3:  As a baseline legacy C-DRX reconfiguration is used to change UE C-DRX configuration once Cell DTX/DRX is activated/deactivated. (20/27)

Proposal 3a: RAN2 to discuss an alternative reconfiguration option where two C-DRX configurations (one legacy and one for cell DTX) are set in advance and the second one would be activated together will cell DTX. (9/27)

=>	Noted

R2-2308738	UE C-DRX configuration used upon cell DTX activation	vivo, Fraunhofer IIS, Huawei, HiSilicon, LG Electronics Inc., MediaTek Inc., NEC, Xiaomi	discussion	Rel-18

Proposal 1: For alignment between UE C-DRX and cell DTX, the gNB can configure an additional UE C-DRX configuration together with cell DTX/DRX parameters, which is enabled implicitly upon cell DTX activation. The additional C-DRX configuration contains optional C-DRX parameters which the UE shall use when cell DTX is activated.

-	ZTE supports an additional C-DRX configuration.  Lenovo thinks it can work but it is a bit more signaling with anticipation that it will use Cell DTX/DRX.   We can also consider other solutions that don’t require explicit configuration.  CATT agrees and we should enable some implicit alignment.    Interdigital explains that the UE doesn’t monitor PDCCH if there is no pending retransmission, so it is sufficient.

-	Apple thinks that this is anyways up to network implementation so we don’t need to have more implementation.   Fujitsu doesn’t see the need as well.  Oppo agrees.

-	LG is concerned that the network would have to reconfigure all UEs, so having a second C-DRX is it is much more efficient than a separate RRC reconfiguration.   Qualcomm doesn’t won’t to modify C-DRX operation.  Additionally the activation/deactivation is not reliable.  Mediatek thinks that this ensures that the alignment can be done by the network.

-	Nokia asks if second configuration is just the offset so the UEs can be aligned.   Vivo explains that sometime the offset needs to be changed and sometimes the periodicity.   CeWit and CATT agrees with Nokia.

-	Samsung thinks that this is not too dynamic that RRC signaling is ok.

-	Qualcomm thinks this is a dangerous change

=>	Noted

R2-2307147	Joint Operation between UE C-DRX and Cell DTX	NEC	discussion	NR_SL_enh2

Proposal-3: When C-DRX and cell DTX OnDuration overlaps partially, whether re-configure UE C-DRX to align with cell DTX pattern is left to network.

R2-2307150	Discussion on Cell DTX/DRX configuration and operation	Xiaomi	discussion	Rel-18

Proposal 6c: Only drx-LongCycleStartOffset is configured for NES mode C-DRX and drx-ShortCycle is not applied in NES mode C-DRX.

Cell DTX/DRX configuration parameters

R2-2307527	Outcome of [Post122][307][NES] DTX/DRX – alignment, single/multiple configurations, parameter values (Huawei)	Huawei, HiSilicon	discussion	Rel-18	Netw_Energy_NR-Core

Proposal 7: RAN2 specifies cellDTX-onDurationTimer (and cellDRX-onDurationTimer) to have the same value range as UE C-DRX on-duration timer. (24/27)

Proposal 8: RAN2 specifies cellDTX-Cycle (and cellDRX-Cycle) to have the same value range as UE C-DRX Long cycle. (24/27)

Proposal 4: Separate DTX and DRX configuration means that the features can be enabled separately. (25/27)

-	Qualcomm is not sure how Cell DTX would without Cell DRX.   Nokia explain that if there is on DRX the network is always listening so there is no issues.  Vodafone thinks this become complicated unnecessarily.   Apple and LG agree with Qualcomm and Vodafone.

Proposal 5: On-duration and Cycle parameters are common between cell DTX and DRX. (16/27)

-	Nokia doesn’t see a need to restrict.

Proposal 5a: RAN2 to discuss whether there is a need to enable a different start offset configuration for cell DTX and cell DRX.

-	Nokia disagrees, transmission and reception can happen at different times.   ZTE agrees with Nokia.

Activation/Deactivation

R2-2307524	Discussion on cell DTX and DRX	Huawei, HiSilicon	discussion	Rel-18	Netw_Energy_NR-Core

Proposal 1:	RAN2 will not introduce additional MAC CE based indication for cell DTX/DRX activation and deactivation.

R2-2307647	Cell DTX-DRX Mechanism	Qualcomm Incorporated	discussion	Rel-18

Proposal 1: RAN2 to introduce a MAC CE for cell DTX/DRX (de)activation. FFS the exact design of this MAC CE.

-	Samsung and Fraunhaufer support the proposal as RAN1 is concerned with reliability.  Nokia and Apple explains that RAN1 already designed one mechanism so why do we need MAC CE.

PDCCH monitoring during non-active period for retransmission.

R2-2308737	Remaining issues for cell DTX-DRX	vivo	discussion	Rel-18

Proposal 5: RAN2 to confirm the following working assumption: When the retransmission timer is running (if C-DRX is configured), the UE is expected to monitor PDCCH, like in legacy.  It is up to the network whether it schedules retransmissions out of the Cell DTX active period, i.e., when the DRX retransmission timer is running, the UE should monitor PDCCH regardless of the Cell DTX.

PDCCH monitoring when C-DRX is not configured

R2-2308727	Leftover issues of Cell DTX/DRX	CATT, Turkcell	discussion	Rel-18	FS_Netw_Energy_NR

Proposal 4: A UE not configured with C-DRX monitors PDCCH irrespective of Cell DTX/DRX.

R2-2307808	Discussion on key issues of Cell DTX/DRX	Apple	discussion	Netw_Energy_NR-Core

Proposal 2: If only Cell DTX/DRX is configured (i.e. UE C-DRX is not configured), retransmission is not allowed during non-active duration.

R2-2308388	Remaining issues on Cell DTX/DRX	InterDigital	discussion	Rel-18	Netw_Energy_NR-Core

Proposal 2: 	Cell DTX in RRC can only be configured when C-DRX is configured. Such restriction can be added in RRC.

Discussions

-	Huawei agrees to link the C-DRX with Cell DTX.  Samsung would prefer to no restrict network configuration.  CATT explains that there are cases where the UE is not configured with Cell DRX.

After offline

-	Most companies agree, but some concerns were raised for the case where the UE doesn’t support C-DRX.   Those UEs can be addressed by other means, like mobility to another cell. Etc.

-	Samsung is concerned for the case where the network hasn’t implemented C-DRX.    Ericsson sees the proposal as we should at least check how it works with C-DRX and then later we can check if it works.

Agreements:

1	Activation/deactivation is per serving cell.  FFS if the configuration is per cell or per MAC entity

2	RAN2 will reuse the start timer formula of the onDurationTimer from UE C-DRX (including SlotOffset) to specify the start of cellDTX-onDurationTimer (and cellDRX-onDurationTimer) in 38.321.

3	The gNB should ensures that there is at least partial overlapping between UE C-DRX on-duration and cell DTX/DRX on-duration.  It is up to network implementation to ensure the alignment.  We will capture this in stage 2 specification.

Understanding is that alignment means that the cell DTX/DRX and C-DRX periodicity should be multiple of each other.   FFS if we anything needs to be specified in stage 3 (i.e. in IE description)

4	As a baseline legacy C-DRX reconfiguration is used to change UE C-DRX configuration once Cell DTX/DRX is activated/deactivated.

5	RAN2 specifies cellDTX-onDurationTimer (and cellDRX-onDurationTimer) to have the same value range as UE C-DRX on-duration timer.

6	RAN2 specifies cellDTX-Cycle (and cellDRX-Cycle) to have the same value range as UE C-DRX Long cycle.

7	Separate DTX and DRX configuration means that the features can be enabled separately (i.e. Cell DTX can be configured without Cell DRX)

8	On-duration and Cycle parameters are common between cell DTX and DRX, when both are configured.  FFS if we have different start offset configuration for cell DTX and cell DRX

9	RAN2 will not introduce a MAC CE for cell DTX/DRX (de)activation.

10	Confirm working assumption, when the retransmission timer is running (if C-DRX is configured), the UE is expected to monitor PDCCH, like in legacy.  It is up to the network whether it schedules retransmissions out of the Cell DTX active period, i.e., when the DRX retransmission timer is running, the UE should monitor PDCCH regardless of the Cell DTX.

11	We focus on the case where DTX in RRC can only be configured when C-DRX is configured.  We will not optimize for the case where C-DRX is not configured.

Extension of Cell DTX activity

R2-2308794	Further aspects on cell DTX/DRX	Ericsson	discussion

Proposal 1	UE specific inactivity timer is introduced to extend on duration time of Cell DTX/DRX, i.e. after expiration of the timer, the UE considers the Cell DTX/DRX to be in non-active period.

R2-2308593	Discussion on DTX/DRX mechanism	LG Electronics Inc.	discussion	Rel-18	Netw_Energy_NR-Core

Proposal 2. Cell DTX pattern is considered as a fixed pattern. (i.e., active period of cell DTX is not extended.)

CG transmissions

R2-2307760	SPS/CG and MAC CE Activation/Deactivation in Cell DTX/DRX	Samsung	discussion	Rel-18

Proposal 4: In case of SPS bundle, the UE receives SPS-PDSCH only if all the SPS occasions within a bundle completely overlaps with cell DTX Active Time.

Proposal 5: In case of CG bundle, the UE transmits CG-PUSCH only if all the CG occasions within a bundle completely overlaps with cell DRX Active Time.

R2-2307178	Various (RRC Procedure, Measurement, SR, CG etc.) alignment aspects 	Lenovo	discussion	Netw_Energy_NR-Core

Proposal 10	UE performs the transmission within a bundle of the configured grant regardless the cell is in Cell Transmission ON or OFF duration.

Multiple Cell DTX/DRX configurations

R2-2307527	Outcome of [Post122][307][NES] DTX/DRX – alignment, single/multiple configurations, parameter values (Huawei)	Huawei, HiSilicon	discussion	Rel-18	Netw_Energy_NR-Core

Proposal 6: As a baseline, RAN2 designs cell DTX/DRX with a single configuration. (16/27)

R2-2307911	Single configuration with multiple DTX_DRX patterns and explicit signalling	BT Plc, Nokia, T-Mobile, Deutsche Telekom, NTT Docomo, KDDI, ZTE	discussion	Rel-18

Proposal 1	RAN2 to agree that a common cell DTX/DRX pattern configuration in the cell may contain multiple cell DTX/DRX patterns where only a single cell DTX/DRX pattern is active at a single point in time.

SR

R2-2307178	Various (RRC Procedure, Measurement, SR, CG etc.) alignment aspects 	Lenovo	discussion	Netw_Energy_NR-Core

Proposal 5	SR can be configured to cater to the important situations (Emergency calls, beam failure

recovery of a BFD-RS set MAC CE) only.

R2-2307177	Remaining issues for Cell DTX_DRX	Samsung Electronics Co., Ltd	discussion	Rel-18	Netw_Energy_NR-Core

Proposal 1: For each SR configuration, network configures whether the UE is allowed to transmit an SR during Cell DRX non-active period.

R2-2308179	Discussion on DTX/DRX mechanism	OPPO	discussion	Rel-18	Netw_Energy_NR

Proposal 2	RAN2 confirms no exceptional handling for the SR transmission/reception during the cell DRX non-active duration.

R2-2308606	Discussion on Cell DTX/DRX	Fujitsu	discussion	Rel-18	Netw_Energy_NR-Core

Proposal 5:	If the SR transmission is allowed during Cell DRX non-active period, the UE should monitor a PDCCH after the SR transmission regardless of Cell DTX configuration.

Proposal 6:	If the SR with high priority is transmitted, the network can schedule a dynamic UL grant for a new transmission during C-DRX active period even during Cell DTX non-active period.

R2-2307787	Remaining issues on Cell DTX/DRX	Nokia, Nokia Shanghai Bell	discussion	Rel-18	Netw_Energy_NR-Core

Proposal 8: If SR is not allowed during non-active period, RACH is allowed for emergency call if the next active period is too far away.

Measurements

R2-2307808	Discussion on key issues of Cell DTX/DRX	Apple	discussion	Netw_Energy_NR-Core

Proposal 11: In order to start RRM/RLM/BFR work, RAN2 send LS to RAN1/RAN4 on whether any change on requirements of measurement and reporting based on SSB/CSI-RS/TRS/PRS.

R2-2307147	Joint Operation between UE C-DRX and Cell DTX	NEC	discussion	NR_SL_enh2

R2-2307150	Discussion on Cell DTX/DRX configuration and operation	Xiaomi	discussion	Rel-18

R2-2307177	Remaining issues for Cell DTX_DRX	Samsung Electronics Co., Ltd	discussion	Rel-18	Netw_Energy_NR-Core

R2-2307178	Various (RRC Procedure, Measurement, SR, CG etc.) alignment aspects 	Lenovo	discussion	Netw_Energy_NR-Core

R2-2307524	Discussion on cell DTX and DRX	Huawei, HiSilicon	discussion	Rel-18	Netw_Energy_NR-Core

R2-2307527	Outcome of [Post122][307][NES] DTX/DRX – alignment, single/multiple configurations, parameter values (Huawei)	Huawei, HiSilicon	discussion	Rel-18	Netw_Energy_NR-Core

R2-2307647	Cell DTX-DRX Mechanism	Qualcomm Incorporated	discussion	Rel-18

R2-2307713	Cell DTX/DRX NES Techniques	III	discussion

R2-2307717	Discussion on cell DTX/DRX	KDDI Corporation	discussion

R2-2307760	SPS/CG and MAC CE Activation/Deactivation in Cell DTX/DRX	Samsung	discussion	Rel-18

R2-2307787	Remaining issues on Cell DTX/DRX	Nokia, Nokia Shanghai Bell	discussion	Rel-18	Netw_Energy_NR-Core

R2-2307808	Discussion on key issues of Cell DTX/DRX	Apple	discussion	Netw_Energy_NR-Core

R2-2307897	TAT aspects under Cell-DTX/Cell-DRX	FGI	discussion

R2-2307898	Discussion on cell DTX and DRX	FGI	discussion

R2-2307911	Single configuration with multiple DTX_DRX patterns and explicit signalling	BT Plc, Nokia, T-Mobile, Deutsche Telekom, NTT Docomo, KDDI, ZTE	discussion	Rel-18

R2-2308179	Discussion on DTX/DRX mechanism	OPPO	discussion	Rel-18	Netw_Energy_NR

R2-2308279	The remaining issues on cell DTX and DRX	ZTE corporation, Sanechips	discussion	Rel-18	Netw_Energy_NR-Core

R2-2308301	Discussion on cell DTXDRX	CMfCC	discussion	Rel-18	Netw_Energy_NR-Core

R2-2308388	Remaining issues on Cell DTX/DRX	InterDigital	discussion	Rel-18	Netw_Energy_NR-Core

R2-2308448	UL considerations for Cell DTX/DRX	NEC Telecom MODUS Ltd.	discussion	R2-2305855

R2-2308493	Further aspects on cell DTX/DRX	Ericsson	discussion	Withdrawn

R2-2308534	Cell DTX/DRX traffic adaptations	Fraunhofer IIS, Fraunhofer HHI	discussion	Rel-18

R2-2308593	Discussion on DTX/DRX mechanism	LG Electronics Inc.	discussion	Rel-18	Netw_Energy_NR-Core

R2-2308606	Discussion on Cell DTX/DRX	Fujitsu	discussion	Rel-18	Netw_Energy_NR-Core

R2-2308703	Support multiple configuration of Cell DTX/DRX	ETRI	discussion	Rel-18

R2-2308727	Leftover issues of Cell DTX/DRX	CATT, Turkcell	discussion	Rel-18	FS_Netw_Energy_NR

R2-2308737	Remaining issues for cell DTX-DRX	vivo	discussion	Rel-18

R2-2308738	UE C-DRX configuration used upon cell DTX activation	vivo, Fraunhofer IIS, Huawei, HiSilicon, LG Electronics Inc., MediaTek Inc., NEC, Xiaomi	discussion	Rel-18

R2-2308794	Further aspects on cell DTX/DRX	Ericsson	discussion

R2-2308886	Cell DTX/DRX NES Techniques	CEWiT	discussion

### 7.3.3	SSB-less Scell operation

Contributions on inter-band CA for FR1 and co-located cells

R2-2307810	Further Discussion on RAN2 work of inter-band SSB-less CA	Apple	discussion	Netw_Energy_NR-Core

Proposal 1: RAN2 focus on Scenario 1 in Rel-18 unless RAN4 has concrete conclusion to specify requirements of Scenario 2a.

Proposal 2: An indication needs to be introduced to indicate which inter-band active serving cell can be used as timing source for the SSB-less SCell. FFS signaling details.

Proposal 3: If RAN4 conclude scenario 1 is feasible, TRS of SSB-less SCell is configured by RRC signalling as in Rel-17, and the UE behavior of SCell activation captured in Section 5.9 of TS 38.321 can be reused.

R2-2308026	Discuss on SSB-less SCell operation in NES	Lenovo	discussion	Rel-18

Proposal 2: RAN2 to discuss whether the SSB-less SCell is configured in the same TAG as the reference serving cell.

R2-2307152	Discussion on inter-band SSB-less SCell	Xiaomi	discussion	Rel-18

Proposal 1: The TRS for fast SCell activation in R17 can be reused for R18 inter-band SSB-less SCell, including TRS configuration and Enhanced SCell Activation/Deactivation MAC CEs for TRS activation.

Proposal 2: For inter-band SSB-less SCell, RRC based SCell activation is not supported, i.e., the sCellState in SCellConfig is not set to “activated”.

R2-2307519	On NES SSB-less SCell operation	Nokia, Nokia Shanghai Bell	discussion	Rel-18	Netw_Energy_NR-Core

R2-2307648	SSB-less Interband CA	Qualcomm Incorporated	discussion	Rel-18

R2-2307810	Further Discussion on RAN2 work of inter-band SSB-less CA	Apple	discussion	Netw_Energy_NR-Core

R2-2308026	Discuss on SSB-less SCell operation in NES	Lenovo	discussion	Rel-18

R2-2308180	Discussion on SSB-less Scell operation	OPPO	discussion	Rel-18	Netw_Energy_NR

R2-2308280	Discussion on SSB-less SCell operation for NES	ZTE corporation, Sanechips	discussion	Rel-18	Netw_Energy_NR-Core

R2-2308359	On SBB/SIB-less NES solutions	Dell Technologies	discussion	Rel-18	FS_Netw_Energy_NR

R2-2308391	SSB-less Scell operation	InterDigital	discussion	Rel-18	Netw_Energy_NR-Core

R2-2308728	Enhancements on SSB-less SCell operation	CATT	discussion	Rel-18	FS_Netw_Energy_NR

### 7.3.4	Cell selection/re-selection

Contributions mechanisms to prevent legacy UEs camping on cells adopting the Rel-18 NES mode

Cell Selection

R2-2308181	Discussion on cell selection reselection	OPPO	discussion	Rel-18	Netw_Energy_NR

Proposal 1: Use cellBarred in MIB to control the legacy UE accessing an NES cell or preventing from an NES cell.

Proposal 2: One single bit is introduced for controlling all NES-capable UEs to access a cell, i.e. all NES-capable UEs are uniformly allowed or prevented from accessing a specific NES cell.

Proposal 3: A new barring indication is introduced in SIB1 to control NES-capable UEs accessing an NES cell. In detail, once cellBarred in MIB is indicated as Barred, the NES-capable UEs need to further check the new barring indication in SIB1.

-	LG thinks that NES UEs would ignore the legacy barring IE if new NES cell barring bit is present, similar to legacy UEs.  Huawei asks if the UE should not check the NES barring bit.  Ericsson thinks it is straightforward that the UE would need to check legacy barring if NES barring IE is not present.  Oppo thinks that we should check the legacy barring.  Apple thinks that the UE should also check the SIB1 NES barring.

R2-2308739	Discussion on cell selection/re-selection	vivo	discussion	Rel-18

Proposal 3: If any introduced cell bar cellBarredNES-r18 is set to not barred and the NES-capable UE is capable of the corresponding NES feature, the NES-capable UE considers the cell as not barred.

Discussion

-	Vivo and Franhoufer thinks that a single bit is not enough as there are other NES techniques, for example, power domain techniques UEs can be treated as legacy UE.

-	LG thinks P1 is agreeable and 1 bit is sufficient.

-	Huawei thinks 1 bit is sufficient, spatial and power domain are per UE configured, so the only NES techniques that has impact on idle UEs is Cell DTX/DRX.  Apple, Xiaomi agrees, and this bit should be future proof.  Ericsson agrees.

-	NEC is concerned that we won’t have enough granularity to discussion.

Discussion on Use cellBarred in MIB to control the legacy UE accessing an NES cell or preventing from an NES cell.

-	Lenovo thinks that we should be careful with MIB barring as we don’t want to prevent legacy from accessing the cell for emergency.

-	InterDigital thinks that there are already tools to bar legacy UEs.

Agreements

-	One single bit in SIB1 is introduced for controlling all “NES-capable UEs” to access a cell.  FFS what “NES capable UE” bit means.  The NES UE always follows the NES bit used for barring, if present.  If not present the UE shall follow legacy barring.

-	No new cell baring techniques for non-NES UEs will be specified.

-	No new cell re-selection techniques will be considered in this Rel-18

Cell Re-selection

R2-2307765	Reselection and Paging handling for NES	Nokia, Nokia Shanghai Bell	discussion	Rel-18	FS_Netw_Energy_NR

Proposal 5: Introduce an indication not requiring SI update procedure to indicate the cell is being shut down and to trigger NES capable UE to bar the cell and perform cell reselection

R2-2308281	Consideration on cell selection and reselection enhancements for NES	ZTE corporation, Sanechips	discussion	Rel-18	Netw_Energy_NR-Core

Proposal 2: Whether to prioritize a NES cell or NES frequency for NES-capable UE cell re-selection depends on whether QoffsetNES is configured for the cell or frequency. And if configured, QoffsetNES is used to prioritize the NES cell or NES frequency for NES-capable UE cell re-selection.

Discussion

-	Qualcomm and LG thinks it is out scope.  Nokia thought this was more for the UE but if the UE vendors are ok with nothing that also good.

R2-2307149	Discussion on RRC_IDLE/INACTIVE UE behavior due to NES	Xiaomi	discussion	Rel-18

R2-2307220	Discussion on cell selection and reselection for NES	Samsung Electronics GmbH	discussion	Rel-18

R2-2307525	Discussion on cell selection/reselection for NES	Huawei, HiSilicon	discussion	Rel-18	Netw_Energy_NR-Core

R2-2307765	Reselection and Paging handling for NES	Nokia, Nokia Shanghai Bell	discussion	Rel-18	FS_Netw_Energy_NR

R2-2307809	Detailed mechanism of legacy UE barring in NES cell	Apple	discussion	Netw_Energy_NR-Core

R2-2308027	Cell selection/re-selection in NES	Lenovo	discussion	Rel-18

R2-2308181	Discussion on cell selection reselection	OPPO	discussion	Rel-18	Netw_Energy_NR

R2-2308281	Consideration on cell selection and reselection enhancements for NES	ZTE corporation, Sanechips	discussion	Rel-18	Netw_Energy_NR-Core

R2-2308333	Discussion on cell barring and reselection for NES	CMCC	discussion	Rel-18	Netw_Energy_NR-Core

R2-2308389	Cell selection and resection for NES	InterDigital	discussion	Rel-18	Netw_Energy_NR-Core

R2-2308449	Procedure for legacy UEs camping on NES cells	NEC Telecom MODUS Ltd.	discussion	R2-2305858

R2-2308494	Remaining aspects for NES Cell selection/reselection	Ericsson	discussion

R2-2308533	Cell Selection and Re-Selection for NES	Fraunhofer IIS, Fraunhofer HHI	discussion	Rel-18

R2-2308681	Access restriction	LG Electronics	discussion	Rel-18	Netw_Energy_NR-Core	Late

R2-2308729	Consideration on Cell Selection/Re-selection on NES cells	CATT	discussion	Rel-18	FS_Netw_Energy_NR

R2-2308739	Discussion on cell selection/re-selection	vivo	discussion	Rel-18

R2-2308784	Access restrition for NES	LG Electronics Inc.	discussion	Rel-18	Netw_Energy_NR-Core

R2-2308831	Discussion on Cell selection	NTT DOCOMO INC.	discussion	Rel-18	Netw_Energy_NR-Core

### 7.3.5	Connected mode mobility

Contributions on CHO procedure enhancement(s) in case source/target cell is in NES mode

### 7.3.5	Connected mode mobility

Contributions on CHO procedure enhancement(s) in case source/target cell is in NES mode

NES Mode

R2-2307649	NES Connected mode mobility	Qualcomm Incorporated	discussion	Rel-18

Observation 1: The exact mechanism the NW implements to save energy is not relevant to CHO enhancements, it is enough from RAN2 point of view to assume that a source/target cell is a cell where it is not preferred for UEs to connect.

Proposal 1: Working Assumption: “NES mode” of source/target cell means a cell where it is not preferred for the UE to connect. It is up to NW implementation how energy is saved in “NES mode” with less or no UEs Connected.

=>	Noted

R2-2308302	Discussion on connected mode mobility	CMCC	discussion	Rel-18	Netw_Energy_NR-Core

Proposal 5: RAN2 needs to clarify whether cell switching off is one case of NES mode and whether it should be indicated to the UE.

=>	Noted

R2-2307766	CHO on NES	Nokia, Nokia Shanghai Bell	discussion	Rel-18	FS_Netw_Energy_NR

Proposal 1: “The NES trigger” would be at least for the use case of turning off the cell (whether other triggers are enabled is FFS and need to wait that WI progresses on other aspects of the WI).

=>	Noted

Discussion

-	Nokia thinks that we should support turning off and we should introduce something to turn off like paging.  Lenovo asks why is paging considered for connected mode.   Nokia explains that the UE monitors paging for SI update.

-	Vodafone thinks that we shouldn’t talk about NES mode, but we should talk about features.  Switching off is one of the techniques and we should design how to enable the feature.

-	Lenovo thinks that we can also consider Cell DTX/DRX with infinite periodicity.  We should have the ability to switch off.

-	CATT thinks that we can use the legacy for switching

-	Huawei would like to make sure that the framework is general

-	Xiaomi thinks that this is not in the WI description.

NES CHO Execution Trigger

R2-2308697	CHO enhancement for different NES use cases	LG Electronics Inc.	discussion	Rel-18

Proposal 2.	CondEvent T1 is supported for UE capable of GNSS or SIB9 to support cell-off use case.

R2-2307475	Discussion on the CHO Enhancements for NES	Google Inc.	discussion	Rel-18

Proposal 1	The time-based CHO condition is not supported in Rel-18 for the NES purpose.

R2-2308740	Conditional handover enhancement for network energy saving	vivo	discussion	Rel-18

Proposal 4: For the explicit signalling for CHO, RAN2 to consider the below options:

- Dedicated MAC CE;

- Short message sent on common PO.

=>	Noted

R2-2307811	Further Discussion on CHO enhancement in NES	Apple	discussion	Netw_Energy_NR-Core

Proposal 5: On the signaling for the UE to determine source cell enters NES mode, introduce indication in SIB1.

-	BT asks what is the point as the UE already knows DTX/DRX and if the cell is off there will be no SIBs.  Apple explains that this indication would be sent a bit in advance of switching off.  Vodafone agrees with Apple.

=>	Noted

Discussion

-	Lenovo thinks that the timer based is dependent on GNSS and we would have to design something different anyways.   For trigger we can use the existing RRC and activation/deactivation

-	Nokia thinks that we need to understand the use case for the timer based case.

-	Oppo thinks that the time based solution is a good one and just focus on the enhancements that are needed.

-	Huawei thinks that we shouldn’t waste time with timer and we should chose between the two options to trigger CHO for switching off and Cell DTX/DRX:

1.  new MAC CE

2.  Indication in SIB

- 	InterDigital thinks that the indication in SIB is very unflexible so we should focus on either MAC CE or the existing indication that is specified in RAN1 for CELL DTX/DRX

-	CMCC thinks that the timer based should be considered as the switching on off is predictable

-	Sharp asks of the indication is sent after the modification notification boundary.  Apple says yes

-	Mediatek is concerned that we are modifying how we are addresing mobility

-	Qualcomm thinks MAC CE is the best option as we don’t do use SIBs for mobility and it will impact idle UEs as they have to re-aquire SIB1.

-	Xiaomi thinks that MAC CE is dedicated signalling so it is not much benefit compared to RRC configuration.   Timer based mechanism is preferred.

-	CATT explains that the operators would turn off cells at a particular time, so we can use the timer based mechanisms.  The UE doesn’t need to have GNSS capabilities it can acquire UTC.  We excluded MAC CE for DTX/DRX so why are we discussing it now.

-	Lenovo clarifies for the cell DTX/DRX we already have mechanisms and now we are just talking about switching off.

-	Sony prefers MAC CE.

-	Nokia asks what is the use case.  It is not to reduce signaling load but rather to do things faster.   We can consider the Cell DTX/DRX L1 indication but we shouldn’t introduce new higher layer indications.   NEC would like to keep option 2 for cell switching.

-	Qualcomm thinks that L1 signaling is unreliable and we should use it for CHO trigger.

-	Ericsson doesn’t think SIB signaling is beneficial, as it is very slow and there isn’t much difference between L1/MAC CE as they both unsecure, so we should consider using the L1 DTX/DRX signaling.   Apple indicates that in LTM L1 group layer 1 mobility was discussed but not agreed due security.   Samsung thinks that we should re-use the L1 signaling

-	Interdigital thinks that based on all comments only the MAC CE seems to not have a problem.   BT agrees and doesn’t want to have multiple solutions, so MAC CE can be used for all cases.

-	Vodafone has some sympathy of modelling cell switch-off with the DTX/DRX

-	Mediatek would like to ensure that we have one framework.   Qualcomm explains that this is not triggering moblity but triggering CHO, so this is not LTM.   Lenovo also clarifies that this has nothing to do with LTM.

-	Lenovo ask why the existing RRC signaling is not sufficient, we can model cell switch off within DRX.   Qualcomm doesn’t think cell switch off cannot be modelled within DTX/DRX

=>	offline (Lily)

R2-2309211

-	Qualcomm thinks that group-based L1 signaling doesn’t work technically.

-	Lenovo asks for MAC CE what happens to the UEs that don’t meet the conditions.  InterDigital explains that those UEs would stay in the cell as this is not a mobility command.

-	Nokia and Ericsson thinks that this MAC CE is redundant and everything is already signalled to the UEs.

-	Qualcomm is concerned that the only thing left is something that doesn’t work.

Proposal 1: (8/14) Group-based L1 signalling for cell DTX/DRX activation is not reused for NES CHO execution triggering.

Proposal 2: (11/16) A new MAC CE is not introduced for NES CHO execution condition.

Proposal 3: (13/18) (At least for cell DTX/DRX) Time-based CHO is not to be considered in NES.

Proposal 4: (9/13) Do not consider using an indication in SIB1 for triggering NES CHO execution condition.

Agreements

1	We will support the CHO triggers for the use case of turning off the cell

2	(At least for cell DTX/DRX) Time-based CHO is not to be considered in NES.

3	Do not consider using an indication in SIB1 for triggering NES CHO execution condition

Target cell

R2-2308608	Discussion on Connected mode mobility for network energy savings	Fujitsu	discussion	Rel-18	Netw_Energy_NR-Core

Proposal 7: 	RAN2 to confirm the UE does not have to obtain the NES mode of the cell from the target cell.

Proposal 8: 	The priority information is additionally provided by the source cell to select a suitable target cell.

Proposal 9: 	The UE selects the CHO candidate cell indicated as a high priority if multiple CHO candidate cells fulfill the condition and the priority information is provided.

Failure Handling

R2-2308697	CHO enhancement for different NES use cases	LG Electronics Inc.	discussion	Rel-18

Proposal 3.	The UE should send an indication indicating that there is no suitable cell for execution conditional mobility.

R2-2307766	CHO on NES	Nokia, Nokia Shanghai Bell	discussion	Rel-18	FS_Netw_Energy_NR

Proposal 2: In case there is no candidate cell fulfilling radio conditions of NES event at point of time when NES trigger is received, we can rely on RRC re-establishment/RLF handling.

R2-2307811	Further Discussion on CHO enhancement in NES	Apple	discussion	Netw_Energy_NR-Core

Proposal 6: Failure handling enhancement (i.e. new UE behavior if there are no triggered cells after the source cell has entered NES mode) is not pursued.

R2-2307151	Discussion on UE mobility due to NES cell	Xiaomi	discussion	Rel-18

R2-2307179	CHO Procedure in NES Mode	Lenovo	discussion	Netw_Energy_NR-Core

R2-2307221	Discussion on Connected mode mobility for NES	Samsung Electronics GmbH	discussion	Rel-18

R2-2307456	Discussion on CHO for NES	Ericsson	discussion	Rel-18	Netw_Energy_NR-Core

R2-2307475	Discussion on the CHO Enhancements for NES	Google Inc.	discussion	Rel-18

R2-2307645	Discussion on CHO enhancement	NEC	discussion	Rel-18	Netw_Energy_NR-Core

R2-2307649	NES Connected mode mobility	Qualcomm Incorporated	discussion	Rel-18

R2-2307766	CHO on NES	Nokia, Nokia Shanghai Bell	discussion	Rel-18	FS_Netw_Energy_NR

R2-2307811	Further Discussion on CHO enhancement in NES	Apple	discussion	Netw_Energy_NR-Core

R2-2307917	Discussion on CHO enhancements for NES	Sharp	discussion

R2-2308031	Discussion on CHO enhancement for NES	Huawei, Turkcell, HiSilicon	discussion	Rel-18	Netw_Energy_NR-Core

R2-2308175	Handover enhancement for NES	Sony	discussion	Rel-18	FS_Netw_Energy_NR

R2-2308182	Discussion on connected mode mobility	OPPO	discussion	Rel-18	Netw_Energy_NR

R2-2308198	CHO enhancement for NES cell	Quectel	discussion

R2-2308282	The remaining issues on connected mode mobility	ZTE corporation, Sanechips	discussion	Rel-18	Netw_Energy_NR-Core

R2-2308302	Discussion on connected mode mobility	CMCC	discussion	Rel-18	Netw_Energy_NR-Core

R2-2308390	CHO for NES	InterDigital	discussion	Rel-18	Netw_Energy_NR-Core

R2-2308608	Discussion on Connected mode mobility for network energy savings	Fujitsu	discussion	Rel-18	Netw_Energy_NR-Core

R2-2308697	CHO enhancement for different NES use cases	LG Electronics Inc.	discussion	Rel-18

R2-2308730	CHO procedure enhancements	CATT	discussion	Rel-18	FS_Netw_Energy_NR

R2-2308740	Conditional handover enhancement for network energy saving	vivo	discussion	Rel-18

### 7.3.6	Others

This will be downprioritized

R2-2307458	MAC CE for activating/deactivating SP CSI report configurations for NES 	Ericsson	discussion	Rel-18	Netw_Energy_NR-Core

R2-2307650	Restricting Paging to limited area	Qualcomm Incorporated	discussion	Rel-18

R2-2308045	Discussion on RAN1 and RAN3 led NES techniques	Huawei, HiSilicon	discussion	Rel-18	Netw_Energy_NR-Core

R2-2308154	Skip monitoring of CSI-RS during non-active periods	Sony	discussion	Rel-18	FS_Netw_Energy_NR

## 7.8	NR support for UAV

(NR_UAV -Core; leading WG: RAN2; REL-18; WID: RP-223545)

Time budget: 1 TU

Tdoc Limitation: 3

### 7.8.1	Organizational

Stage 2 running CR expected as input to this meeting

Expected input: Running CRs for 38.331 (Qualcomm), 38.300 (Nokia)

Expected input after capability discussions: 38.306 (Huawei)

R2-2307034	Reply LS to RAN2 on flightpath information forwarding for UAV (R3-233493; contact: ZTE)	RAN3	LS in	Rel-18	NR_UAV-Core	To:RAN2	Cc:SA2

=>	Noted

R2-2307059	Reply LS on BRID and DAA broadcast over LTE and NR PC5 (S2-2307781; contact: LGE)	SA2	LS in	Rel-18	NR_UAV-Core, UAS_Ph2	To:RAN2

=>	Noted

R2-2307582	Uncrewed Aerial Vehicles in Rel-18 - Updated Workplan	Nokia, Nokia Shanghai Bell	Work Plan	Rel-18	NR_UAV-Core

=>	Noted

R2-2307583	38.300 Running CR for Rel-18 UAVs	Nokia, Nokia Shanghai Bell	draftCR	Rel-18	38.300	17.5.0	NR_UAV-Core

=>	The CR is endorsed

R2-2307634	RRC Running CR (Introduction of Aerial Support)	Qualcomm Incorporated	draftCR	Rel-18	38.331	17.5.0	B	NR_UAV-Core

=>	The CR will used as a baseline for POST meeting email discussion

### 7.8.2	Measurement reporting for mobility and interference control

Contributions should focus on further details related enhancement to measurement reports taking into account agreements made in RAN2#121bis-e

Combination of Ax/Hx

Supported Ax

R2-2308833	Discussion on combination event for NR UAV	vivo

Proposal 1：Event A3, A4 and A5 can be considered to combined with Event H1/H2.

R2-2308702	Measurement report enhancement for NR UAV	Huawei, HiSilicon

Proposal 3: Beside for event A4, the combination of the other event Ax and event H1/H2 should not be introduced.

Stage-3 details of Ax/Hx combination

R2-2307735	On measurement reporting enhancement	ZTE

Proposal 1: An unified solution, i.e. new event type, is considered to implement both height-dependent MR configuration and combination of events.

Proposal 2: To introduce new event types on the combination of event A3/A5 and event H1/H2.

Proposal 3: The new event type can include both H1 and H2 threshold, or one of H1 and H2 threshold.

R2-2308833	Discussion on combination event for NR UAV	vivo

Proposal 2: The entering condition for the combination event can be satisfied when both entering condition of event Ax and event H1/H2 are fulfilled.

Proposal 3: The network can configure a timeToTrigger for combination event. When the entering condition of the combination is satisfied and both event Ax and event H1/H2 are fulfilled during TimeToTrigger, the UE can trigger the measurement reporting procedure.

Proposal 4: The UE can trigger the measurement reporting procedure when either of the event Ax and event H1/H2 satisfies the leaving condition.

Discussion

-	Nokia thinks that defining height ranges would be simpler.   Qualcomm thinks that creating new events would be simpler.   LG agrees with Qualcomm

-	Samsung thinks that Qualcomm comment is valid.

Proposal 3: The new event type can include both H1 and H2 threshold, or one of H1 and H2 threshold.

-	Qualcomm explains that in RRC we cannot have both threshold.  Huawei agrees.   NEC would like to have a lower bound and higher bound.   Qualcomm explains that we implement the height ranges with multiple events.

-	Huawei thinks that only thing we need is a hysteresis on top of multiple ranges.  Nokia thinks that if we just use a hysteris how do you avoid the ping-pong.  Huawei explains that you don’t use hysteresis to implement multiple ranges.    You use it for case where you have a drone flying between the two ranges to avoid.

-	Nokia would like to study this further.  Huawei thinks flexibility is very important and Samsung thinks that Nokia’s concern can be handled by implementation.

Hysteresis

R2-2307735	On measurement reporting enhancement	ZTE

Proposal 4: To introduce hysteresis for the new event types along with H1/H2 threshold, i.e. similar to the hysteresis introduced for height-dependent SSB-ToMeasure configuration.

R2-2308605	On Height-dependent Measurement Report Configuration for UAVs	Nokia, Nokia Shanghai Bell

Proposal 4: Do not consider the hysteresis parameter when evaluating the triggering or entering conditions of Event H1 and Event H2 when the trigger is due to an aerial UE already satisfying the triggering or entering conditions of either Event HX immediately upon being configured with one or more of Event(s) HX.

Discussion

-	Nokia is concerned with the hysteresis for each threshold.  Samsung understand Nokia’s concern but is not sure we need to address it, we can think until next meeting.  Huawei and Qualcomm thinks that is natural to have a hysteresis for each threshold in the report.  CMCC thinks that hysteresis can solve the ping pong issues.  Qualcomm agrees.

Discussion on event A3/A5

-	Huawei thinks that A3/A5 will not work with vertical mobility as the cells can vanish very quickly and we should just do it for A4 and not try to optimize.

-	LG thinks that it can be advantageous to combine A3 and A5.  ZTE and Samsung agree.

-	Qualcomm has sympathy for Huawei.  Apple thinks that the new event is for interference control and not for mobility.   Samsung thinks that A3 and A5 are also for mobility handling so it useful to combine.

-	Ericsson thinks that we can combine any event and you can combine them at the measurement ID levels.   Qualcomm thinks that we haven’t yet decided.

-	Nokia explains that A4 is important

-	Qualcomm indicates that if we agreed to A3/A5 we need to write 6 events.

Information within MR triggered by height

R2-2308821	Discussion on measurement reporting enhancements for NR UAV	Xiaomi

Proposal 1: A new indicator configured by the network for UE height reporting can be introduced in report configuration.

Proposal 2: For measurement reporting triggered by event H1 and event H2, existing location reporting mechanism can be reused. If the includeCommonLocationInfo is configured by network, UE location information, i.e. CommonLocationInfo shall be included in the measurement report.

Proposal 3: Horizontal and vertical speed can be included in location information.

Proposal 4: For measurement reporting triggered by event H1 or event H2, measurement results of serving cells and neighbour cells can be included in the content of the measurement report based on the network configuration.

SSB-toMeasure & numberOfTriggeringCells

SSB-toMeasure

R2-2307734	On height-dependent SSB-ToMeasure configuration	ZTE

Proposal 5: It’s left to UE implementation on how to handle the change of SSB-ToMeasure configuration due to change of height range.

Proposal 6:To capture in spec the condition that UE starts and stops to apply the height dependent SSB-ToMeasure configuration.

Proposal 7: To extend height-dependent SSB-ToMeasure to RRM measurement for cell re-selection, Idle/Inactive measurements and RACH resource selection.

-	Qualcomm supports the proposal.   Samsung thinks this is out of WI scope.   Qualcomm doesn’t thinks it is out scope as it is not limited to RRC Connected.   Nokia explains that in LTE it is limited to CONNECTED and companies may have made a similar assumption.   Huawei thought that this is not a bad thing to have but we have already doubled the content.

numberOfTriggeringCells: Config details

R2-2307681	Further discussion on NR support for UAV	NTT DOCOMO, INC

Proposal1: Introduce multiple event A4 thresholds and numbeOftTriggeringCells values for different height ranges in EventTriggerConfig.

numberOfTriggering cells: Cell handling

R2-2308702	Measurement report enhancement for NR UAV	Huawei, HiSilicon

Modified online Proposal 6: RAN2 should consider that the UE keeps the cell in cellTriggeredList when the UE moves to a new height region with a same/different numberOfTriggeringCells value.

-	Huawei has a preference to keep the old results.  Samsung thinks that when the UE moves to a new range it should apply new measurement results.  When the UE reconfires offset it discards measurement.   LG thinks that if we introduce new event we resolve the problem with proposal 6.    Nokia and ZTE has the same view.

-	Qualcomm thinks that we should keep celltriggered list as it is.   Ericsson agrees and doesn’t think we should leave it up to UE implementation.

R2-2308833	Discussion on combination event for NR UAV	vivo

Proposal 7: For combination event, if the event H1/H2 satisfies the leaving condition, the UE should empty the cellsTriggeredList. if the event Ax satisfies the leaving condition, the UE should remove the concerned cell in the cellsTriggeredList

R2-2308651	Discussion on Measurement Reporting for NR UAV	China Telecom

Proposal 5: Whether to discard or keep the old cells in the cellTriggeredList when the numberOfTriggeringCells is changed in a new height region is left to UE implementation.

numberOfTriggering cells: Interference detection

R2-2308605	On Height-dependent Measurement Report Configuration for UAVs	Nokia, Nokia Shanghai Bell

Proposal 2: Study a method which allows a UE to measure all SSBs, for the purpose interference reporting, for events configured with numberOfTriggeringCells.

Height range details

Maximum height

R2-2307635	Measurement and reporting enhancements	Qualcomm

Proposal 3. For RRC signalling, the maximum UE height both for configuration and reporting is 20km.

R2-2308702	Measurement report enhancement for NR UAV	Huawei, HiSilicon

Proposal 1: The maximum altitude for UAV applications in TS 22.125 is 300m, and four height ranges should be the maximum.

Discussion

-	Nokia asks what does the value 20km represent.  QC explains it is HAPS.

-	Samsung is realistic and supports Huawei.

-	CMCC thinks that we should be future proof as we now have the requirements include 600m

-	Huawei thinks that we can be future proof but we should ask how many zones we need.

-	Nokia agrees that we should be realistic.

-	CMCC doesn’t think we can rely on the LTE max height and in NR we can meet new requirement.  QC agrees with CMCC as the range in LTE was 300m and with NR we can have a bigger range with NTN.

-	Vodafone asks what the height is in relation to, the ground?  QC explains it is according to sea level.

-	Samsung explains that SA1 requirements has 300m max in the spec.

Number of supported height ranges

R2-2308832	Discussion on measurement reporting enhancement for NR UAV	vivo

Proposal 2: Two height ranges are introduced for height-dependent MO configurations.

R2-2308707	Remaining issues on measurement reporting enhancements in NR UAV	Samsung Electronics Nordic AB

Proposal 4: Maximum number of height ranges in height-based list of SSB-ToMeasure in MeasObjectNR is 4.

R2-2307635	Measurement and reporting enhancements	Qualcomm

Proposal 1.	For height-based list of SSB-ToMeasure, number of height ranges is 32.

Configuration details

R2-2307734	On height-dependent SSB-ToMeasure configuration	ZTE

Proposal 4: It’s up to network implementation to ensure there is no overlapping on height ranges.

-	Huawei thinks it should be - the handling of possible overlap of height ranges it is up to network control.  Qualcomm asks then what does the UE do if it overlaps.

-	Qualcomm and Apple asks if can the UE be in two height ranges then what happens.

-	Samsung understands that the network should avoid these situations.   Nokia would like to separate the thresholds from the hysteresis.  We should avoid overlapping ranges.

R2-2307635	Measurement and reporting enhancements	Qualcomm

Proposal 4. Signalling should allow the network to simultaneously configure height-based event thresholds for the whole range of possible UE heights.

-	Samsung and InterDigital thinks it is good for the network to support the full range.  Nokia agrees but we want to ensure that we don’t define too many ranges.

R2-2308702	Measurement report enhancement for NR UAV	Huawei, HiSilicon

Proposal 2: It is left to the network implementation the number of height ranges that can be configured for the UAV, i.e., the network does not need to always configure the maximum height ranges for the UE.

Agreements:

Measurement reporting

- New event type is introduced to implement both height-dependent MR configuration and combination of events

- The new event type includes only one of H1 or H2 thresholds.   A hysteresis for each threshold will be introduced.  A single time to trigger for the event will be introduced. Rapporteur will implement the solution and we can come back next meeting if there are technical issues.

- Event combination of H1/H2 with A3, A4, A5 will be support

- When the UE moves to a new height region with a different numberOfTriggeringCells value for one configured event, it is up to UE implementation whether the UE keeps the old measurement report or cellstriggeredlist

- At least 300m above ground level will be supported.  FFS if higher ranges can/need to be considered.

- Signalling should allow the network to simultaneously configure height-based event thresholds for the whole range of possible UE heights

Flight path update/reporting

- No delta signaling for updated flight path will be supported

- Confirm that same indication is used for both initial and updated flightpath plan

- If any waypoint/timestamp has changed more than the configured threshold the UE triggers flightpath update indication.

- When both distance and time threshold are configured, the flightpath update can be triggered if either of those conditions are met.   No prohibit timer will be introduced.

No transmit zone

R2-2307463	UAV measurement reports 	Ericsson

Proposal 1	RAN2 to study and specify mechanisms to fulfill requirements on no-transmit zone in ECC Decision (22)07 in the ongoing WI.

Proposal 2	RAN2 to specify signalling that allows NTZ-affected cells to bar aerial UEs. FFS UE signaling

R2-2307441	Discussion on Measurement Reports Enhancements	NEC

R2-2307869	Measurement reporting enhancement in UAV	Apple

R2-2307996	Discussion on height dependent measurement for NR UAV	Lenovo

R2-2308298	Discussion on the FFS issues for measurement reporting	CMCC

R2-2308651	Discussion on Measurement Reporting for NR UAV	China Telecom

R2-2308686	Discussion on no-transmit-zone	LG

R2-2308797	Measurement Report Enhancement	LG

### 7.8.3 	Flight path reporting

Contributions on enhancements to flight path reporting

Delta flightpath report

No delta signalling

R2-2307997	Remaining issues of flight path reporting for NR UAV	Lenovo

Proposal 3: not support delta signaling for flight path update report

Timestamp only

R2-2307279	Consideration on flight path reporting for NR UAV	DENSO

Proposal 1: RAN2 to allow UE to update flight path partially at least for the timestamps

Proposal 2: RAN2 to introduce index for each waypoint so that UE can update flight path partially

Proposal 3: Define wayPointLocation as optional field to allow UE to update timeStamp only

Timestamp and waypoint

R2-2307636	Flight path reporting enhancements	Qualcomm

Proposal 2. Signalling of the changes in flight path information by the UE is supported.

Proposal 3. Add a Way Point ID to identify a particular waypoint in the UE-reported flight path.

Proposal 4. Use xxToAddModList and xxToRemoveList to enable signalling of the changes in the flight path.

Other

R2-2308299	Considerations on the remaining issues for flight path reporting	CMCC

Proposal 1: Reporting whole or delta flight path after updated could be left to implement.

Discussion

-	LG has changed their mind would like to support delta reporting as the UE can recognize which waypoint has been updated.

-	Huawei has also changed their mind as we couldn’t come up with a unified solution with the proponents.   Also the AS doesn’t know much about the flight path so we don’t have a way to engineer anything.

Flightpath update notification

Contents of flightpath update notification

R2-2307584	Even Further Details on Flight Path Plan (FPP)	Nokia, Nokia Shanghai Bell

Proposal 7:  Confirm that a single indication is used for both initial and updated flightpath plan.

R2-2308467	UAV Flight Path Reporting	Ericsson España S.A.

Proposal 4: Include codepoints along with flight path update indicator to indicate the triggering condition(s) for the flight path update.

-	Ericsson thinks the information on the condition can be used by network to decide whether to request the update.  Samsung doesn’t think it is needed as it configures the criteria anyways.  NEC, QC, Nokia and ZTE also doesn’t think it is needed.   Huawei would like to give Ericsson one more chance.

R2-2308528	Flightpath update notification for UAV	InterDigital

Proposal 5: NW can configure UE to report assistance information about the flight path update.

Proposal 6: Flight path update assistance information can include if only time information has changed.

Notification triggering: Details

R2-2308834	Discussion on flight path reporting	vivo

Proposal 1: a waypoint/timestamp delta threshold with value zero can be optionally configured.

Proposal 2: if any waypoint/timestamp has changed more than the configured threshold except that 0 or infinity, the UE triggers flightpath update indication.

R2-2307442	Discussion on Flight Path Reporting	NEC

Proposal 1:	Consider following conditions to trigger flightpath update notification:

-	More than M waypoints’ timestamps have changed from the reported FPP, and the change is more than a configurable time threshold

-	More than N waypoints’ locations have changed from the reported FPP, and the change is more than a configurable distance threshold

Discussion

-	Vivo and Qualcomm think that zero means it is up the UE if reports

-	Huawei thinks that the threshold should be controlled on waypoint basis and have option to report always.

-	Interdigital agrees with proposal 1 from NEC.   Qualcomm thinks that if any waypoint cross the threshold the UE should report.    Samsung and LG agrees with Qualcomm.

-	Nokia thinks it is important to differentiate between waypoints and it is should be up to the network if the timing of a waypoint or multiple way point has changed and it should be configurable.

-	ZTE agrees with Qualcomm and we agreed to have something simple last meeting.

-	Ericsson thinks it makes sense to have the number of way points.

-	Nokia is concerned that if we have triggers on single way point this will force the network to configure a bigger threshold.

-	QC asks what about - distance and time – is it “and” or “or”.   Huawei thinks that we should configure both and be reported independently.  Question is do we also put the option to have “and”.   Samsung thinks that from UE perspective the UE should report if either of them change.  Nokia understands that anyway if one changes the other should change anyways.

Discussion on prohibit timer for flight path update

-	Samsung thinks that if no threshold is configured the UE would report frequently.

-	Nokia doesn’t think this is needed.  InterDigital also doesn’t think this is a big problem, we have many ways to control the reporting, including setting threshold and not requesting update at all.

Notification triggering: Configuration aspects

R2-2308704	Further discussion on flight path reporting	Huawei, HiSilicon

Proposal 1: RAN2 should clarify that the gNB can configure the delta time AND/OR the delta distance for the flight path update indication trigger to the UE rather than the delta time OR the delta distance.

Proposal 5: If the source gNB has a flight path (update) indication and the old flight path, the source gNB should forward the flight path (update) indication to the target gNB during the handover procedure.

-	Vodafone things that this scenario can happen, as there can be two vendors with different feature capabilities.   InterDigital thinks that this can be solved by adding the flightupdate bit on source to targe cell information.  Nokia thinks that the scenario is valid but we can already address it but sending the initial messages to target cell.

-	Qualcomm thinks scenario is valid and agrees with Nokia.   Samsung and CMCC also thinks that the scenario is valid but it can be addressed.   CMCC thinks that we should keep it simple.   Ericsson also thinks that there are mechanisms in place to retrieve the information.

=>	The scenario is valid but current solutions should already address these scenarios.

=>	Noted

R2-2307279	Consideration on flight path reporting for NR UAV	DENSO

Proposal 5: The mechanism of ReportConfig for measurement report could be used for the triggering condition configuration by the network

Flightpath Report Contents

Timestamp details

R2-2307918	Discussion on flight path reporting for NR UAV	Sharpn

Proposal 1: The UE includes time stamp information in flight path reporting in UEInformationResponse message when includeTimeStamp in UEInformationRequest message is configured as LTE.

Proposal 2: A parameter similar as includeTimeStamp can be added in RRCReconfiguration message to control whether timestamp needs to be reported for flight path update.

R2-2308467	UAV Flight Path Reporting	Ericsson España S.A.

Proposal 5	To increase the informativeness of flight path reporting, include hysteresis, or time window to the time stamp of a UTC waypoint.

Waypoint details

R2-2307584	Even Further Details on Flight Path Plan (FPP)	Nokia, Nokia Shanghai Bell

Proposal 1: Use OCTET STRING to convey the LocationCoordinates in the WayPoint definition.

Proposal 2: Use the name WayPoint instead of WayPointLocation in NR.

Proposal 3: Use the above ASN.1 WayPoint definition as starting point to define the content of the FlightPathInfoReport with the flightPath-r18 as mandatory field in this IE.

Proposal 4: Flight path plan in NR Rel-18 distinguishes the start- and end-related waypoints from other reported waypoints. These waypoints shall contain timestamp. Other remaining timestamps may have no spatial or temporal requirements.

R2-2307736	On flight path reporting	ZTE

R2-2307870	Flight path reporting in UAV	Apple

R2-2308529	Flightpath reporting for UAV	InterDigital

R2-2308653	Discussion on Flight Path Reporting for NR UAV	China Telecom

R2-2308692	Discussion on flight path reporting	Samsung

R2-2308720	Discussion on triggering of flight path report	ASUSTeK

R2-2308798	Flight Path Information Report	LG

R2-2308822	Discussion on flight path reporting for NR UAV	Xiaomi

### 7.8.4	Subscription-based aerial-UE identification

Contributions should focus on signaling required to support subscription-based aerial-UE identification

This AI will not be treated and no contributions are expected, as no further NR enhancements will be pursued.

### 7.8.5	UAV identification broadcast

UAV identification broadcast using PC5-U will be treated with higher priority.

LS from SA2

R2-2307585	On the Remaining Aspects for Supporting PC5-based BRID and DAA	Nokia, Nokia Shanghai Bell

Proposal 1: Unicast-based DAA can be supported in Rel-18, but only its introduction does not require additional RAN2 work besides defining a sidelink capability.

-	Ericsson asks why we need a new capability.  If the UE supports SL it can do unicast, broadcast.  Nokia explains that the intention is that the UE reports that the UE supports unicast or broadcast DAA.  Samsung also doesn’t think we need a new capability.

Proposal 2: RAN2 updates the NR specification (e.g. by pointing to a proper table with a list of PQIs), but only after the topic is resolved in SA2.

Proposal 3: RAN2 is asked to consider if separate PQI for BRID and DAA over NR SL justifies the introduction of a dedicated SL Tx resource pool.

=>	Noted

R2-2308560	Resources for broadcast of BRID and DAA	Beijing Xiaomi

Proposal 1: RAN2 do not introduce separate resource pools for support A2X broadcast of BRID and DAA transmission over PC5 interface in REL-18

Proposal 2: RAN2 replies to SA2 to confirm that for REL-18 RAN2 has no requirement for new PQIs for A2X service identification in order to facilitate differentiated handling between V2X and A2X services.

=>	Noted

R2-2307637	Remaining aspects of PC5-based BRID and DAA support	Qualcomm

Proposal 3. Separate SL resource pool for BRID and DAA broadcast is supported.

=>	Noted

Discussion on separate resource pools

-	Qualcomm explains that if you have a separate pool then there is no collision with other services and quality can be comprised.   Ericsson agrees.

-	Xiaomi explain that it is makes dimensioning difficult and separating the pools means taking the resources from other services.  Nokia has similar concerns with Xiaomi.

-	Huawei point of view there is no need to separate the pools.   Samsung would like to have some flexibility and be able to configure separate or common pool.

-	Huawei thinks that maybe we can see if we can have a simple way of indicating whether a V2X pool can be used.

Agreements:

- Unicast-based DAA can be supported in Rel-18, but only if its introduction does not require additional RAN2 work besides defining a capability (if needed)

- RAN2 will support (pre-) configured separate SL resource pool for BRID and DAA broadcast

## 7.18	Mobile Terminated Small Data Transmission

(NR_NR_MT_SDT-Core; leading WG: RAN2; REL-18; WID: RP-222993)

Time budget: 0.5 TU

Tdoc Limitation: 2 tdoc

This WI is planned to be concluded and finalized at current meeting.

### 7.18.1	Organizational

Running CRs expected as input in this meeting: 38.300 (Nokia), 38.331 (ZTE), 38.321 (Huawei), 38.306 (Intel).

R2-2307128	Introduction of MT-SDT to MAC spec	Huawei, HiSilicon	CR	Rel-18	38.321	17.5.0	1634	-	B	NR_MT_SDT-Core

=>	The CR is endorsed

R2-2307129	Summary of [Post122][309][MT-SDT] 38.321 Running CR (Huawei)	Huawei, HiSilicon	discussion	Rel-18	NR_MT_SDT-Core

Proposal1: Confirm that the condition for data volume threshold is not applicable for Small Data Transmission procedure triggered for MT-SDT.

-	LG thinks that there is a case where large SDT data is generated at paging reception

-	Intel explains that this was already agreed and discussed and the rapporteur just made this proposal to remove the editor’s note from CR.   LG thinks that we agreed to the RRC layer and we need to discuss the MAC layer.  Huawei has same understanding as Intel.

-	Sony asks how the Network knows as one is DL and data volume is for UL.

-	ZTE explains that we have two procedures, MO-SDT and MT-SDT and the UE will check for MO-SDT and we agreed to keep it separate and it is up to the UE.  Even if the UE is paged it is up to the UE to check.

-	LG asks if the UE can send UL data after MT-SDT is triggered even if there is large data volume.  ZTE thinks that it can, and the UE will send the BSR and it is up the network how it handles, same as getting more data while doing MO-SDT.

Proposal2: Discuss whether it is possible to configure two different RSRP thresholds for MO-SDT and MT-SDT.

-	Vodafone asks why we have two different RSRP thresholds.  Lenovo explains that UL and DL performance is different and link budgets are different.

-	Intel doesn’t think that having a single threshold is sufficient.

-	Sony is good with this agreement.  Nokia agrees and anyways it is better to have different parameters in ASN.1.  Vivo agrees with Lenovo and we need this threshold in case the UE cannot read the Rel-17 parameter.

-	Mediatek this that single threshold is simpler.   Ericsson explains that the nature of the transmissions are quite different and agree with Nokia

-	LG thinks that once the procedure is initiated the UE can transmit both UL and DL and thinks that this can cause trouble if the UE starts MT-SDT with lower threshold and then gets UL data, then data can fail if the DL threshold was too low.

-	Vodafone is not sure how big the difference really is and perhaps one value is sufficient.

-	ZTE doesn’t see the need for this.   Huawei thinks that a single value is sufficient.

-	Intel suggests an optional threshold in the MT-SDT configuration can be a way forward.  If the network wants to indicated a separate one it can include otherwise it can use the same as MO-SDT.  Nokia is ok with this proposals but we have to make sure that if MT-SDT only is configured this should be mandatory

=>	Noted

R2-2307529	RRC Running CR review report (Post122 email 312)	ZTE Corporation (rapporteur)	report

Proposal 1: A separate RSRP threshold for MT-SDT can be configured even if the network configures the corresponding threshold for MO-SDT (i.e. UE uses the corresponding threshold depending on whether MO or MT procedure is triggered)

=>	Noted

R2-2307530	Introduction of MT-SDT	ZTE Corporation (rapporteur)	CR	Rel-18	38.331	17.5.0	4194	-	B	NR_MT_SDT-Core, TEI18

=>	Use this as baseline, rapporteur will take comments into account and continue the review over email

R2-2308082	UE capabilities for Rel-18 MT-SDT WI	Intel Corporation	draftCR	Rel-18	38.306	17.5.0	B	NR_MT_SDT-Core

R2-2308083	UE capabilities for Rel-18 MT-SDT WI	Intel Corporation	draftCR	Rel-18	38.331	17.5.0	B	NR_MT_SDT-Core

R2-2308926	Introduction of MT-SDT in Stage-2	Nokia, Nokia Shanghai Bell	draftCR	Rel-18	38.300	17.5.0	NR_MT_SDT-Core

Agreements

- From RAN2 point of view, the WI is considered complete

### 7.18.2	Control plane aspects

R2-2308077	MT SDT – CP Open Topics on Capabilities and Configurations (including CG-SDT)	Intel Corporation	discussion	Rel-18	NR_MT_SDT-Core

UE capabilities

Proposal 1.	A new optional radio capability signaling (e.g., MT-SDT-r18) is defined to indicate UE’s support of Rel-18 MT-SDT.

Proposal 1.1.	MT-SDT-r18 indicates whether the UE supports transmission of data over allowed radio bearers in RRC_INACTIVE state via Random Access procedure (i.e., RACH) or via configured grant type 1 (i.e., CG-SDT), as specified in TS 38.331 [9].

Proposal 1.1.1.	If a UE supports MT-SDT-r18, this UE shall also support Rel-17 RA-SDT (i.e., ra-SDT-r17).

-	Qualcomm is wondering why we should have a dependency with Rel-17 SDT.  If the UE support MT-SDT it doesn’t mean it has to support MO-SDT.   Intel is ok to not link them, but the thought was that the network can configure them separately, but from the UE perspective it can do UL and DL.

-	Huawei thinks that for RA-SDT we can keep them separately, but for CG SDT there should be a link.

-	Vodafone asks if the intention is that the UE can indicated separately that it support CG-SDT.

Proposal 2.	A separate capability (e.g., srb-MT-SDT-r18) is defined to indicate the support of MT-SDT over SRB2. I.e., two new radio capabilities would be defined e.g., MT-SDT-r18 and srb2-MT-SDT-r18.

-	Huawei asks what is the reason to separate this.  Intel explains that some network vendors and UE vendors had some concerns.

-	Nokia thinks that we should not have a separate capability.   Huawei clarifies that they want to link it with MO-SDT SRB2.   Apple thinks that if the UE supports MO-SDT then Huawei’s suggestion makes sense but in the case that we only have MT-SDT support then we need two capabilities.

-	Qualcomm still thinks that we need a separate capability.

Proposal 2.1.	If a UE supports srb2-MT-SDT-r18, this UE shall also support mt-SDT-r18 and Rel-17 SRB-SDT (i.e., srb-SDT-r17).

Proposal 3.	A new optional radio capability signaling (e.g., sdt-RACH-r18) is defined to indicate UE’s support to select RACH resources instead of configured grant type 1 resource when triggering resume for SDT or MT-SDT and next CG-SDT resource is too far as specified in TS 38.331.

-	LG wonders why we want to introduce a new capability for such a small little feature.  Intel is concerns because it impact the Rel-17 SDT and it is too late to update the Rel-17 capabilities.

-	Nokia thinks that at least for Rel-18 it should be mandatory if you support MT-CG-SDT.   LG agrees.

-	ZTE and NEC think that this is linked to the longer CG period.   LG doesn’t see the link.

Proposal 3.1.	If a UE supports sdt-RACH-r18, this UE shall also support Rel-17 CG-SDT (i.e., cg-SDT-r17).

Proposal 4.	If Proposal 1 to Proposal 3 are agreed, to adopt as baseline the TP captured in the UE capabilities draft CRs for Rel-18 MT-SDT to TS 38.306 available in R2-2308082 and to TS 38.331 available in R2-2308083.

Proposal 5.	A new MT-SDT common SIB configuration (e.g., mt-SDT-ConfigCommonSIB-r18) is defined including the following IEs: sdt-RSRP-Threshold, sdt-LogicalChannelSR-DelayTimer and t319a. The following behaviour is also captured in 38.331 specification:

Proposal 5.1.	When network allows initiation of resume due to MT-SDT and MO-SDT, mt-SDT-ConfigCommonSIB-r18 is signalled but does not include IEs already included in SDT-ConfigCommonSIB-r17 to avoid unnecessary signaling. I.e., UE initiating resume due to MT-SDT also applies the applicable Ies signal in SDT-ConfigCommonSIB-r17).

Proposal 5.2.	When network only allows initiation of resume due to MT-SDT (and not MO-SDT, i.e., network does not signal SDT-ConfigCommonSIB-r17), the required IEs are provided via the new mt-SDT-ConfigCommonSIB-r18.

Proposal 6.	For the BR configuration associated to MT-SDT and provided in RRCRelease message, to discuss whether to define it via: approach (1) a new/independent MT-SDT specific RB configuration, or approach (2) extend the definition of Rel-17 SDT specific RB configurations to also allow its usage for MT-SDT only and both MT-SDT and MO-SDT (with a new signaling indicating the allowed SDT operation).

Proposal 7.	To define cg-SDT-MaxDurationToNext-CG-Occasion-r18 per LCH.

Proposal 8.	For the values of cg-SDT-MaxDurationToNext-CG-Occasion-r18, to consider also add one more value smaller and higher than 100 msec (e.g., 40 and 300 msec).

Discussion on whether only MT-SDT can be configured per UE even if the cell supports MO-SDT

-	Ericsson, Nokia thinks that this should be supported.   Huawei doesn’t see the reason why the network would support MO-SDT but not support MT-SDT.

Agreements

- Confirm that the condition for data volume threshold is not applicable for Small Data Transmission procedure triggered for MT-SDT

- Sdt-RSRP threshold is included in MT-SDT configuration.  Optional IE, but mandatorily present if only MT-SDT is configured.   The network can configure the MT and MO SDT threshold differently if it wants.

- Specify the setting of mt-SDT indication in the resume cause in section 5.3.2 (i.e. at the time of paging reception).

- Discuss per LCH vs Pre UE configuration of cg-SDT-MaxDurationToNext-CG-Ocacassion-r18 as part of the TEI18 discussion together with the extended CG period discussion. Note regardless of the final outcome of extended CG period, this IE will be specified in some form eventually as originally agreed (only details are FFS)

- When a UE receives a paging message indicating MT-SDT and upon first transmission of the CCCH message (i.e., when conditions for initiating MT-SDT are fulfilled), the UE should start timer T319a.

- As already captured in running spec, only MT-SDT cannot be configured per UE, if the cell supports both MO-SDT and MT-SDT

UE capabilities

- A new optional radio capability signaling (e.g., MT-SDT-r18) is defined to indicate UE’s support of Rel-18 MT-RA-SDT-r18.  MT-RA-SDT-r18 indicates whether the UE supports initiation of MT-SDT procedure and transmission/reception in RRC_INACTIVE state via Random Access procedure (i.e., RACH).

- Separate UE capability is used to indicate support MT-CG-SDT.  If the UE support MT-CG-SDT, it has to support MT-RA-SDT.

- SRB2 capability will be captured by adding “or MT-SDT” Rel-17 srb-sdt-r-17 capability.  (i.e. no separate capability)

- FFS how/whether we define UE capability to indicate UE’s support to select RACH resources instead of configured grant type 1 resource when triggering resume for SDT or MT-SDT and next CG-SDT resource is too far as specified in TS 38.331.

R2-2309141	Report of [AT123][301][MT-SDT] 38.331 Running CR (ZTE)

=>	Noted

R2-2307672	RRC configuration and the UE capability for MT-SDT	Xiaomi	discussion	Rel-18	NR_MT_SDT-Core

Proposal 1: Introduce 1 per-UE capability bit for supporting “RACH-based MT-SDT”.

Proposal 2: Introduce 1 per-Band capability bit for supporting “CG-based MT-SDT”.

Proposal 3: Introduce 1 per-UE capability bit for supporting “SRB2-based MT-SDT”.

Proposal 4: The UE supporting SRB2-based MT-SDT shall also support “RACH-based MT-SDT” or “CG-based MT-SDT”.

Proposal 5: The MT-SDT UE capability bits are independent from the MO-SDT UE capability bits (i.e. ra-SDT-r17, cg-SDT-r17, srb-SDT-r17 and pusch-Repetition-CG-SDT-r17).

Proposal 6: Introduce two new RRC configurations (i.e. mt-SDT-DRB-List-r18 for DRB and mt-SDT-SRB2-Indication-r18 for SRB2) in RRCRelease message for configuring MT-SDT RB.

-	ZTE doesn’t see why two separate configurations.  Xiaomi explains that there is an error.   ZTE wants to ensure that the radio bearer configuration is common as we agreed before.  Intel agrees but the network must ensure it can configure only MT-SDT.

R2-2308170	Control plane aspects for MT-SDT	Sony	discussion	Rel-18	NR_MT_SDT-Core

Proposal 2: When a UE receives a paging message indicating MT-SDT and upon first transmission of the CCCH message (i.e., when conditions for initiating MT-SDT are fulfilled), the UE should start timer T319a.

R2-2307117	Remaining Issues on MT-SDT  from CP	vivo	discussion	Rel-18	NR_MT_SDT-Core

R2-2307173	Remaining Control plane issues of MT SDT Procedure in RRC_INACTIVE state	Samsung Electronics Co., Ltd	discussion	Rel-18	NR_MT_SDT-Core

R2-2307518	CP aspects for MT-SDT procedure	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_MT_SDT-Core

R2-2307803	Consideration on MT-SDT from CP perspective	LG Electronics Inc.	discussion	Rel-18	NR_MT_SDT-Core

R2-2307845	Control plane aspects of MT-SDT	Apple	discussion	Rel-18	DUMMY

R2-2307952	Further MT-SDT discussion	Ericsson	discussion	NR_MT_SDT-Core

R2-2307959	remaining CP details for MT-SDT	Lenovo	discussion	Rel-18	NR_MT_SDT-Core

R2-2308349	Control plane aspects of MT-SDT	Huawei, HiSilicon	discussion	Rel-18	NR_MT_SDT-Core

R2-2308364	The remaining issues on CP aspects for MT-SDT	CATT	discussion	Rel-18	NR_MT_SDT-Core

R2-2308405	Remining issues on control plane aspects of MT-SDT	Qualcomm Incorporated	discussion	NR_MT_SDT-Core

R2-2308655	Discussion on CP aspects of MT-SDT	China Telecom	discussion

### 7.18.3	User plane aspects

R2-2307934	Handling BWP restrictions in MT-SDT	Ericsson, T-Mobile USA, Deutsche Telekom	discussion	NR_MT_SDT-Core

Proposal 1	RAN2 handles BWP restriction in MT-SDT with option 2, introducing a Channel Quality Indication (CQI) in Message 3 or Msg A.

Option 1: Remove the CORESET 0 restriction for DL BWP for SDT from/after first DL MT-SDT data

Option 2: Introduce Channel Quality Indication (CQI) in Message 3 or MsgA

-	LG doesn’t think anything is needed as subsequent transmissions can handle additional data.  Ericsson thinks it is true but it would be very inefficient.

-	Sony thinks that there are other solutions, like reusing MBS solutions.

-	ZTE likes this but thinks that this is out of scope of WI.  It can also be done as a TEI 18.

-	CATT thinks that network implementation can solve this issue.

-	LG explains that we discussed this in MO-SDT and concluded it was not needed and in terms of data rate limitations, that’s why we subsequent data.

Proposal 2	Introduce a DCQR MAC CE which can be multiplexed into Msg3, MsgA or the initial CG-SDT transmission in the MT-SDT procedure.

=>	Noted

R2-2308169	Beam failure recovery for Rel-18 SDT	Sony, Nokia, Nokia Shanghai Bell, Samsung	discussion	Rel-18	NR_MT_SDT-Core

Proposal 1: For beam failure recovery in Rel-18 SDT, during ongoing RA based MO-SDT or MT-SDT procedure (performed over RACH) if the RSRP value of the current SSB (i.e., SSB selected in the last random access procedure during the ongoing SDT procedure) is less than a pre-configured threshold, a UE should trigger RACH procedure similar to CG-SDT procedure in Rel-17 SDT.

-	LG doesn’t think this is needed.  Qualcomm doesn’t think there is a beam failure recovery.  Vivo doesn’t thinks this is a critical and the UE can rely on regular measurements for cell reselection.

-	Nokia explains that the UE wouldn’t do cell reselection if the quality of the cell is good enough.  The UE needs to decode the SSB to be able to transmit so we should check similar to CG-SDT.  Huawei has some sympathy for the proposal.

-	Apple wants to ensure that this doesn’t introduce any new requirements.  Nokia explains that we wouldn’t specify requirements as on the rx side anyways, but if there is a problem it would allow the UE to recover.   Qualcomm thinks that there may be UE impact as the UE would need to know when it can check.

-	ZTE has some sympathy on the proposal.   Qualcomm thinks that there may be RAN4 impact for example when the UE should check the SSBs as currently there are no requirements.

-	Intel would like to specify for MO-SDT and MT-SDT if we do it.

=>	We will come back to next meeting, to specify if there are negligible other WG impacts and no new RAN4 requirements imposed on UE.

R2-2308242	User plane aspects of MT-SDT	NEC	discussion	Rel-18	NR_MT_SDT-Core

Proposal 1: For MO-SDT and MT-SDT, if the next CG resource is too far and RA resource is available, the UE performs RA-SDT.

Proposal 2: For MO-SDT, if the next CG resource is too far, and RA-SDT resource is not configured, the UE initiates legacy RRC resume procedure.

Proposal 3: The offset threshold to determine whether CG-SDT resource can be selected should be configured in RRCRelease message.

=>	Noted

R2-2307804	Remaining UP issues on MT-SDT	LG Electronics Inc.	discussion	Rel-18	NR_MT_SDT-Core

Proposal 1. If the time gap between SDT initiation and the next CG occasion is larger than the threshold, the UE should check whether there is an available RA occasion until the next CG occasion.

-	Nokia thinks that this is network configuration issue.  Samsung thinks that the RA occasions will be quite close/short so this is not an issue.  NEC agrees.

=>	Not supported

Proposal 2. DVT condition should be checked in MAC in MT-SDT procedure, same as legacy.

Proposal 3. configuredGrantType1Allowed should be considered for the condition of MT-SDT initiation.

R2-2308927	Selection between CG-SDT and RACH based SDT	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_MT_SDT-Core

Proposal 1: Modify the LCH restrictions condition to initiate CG-SDT to account a scenario where MT-SDT is to be initiated while there is no UL RB with data available for transmission.

-	Intel thinks that this is already possible in the specs.  Apple agrees but current CR doesn’t include it.

-	ZTE indicates that we agreed that we will allow CCCH transmission.

=>	Modify the LCH restrictions condition to initiate CG-SDT to account a scenario where MT-SDT is to be initiated.

=>	Noted

R2-2307960	Remaining UP details for MT-SDT	Lenovo	discussion	Rel-18	NR_MT_SDT-Core

Proposal 1: Confirm the RAN2#122 agreement that RSRP thresholds for MT-SDT and MO-SDT can be configured separately.

Proposal 2: The RSRP thresholds applied for MT-SDT are either signalled within the RRCRelease message or broadcast in the cell.

Proposal 3: It is not necessary to perform carrier selection for MT-SDT and only the SDT resources on NUL carrier should be used to respond to MT-SDT.

R2-2307118	Remaining Issues on MT-SDT  from UP	vivo	discussion	Rel-18	NR_MT_SDT-Core

R2-2307174	Remaining user plane issues of MT SDT Procedure in RRC_INACTIVE state	Samsung Electronics Co., Ltd	discussion	Rel-18	NR_MT_SDT-Core

R2-2307273	Discussion on user plane issues for MT-SDT	Continental Automotive	discussion	Rel-18

R2-2307846	MT-SDT over CG-SDT	Apple	discussion	Rel-18	DUMMY

R2-2308081	MT SDT – UP Open Topics on ROHC and DVT	Intel Corporation	discussion	Rel-18	NR_MT_SDT-Core

R2-2308171	User plane aspects for MT-SDT	Sony	discussion	Rel-18	NR_MT_SDT-Core

R2-2308348	User plane aspects of MT-SDT	Huawei, HiSilicon	discussion	Rel-18	NR_MT_SDT-Core

R2-2308406	Remining issues on user plane aspects of MT-SDT	Qualcomm Incorporated	discussion	NR_MT_SDT-Core

R2-2308656	Discussion on UP aspects of MT-SDT	China Telecom	discussion

## 7.23	Timing Resiliency and URLLC Enh

(NR_TRS_URLLC; leading WG: RAN3; REL-18; WID: RP-230754)

Time budget: 0.5 TU

Tdoc Limitation: 1 tdoc

R2-2307654	Clock Quality Report Delivery	Qualcomm Incorporated	discussion	Rel-18

### 7.23.1   Organizational

Incoming LSs, Rapporteur input etc.

Expected inputs to next meeting, running CRs for the following: 38.300 [Nokia], 38.331 [Ericsson],

R2-2307051	Response to Reply LS on Proposed method for Time Synchronization status reporting to UE(s) (S1-231285; contact: Nokia)	SA1	LS in	Rel-18	TRS_URLLC-NR	To:CT1, RAN2	Cc:SA2, RAN3

R2-2307791	Stage 2 running CR on timing resiliency and URLLC	Nokia, Nokia Shanghai Bell	discussion	Rel-18	TRS_URLLC-NR-Core

=>	The CR is endorse

R2-2308531	Introduction of URLLC and Timing Resiliency	Ericsson	CR	Rel-18	38.331	17.5.0	4258	-	B	TRS_URLLC-NR-Core

### 7.23.2   General

No contributions on BAT offset derivation are expected

R2-2307792	5GS network timing synchronization status and reporting	Nokia, Nokia Shanghai Bell	discussion	Rel-18	TRS_URLLC-NR-Core

Proposal 1: Confirm in RAN2 that no AS capability is needed for the UE to support clock quality information mechanisms.

-	ZTE thinks that the AS capability is needed as the information is sent to UE by dedicated signaling.  Vivo thinks that we can have a capability without signaling.   Huawei, Xiaomi, agree with Nokia, gNB already knows.  Apple is not against to have a capability, but since since we have a NAS capability no need for one.  Vodafone is against having the capability.  Samsung also agres with nokia

Proposal 2: Confirm the gNB is always broadcasting Event ID in SIB9 if it supports the feature.

-	Huawei thinks that SA2 doesn’t already support this so we would have to send an LS.   CATT thinks that they are still discussing and they may align.  SA2 can update if there is something.

Proposal 3: DLInformationTransfer message is extended to include clock quality information reporting towards the UE (i.e., event ID, clock quality metrics or clock quality indication).

-	ZTE asks if we can send event ID with dedicated signaling.  Nokia thinks it is good to be able to signal.   Vivo thinks that we don’t need to provide event ID in connected mode.   Samsung thinks that the event ID should be provided to the UE by dedicated siganling when moving from connected to idle or inactive.

Agreement

- Confirm in RAN2 that no AS capability is needed for the UE to support clock quality information mechanisms.

- Confirm the gNB is always broadcasting Event ID in SIB9 if it supports the feature.  Send LS to SA2 to information.

- DLInformationTransfer message is extended to include clock quality information reporting towards the UE (i.e., clock quality metrics or clock quality indication).  FFS if event ID is included to be discussed in email discussion for 331

R2-2309264	LS on Event ID broadcast in SIB9	RAN2	LS out	Approval	7.23.2	Rel-18	TRS_URLLC-NR-core	SA2

=>	The LS is approved (by email discussion)

R2-2307114	Discussion on Timing Synchronization Status Monitoring	vivo	discussion	Rel-18	TRS_URLLC-NR-Core

Proposal 2: The NAS layer of the UE shall indicate the AS layer to enable the function of timing synchronization status change.

Proposal 4: Upon released from CONNECTED to IDLE/IANCTIVE, UE attempts to acquire the current report ID.

-	Qualcomm thinks that the UE doesn’t need to reaquire anything.  Nokia thinks that we can rely on the latest SIB information.

Proposal 5: UE updates the locally stored reference report ID, once initiating RRC Setup/Resume for Clock Quality Information.

Proposal 6: UE flushes the locally stored reference report ID when moving into a TRS-disabled gNB.

-	Ericsson explains that we need to define something explicit.

=>	discuss in email discussion for 38.331

Proposal 7: RAN2 to discuss whether/how to differentiate UE accessing the network for Clock Quality Information.

R2-2307502	Remaining issues for NR Timing Resiliency	Ericsson	discussion	Rel-18

Proposal 2	RAN2 to discuss how the UE indicates it is interested to receive the clock quality information during the SDT procedure and if a new resume cause value should be defined.

-	ZTE thinks that it would be benefito provide an additional cause in resume message.  ZTE explains it is very important for the AS layer as it can reduce signaling overhead.

-	Qualcomm doesn’t want to change anything to SDT.

-	Nokia assumed we wouldn’t modify the resume cause.  Vivo agrees with Nokia the network implementation can identify the UE and it’s capability.  Xiaomi agrees and we can just use MO signaling.  Samsung htinks that this doesn’t happen often so we can rely on existing signaling.  Huawei also supports to not do anything new.

-	ZTE thinks that UE capability cannot help the network determine whether it is normal transmission or clock information.

-	Intel thinks that we can maybe extend MT-SDT to get triggered by clock information

=>	As a baseline no change to SDT will be support

R2-2307352	RAN2 Impact of 5GS network timing synchronization status and reporting	CATT	discussion	Rel-18	TRS_URLLC-NR-Core

R2-2307560	Discussion on remaining issues for TRS	Huawei, Hisilicon, China Southern Power Grid	discussion	TRS_URLLC-NR-Core

R2-2307600	Remaining issues of time synchronization status and reporting	ZTE Corporation, Sanechips	discussion	Rel-18	TRS_URLLC-NR-Core

R2-2307759	Open Issues on Timing Synchronization	Samsung	discussion	Rel-18

R2-2307782	5GS network timing synchronization status and reporting	Xiaomi	discussion	Rel-18	TRS_URLLC-NR-Core

R2-2307838	UE Access for 5GS Network Timing Synchronization	Apple	discussion	DUMMY

R2-2308308	Discussion on the network timing synchronization status monitoring	CMCC	discussion	Rel-18

R2-2308658	Discussion on Time Synchronization Status and Reporting	China Telecom	discussion

# Summary

## MT-SDT Work Item

Agreements

- From RAN2 point of view, the WI is considered complete

## Email discussions

[POST123][309][R18 URLLC] Running 38.331 (Ericsson)

Scope : Discuss stage-3 signaling details

Review running CR

Outcome: CR to be submitted to next meeting

Deadline: long

[POST123][301][MT-SDT] CR to 38.331 (ZTE)

Scope : Agree in principle with running CR

Outcome: CR to be submitted to plenary

Deadline: short

[POST123][302][MT-SDT] CR to 38.331 (Huawei)

Scope : Agree in principle with running CR

Outcome: CR to be submitted to plenary

Deadline: short

[POST123][303][MT-SDT] CR to 38.306 (Intel)

Scope : review running CR

Outcome: CR to be submitted to next meeting

Deadline: long

[POST123][310][UAV] Running CR 38.300 (Nokia)

Scope : Review running CR

Outcome: CR to be endorsed

Deadline: two weeks

[POST123][311][UAV] Running CR 38.331 (Qualcomm)

Scope : Review running CR

Outcome: CR to be submitted to next meeting

Deadline: long

[POST123][312][NES] Running CR 38.331 (Huawei)

Scope : Review running CR and discuss issue configuration per serving cell or MAC entity

Outcome: CR to be submitted to next meeting

Deadline: long

[POST123][313][NES] Running CR 38.300 (Ericsson)

Scope : Review running CR

Outcome: CR to be endorsed

Deadline: two weeks

[POST123][314][NES] Running CR 38.321 (InterDigital)

Scope : Review running CR

Outcome: CR to be submitted to next meeting

Deadline: long

[POST123][315][NES] Running CR 38.304 (InterDigital)

Scope : Review running CR

Outcome: CR to be submitted to next meeting

Deadline: long