3GPP TSG RAN WG2 Meeting #124       	      	                	               R2- 2312036

Chicago, USA, 13-17 Nov, 2023

Agenda Item:	7.16.2.3

Source:	Intel Corporation

Title:	Proactive and reactive model transfer/delivery

Document for:	Discussion and Decision

# Introduction

During RAN2 #121 meeting, following solutions were agreed and their analyzed pros/cons are summarized in the table.

- Solution 1a: gNB can transfer/deliver AI/ML model(s) to UE via RRC signalling. - Solution 2a: CN (except LMF) can transfer/deliver AI/ML model(s) to UE via NAS signalling. - Solution 3a: LMF can transfer/deliver AI/ML model(s) to UE via LPP signalling. - Solution 1b: gNB can transfer/deliver AI/ML model(s) to UE via UP data. - Solution 2b: CN (except LMF) can transfer/deliver AI/ML model(s) to UE via UP data. - Solution 3b: LMF can transfer/deliver AI/ML model(s) to UE via UP data. - Solution 4: Server (e.g. OAM, OTT) can transfer/delivery AI/ML model(s) to UE (e.g. transparent to 3GPP).

RAN2 #123 meeting further discussed two approaches as model transfer/delivery, i.e. reactive model transfer/delivery and proactive model transfer/delivery.

In this contribution, We provide our understanding of specification impact of reactive and proactive model transfer/delivery methods.

# Discussion

## Proactive/Reactive Model Transfer/Delivery

During RAN2 #123 meeting, following two options for model transfer/delivery were discussed:

- Model transfer/delivery can be initiated in following two ways: - Reactive model transfer/delivery: an AI/ML model is downloaded when it is needed due to changes in scenarios, configurations, or sites. - FFS: Proactive model transfer/delivery: AI/ML models are pre-download to UE, and a model switch is performed when changes in scenarios, configurations, or sites occur.

Except different latency requirement as discussed in [1], in our understanding, whether to choose proactive or reactive model transfer/delivery also depends on the storage/memory available at the UE side. In proactive model transfer/delivery method, multiple AI/ML models will be pre-downloaded and stored at the UE. This will cost more memory compared to reactive model transfer/delivery, as the later one only needs to support a memory size that can hold a max size AI/ML model (per sub-use case).

Observation 1: Proactive model transfer/delivery requires more memory at the UE side to store multiple AI/ML models.

Furthermore, it was agreed in RAN2 #123 meeting:

- RAN2 assumes that for UE-side AIML, the UE may inform the RAN about applicability conditions of AIML algorithm(s) available to the UE, to support RAN control (e.g. activation/deactivation/switching).

Reactive model transfer/delivery requires the UE to report its applicable conditions to the network, either dynamically or statically. However, for proactive model transfer/delivery, as discussed in our companion contribution [2], UE doesn’t need to report applicable conditions, as model switching can be done by UE itself according to its own collected information.

Observation 2: If proactive model transfer/delivery is used, for UE-sided model, UE does not need to inform RAN about applicability conditions of AIML algorithms available to the UE.

Based on above analysis, we further summarized a table comparing proactive/reactive model transfer/delivery.

|  | Proactive model transfer/delivery | reactive model transfer/delivery |
| --- | --- | --- |
| latency requirement | No requirement | Relaxed |
| memory cost at UE | High | Low |
| applicability condition reporting for UE-sided model | Not needed (UE can switch models on its own) | Needed (need NW decision for model transfer/delivery) |

Proposal 1: RAN2 to adopt above table when comparing proactive and reactive model transfer/delivery.

# Conclusion

In this contribution, we analyze the general architecture impact to model transfer method:

Observation 1: Proactive model transfer/delivery requires more memory at the UE side to store multiple AI/ML models.

Observation 2: If proactive model transfer/delivery is used, for UE-sided model, UE does not need to inform RAN about applicability conditions of AIML algorithms available to the UE.

Proposal 1: RAN2 to adopt below table when comparing proactive and reactive model transfer/delivery.

|  | Proactive model transfer/delivery | reactive model transfer/delivery |
| --- | --- | --- |
| latency requirement | No requirement | Relaxed |
| memory cost at UE | High | Low |
| applicability condition reporting for UE-sided model | Not needed (UE can switch models on its own) | Needed (need NW decision for model transfer/delivery) |

# References

- R2-2308178, Discussion on AI/ML Model Transfer/Delivery, MediaTek Inc.

- R2-2313176, LCM signaling

- TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface