3GPP TSG-RAN WG2 Meeting#124       					   	   			R2-23xxxxx

Chicago, USA, Nov. 13th – 17th, 2023

Agenda item:		7.7.4.1

Title: 				Remaining issues on cell reselection enhancements

Source: 			ZTE Corporation, Sanechips

Document for: 	Discussion and Decision

- Introduction

During the 38.304 CR drafting, the wording has been changed back and forth for the location based measurement initiation and TN measurement relaxation.

In this paper, we address the wording suggested in the email discussions during the CR drafting, targeting to make a final decision so that there would be no repeated discussion in the future.

- Discussion

  - Location based measurement initiation for earth moving cell

The following options have been raised during the CR drafting.

- Option 1: Separate capability description for location-based measurement initiation in quasi-earth-fixed system and earth moving system

The changes are as follows [1]:

-	If the serving cell fulfils Srxlev > SIntraSearchP and Squal > SIntraSearchQ:

-	If distanceThresh and referenceLocation are broadcasted in SIB19, and if UE supports location-based measurement initiation for NTN quasi-Earth-fixed system and has obtained its location information:

-	If the distance between UE and the serving cell reference location referenceLocation is shorter than distanceThresh, the UE may not perform intra-frequency measurements;

-	Else, the UE shall perform intra-frequency measurements;

- 	else if distanceThresh and movingReferenceLocation are broadcasted in SIB19, and if UE supports location-based measurement initiation for NTN Earth-moving system and has obtained its location information:

- 	If the distance between UE’s location and the serving cell reference location determined based on movingReferenceLocation is shorter than distanceThresh, the UE may not perform intra-frequency measurements;

- 	Else, the UE shall perform intra-frequency measurements;

- Option 2: Common capability description for location-based measurement initiation in quasi-earth-fixed system and earth moving system

The changes are as follows:

-	If the serving cell fulfils Srxlev > SIntraSearchP and Squal > SIntraSearchQ:

-	If distanceThresh and referenceLocation are broadcasted in SIB19, and if UE supports location-based measurement initiation and has obtained its location information:

-	If the distance between UE and the serving cell reference location referenceLocation is shorter than distanceThresh, the UE may not perform intra-frequency measurements;

-	Else, the UE shall perform intra-frequency measurements;

- 	else if distanceThresh and movingReferenceLocation are broadcasted in SIB19, and if UE supports location-based measurement initiation and has obtained its location information:

- 	If the distance between UE’s location and the serving cell reference location determined based on movingReferenceLocation is shorter than distanceThresh, the UE may not perform intra-frequency measurements;

- 	Else, the UE shall perform intra-frequency measurements;

Considering that separate UE capabilities have been defined in email discussion [Post123bis] [310] [NR-NTN Enh] UE caps running CR (Intel) [2]:

We understand option 1 would be more appropriate.

Proposal 1: Separate capability description for location-based measurement initiation for quasi-earth-fixed system and earth moving system is used in 38.304 to align with the capability definition in 38.306.

  - Term for TN measurement relaxation

Two terms have been raised during CR drafting on skipping or relaxing measurements on TN cells where there is no coverage, which would impact the description in both 38.306 for UE capabilities and 38.304 for the actual UE behavior.

- Term 1: TN measurement relaxation

- Term 2: Skipping TN measurement

We understand both terms are reasonable but the term has been changed back and forth in 38.306 and 38.304 CR drafting, all we need is a decision so that there would be no repeated discussion in the future.

Proposal 2: Choose one term from the following for skipping or relaxing measurements on TN cells where there is no coverage and such term should be aligned in both 38.306 and 38.304:

- Term 1: TN measurement relaxation

- Term 2: Skipping TN measurement

- Conclusion and proposals

Based on the analysis in previous sections, the following observations and proposals are given:

Proposal 1: Separate capability description for location-based measurement initiation for quasi-earth-fixed system and earth moving system is used in 38.304 to align with the capability definition in 38.306.

Proposal 2: Choose one term from the following for skipping or relaxing measurements on TN cells where there is no coverage and such term should be aligned in both 38.306 and 38.304:

- Term 1: TN measurement relaxation

- Term 2: Skipping TN measurement

- References

[1] R2-2312210_Introduction of NR NTN enhancements in 38.304

[2] R2-2312163_ UE capabilities for Rel-18 NR NTN Enh. WI_38.306