3GPP TSG RAN WG2 Meeting #124                                                        R2-2313961

# Chicago, US, November 13-17, 2023

Agenda item:	7.7.1

Source:	Intel Corporation

Title:	Remaining open topics on UE capabilities for Rel-18 NR NTN Enh.

Document for:	Discussion and decision

# Introduction

This document aims to discuss remaining open issues, if any, and review the running CRs on UE capabilities for Rel-18 NR NTN Enhancement.

[AT124][305][NR-NTN Enh] UE Caps CRs (Intel)

Scope: Update the running drafts CRs with meeting agreements

Intended outcome: Endorsed draft CRs

Deadline for companies' feedback:  Thursday 2023-11-16 17:00 CST

Deadline for endorsed CRs (in R2-2313775 and R2-2313776):  Friday 2023-11-17 08:00 CST

Companies are invited to provide their inputs before Thursday 2023-11-16 17:00 CST.

# Companies’ point of contact (PoC)

| Company’s name | PoC’s name | PoC’s email address |
| --- | --- | --- |
| Ericsson | Ignacio Pascual | Ignacio.pascual.pelayo@ericsson.com |
| Lenovo | Min Xu | xumin13@lenovo.com |
| MediaTek | Abhishek Roy | Abhishek.Roy@mediatek.com |
| Huawei, HiSilicon | Lili Zheng | zhenglili4@huawei.com |
| Thales | Flavien Ronteix | Flavien.ronteix-jacquet@thalesaleniaspace.com |
| Samsung | Shiyang Leng | shiyang.leng@samsung.com |

# Discussion

## Location based CHO

The following discussion points address the following editor’s note for locationBasedCondHandoverNTN-r18:

- Editor’s note: FFS whether any change or update is needed considering how locationBasedCondHandover-r17 is defined”, or whether location-based conditional handover for moving cell refers to source cell, target cell or both

- Should the description of locationBasedCondHandoverNTN-r18 UE capability be updated to clarify whether the location-based conditional handover for moving cell refers to source cell, target cell or both? If so, please suggest corresponding update of the TP.

| Company’s name | Yes/No | Comments, if any |
| --- | --- | --- |
| Ericsson | No | In our understanding, if a UE supports moving cells, it does not really matter which type of cell the source/target of the handover are. |
| Qualcomm |  | We expect both source and target cells are moving. Otherwise, there will be issue in configuration of reference locations. Is the intention to allow UE not support the case source is fixed cell, and target is moving cell? |
| Lenovo | No | The UE can handle conditional mobility for both EFC and EMC as long as it supports. |
| MediaTek | No |  |
| Huawei, HiSilicon | No | This is a UE capability about predicting the trajectory of reference location, and it doesn’t matter whether it is source cell or target cell. |
| Thales | No |  |
| Samsung | No | No difference between source cell or target cell, this is for any earth-moving cell. |

[Summary report] [Rapp(v1)] All companies who are OK with current TP which describes UE capability for locationBasedCondHandoverNTN-r18.

- Should the description of locationBasedCondHandoverNTN-r18 UE capability be further updated? If so, please suggest corresponding update of the TP.

| Company’s name | Yes/No | Comments, if any |
| --- | --- | --- |

## Unchanged PCI

The following discussion point address the following editor’s note captured for unchangedPCI-NTN-SoftSwitch-r18 and unchangedPCI-NTN-HardSwitch-r18:

- Editor’s note: FFS whether further changes may be needed after further progressing on the design to support unchanged PCI with soft and hard switch

- Do you have concerns on having two separate UE capabilities to indicate UE’s support of unchanged PCI with soft and hard switch? If yes, please justify this and also explain your preference and if applicable, your suggested/updated TP.

| Company’s name | Yes /No | Comments, if any |
| --- | --- | --- |
| Ericsson | No | Even though UE behavior is slightly different between soft and hard switch, this can be captured in a single capability. |
| Qualcomm | No | We prefer having two different capabilities for hard an soft. |
| Lenovo | No |  |
| MediaTek | No |  |
| Huawei, HiSilicon | Yes | The hard/soft switching is only a deployment issue, we think the UE either supports both scenarios or not support the unchanged PCI feature at all. Also in Thursday CB, it was agreed that even for soft satellite switching the NW implementation can choose not to configure t-Start, and from UE perspective it is actually a hard switching case, in this case which capability should be referred to? Soft or hard? Therefore we think a single capability is enough. BTW, Ericsson’s comment also speaks in favor of single capability so it should be a “Yes”? |
| Thales | Yes see comment | We share the Huawei’s view, the hard or soft switching scenario is a network implementation issue. Try to manage at the UE side with only one capability nchangedPCI-NTN-r18. However, we are aware that it could be difficult to have only one capability for soft and hard are if the requirements are too different, in that case, the answer is No. |
| Samsung | No, prefer two capabilities | From UE perspective, if NW configures a soft switch (i.e., by indicating T-start), UE has to applied the additional SSB timing information to find the SSBs from the target satellite. This is a different UE behavior than hard switch. Prefer two separate UE capabilities. |

[Summary report] [Rapp(v1)] Views are split whether to have two separate UE capabilities to indicate UE’s support of unchanged PCI with soft and hard switch:

- 1 capability is supported by 4 companies (Ericsson, Huawei, HiSilicon, Thales), highlighting:

  - Hard/soft switching is only a network deployment issue.

  - UE either supports both scenarios or not support the unchanged PCI feature at all.

- 2 capabilities is supported by 4 companies (Qualcomm, Lenovo, MediaTek, Samsung), highlighting:

  - If NW configures a soft switch (i.e., by indicating T-start), UE has to apply the additional SSB timing information to find the SSBs from the target satellite. This is a different UE behavior than hard switch.

This topic might require further discussion and potentially a “middle ground” solution could be considered. On summary, the following options could be considered:

- Option 1) [4/8] Two UE capabilities, unchangedPCI-NTN-SoftSwitch-r18 and unchangedPCI-NTN-HardSwitch-r18.

- Option 2) [4/8] One UE capability, unchangedPCI-NTN-r18.

- Option 3) Two UE capabilities with some dependencies: unchangedPCI-NTN-HardSwitch-r18 can be supported by itself; but if UE supports unchangedPCI-NTN-SoftSwitch-r18, UE is required to also indicate the support of unchangedPCI-NTN-HardSwitch-r18

[Summary report] [Rapp(v2)] The following inputs were provided over reflector by session chair:

- Change the feature name e.g., to "satellite switch with re-sync" (i.e. softSatelliteSwitch-Resync-NTN-r18 and hardSatelliteSwitch-Resync-NTN-r18)

- Ideally scenario is having 1 capability (option 2), otherwise some dependencies (options 3) seems preferable.

- Option 1 is not good as it will be almost impossible for a NW supporting "satellite switch with re-sync" to handle both the UE types.

- Moreover, even if we go for option 3, we should still clarify that a UE supporting only hardSatelliteSwitch-Resync-NTN-r18 will anyway be able to consider (and adapt to) "SSB time offset" after T-service, otherwise it will again be impossible for a NW implementing soft "satellite switch with re-sync" (and then implementing a "SSB time offset" different than 0) to support UE supporting only hardSatelliteSwitch-Resync-NTN-r18.

- For UE capability(ies) that indicate the support of satellite switch with re-sync (i.e., unchanged PCI) with hard and soft switch, to further discuss:

  - Option 1) [4/8] Two UE capabilities, softSatelliteSwitch-Resync-NTN-r18 and hardSatelliteSwitch-Resync-NTN-r18.

  - Option 2) [4/8] One UE capability, satelliteSwitch-Resync-NTN-r18.

  - Option 3) Two UE capabilities with some dependencies: hardSatelliteSwitch-Resync-NTN-r18 can be supported by itself; but if UE supports softSatelliteSwitch-Resync-NTN-r18, UE is required to also indicate the support of hardSatelliteSwitch-Resync-NTN-r18.

    - If option 3) is agreed, to also discuss whether the description of hardSatelliteSwitch-Resync-NTN-r18 capability in draftCR to 38.306 also captures that UE only supporting hardSatelliteSwitch-Resync-NTN-r18 shall apply "SSB time offset" at or after T-service.

## TP changes (if any) to draftCRs on UE Capab.

- Please indicate if you have any suggested changes to current TP to 38.306 or 38.331 on UE Capabilities for Rel-18 NR NTN Enh which are not covered by previous questions. Note that draftCR to 38.306 has a minor updated (with new changes tracked under “Rapp (v1)”) considering the following RAN2#124 agreement:

- Adopt the terminology “Skipping TN measurement” in both 38.304 and 38.306

| Company’s name | Section/Parameter | Comments, if any |
| --- | --- | --- |

## New UE Capabilities to add (if any)

- Please indicate if there are any other UE capabilities that needs to be discussed/defined by RAN2 for Rel-18 NR NTN Enhancements which are not covered by previous questions.

| Company’s name | Comments, if any |
| --- | --- |

On summary:

- The UE Capability draftCRs for Rel-18 NR NTN Enh are updated based on above agreements and all the Editor’s notes are removed.

# Conclusion

The proposals captured are the following:

Proposal 1.	For UE capability(ies) that indicate the support of satellite switch with re-sync (i.e., unchanged PCI) with hard and soft switch, to further discuss:

Proposal 1.1.	Option 1) [4/8] Two UE capabilities, softSatelliteSwitch-Resync-NTN-r18 and hardSatelliteSwitch-Resync-NTN-r18.

Proposal 1.2.	Option 2) [4/8] One UE capability, satelliteSwitch-Resync-NTN-r18.

Proposal 1.3.	Option 3) Two UE capabilities with some dependencies: hardSatelliteSwitch-Resync-NTN-r18 can be supported by itself; but if UE supports softSatelliteSwitch-Resync-NTN-r18, UE is required to also indicate the support of hardSatelliteSwitch-Resync-NTN-r18.

Proposal 1.3.1.	If option 3) is agreed, to also discuss whether the description of hardSatelliteSwitch-Resync-NTN-r18 capability in draftCR to 38.306 also captures that UE only supporting hardSatelliteSwitch-Resync-NTN-r18 shall apply "SSB time offset" at or after T-service.

Proposal 2.	The UE Capability draftCRs for Rel-18 NR NTN Enh are updated based on above agreements and all the Editor’s notes are removed.