3GPP TSG-RAN WG2 Meeting #125 meeting	R2-2400226

Athens, Greece, 26th February – 1st March 2024

Agenda item:		7.5.3.1

Source:		Lenovo

Title:		Corrections for padding BSR for refined BSR

Document for:		Discussion and Decision

- Introduction

- In this contribution we discuss the corrections for padding BSR for refined long BSR.

- Discussion

We think it is also beneficial to support padding BSR for refined long BSR as in legacy. If RAN2 agree to introduce padding BSR for Refined BSR MAC CE, then there are 4 possible padding BSR formats for a Refined BSR: Refined Long BSR, Refined Long Truncated BSR, Refined Short Truncated BSR or Refined Short BSR. In legacy, there are Short BSR and Short Truncated BSR, Long BSR, Long Truncated BSR.

| Type | Refined Long BSR | Refined Long Truncated BSR | Refined Short /Refined Short Truncated BSR | Long BSR | Long Truncated BSR | Short BSR /Short Truncated BSR |
| --- | --- | --- | --- | --- | --- | --- |
| Size of MAC CE | 3+ bytes | 3+ bytes | 2 bytes | 2+ bytes | 1byte or 2+ bytes | 1 byte |

UE selects the LCGs with data available for transmission in decreasing order of the LCH priority based on size of the padding bits similar as legacy.

- If the number of padding bits is large enough to accommodate Refined Long BSR MAC CE plus its subheader and there’s an LCG configured with additionalBSR-TableAllowed with data available for transmission, UE reports the Refined Long BSR MAC CE.

- If UE has more than one LCG with data available for transmission, and the number of the padding bits is not sufficient to accommodate the Refined Long BSR MAC CE but sufficient to accommodate a Long truncated refined BSR MAC CE including buffer size at least for one LCG with configured with additionalBSR-TableAllowed, UE reports the truncated Refined Long BSR MAC CE.

- If there’s only one LCG configured with additionalBSR-TableAllowed with data available for transmission, and the number of the padding bits is not sufficient to accommodate the Refined Long BSR MAC CE plus its subheader and, and if the number of padding bit is sufficient to accommodate refined short BSR MAC CE, UE reports the Refined Short BSR MAC CE as depicted in Figure 1 for the LCG.

Figure 1. Refined Short and Refined Short Truncated BSR MAC CE.

Figure 2. Refined Long and Refined long truncated BSR MAC CE.

Figure 3. Long BSR MAC CE.

Figure 4. short and short truncated BSR MAC CE.

Proposal 1.	RAN2 to agree to introduce padding BSR for Refined BSR MAC CE.

Proposal 2.	If the number of padding bit is large enough to accommodate Refined Long BSR MAC CE plus its subheader and there’s an LCG configured with additionalBSR-TableAllowed with data available for transmission, UE reports the padding Refined Long BSR MAC CE.

Proposal 3.	If the number of the padding bits is not sufficient to accommodate the Refined Long BSR MAC CE, UE selects the LCGs with data available for transmission in decreasing order of the LCH priority based on size of the padding bits.

- If there’s only one LCG configured with additionalBSR-TableAllowed with data available for transmission, and the number of the padding bits is not sufficient to accommodate the Refined Long BSR MAC CE plus its subheader and, and if the number of padding bits is sufficient to accommodate refined short BSR MAC CE, UE reports the Refined Short BSR MAC CE as depicted in Figure 1 for the LCG.

- If UE has more than one LCG with data available for transmission, and the number of the padding bits is not sufficient to accommodate the Refined Long BSR MAC CE but sufficient to accommodate Long truncated refined BSR MAC CE including buffer size at least for one LCG with configured with additionalBSR-TableAllowed, UE reports the truncated Refined Long BSR MAC CE.

- If UE has more than one LCG with data available for transmission, and the number of the padding bits is equal to the size of the Refined Short Truncated BSR at least for one LCG with configured with additionalBSR-TableAllowed, UE reports the truncated Refined Short Truncated BSR MAC CE.

- Conclusion

In this contribution, we discuss XR-awareness. According to above observations, we have the following proposals:

Proposal 1.	RAN2 agree to introduce padding BSR for Refined BSR MAC CE.

Proposal 2.	If the number of padding bit is large enough to accommodate Refined Long BSR MAC CE plus its subheader and there’s an LCG configured with additionalBSR-TableAllowed with data available for transmission, UE reports the padding Refined Long BSR MAC CE.

Proposal 3.	If the number of the padding bits is not sufficient to accommodate the Refined Long BSR MAC CE, UE selects the LCGs with data available for transmission in decreasing order of the LCH priority based on size of the padding bits.

- If there’s only one LCG configured with additionalBSR-TableAllowed with data available for transmission, and the number of the padding bits is not sufficient to accommodate the Refined Long BSR MAC CE plus its subheader and, and if the number of padding bits is sufficient to accommodate refined short BSR MAC CE, UE reports the Refined Short BSR MAC CE as depicted in Figure 1 for the LCG.

- If UE has more than one LCG with data available for transmission, and the number of the padding bits is not sufficient to accommodate the Refined Long BSR MAC CE but sufficient to accommodate Long truncated refined BSR MAC CE including buffer size at least for one LCG with configured with additionalBSR-TableAllowed, UE reports the truncated Refined Long BSR MAC CE.

- If UE has more than one LCG with data available for transmission, and the number of the padding bits is equal to the size of the Refined Short Truncated BSR at least for one LCG with configured with additionalBSR-TableAllowed, UE reports the truncated Refined Short Truncated BSR MAC CE.

- References

TS 38.321 v18.0.0

- TP

The Annex A provides Text Proposal to TS 38.321 for Proposals.

Next Modified Subclause

For Padding BSR, the MAC entity for which logicalChannelGroupIAB-Ext is not configured by upper layers shall:

- if for at least one LCG configured with additionalBSR-TableAllowed, the amount of UL data available for transmission is within the buffer sizes specified in Table 6.1.3.1-3:

- if the number of padding bits is equal to or larger than the size of the Refined Long BSR plus its subheader:

3> report Refined Long BSR for all LCGs which have data available for transmission.

2> if the number of padding bits is larger than the size of the Refined Short BSR plus its subheader and less than the size of the the Refined Long BSR plus its subheader:

3> if more than one LCG has data available for transmission when the BSR is to be built:

4> report Refined Long Truncated BSR for all LCGs which have data available for transmission following a decreasing order of the highest priority logical channel (with or without data available for transmission) in each of these LCG(s), and in case of equal priority, in increasing order of LCGID.

2> if the number of padding bits is equal to the size of the Refined Short Truncated BSR plus its subheader:

3> if more than one LCG has data available for transmission:

4> report Refined Short Truncated BSR of the LCG with the highest priority logical channel with data available for transmission;

- if only one LCG has data available for transmission:

2> if the number of padding bits is equal to the size of the Refined Short BSR plus its subheader:

3> report Refined Short BSR for all LCGs which have data available for transmission.

- Else:

1>	if the number of padding bits is equal to or larger than the size of the Short BSR plus its subheader but smaller than the size of the Long BSR plus its subheader:

2>	if more than one LCG has data available for transmission when the BSR is to be built:

3>	if the number of padding bits is equal to the size of the Short BSR plus its subheader:

4>	report Short Truncated BSR of the LCG with the highest priority logical channel with data available for transmission.

3>	else:

4>	report Long Truncated BSR of the LCG(s) with the logical channels having data available for transmission following a decreasing order of the highest priority logical channel (with or without data available for transmission) in each of these LCG(s), and in case of equal priority, in increasing order of LCGID.

2>	else:

3>	report Short BSR.

1>	else if the number of padding bits is equal to or larger than the size of the Long BSR plus its subheader:

2>	report Long BSR for all LCGs which have data available for transmission.

Next Modified Subclause

#### 6.1.3.1	Buffer Status Report MAC CEs

Buffer Status Report (BSR) MAC CEs consist of either:

-	Short BSR format (fixed size); or

-	Extended Short BSR format (fixed size); or

-	Long BSR format (variable size); or

-	Refined Long BSR format (variable size); or

- 	Refined Long Truncated BSR format (variable size); or

-	Refined Short Truncated BSR format (fixed size); or

-	Refined Short BSR format (fixed size); or

-	Extended Long BSR format (variable size); or

-	Short Truncated BSR format (fixed size); or

-	Extended Short Truncated BSR format (fixed size); or

-	Long Truncated BSR format (variable size); or

-	Extended Long Truncated BSR format (variable size).

Pre-emptive BSR MAC CE consists of:

-	Pre-emptive BSR format (variable size); or

-	Extended Pre-emptive BSR format (variable size).

The BSR formats are identified by MAC subheaders with LCIDs as specified in Table 6.2.1-2.

The Refined Long BSR format, Extended BSR formats and Pre-emptive BSR format are identified by MAC subheaders with eLCIDs as specified in Table 6.2.1-2b.

The fields in the BSR MAC CE are defined as follows:

-	LCG ID: The Logical Channel Group ID field identifies the group of logical channel(s) whose buffer status is being reported. The length of the field is 3 bits for the case of Short BSR and Short Truncated BSR formats, and 8 bits for the case of Extended Short BSR and Extended Short Truncated BSR formats;

-	LCGi: For the Long BSR format, Refined Long BSR format, Extended Long BSR format, Pre-emptive BSR format, and Extended Pre-emptive BSR format, this field indicates the presence of the Buffer Size field for the logical channel group i. The LCGi field set to 1 indicates that the Buffer Size field for the logical channel group i is reported. The LCGi field set to 0 indicates that the Buffer Size field for the logical channel group i is not reported. For the Long Truncated BSR format and the Extended Long Truncated BSR format, this field indicates whether logical channel group i has data available. The LCGi field set to 1 indicates that logical channel group i has data available. The LCGi field set to 0 indicates that logical channel group i does not have data available;

-	BTi: This field is included only in the Refined Long BSR format. This field is present only if the corresponding LCGi is set to 1; otherwise, this field is reserved. If present, this field indicates which buffer size table is used to set the Buffer Size field for the logical channel group i. The BTi field set to 1 indicates that the buffer size table specified in Table 6.1.3.1-3 is used for the logical channel group i. The BTi field set to 0 indicates that the buffer size table specified in Table 6.1.3.1-2 is used for the logical channel group i;

-	Buffer Size: The Buffer Size field identifies the total amount of data available according to the data volume calculation procedure in TSs 38.322 [3] and 38.323 [4] across all logical channels of a logical channel group after the MAC PDU has been built (i.e. after the logical channel prioritization procedure, which may result the value of the Buffer Size field to zero). The amount of data is indicated in number of bytes. The size of the RLC headers and MAC subheaders are not considered in the buffer size computation. The length of this field for the Short BSR format and the Short Truncated BSR format is 5 bits. The length of this field for the Extended Short BSR format and the Extended Short Truncated BSR format is 8 bits. The length of this field for the Long BSR format, the Refined Long BSR format, the Long Truncated BSR format, the Extended Long BSR format, and the Extended Long Truncated format is 8 bits. The values for the 5-bit Buffer Size fields are shown in Tables 6.1.3.1-1. The values for the 8-bit Buffer Size fields in the Long BSR format, the Long Truncated BSR format, the Extended Long BSR format, and the Extended Long Truncated format are shown in Table 6.1.3.1-2. For the Refined Long BSR format, if an LCG is configured with additionalBSR-TableAllowed and the amount of data for an LCG is within the buffer sizes specified in Table 6.1.3.1-3, the MAC entity shall use the buffer sizes specified in Table 6.1.3.1-3 to set the value of this field; otherwise, the MAC entity shall use Table 6.1.3.1-2 instead. For the Long BSR format, the Refined Long BSR format, the Long Truncated BSR format, the Extended Long BSR format, and the Extended Long Truncated format, the Buffer Size fields are included in ascending order based on the LCGi. For the Long Truncated BSR format and the Extended Long Truncated format the number of Buffer Size fields included is maximised, while not exceeding the number of padding bits. For the Pre-emptive BSR format and the Extended Pre-emptive BSR format, the Buffer Size field identifies the total amount of the data expected to arrive at the IAB-MT of the node where the Pre-emptive BSR/Extended Pre-emptive BSR is triggered and does not include the volume of data currently available in the IAB-MT. Pre-emptive BSR format is identical to the Long BSR format. Extended Pre-emptive BSR format is identical to the Extended Long BSR format.

NOTE 1:	For the Pre-emptive BSR, if configured, the LCGs to be reported, the expected data volume calculation, the exact time to report Pre-emptive BSR and the associated LCH are left to implementation.

NOTE 2:	The mapping of LCGs between the ingress and egress links of an IAB node for purposes of determining expected change in occupancy of IAB-MT buffers (to be reported as Pre-emptive BSR) is left to implementation.

NOTE 3:	The number of the Buffer Size fields in the Long BSR, Extended Long BSR, Long Truncated BSR, and Extended Long Truncated BSR format can be zero.

NOTE 4:	The Extended versions of the BSR formats can only be used by IAB nodes.

Figure 6.1.3.1-1: Short BSR and Short Truncated BSR MAC CE

Figure 6.1.3.1-2: Long BSR, Long Truncated BSR, and Pre-emptive BSR MAC CE

Figure 6.1.3.1-3: Extended Short BSR and Extended Short Truncated BSR MAC CE

Figure 6.1.3.1-4: Extended Long BSR, Extended Long Truncated BSR, and Extended Pre-emptive BSR MAC CE

Figure 6.1.3.1-5: Refined Long BSR MAC CE

Figure 6.1.3.1-x: Refined Short and Refined Short Truncated BSR MAC CE

Next Modified Subclause

Table 6.2.1-2b: Values of one-octet eLCID for UL-SCH

| Codepoint | Index | LCID values |
| --- | --- | --- |
| 0 to 222 | 64 to 286 | Reserved |
| 223 | 287 | Multiple Entry PHR with assumed PUSCH MAC CE (four octets Ci) |
| 224 | 288 | Multiple Entry PHR with assumed PUSCH MAC CE (one octets Ci) |
| 225 | 289 | Single Entry PHR with assumed PUSCH MAC CE |
| 226 | 290 | SL-PRS Resource Request |
| 227 | 291 | Refined Long BSR |
| 228 | 292 | Delay Status Report |
| 229 | 293 | Enhanced Multiple Entry PHR for multiple TRP (four octets Ci) |
| 230 | 294 | Enhanced Multiple Entry PHR for multiple TRP (one octets Ci) |
| 231 | 295 | Enhanced Single Entry PHR for multiple TRP |
| 232 | 296 | Enhanced Multiple Entry PHR (four octets Ci) |
| 233 | 297 | Enhanced Multiple Entry PHR (one octets Ci) |
| 234 | 298 | Enhanced Single Entry PHR |
| 235 | 299 | Enhanced BFR (one octet Ci) |
| 236 | 300 | Enhanced BFR (four octet Ci) |
| 237 | 301 | Truncated Enhanced BFR (four octet Ci) |
| 238 | 302 | Positioning Measurement Gap Activation/Deactivation Request |
| 239 | 303 | IAB-MT Recommended Beam Indication |
| 240 | 304 | Desired IAB-MT PSD range |
| 241 | 305 | Desired DL Tx Power Adjustment |
| 242 | 306 | Case-6 Timing Request |
| 243 | 307 | Desired Guard Symbols for Case 6 timing |
| 244 | 308 | Desired Guard Symbols for Case 7 timing |
| 245 | 309 | Extended Short Truncated BSR |
| 246 | 310 | Extended Long Truncated BSR |
| 247 | 311 | Extended Short BSR |
| 248 | 312 | Extended Long BSR |
| 249 | 313 | Extended Pre-emptive BSR |
| 250 | 314 | BFR (four octets Ci) |
| 251 | 315 | Truncated BFR (four octets Ci) |
| 252 | 316 | Multiple Entry Configured Grant Confirmation |
| 253 | 317 | Sidelink Configured Grant Confirmation |
| 254 | 318 | Desired Guard Symbols |
| 255 | 319 | Pre-emptive BSR |
| Xxx1 | Yyy1 | Refined Long Truncated BSR |
| Xxx2 | Yyy2 | Refined Short Truncated BSR |
| Xxx3 | Yyy3 | Refined Short BSR |

End of Changes