3GPP TSG-RAN WG2 Meeting #125	R2-2400485

Athens, Greece,  Feb. 26th – Mar. 1st, 2024

Agenda Item:	7.3.2

Source:	OPPO

Title:	Correction on the SP CSI reporting

Document for:	Discussion, Decision

Introduction

This paper will focus on the SP CSI reporting.

Discussion

In Rel-18 NES, it was agreed in RAN1 that the UE is not expected to transmit the SP CSI report during the cell DRX non-active period.

Agreement From RAN1 point of view, Rel-18 UE supporting cell DRX is not expected to transmit the following signals/channels to the gNB during non-active periods of cell DRX. The list of signals/channels may be updated based on RAN2/RAN4 input and other signals/channels are not precluded from further discussions. - Periodic/Semi-persistent CSI report - Periodic/Semi-persistent SRS - FFS: SRS for positioning - FFS: - HARQ feedback for SPS PDSCH - FFS whether there will be exception case(s) for UE transmitting listed signals/channels during non-active periods of DRX - FFS Whether the listed signals/channels can be configurable by gNB - FFS: Whether the same or different UE behavior is applicable with or without C-DRX - FFS: RAN1 to consider impact on system if the channels/signals are not transmitted during non-active period Agreement From RAN1 point of view, Rel-18 UE supporting cell DTX does not expect to receive and/or process the following signals/channels from the gNB, during non-active periods of cell DTX. The list of signals/channels may be updated based on RAN2/RAN4 input and other signals/channels are not precluded from further discussions. - Periodic/Semi-persistent CSI-RS configured in CSI report configuration in CSI-ReportConfig with reportQuantity including RI (for CSI reporting) - FFS: - PDCCH in USS - UE behaviour for retransmission - if some specific RNTI scrambled PDCCH in USS will be excluded from cell DTX operation - PDCCH in Type-3 CSS - UE behaviour for retransmission - if some specific RNTI scrambled PDCCH in Type-3 CSS will be excluded from cell DTX operation - PRS - CSI-RS configured by measObjectNR (for RRM) - CSI-RS associated with RadioLinkMonitoringConfig and BeamFailureDectection (for RLM and BFD) - Periodic CSI-RS configured with trs-Info ‘true’ (for tracking) - Periodic/Semi-persistent CSI-RS (for BM) - FFS on how to differentiate (if needed) with other CSI-RS used for CSI reports for BM - FFS: Whether the same or different UE behaviour is applicable with or without C-DRX - FFS: Whether the list of impacted signals/channels can be configurable - FFS: Whether there will be exception case(s) for UE receiving and/or processing listed signals/channels during non-active periods of DTX - FFS: RAN1 to consider impact on system if the channels/signals are not transmitted during non-active period

And, the current RRC spec supports the transmission of SP CSI reports via either PUSCH or PUCCH.

- – CSI-ReportConfig The IE CSI-ReportConfig is used to configure a periodic or semi-persistent report sent on PUCCH on the cell in which the CSI-ReportConfig is included, or to configure a semi-persistent or aperiodic report sent on PUSCH triggered by DCI received on the cell in which the CSI-ReportConfig is included (in this case, the cell on which the report is sent is determined by the received DCI). See TS 38.214 [19], clause 5.2.1. CSI-ReportConfig information element -- ASN1START -- TAG-CSI-REPORTCONFIG-START CSI-ReportConfig ::= SEQUENCE { reportConfigId CSI-ReportConfigId, carrier ServCellIndex OPTIONAL, -- Need S resourcesForChannelMeasurement CSI-ResourceConfigId, csi-IM-ResourcesForInterference CSI-ResourceConfigId OPTIONAL, -- Need R nzp-CSI-RS-ResourcesForInterference CSI-ResourceConfigId OPTIONAL, -- Need R reportConfigType CHOICE { periodic SEQUENCE { reportSlotConfig CSI-ReportPeriodicityAndOffset, pucch-CSI-ResourceList SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUCCH-CSI-Resource }, semiPersistentOnPUCCH SEQUENCE { reportSlotConfig CSI-ReportPeriodicityAndOffset, pucch-CSI-ResourceList SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUCCH-CSI-Resource }, semiPersistentOnPUSCH SEQUENCE { reportSlotConfig ENUMERATED {sl5, sl10, sl20, sl40, sl80, sl160, sl320}, reportSlotOffsetList SEQUENCE (SIZE (1.. maxNrofUL-Allocations)) OF INTEGER(0..32), p0alpha P0-PUSCH-AlphaSetId }, aperiodic SEQUENCE { reportSlotOffsetList SEQUENCE (SIZE (1..maxNrofUL-Allocations)) OF INTEGER(0..32) } },

While the current MAC spec only covers the possibility of the SP CSI report on PUSCH.

- 5.34.3 Cell Discontinuous Reception [omitted] 1> if cell DRX is activated and the Serving Cell is not in the cell DRX Active Period: 2> not instruct the physical layer to signal a SR on a PUCCH resource for SR; 2> not increment the SR_COUNTER for a SR; 2> not start the sr-ProhibitTimer for a SR; 2> not deliver any configured uplink grant and the associated HARQ information to the HARQ entity; 2> not instruct a HARQ process associated with a configured uplink grant to trigger a new transmission or a retransmission; 2> not report periodic CSI on PUCCH and semi-persistent CSI configured on PUSCH; 2> if an emergency service is initiated by upper layers and this Serving Cell is the SpCell: 3> initiate a Random Access procedure (as specified in clause 5.1.1).

Thus, we suggest updating the MAC spec to align between the RAN2’s spec and the RAN1’s requirement.

Update the MAC spec to prohibit the MAC entity from reporting semi-persistent CSI via either PUSCH or PUCCH during non-active periods of cell DRX.

Conclusion

We have the following proposals:

Proposal 1	Update the MAC spec to prohibit the MAC entity from reporting semi-persistent CSI via either PUSCH or PUCCH during non-active periods of cell DRX.

Text Proposal

- 5.34.3	Cell Discontinuous Reception

Cell DRX is configured if cellDTXDRXconfigType is set to drx or dtxdrx. Cell DRX operation is activated and deactivated for each Serving Cell by:

-	receiving a cell DRX indication from lower layers indicating activation or deactivation of cell DRX operation, as specified in TS 38.213 [6];

-	configuring CellDTXDRX-Config by upper layers: if cell DRX is configured and cellDTXDRXactivationStatus is set to activated, cell DRX operation is activated upon cell DRX configuration; if cell DRX is configured and cellDTXDRXactivationStatus is set to deactivated, cell DRX operation is deactivated upon cell DRX configuration; if CellDTXDRX-Config is released, cell DRX operation is deactivated and all the corresponding configurations are released.

When cell DRX is configured and activated for a Serving Cell, the cell DRX Active Period includes the time while:

-	celldtxdrx-onDurationTimer is running for the associated Serving Cell.

For each Serving Cell configured with cell DRX, the MAC entity shall:

1>	if cell DRX is activated for this Serving Cell:

2>	if [(SFN × 10) + subframe number] modulo (celldtxdrx-Cycle) = (celldtxdrx-StartOffset):

3>	start celldtxdrx-onDurationTimer for this serving cell after celldtxdrx-SlotOffset from the beginning of the subframe.

1>	if cell DRX is activated and the Serving Cell is not in the cell DRX Active Period:

2>	not instruct the physical layer to signal a SR on a PUCCH resource for SR;

2>	not increment the SR_COUNTER for a SR;

2>	not start the sr-ProhibitTimer for a SR;

2>	not deliver any configured uplink grant and the associated HARQ information to the HARQ entity;

2>	not instruct a HARQ process associated with a configured uplink grant to trigger a new transmission or a retransmission;

2>	not report CSI on PUCCH and semi-persistent CSI configured on PUSCH;

2>	if an emergency service is initiated by upper layers and this Serving Cell is the SpCell:

3>	initiate a Random Access procedure (as specified in clause 5.1.1).

Reference

[1] RP-223540 New WID: Network energy savings for NR

[2] TR 38.864 Study on network energy savings for NR