3GPP TSG-RAN WG2 Meeting #125	R2-2400489

Athens, Greece,  Feb. 26th – Mar. 1st, 2024

Agenda Item:	7.5.3.2

Source:	OPPO

Title:	The impact on DSR due to intra-UE prioritization

Document for:	Discussion, Decision

Introduction

This paper would like to discuss the impact on DRS due to intra-UE prioritization.

Discussion

In the current MAC spec, the remaining time reported in the DSR MAC CE is derived based on the reference time related to the point of the first transmission of that information.

- 6.1.3.72 Delay Status Report MAC CE The Delay Status Report (DSR) MAC CE is identified by MAC subheader with an eLCID as specified in Table 6.2.1-2b. The fields in the DSR MAC CE are defined as follows: - LCGi: This field indicates the presence of delay information (i.e. the Remaining Time and Buffer Size fields) for the LCG i. The LCGi field set to 1 indicates that the delay information for the LCG i is reported. The LCGi field set to 0 indicates that the delay information for the LCG i is not reported; - Remaining Time: This field indicates the shortest remaining value of PDCP discardTimer (described in clause 7.3 in TS 38.323 [4]) among all PDCP SDUs buffered for an LCG, at the time of the first symbol of the first PUSCH transmission that includes this DSR MAC CE. The length of this field is 6 bits. The value r in this field indicates a remaining time within the range of (r, r + 1] msec; - BT: This field is present only if the corresponding LCG is configured with additionalBSR-TableAllowed; otherwise, this field is reserved. If present, the BT field set to 1 indicates that the buffer sizes specified in Table 6.1.3.1-3 are used to set the value of the Buffer Size field, while the BT field set to 0 indicates that the buffer sizes specified in Table 6.1.3.1-2 are used instead; - Buffer Size: The Buffer Size field indicates the total amount of delay-critical UL data for an LCG according to the data volume calculation procedure specified in clause 5.5 in TS 38.322 [3] and clause 5.6 in TS 38.323 [4] for the associated RLC and PDCP entities, respectively, after the MAC PDU has been built. If the corresponding LCG is configured with additionalBSR-TableAllowed and the amount of delay-critical UL data for an LCG is within the buffer sizes specified in Table 6.1.3.1-3, the MAC entity shall use the buffer sizes specified in Table 6.1.3.1-3 to set the value of this field; otherwise, the MAC entity shall use Table 6.1.3.1-2 instead. This field is indicated in number of bytes. The length of this field is 8 bits.

Due to the agreements achieved in RAN2#122, one issue left is whether intra-UE prioritization would impact the remaining time determination.

- When/if UE reports remaining time, the reference time for the remaining time is determined from the point of the first transmission of the information. FFS if intra-UE prioritization can impact this.

In the intra-UE prioritization procedure, the MAC layer triggers a new transmission for the deprioritized MAC PDU when criteria are fulfilled. In this case, the MAC layer directly delivers the deprioritized MAC PDU to the identified HARQ process instead of invoking the Multiplexing and assembly entity to regenerate a new MAC PDU. If the remaining time to be reported is already contained in the deprioritized MAC PDU, the MAC layer has no chance to update the remaining time contained, even if it is the time information is outdated. Similar issues also exist in e.g. the BSR/PHR reporting and no enhancement has been agreed upon before. Thus, we prefer not to impact the remaining time determination due to intra-UE prioritization.

No impact is introduced on the remaining time determination for DSR due to intra-UE prioritization.

Conclusion

We have the following proposals:

Proposal 1	No impact is introduced on the remaining time determination for DSR due to intra-UE prioritization.

Reference

[1] RP-230786 Updated WID on XR Enhancements for NR, Nokia, Qualcomm

[2] TR 23.700-60 Study on XR (Extended Reality) and media services

[3] R2-2302309 TR 38.835 v1.0.2, Nokia (Rapporteur)