3GPP TSG-RAN WG2 #125	Tdoc R2-2400590

Athens, Greece, February 26 – March 01, 2024

Agenda Item:	7.3.3

Source:	Ericsson

Title:	Open issue: mTRP and NES

Document for:	Discussion, Decision

# 1	Introduction

The following open issue was raised for mTRP:

For the R1 agreement “Cell DTX/DRX operation is only supported for sTRP”, R2 has received an LS from R1 (R1-2312409) stating “RAN1 notes that it is up to RAN2 to decide whether/how to capture the above agreement in RAN2 specifications.”

This contribution discusses how to address this open issue.

# 2	Discussion

The RAN1 agreement states that cell DTX/DRX configuration is only supported for sTRP. This could be captured as a configuration restriction in the field description of cellDTXDRX-Config e.g.:

cellDTXDRX-Config Used to configure cell DTX/DRX for the serving cell, as specified in TS 38.321 [3]. A maximum of two cell DTX/DRX patterns can be configured per MAC entity for different serving cells. The two configured patterns are aligned, that the start and slot offset are common and the periodicity of one pattern is an integer multiple of the other. This field is only configured in case of sTRP.

However, to be more future proof (e.g. if support for mTRP and cell DTX/DRX is introduced in the future), it is cleaner to include this restriction in the field description of the UE capability for cell DTX/DRX (nes-CellDTX-DRX-r18). Otherwise, if in a future release the support of mTRP and cell DTX/DRX is specified, we would have to remove the restriction from 38.331 and anyway capture it in 38.306 along with a new capability to indicate support of e.g. mTRP and cell DTX/DRX.

Even if no support for mTRP and NES is introduced in the future, the restriction agreed by RAN1 is related to what the UE supports and thus makes sense to include in the UE capabilities.

The restriction agreed for cell DTX/DRX configuration is related to UE capabilities.

Capture the restriction that cell DTX/DRX operation is only supported for sTRP in 38.306 in the nes-CellDTX-DRX-r18 field description.

A TP for 38.306 is provided in section 4.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	The restriction agreed for cell DTX/DRX configuration is related to UE capabilities.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Capture the restriction that cell DTX/DRX operation is only supported for sTRP in 38.306 in the nes-CellDTX-DRX-r18 field description.

# 4	TP for 38.306

| nesBasedCondHandoverWithDCI-r18 Indicates whether the UE supports DCI-based enabling/disabling NES-specific CHO execution condition, i.e. NES-specific CHO execution condition based on source cell NES mode indicated via DCI format 2_9 as specified in TS 38.331 [9]. A UE supporting this feature shall also indicate the support of condHandover-r16. UE shall set the capability value consistently for all FDD-FR1 bands, all TDD-FR1 bands, all TDD-FR2-1 bands and all TDD-FR2-2 bands respectively. | Band | No | N/A | N/A |
| --- | --- | --- | --- | --- |
| nes-CellDTX-DRX-r18 Indicates whether the UE supports cell DTX and/or DRX operation by RRC configuration. The supported number of cell DTX/DRX patterns per cell group is 2, regardless of each pattern is for cell DTX only, cell DRX only, or both. The cell DTX/DRX operation is only supported for sTRP. | Band | No | N/A | N/A |
| nes-CellDTX-DRX-DCI2-9-r18 Indicates whether the UE supports cell DTX/DRX configuration activation and deactivation via DCI 2_9. A UE supporting this feature shall also indicate support of nes-CellDTX-DRX-r18. | Band | No | N/A | N/A |