3GPP TSG-RAN WG2 #125	Tdoc R2-2400591

Athens, February-March 2024

Agenda Item: 7.20.3

Source:	Ericsson

Title:	H045(on CodebookConfig-r18)

Document for:	Discussion, Decision

# Introduction

| ID | Delegate | Work Item | Class | TDoc | Status | Processed | Proposed Conclusion | Description | Proposed Change | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H045 |  | MIMO | 2 | None | PropDisagree |  |  | Wrong place for this field. | According to the RAN1 RRC parameter list, this should be configured in NZP-CSI-RS-ResourceSet, instead of in CodebookConfig. Because it configures the offset between CSI-RS resources, it should be in resource set configuration. Move it to NZP-CSI-RS-ResourceSet. | [QC v125] agree. [Ericsson-Helka-Liina] that was the initial input but the parameter seems specific to Dopplercodebook and not a general CSI-RS resource parameters. What is the issue of having it here? |

# CodebookConfig

In:

CodebookConfig-r18  ::=                   SEQUENCE {

codebookType                              CHOICE {

type2                                     SEQUENCE {

typeII-CJT-r18                            SEQUENCE {

n1-n2-codebookSubsetRestrictionList-r18   SEQUENCE (SIZE (1..4)) OF CBSR-r18,

paramCombination-CJT-r18                  INTEGER (1..7),

paramCombination-CJT-L-r18                SEQUENCE (SIZE (1..4)) OF INTEGER (1..5)     OPTIONAL,  -- Need R

restrictedCMR-Selection-r18               ENUMERATED {enable}                          OPTIONAL,  -- Need R

valueOfO3-r18                             ENUMERATED {n1, n4}                          OPTIONAL,  -- Need R

numberOfPMI-SubbandsPerCQI-Subband-r18    INTEGER(1..2)                                OPTIONAL,  -- Need R

typeII-RI-Restriction-r18                 BIT STRING (SIZE (4)),

codebookMode-r18                          INTEGER (1..2)                               OPTIONAL   -- Need R

},

typeII-CJT-PortSelection-r18              SEQUENCE {                paramCombination-CJT-PS-r18               INTEGER (1..5),

paramCombination-CJT-PS-alpha-r18         SEQUENCE (SIZE (1..4)) OF INTEGER (1..8)     OPTIONAL,  -- Need R

restrictedCMR-Selection-r18               ENUMERATED {enable}                          OPTIONAL,  -- Need R

valueOfO3-r18                             ENUMERATED {n1, n4}                          OPTIONAL,  -- Need R

valueOfN-CJT-r18                          ENUMERATED {n2, n4}                          OPTIONAL,  -- Need R

numberOfPMI-SubbandsPerCQI-Subband-r18    INTEGER(1..2)                                OPTIONAL,  -- Need R

typeII-PortSelectionRI-Restriction-r18    BIT STRING (SIZE (4)),

codebookMode-r18                          INTEGER (1..2)                               OPTIONAL   -- Need R

},

typeII-Doppler-r18                        SEQUENCE {

n1-n2-codebookSubsetRestriction-r18       CBSR-r18,

paramCombination-Doppler-r18              INTEGER (1..9),

td-dd-config-r18                          TD-DD-Config-r18                             OPTIONAL,  -- Need R

numberOfPMI-SubbandsPerCQI-Subband-r18    INTEGER(1..2)                                OPTIONAL,  -- Need R

predictionDelay-r18                       ENUMERATED {m0,n0,n1,n2 }                    OPTIONAL,  -- Need R

typeII-RI-Restriction-r18                 BIT STRING (SIZE (4))

},

typeII-DopplerPortSelection-r18           SEQUENCE {

paramCombinationDoppler-PS-r18            INTEGER (1..8),

td-dd-config-r18                          TD-DD-Config-r18                             OPTIONAL,  -- Need R

valueOfN-Doppler-r18                      ENUMERATED {n2, n4}                          OPTIONAL,  -- Need R

numberOfPMI-SubbandsPerCQI-Subband-r18    INTEGER(1..2)                                OPTIONAL,  -- Need R

predictionDelay-r18                       ENUMERATED {m0,n0,n1,n2 }                    OPTIONAL,  -- Need R

typeII-PortSelectionRI-Restriction-r18    BIT STRING (SIZE (4))

}

}

}

}

CBSR-r18 ::=    CHOICE {

two-one                                BIT STRING (SIZE (8)),

two-two                                BIT STRING (SIZE (27)),

four-one                               BIT STRING (SIZE (16)),

three-two                              BIT STRING (SIZE (35)),

six-one                                BIT STRING (SIZE (24)),

four-two                               BIT STRING (SIZE (43)),

eight-one                              BIT STRING (SIZE (32)),

four-three                             BIT STRING (SIZE (59)),

six-two                                BIT STRING (SIZE (59)),

twelve-one                             BIT STRING (SIZE (48)),

four-four                              BIT STRING (SIZE (75)),

eight-two                              BIT STRING (SIZE (75)),

sixteen-one                            BIT STRING (SIZE (64))

}

TD-DD-Config-r18 ::=        SEQUENCE {

vectorLengthDD-r18           ENUMERATED {n1,n2,n4,n8 },

unitDurationDD-r18           ENUMERATED {m1,m2}                                    OPTIONAL,  -- Need R

aperiodicResourceOffset-r18  INTEGER(1..2)                                         OPTIONAL,  -- Need R

tdCQI-r18                    ENUMERATED {n11,n12,n2 }                              OPTIONAL   -- Need R

}

TD-DD-Config field descriptions

aperiodicResourceOffset Offset m between two consecutive aperiodic CSI-RS resources. See TS 38.214 [19], clause 5.2.1.4.

tdCQI Number of TD CQIs (X) in each SB and ref slot(s) + ref W2(s). 1-1: 1 TD CQI, ref = 1st slot and 1st W2 1-2: 1 TD CQI, ref = 1st and last slots, and 1st and last W2 2: 2 TC CQIs (see TS 38.214 [19], clause 5.2.1.4).

unitDurationDD DD/TD unit duration (in slots), for AP-CSI-RS only. Note: For P/SP-CSI-RS, d is set to CSI-RS periodicity (see TS 38.214 [19], clause 5.2.1.4). If the value of aperiodicResourceOffset is '1', network configures value 'm1' for this field.

vectorLengthDD Doppler-/time-domain (DD/TD) basis vector length; See TS 38.214 [19], clause 5.2.1.4.

The parameter aperiodicResourceOffset is meaningful only when configured with Codebook Type II Doppler, hence it is placed in the codebookConfig. However, it is relaed to CSI-RS resources and could be placed there as well but would require configuration limitation.

- Propose to reject RILH045.

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	Propose to reject RILH045.