3GPP TSG-RAN WG2 #125	R2-2400634

Athens, Greece, February 2024

Agenda Item:	7.9.4

Source:	OPPO

Title:	Discussion on left issues for SRAP

Document for:	Discussion, Agreement

Introduction

This paper will discuss the left issues of SRAP specification.

Discussion

Issue-1: Other error handling at SRAP layer

In R17 U2N Relay, error handling is needed for the following cases:

For U2N Remote UE, the Bearer ID and UE ID are configured, when the SRAP PDU with an unknown Bearer ID or UE ID is received;

For U2N Relay UE, the Bearer ID and UE ID are configured, when the SRAP PDU with an unknown Bearer ID or UE ID is received; Or the UE ID and Bearer ID in the received SRAP PDU don’t match with the concerned ingress link.

Similar to R18 U2U Relay, at the U2U Remote UE side, the wrong packet may be received with the wrong UE ID fields or the wrong Bearer ID field;

At U2U Relay UE side, the wrong packet may be received with the wrong source UE ID field or wrong Bearer ID field; Or the UE ID and Bearer ID in the received SRAP PDU don’t match with the concerned ingress link.

So by following the R17 mechanism, the text proposed for the error case handling for U2U Relay in SRAP specification can be as follows:

For U2U Remote UE, when a SRAP Data PDU with SRAP header that contains UE ID fields which does not match sl-PeerRemoteUE-LocalIdentity and sl-RemoteUE-LocalIdentity included in sl-SRAP-ConfigPC5 or a SRAP Data PDU with SRAP header that contains BEARER ID field which does not match slrb-PC5-ConfigIndex included in SLRB-Config is received, the SRAP entity shall: - Discard the received SRAP Data PDU. For U2U Relay UE, when a SRAP Data PDU with SRAP header that contains a UE ID which does not match sl-PeerRemoteUE-LocalIdentity included in sl-SRAP-ConfigPC5 configured by the U2U Relay UE or a SRAP Data PDU with SRAP header contains BEARER ID field which does not match slrb-PC5-ConfigIndex included in SLRB-Config is received, or when a SRAP Data PDU that contains a UE ID which does not match the concerned sl-RemoteUE-LocalIdentity corresponding to sl-RemoteUE-L2Identity of the ingress link is received by U2U Relay UE, the SRAP entity shall: - Discard the received SRAP Data PDU.

For R18 L2 U2U Relay, add the error handling at U2U Relay UE can Remote UE as the above TP.

Issue-2: Whether the egress link determination at U2U Remote UE is needed

For egress link determination, in R17 U2N Relay, the egress link determination is only needed for L2 U2N Relay UE but not L2 U2N Remote UE. While for L2 U2U Relay, since a U2U Remote UE can have multiple U2U Relay UEs to establish multiple U2U links, seems there is a need to also have egress link determination operation at U2U Remote UE.

The proposed text can be added as follows:

#### 5.3a.1.x Egress link determination For a U2U SRAP Data PDU to be transmitted, SRAP entity shall: - Determine the egress link on PC5 interface towards the U2U relay UE based on the determined UE ID fields in accordance with clause 5.3a.1.3.

For R18 L2 U2U Relay, add the egress link determination procedure for the transmitting operation for U2U Remote UE.

Conclusion

We have the following proposals:

Proposal 1	For R18 L2 U2U Relay, add the error handling at U2U Relay UE can Remote UE as the above TP.

Proposal 2	For R18 L2 U2U Relay, add the egress link determination procedure for the transmitting operation for U2U Remote UE.