3GPP TSG-RAN WG2 Meeting #125	R2-2400814

Athens, Greece, 26th February - 1st March 2024

Agenda Item:	7.4.1.1

Source:	Huawei, HiSilicon

Title:	Discussion and draft reply LS on n-TimingAdvanceOffset for PDCCH order RACH

Document for: Discussion

# Introduction

In LTM, the UE can send a RA preamble to candidate cells to obtain the TA for RACH-less cell switch. For this procedure, RAN4 identified an issue and sent an LS to RAN2:

R4-2321389, LS on n-TimingAdvanceOffset for PDCCH order RACH During RAN4#109 meeting, RAN4 has found that n-TimingAdvanceOffset is not included in RAN2 configuration for PDCCH-order RACH on target neighbour cell. When UE is configured to send PDCCH-ordered RACH on target neighbour cell, it is necessary for UE to know the value of n-TimingAdvanceOffset for the target neighbour cell.

In this contribution, we discuss the n-TimingAdvanceOffset configuration and provide a draft reply LS in annex.

# Discussion

The n-TimingAdvanceOffset was introduced since Rel-15 and it is configured under ServingCellConfigCommon. It indicates the N_TA-Offset to be applied for all uplink transmissions on this serving cell. It is an optional need S field with the following description: “If the field is absent, the UE applies the value defined for the duplex mode and frequency range of this serving cell. See TS 38.133, table 7.1.2-2.” The table is shown below:

TS 38.133, table 7.1.2-2. Frequency range and band of cell used for uplink transmission; (Unit: TC). FR1 FDD or TDD band with neither E-UTRA–NR nor NB-IoT–NR coexistence case; 25600 (Note 1). FR1 FDD band with E-UTRA–NR and/or NB-IoT–NR coexistence case; 0 (Note 1). FR1 TDD band with E-UTRA–NR and/or NB-IoT–NR coexistence case; 39936 (Note 1). FR2; 13792. Note 1: The UE identifies based on the information n-TimingAdvanceOffset as specified in TS 38.331 [2]. If UE is not provided with the information n-TimingAdvanceOffset, the default value of is set as 25600 for FR1 band. In case of multiple UL carriers in the same TAG, UE expects that the same value of n-TimingAdvanceOffset is provided for all the UL carriers according to clause 4.2 in TS 38.213 [3] and the value 39936 of can also be provided for a FDD serving cell. Note 2: Void.

Observation 1: When the n-TimingAdvanceOffset in ServingCellConfigCommon is absent, the UE applies a value according to table 7.1.2-2 in TS 38.133, based on the duplex mode and frequency range of the serving cell.

In Rel-18 LTM, the n-TimingAdvanceOffset has already been added to EarlyUL-SyncConfig for candidate cells in TS 38.331 v18.0.0, so RAN2 already fixed the issue mentioned by RAN4. However, currently, it is an optional Need R field, without the description of the UE behaviour when the n-TimingAdvanceOffset is absent in EarlyUL-SyncConfig.

Observation 2: The n-TimingAdvanceOffset in EarlyUL-SyncConfig is optional Need R field in TS 38.331 v18.0.0, without the description of the UE behaviour when the n-TimingAdvanceOffset is absent.

We believe a similar description can be added for n-TimingAdvanceOffset in EarlyUL-SyncConfig, e.g., if the field is absent, the UE applies the value according to the duplex mode and frequency range of this candidate cell as specified in TS 38.133, table 7.1.2-2. In addition, the UE is able to determine the duplex mode and frequency range of a candidate cell by frequencyInfoUL-r18 configured in EarlyUL-SyncConfig-r18.

Therefore, RAN2 can reply that the n-TimingAdvanceOffset has been added and ask RAN4 to confirm whether the above understanding is correct, i.e., if the field is absent, the UE applies the value according to the duplex mode and frequency range of this candidate cell as specified in TS 38.133, table 7.1.2-2.

Proposal 1: Reply to RAN4 that RAN2 has added n-TimingAdvanceOffset for candidate cells. Request RAN4 to confirm whether the following description is correct: if the field is absent, the UE applies the value according to the duplex mode and frequency range of this candidate cell as specified in TS 38.133, table 7.1.2-2.

# Conclusion

This contribution makes the following proposals:

Proposal 1: Reply to RAN4 that RAN2 has added n-TimingAdvanceOffset for candidate cells. Request RAN4 to confirm whether the following description is correct: if the field is absent, the UE applies the value according to the duplex mode and frequency range of this candidate cell as specified in TS 38.133, table 7.1.2-2.

A draft reply LS is in the annex.

# Reference

- R4-2321389, LS on n-TimingAdvanceOffset for PDCCH order RACH.

# Annex: draft reply RS

3GPP TSG-RAN WG2 Meeting #125	R2-24xxxxx

# Athens, Greece, 26 February - 1 March 2024

Title:	(Draft) reply LS on n-TimingAdvanceOffset for PDCCH order RACH

Response to:	R4-2321389, LS on n-TimingAdvanceOffset for PDCCH order RACH

Release:	Rel-18

Work Item:	NR_Mob_enh2-Core

Source:	Huawei (to be RAN2)

To:	RAN WG4

Cc:	RAN WG1

Contact Person:

#### Name:	David Lecompte

E-mail Address:	david.lecompte@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

RAN2 would like to thank RAN4 for the LS on n-TimingAdvanceOffset for PDCCH order RACH.

During RAN4#109 meeting, RAN4 has found that n-TimingAdvanceOffset is not included in RAN2 configuration for PDCCH-order RACH on target neighbour cell. When UE is configured to send PDCCH-ordered RACH on target neighbour cell, it is necessary for UE to know the value of n-TimingAdvanceOffset for the target neighbour cell.

RAN2 has added n-TimingAdvanceOffset in EarlyUL-SyncConfig in TS 38.331 v18.0.0 as an optional field. However, the current field description does not specify the UE behaviour when n-TimingAdvanceOffset is absent. RAN2 understands that if the field is absent, the UE applies the value according to the duplex mode and frequency range of this candidate cell as specified in TS 38.133, table 7.1.2-2, which is similar with the description for n-TimingAdvanceOffset in ServingCellConfigCommon. In addition, the UE is able to determine the duplex mode and frequency range of a candidate cell by frequencyInfoUL-r18 configured in EarlyUL-SyncConfig-r18.

RAN2 would like to request RAN4 to confirm whether the above understanding is correct. If it is correct, RAN2 further request RAN4 to check whether the description for table 7.1.2-2 in TS 38.133 needs to be updated.

2. Actions:

To RAN4: RAN2 respectfully ask RAN4 to check the above RAN2 understanding.

3. Date of Next TSG-RAN WG2 Meetings:

RAN2#125-bis	15 - 19 Apr 	Changsha, China

RAN2#126	20 - 24 May 	Fukuoka, Japan