# 3GPP TSG RAN2#125  	                                                      R2-2401049

# Athens, Greece, Feb 26th – Mar 1st , 2023

Agenda item:	6.1.3

Title:	Considerations On the LS About TRS Configuration for idle/inactive UEs

Source:	ZTE Corporation, Sanechips

Document for:	Discussion and Decision

Introduction

- There is an LS received from RAN1 regarding the TRS resources for idle/inactive UE, and the following agreements are achieved in RAN1:

Agreement:

RAN1 confirms that the intended periodicity in periodicityAndOffset under TRS-ResourceSet-r17 range was 10, 20, 40 or 80ms as agreed RAN1#105-e and sends LS to RAN2 to update, if feasible, the periodicityAndOffset to cover these periodicities for Rel-18

- Also indicate that RAN2 avoid potential NBC impact for Rel-17 UEs

Conclusion

The interpretation of frequencyDomainAllocation under TRS-ResourceSet-r17 (Idle mode TRS) follows the same behaviour in TS38.211 as row1 in frequencyDomainAllocation under CSI-RS-ResourceMapping (CSI-RS for tracking).

- It is up to RAN2 to decide whether to clarify the field description of frequencyDomainAllocation in TRS-ResourceSet-r17 in specifications”

- RAN 1 require RAN2 to have actions for the newly achieved agreements.

Discussion

Periodicity of TRS Resource Set

In the LS, the first agreement achieved in RAN1 is to ask RAN2 whether it is feasible to change the periodicity of TRS resource set for idle/inactive UE from 10/20/40/80 slot to 10/20/40/80 ms in Rel-18.

In the current specification, the trs-ResourceSetConfig-r17 is contained in SIB17 and the TRS resource sets included in trs-ResourceSetConfig shall be activated upon the PEI or PDCCH for the P-RNTI and the indBitID configured in each TRS resource set.

TS 38.213

A UE in RRC_IDLE state or RRC_INACTIVE state can be provided by trs-ResourceSetConfig a set of TRS occasions [6, TS 38.214]. If trs-ResourceSetConfig is provided, a DCI format 2_7, if pei-SearchSpace is provided, and a DCI format 1_0 with CRC scrambled by P-RNTI includes a TRS availability indication field [4, TS 38.212] that provides a bitmap to groups of TRS resource sets where the configuration of each TRS resource set includes an association to a bit of the bitmap. The UE can be additionally provided a multiple, by validityDuration, for a number of frames provided by defaultPagingCycle for TRS resource sets with indicated presence; if validityDuration is not provided, the multiple is equal to 2.

In order to avoid NBC to R17 UE, there is only one way to do for SIB 17, that is, to add a new TRS configuration for R18 UE (e.g. trs-ResourceSetConfig-r18).

In order to avoid NBC issue to R17 UE, from RRC signaling perspective, the only way is to add a new TRS configuration for R18 UE particularly in SIB17.

However, even though we can have a new TRS configuration for R18 UE, the DCI/PEI for activating TRS resource set is also a broadcast information to both R17 UE and R17 UE. That means the current TRS activation indication in DCI/PEI will be valid to both R17 UE and R18 UE. As a result, one bit in TRS activation field will activate the TCI resource set with an intBitID configured in trs-ResourceSetConfig-r17 and the TCI resource set with the same intBitID configured in trs-ResourceSetConfig-r18. To deal with such control chaos, one direct way is to enhance the current DCI/PEI by adding the TRS resource set activation field in DCI/PEI particularly for R18 UE.

In order to avoid NBC impact of TRS resource set activation for R18 UE on R17 UE, the DCI/PEI shall have an extension of the TRS activation field particularly for R18 UE.

In one implementation for observation 2, to utilize the short message which can be handling by RAN2,the current short message have 4 reserved bits left which can be used for extension of the TRS activation field for R18 UE. However, in our understanding, the short message is normally used for urgent broadcast information such as: system information change, etws, paging change. The TRS for idle/inactive UE is not as much urgent as those information. We tend not to touch short message for TRS resource set activation/deactivation.

For extension of TRS resource set activation field in DCI 1-0, the short message shall not be touched from RAN2 perspective.

If short message is not utilized, the extension of the TRS activation field in DCI/PEI shall be discussed in RAN1 not RAN2. In this sense, RAN 2 cannot achieve any solution to avoid NBC impact for R17 UE to make the TRS periodicity of 10/20/40/80 ms for R18 UE.

There is no any solution in RAN2 can be achieved for making the TRS periodicity of 10/20/40/80 ms for R18 UE without any NBC impact on R17 UE.

In this sense, we think we should tell RAN1 what we understand on the issue raised by RAN LS through an LS back, the draft LS back is exhibited in the Annex part.

Send an LS back regarding the RAN2 understanding.

FrequencyDomainAllocation

Regarding the frequencyDomainAllocation in SIB17, it is defined as 4-bit Bit Sting type parameter and with the follow description:

frequencyDomainAllocation-r17              BIT STRING (SIZE (4)),

frequencyDomainAllocation

Indicates the offset of the first RE to RE#0 in a RB in row1.

It is actually a bit coarse to describe the frequency domain allocation for TRS resource set since there is no any reference table to be indicated about the row1. It is better to refine the wording and include in the rapporteur CR.

RAN2 is kindly asked to refine the field description of frequencyDomainAllocation in TRS-ResourceSet-r17 as ‘This field indicates the offset of the first RE to RE#0 in row 1 of Table 7.4.1.5.3 -1 as defined in TS 38.211 [16]’ , and capture refined wording in rapporteur CR for both R17/R18 RRC specification.

Conclusion

In this contribution, we provide our views on the TRS configuration triggered by RAN1 LS. We have the following observations and proposals:

Observation 1: In order to avoid NBC issue to R17 UE, from RRC signaling perspective, the only way is to add a new TRS configuration for R18 UE particularly in SIB17.

Observation 2: In order to avoid NBC impact of TRS resource set activation for R18 UE on R17 UE, the DCI/PEI shall have an extension of the TRS activation field particularly for R18 UE.

Proposal 1: For extension of TRS resource set activation field in DCI 1-0, the short message shall not be touched from RAN2 perspective.

Proposal 2: There is no any solution in RAN2 can be achieved for making the TRS periodicity of 10/20/40/80 ms for R18 UE without any NBC impact on R17 UE.

Proposal 3: Send an LS back regarding the RAN2 understanding.

Proposal 4: RAN2 is kindly asked to refine the field description of frequencyDomainAllocation in TRS-ResourceSet-r17 as ‘This field indicates the offset of the first RE to RE#0 in row 1 of Table 7.4.1.5.3 -1 as defined in TS 38.211 [16]’ , and capture refined wording in rapporteur CR for both R17/R18 RRC specification.

References

[1] R1-2312620 LS on periodicity of TRS resources for idle/inactive UEs

Annex(Draft LS)

- Overall Description:

RAN2 thank RAN1 about the question of the periodicityAndOffset for TRS-ResourceSet-r17. After discussion, RAN2 finds the NBC impact for R-17 UE is inevitable unless there is extension of the TRS resource set activation field in PEI/DCI for R18 UE particularly.

2. Action:

To RAN1 group.

ACTION: 	RAN2 respectfully requests RAN1 to kindly take the above into account in their work.