3GPP TSG-RAN WG2 Meeting #125	R2-2401210

Athens, Greece, Feb. 26th – Mar. 1st, 2024

Agenda item:	7.5.3.4

Source:	China Telecom

Title:	Remaining issues on DRX for XR

Document for:	Discussion and Decision

- Introduction

The XR WID was completed in RAN#102 meeting. During XR RRC specification offline discussion, the rapporteur had identified the following open issue for XR:

- Should it be possible to configure short non-integer DRX together with long integer DRX cycle. This might require some checking of MAC specifications as well, so this is something to potentially address in the next meeting.

This open issue has MAC impact. This contribution aims to discuss the open issues on the configuration of short non-integer DRX together with long integer DRX cycle and provide some proposals.

- Discussion

DRX has been considered as a useful mechanism to reduce UE power saving since 3G era. In legacy DRX mechanism, in case of the short DRX cycle in rational number is configured, the length of long DRX cycle should be an integer multiple of the short DRX cycle. The value of long DRX cycle is 2 or 3 times that of short DRX cycle. In order to support the traffic requirement for XR, non-integer long and short DRX cycles were introduced in Rel-18 XR WI. The following short and long DRX cycles in rational number are supported for XR traffic.

drx-LongCycleStartOffset            CHOICE {

ms10                                INTEGER(0..9),

ms20                                INTEGER(0..19),

ms32                                INTEGER(0..31),

ms40                                INTEGER(0..39),

ms60                                INTEGER(0..59),

ms64                                INTEGER(0..63),

ms70                                INTEGER(0..69),

ms80                                INTEGER(0..79),

ms128                               INTEGER(0..127),

ms160                               INTEGER(0..159),

ms256                               INTEGER(0..255),

ms320                               INTEGER(0..319),

ms512                               INTEGER(0..511),

ms640                               INTEGER(0..639),

ms1024                              INTEGER(0..1023),

ms1280                              INTEGER(0..1279),

ms2048                              INTEGER(0..2047),

ms2560                              INTEGER(0..2559),

ms5120                              INTEGER(0..5119),

ms10240                             INTEGER(0..10239)

},

shortDRX                            SEQUENCE {

drx-ShortCycle                      ENUMERATED  {

ms2, ms3, ms4, ms5, ms6, ms7, ms8, ms10, ms14, ms16, ms20, ms30, ms32, ms35, ms40, ms64, ms80, ms128, ms160, ms256, ms320, ms512, ms640, spare9, spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1 },

drx-ShortCycleTimer                 INTEGER (1..16)

}                                                                                                           OPTIONAL,   -- Need R

drx-SlotOffset                      INTEGER (0..31)

DRX-ConfigExt2-v1800- ::=                SEQUENCE {

drx-NonIntegerLongCycleStartOffset-r18  CHOICE {

ms1001over240                           INTEGER(0..3),

ms25over6                               INTEGER(0..3),

ms25over3                               INTEGER(0..7),

ms1001over120                           INTEGER(0..7),

ms100over9                              INTEGER(0..10),

ms25over2                               INTEGER(0..11),

ms40over3                               INTEGER(0..12),

ms125over9                              INTEGER(0..12),

ms50over3                               INTEGER(0..15),

ms1001over60                            INTEGER(0..15),

ms125over6                              INTEGER(0..19),

ms200over9                              INTEGER(0..21),

ms250over9                              INTEGER(0..26),

ms100over3                              INTEGER(0..32),

ms1001over30                            INTEGER(0..32),

ms125over3                              INTEGER(0..40),

ms1001over24                            INTEGER(0..40),

ms200over3                              INTEGER(0..65),

ms1001over15                            INTEGER(0..65),

ms250over3                              INTEGER(0..82),

ms1001over12                            INTEGER(0..82),

ms400over3                              INTEGER(0..132)

},

shortDRX-r18                            SEQUENCE {

drx-NonIntegerShortCycle-r18            ENUMERATED {ms1001over240, ms25over6, ms25over3, ms1001over120, ms100over9, ms25over2, ms40over3, ms125over9, ms50over3, ms1001over60, ms125over6, ms200over9, ms100over3, ms1001over30, ms125over3, ms1001over24, ms200over3, spare15, spare14, spare13, spare12, spare11, spare10, spare9, spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1},

drx-ShortCycleTimer-r18                 INTEGER (1..16)

}                                                                                                            OPTIONAL,   -- Need R

For example, the non-integer long DRX cycle, e.g. 125/3ms, are introduced as 3 times of the non-integer short DRX cycle, e.g. 125/9ms. For some non-integer short DRX cycles, e.g. 40/3ms, there exist corresponding integer long DRX cycle, e.g. 40ms.

Observation: For some non-integer short DRX cycles, there exist corresponding integer long DRX cycle.

However, it can be also observed that for some non-integer short DRX cycle, e.g. 25/3ms, there is no corresponding integer long DRX cycle. And the current the RRC specification does not support to configure the non-integer short DRX cycle and integer long DRX cycle together. To address the above issues, in our contribution [2], it is proposed to support the configuration of integer long DRX cycle together with the non-integer short DRX cycle, and introduce more integer long DRX cycles values corresponding to non-integer short DRX cycles.

Per current TS38.321, to address SFN warp around issue, a new counter DRX_SFN_COUNTER is introduced for non-integer DRX cycle. However, it based on the assumption that the length of long DRX cycle is an integer multiple of the short DRX cycle. And the drx-StartOffset is common for non-integer long DRX cycle and non-integer short DRX cycle. So, it is sufficient to only enable the counter DRX_SFN_COUNTER when the drx-NonIntegerLongCycleStartOffset is configured. However, to support the configuration of the non-integer short DRX cycle and integer long DRX cycle together, the new counter DRX_SFN_COUNTER also needs to be enabled when the drx-NonIntegerShortCycle is configured.

Proposal: To support the configuration of the non-integer short DRX cycle together with integer long DRX cycle, new counter DRX_SFN_COUNTER also needs to be enabled when the drx-NonIntegerShortCycle is configured.

- Conclusions

Based on the above analysis, we have the following observation and proposal:

Observation: For some non-integer short DRX cycles, there exist corresponding integer long DRX cycle.

Proposal: To support the configuration of the non-integer short DRX cycle together with integer long DRX cycle, new counter DRX_SFN_COUNTER also needs to be enabled when the drx-NonIntegerShortCycle is configured.

- References

- R2-2312604, Report of [POST123bis][023][XR] 38.331 Running CR (proposals on open issues for RRC CR of XR enhancements WI), Chicago, US

- R2-231xxxx Introduction of XR enhancements into TS 38.331, Chicago, US

Annex: TP to TS38.321

////////////////////////////////////////////////////////////////////////start of change ////////////////////////////////////////////////////////////////////////

The following UE variable is used for the DRX operation if drx-NonIntegerLongCycleStartOffset or drx-NonIntegerShortCycle is configured:

-	DRX_SFN_COUNTER: the counter that increments when SFN changes to 0. This counter can be implemented with a maximum value of 65535.

////////////////////////////////////////////////////////////////////////unchanged is skipped ////////////////////////////////////////////////////////////////////////

1>	if the drx-NonIntegerLongCycleStartOffset or drx-NonIntegerShortCycle is configured:

2>	increment DRX_SFN_COUNTER by 1 in the first symbol of a slot in which SFN changes to 0;

2>	if DRX is (re-)configured by RRC:

3>	set DRX_SFN_COUNTER to 0 in the first symbol of the slot immediately after the successful completion of the RRC (re-)configuration;

////////////////////////////////////////////////////////////////////////end of change ////////////////////////////////////////////////////////////////////////