3GPP TSG-RAN WG2 Meeting #125	R2-2401253

Athens, Greece, February 26 – March 1, 2024

Agenda item:	7.2.6

Source: 	Qualcomm Incorporated

Title: 	MAC Open Issue CA#02: MAC CE for activation/deactivation of aggregated SP SRS for positioning

Document for:		Discussion and Decision

# 1.	Introduction

This contribution discusses the following Open Issue CA#2:

| CA#02 | TBD | FFS whether to reuse the current MAC CE or design a new MAC CE for activation/deactivation of SP positioning SRS with multiple carrier indications |
| --- | --- | --- |

# 2.	Background

At RAN2#123, the following LS from RAN1 was received [1]:

1. Overall Description: With regard to SRS bandwidth aggregation for NR positioning, RAN1 made the following working assumption, and would like RAN2 to check the feasibility. Working assumption For semi-persistent positioning SRS for bandwidth aggregation, a single MAC CE can activate or deactivate: - SRS resource set(s) in one or two or three of three aggregated carriers - SRS resource set(s) in one or two of two aggregated carriers. Note: the single spatial relation is indicated by the MAC CE for each of two or three aggregated SRS resources. Send an LS to RAN2 to confirm the feasibility.. 2. Actions: To RAN2: ACTION: RAN1 respectfully asks RAN2 to check the feasibility of the working assumption and inform RAN1 of RAN2’s conclusion on the feasibility.

RAN2 made the following agreement [2]:

Agreement For activation/deactivation of aggregated SRS across two or three carriers, a single MAC CE is used. FFS if it can be a legacy MAC CE or a new one is needed.

Consequently, RAN1 turned the Working Assumption above into an agreement.

# 3.	Discussion

Aggregation (linkage) information is configured via RRC:

SRS-PosResourceSetLinkedForAggBWList-r18 ::= SEQUENCE (SIZE(1..maxNrOfLinkedSRS-PosResourceSet-r18)) OF SRS-PosResourceSetLinkedForAggBW-r18 SRS-PosResourceSetLinkedForAggBW-r18 ::= SEQUENCE { srs-PosResourceSetLinked-r18 SRS-PosResourceSetId-r16, freqInfo-r18 ARFCN-ValueNR OPTIONAL, -- Need R ul-bwp-ID BWP-Id OPTIONAL -- Need R } maxNrOfLinkedSRS-PosResourceSet-r18 INTEGER ::= 3

According to RAN1, the MAC-CE must be able to

-	activate/deactivate SRS resource set(s) in one or two or three of three aggregated carriers;

-	activate/deactivate SRS resource set(s) in one or two of two aggregated carriers;

-	indicate a single spatial relation for each of two or three aggregated SRS resources.

The Spatial Relation Information of "SP Positioning SRS Activation/Deactivation MAC CE" would suffice.

However, there is only one reserved bit in Octet 2, which is not sufficient to indicate 1, 2, or 3 aggregated carrier.

SP Positioning SRS Activation/Deactivation MAC CE

Some fields in "SP Positioning SRS Activation/Deactivation MAC CE" would not be needed, since they can be inferred from the RRC configuration:

-	Positioning SRS Resource Set's Cell ID, which indicates the identity of the Serving Cell, which contains the activated/deactivated SP Positioning SRS Resource Set. The identity of the linked SP Positioning SRS Resource Set would be known from RRC SRS-PosResourceSetLinkedForAggBWList.

-	Positioning SRS Resource Set's BWP ID, which indicates the UL BWP containing the activated/deactivated SP Positioning SRS Resource Set. The BWP ID of the linked SP Positioning SRS Resource Set would be known from RRC SRS-PosResourceSetLinkedForAggBWList.

-	C-field, which indicates whether Resource Serving Cell ID field(s) and Resource BWP ID field(s) within the field Spatial Relation for Resource ID i are present (in case of CSI-RS and SRS). The Resource Serving Cell ID field(s) and Resource BWP ID field(s) in the field Spatial Relation for Resource IDi (CSI-RS, SRS) can always be present.

-	SUL field, which indicates whether the MAC CE applies to the NUL carrier or SUL carrier configuration. The carrier configuration would be known from RRC SRS-PosResourceSetLinkedForAggBWList.

The activated/deactivated carriers could be indicated with a 3-bit map with reference to the RRC SRS-PosResourceSetLinkedForAggBWList.

Therefore, a single octet for the "SP Aggregated Positioning SRS Activation/Deactivation MAC CE" would be enough, together with the Spatial Relation octets.

A corresponding text proposal for TS 38.321 is provided in the Annex of this contribution.

# 4.	Summary

This contribution discussed the MAC-CE for activation/deactivation of aggregated SRS for positioning Resource Sets. A text proposal for TS 38.321 is provided in the Annex of this contribution.

Proposal:	Agree the TP in the Annex of this contribution.

# References

[1]	R2-2307010 (R1-2306214), "LS to RAN2 on SRS bandwidth aggregation for positioning", RAN1.

[2]	R2-2309117, "Reply LS on SRS bandwidth aggregation for positioning", RAN2.

# Annex: TP for TS 38.321

#### 6.1.3.XX	SP Aggregated Positioning SRS Activation/Deactivation MAC CE

The SP Aggregated Positioning SRS Activation/Deactivation MAC CE is identified by a MAC subheader with eLCID as specified in Table 6.2.1-1b. It has a variable size with following fields:

-	A/D: This field indicates whether to activate or deactivate indicated aggregated SP Positioning SRS resource sets. The field is set to 1 to indicate activation, otherwise it indicates deactivation;

-	S: This field indicates whether the fields Spatial Relation for Resource IDi for the positioning SRS resource i within the positioning SRS resource set are present. If the field is set to 1, the fields Spatial Relation for Resource IDi are present; otherwise, they are absent;

-	C1, C2, C3: These fields indicate the carriers which are linked for SRS for positioning bandwidth aggregation configured in SRS-PosResourceSetLinkedForAggBWList specified in TS 38.331 [5]. C1 corresponds to the first entry in SRS-PosResourceSetLinkedForAggBWList, C2 to the second entry in SRS-PosResourceSetLinkedForAggBWList, and C3 to the third entry in SRS-PosResourceSetLinkedForAggBWList. The field is set to 1 to indicate activation or deactivation of the linked SRS for positioning resource set; otherwise it is set to 0.

-	Spatial Relation for Resource IDi: The field Spatial Relation for Resource IDi is only present if MAC CE is used for activation, i.e. the A/D field is set to 1. There are 4 types of Spatial Relation for Resource IDi, which is indicated by the F (F0 and F1) field within. The fields within Spatial Relation for Resource IDi are the same as shown in Figures 6.1.3.36-2 to 6.1.3.36-5 for the 4 types of Spatial Relations for Resource IDi;

-	R: Reserved bit, set to 0.

Figure 6.1.3.XX-1: SP Aggregated Positioning SRS Activation/Deactivation MAC CE