3GPP TSG-RAN WG2 Meeting #125	R2-2401268

# Athens, Greece, 26th Feb – 1st Mar, 2024

Agenda Item:		7.24.2

Source: 	Huawei, HiSilicon

Title: 	Discussion on the reply to SA2 on RedCap UE MBS Broadcast reception

Document for:	Discussion

1	Introduction

In the last meeting, SA2 sent an LS to RAN2 and RAN3, and ask the questions on FSA ID, as below:

1. Overall Description: SA2 thanks RAN2 for the feedback. RAN2 has replied to SA2´s question 1: An indication that an MBS broadcast session is intended to be received by both non-RedCap UE and RedCap UE may assist the gNB to decide when to transmit the session on both default and RedCap CFR and avoid waste of resources when this is not needed. SA2 would like to inform that it agreed the attached CR against TS 23.247 to reflect that reply. SA2 understands that for an MBS session intended for both RedCap and non-RedCap UEs separate radio resources may be allocated by RAN nodes for RedCap and non-RedCap UEs. It is, however, unclear whether a single MBS Frequency Selection Area (FSA) ID can apply to RedCap UEs and non-RedCap UEs in the same MBS session. If it is not the case, whether it is feasible to adopt different MBS FSA IDs for RedCap and non-RedCap UEs. SA2 would thus like to ask RAN2 to answer the following related questions: Q1: SA2 would like to ask RAN2 to confirm the feasibility of having the same MBS FSA ID for the RedCap UEs and non-RedCap UEs in the same MBS session . Q2: If the answer to Q1 is no, could RedCap UEs and non-RedCap UEs in the same MBS session use separate MBS FSA ID(s)? SA2 would thus like to ask RAN3 to answer the following related question: Q3: If the answer to Q1 is no, and RedCap UEs and non-RedCap UEs in the same MBS session use separate MBS FSA ID(s), is there a need for CN to indicate to NG-RAN which FSA ID is aimed for RedCap UEs and which for non-RedCap UEs? 2. Actions: To RAN2, RAN3: ACTION: SA2 asks RAN2 and RAN3 to kindly answer the above questions.

In this contribution, we will discuss the issue of FSA ID for both RedCap UEs and non-RedCap UEs, and provide the answer to SA2 from RAN2 point of view.

2	Discussion

Based on the definition in TS 23.247, the MBS FSA ID identifies a preconfigured area within, and in proximity to, which the cell(s) announces the MBS FSA ID and the associating frequency. And there is also a note clarifying that the same MBS FSA ID(s) can be assigned to multiple MBS Broadcast sessions. In TS 38.413, upon receiving the MBS Session FSA ID List IE, the NG-RAN uses it to determine which cells/frequencies within the MBS service area to broadcast MBS data.

In TS 38.331, SIB21 contains the mapping between the current and/or neighbouring carrier frequencies and MBS Frequency Selection Area Identities (FSAI). Therefore, the finest granularity mapped to the FSA is the ARFCN-ValueNR, which is used for determining priority of the frequency during cell reselection. The frequency cannot be used to identify different CFRs for Redcap UEs and non-Redcap UEs.Currently, there is no redcap specific frequency, and both Redcap UE and non-Redcap UEs are distributed in the same network. There is no clear benefit to use separate MBS FSA ID for the RedCap UEs and non-RedCap UEs from RAN2 point of view.

With the above analyses, RAN2 only needs to confirm the feasibility of having the same MBS FSA ID for the RedCap UEs and non-RedCap UEs in the same MBS session.

Proposal: Draft an LS to confirm the feasibility of having the same MBS FSA ID for the RedCap UEs and non-RedCap UEs in the same MBS session.

3	Conclusion

In this contribution, we discussed the issue on FSA for Redcap UE. Based on our discussion, we conclude with the following proposals:

Proposal: Draft an LS to confirm the feasibility of having the same MBS FSA ID for the RedCap UEs and non-RedCap UEs in the same MBS session.

4	Reference

- S2-2401506		Reply LS on RedCap UE MBS Broadcast reception	Source: SA2, To:	RAN2, RAN3