3GPP TSG RAN WG2 Meeting #125	                                                    R2-2401427

# Athens, Greece, 26 February - 1 March, 2024

Agenda item:		7.14.4

Source:			Huawei, HiSilicon

Title:		Discussions on open issues for UE capabilities

Document for:	 	Discussion and decision

Introduction

This paper is to discuss open issues for UE capabilities.

Discussion

According to 38.306 rapporteur for R18 QoE, the following open issues are identified:

Open Issue 1: Whether RedCap UE should have the same minimum memory size for QoE in RRC_IDLE/RRC_INACTIVE as non-RedCap UE? Open Issue 2: Whether eRedCap UE shoud have the same minimum memory size for QoE in RRC_IDLE/RRC_INACTIVE as RedCap UE?

For AS buffer size for RedCap/eRedCap UE, the following options were discussed:

- 1st option: The minimum memory requirement is 64KB for RedCap/eRedCap UEs, for both Idle/Inactive and paused QoE reports

- 2nd option: Minimum memory for Idle/Inactive is 64KB, and this memory is in addition to 64KB used for QoE report storage during pause

The main difference is the extra 64KB on UE AS layer. For chipset vendors, this difference may be quite significant, e.g. increase costs.

Firstly, we think both options can work. Secondly, RedCap UEs should be of low cost, and thus smaller memory size can fit the goal. Otherwise, there may be less interests from chipset vendors to implement this feature. In this case, we see some benefits of the smaller value. Lastly, RAN2 agreed that UEs supporting idle/inactive QoE shall support one of the Rel-17 QoE UE capabilities. For the 1st option, it does not require extra memory for UEs to implement idle/inactive QoE feature on top of Rel-17 QoE features.

In general, we slightly prefer the 1st option.

For stage-3 details, we think the existing UE capability bit supportOfRedCap-r17 can be used for RedCap UEs. For eRedCap UEs, the WI has been closed and the WI summary can be found in [1]. In TS 38.306 v18.0.0, supportOfERedCap-r18 is for eRedCap UEs. In general, we think eRedCap UE can have the same minimum memory size for QoE as RedCap UE.

Proposal 1: The minimum memory requirement is 64KB total for both IDLE/INACTIVE and paused QoE reports for RedCap (supportOfRedCap-r17)/eRedCap UE(supportOfERedCap-r18).

Conclusion

Based on the discussion in this paper, we propose the following:

Proposal 1: The minimum memory requirement is 64KB total for both IDLE/INACTIVE and paused QoE reports for RedCap (supportOfRedCap-r17)/eRedCap UE(supportOfERedCap-r18).