3GPP TSG RAN WG2#126	R2-2406009

Fukuoka, Japan, 20th - 24th May 2024

3GPP TSG RAN WG1 #117	R1-2405671

Fukuoka city, Fukuoka, May 20th – 24th, 2024

# Title:	Reply LS on parameters used for CG RACH-less Handover from LTM perspective

# Response to:	R1-2403829/R2-2403996

# Release:	Rel-18

# Work Item:	NR_mob_enh2-Core

Source:	RAN1

To:	RAN2

Cc:

Contact Person:

Name:	Yosuke Akimoto

E-mail Address:	akimoto.yosuke at fujitsu.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None

1. Overall Description:

RAN1 has noticed that the RAN2 question described in “LS on parameters used for CG RACH-less Handover“ (R1-2403829/R2-2403996) is also related to NR_mob_enh2-Core work item.

CG-RRC-Configuration-r18 ::=   SEQUENCE {

cg-RRC-RetransmissionTimer-r18 INTEGER (1..64)                                               OPTIONAL,   -- Need R

cg-RRC-RSRP-ThresholdSSB-r18   RSRP-Range                                                    OPTIONAL,   -- Need R

rrc-SSB-Subset-r18             CHOICE {

shortBitmap-r18                BIT STRING (SIZE (4)),

mediumBitmap-r18               BIT STRING (SIZE (8)),

longBitmap-r18                 BIT STRING (SIZE (64))

}                                                                                            OPTIONAL,   -- Need S

rrc-SSB-PerCG-PUSCH-r18        ENUMERATED {oneEighth, oneFourth, half, one, two, four, eight, sixteen}  OPTIONAL,   -- Need M

rrc-P0-PUSCH-r18               INTEGER (-16..15)                                             OPTIONAL,   -- Need M

rrc-Alpha-r18                  ENUMERATED {alpha0, alpha04, alpha05, alpha06, alpha07, alpha08, alpha09, alpha1} OPTIONAL, -- Need M

rrc-DMRS-Ports-r18             CHOICE {

dmrsType1-r18                  BIT STRING (SIZE (8)),

dmrsType2-r18                  BIT STRING (SIZE (12))

}                                                                                             OPTIONAL,  -- Need M

rrc-NrofDMRS-Sequences-r18  INTEGER (1..2)                                                    OPTIONAL,  -- Need M

...

}

RAN1 discussed the necessity of the power control parameters used for CG-PUSCH transmission for LTM, which has been defined in TS38.331 v18.1.0. The consensus is that the underlined parameters as below, i.e. rrc-P0-PUSCH-r18 and rrc-Alpha-r18, are not applicable for LTM from RAN1 perspective.

2. Actions:

To RAN2 group.

ACTION: 	RAN1 respectfully asks RAN2 to update the ASN.1, if necessary.

3. Date of Next TSG RAN WG1 Meetings:

TSG RAN WG1 Meeting #118	19th– 23rd August 2024	Maastricht, NE

TSG RAN WG1 Meeting #118-bis	14th– 18th October 2024	 TBC, CN