3GPP TSG RAN WG2 #127                                                                                   R2-2406484

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Source:	CATT

Title:	Discussion on the Random Access for Ambient IoT

Agenda Item:	8.2.4

Document for:	Discussion and Decision

- Introduction

This paper further discusses the details related to the RACH procedure for A-IoT, including the consideration on “3-step” CBRA, “2-step” CBRA and CFRA. Section 2 gives our discussion, and followed by Section 3 that summarizes the conclusion of this paper.

- Discussion

  - Further consideration on CBRA

  - Details on “3-step” CBRA

On the general framework for the “3-step” CBRA, we had reached the following agreements in RAN2#126 meeting [1],

A-IoT Msg1: the device sends an ID to the reader. ID is a random ID generated by device (FFS how it is generated, e.g. randomly generated or generated based on Device ID). FFS on ID size. This doesn’t preclude any other RAN1 agreed information A-IoT Msg2: the reader echos the ID received in Msg1. Further information may be included in mgs2 based on RAN1 agreements A-IoT Msg3: device sends Device ID and/or any other upper layer data (depending on upper layer request) The device considers the contention resolution as successful, if the Msg2 including the same random ID in Msg1 is received. RAN2 assumes the size of random ID in Msg1 should be sufficient for contention resolution purpose. “Msg4” (i.e. the subsequent R2D transmission after D2R transmission) does not need to be always sent in random access. “Msg4” can be considered to handle the Msg3 transmission failure (due to various reasons). “Msg4” usage/presence can be further discussed. RAN2 will not use “Msg4” term for further discussion of the random access.

- How to generate random ID in A-IoT Msg1

For the “3-step” CBRA, the A-IoT Msg1 will send a random ID for contention resolution, which shares the similar way as that used for RFID. Regarding how to generate this ID, one way is to randomly generate it within the ID size. The controversy is some companies suggested using the variants based device ID (e.g. the partial device ID) for contention resolution. Note that the format of device ID depends on SA2 design, so there may be a risk that the partial device ID is not unique among all devices, which does not benefit the contention resolution. We think it is straightforward to use the random number for contention resolution. Therefore, it is proposed that,

Proposal 1: The random ID used in A-IoT Msg1 for “3-step” CBRA is randomly generated by the device within the ID size.

- ID size of the random ID in A-IoT Msg1

Regarding the random ID size, a fixed value helps to reduce the complexity for the device. If multiple devices select the same random ID, upon receiving the contention resolution, multiple devices will share the Msg3 resource for UL data transmission, which causes conflict. From our view, this can be considered as corner case with the following consideration,

- A large ID size, e.g. the 16-bit size, ensures enough random diversity between devices;

- With the same random ID for multiple devices, these devices can be distributed in different access occasions based on slot-ALOHA;

Even above mentioned corner case occurs, it can be left to implementation, i.e. the CN can re-trigger the inventory or command service for the conflicted devices. So it is feasible to use a fixed ID size to generate the random ID for contention resolution.

Observation 1: With a large enough ID size and slot-ALOHA, it is feasible to use a fixed ID size to generate the random ID for contention resolution.

However, we agree it also has benefits if the ID size can be indicated by the reader. By this way, the device does not need to always send a large random ID as it will require more radio resource and increase the decoding complexity.

We have slight prefer on the fixed size but still open to discuss, so it is proposed that,

Proposal 2: With a large enough ID size and slot-ALOHA, it is feasible to use a fixed ID size to generate the random ID for contention resolution. RAN2 to discuss whether the ID size in A-IoT Msg1 can be indicated by the reader.

- Whether to need Msg4

Another left issue is whether to have Msg4 to handle the Msg3 transmission failure. According to RAN1 agreements,

Study time-domain multiple access of D2R transmissions. Further details, including pros/cons, are FFS. Study frequency-domain multiple access of D2R transmissions, at least by utilizing a small frequency-shift in baseband. Further details, including pros/cons, are FFS. Whether code-domain multiple access is feasible and necessary for D2R transmissions for all devices is FFS.

The multiple access mechanism is under RAN1 discussion, so even multiple devices are successfully decoded in A-IoT Msg1, the subsequent PDRCH transmission should be allocated with orthogonal resource to avoid interference. That is to say, the failure of Msg3 may occur only when the channel quality of PDRCH is bad enough, instead of conflicted resource.

Observation 2: The failure of Msg3 transmission is mainly caused by the bad channel quality of PDRCH.

Nevertheless, this issue can also be mitigated by the physical layer repetition,

Define repetition types for study purposes as follows: - Block level: All the bits received from higher layers and/or physical layer (according to what is present) after CRC attachment (if used) are blockwise repeated Rblock times - Bit level type 1: Each bit after CRC attachment (if used) is repeated Rbit times - Bit level type 2: Each bit after both CRC attachment (if used) and FEC (if used) is repeated Rbit times - Chip level: Each chip after line coding (if used) or after square wave modulation (if used) is repeated Rchip times - NOTE: Equivalent to extending the duration of each chip by Rchip times For D2R, study at least block-level and bit-level repetition type 1 and type 2.

So from RAN2 perspective, we see no sufficient motivation to introduce Msg4 for Msg3 failure handling. Even Msg3 failure occurs with physical repetition, as our previously mentioned, it can be left to implementation, i.e. the CN can re-trigger the inventory or command service. By this way, the device does not need to be aware of Msg3 failure.

Therefore, it is suggested that,

Proposal 3a: Do not introduce Msg4 to handle the Msg3 transmission failure.

Proposal 3b: It’s up to CN implementation to re-trigger the inventory or command service for the situation where the reader fails to decode Msg3.

    - Details on “2-step” CBRA

On the general framework for the “2-step” CBRA, we had reached the following agreements in RAN2#126 meeting [1],

A-IoT Msg1: The device sends Device ID and/or any other upper layer data (depending on upper layer request). FFS what device ID is and whether an additional random ID is needed. This doesn’t preclude any other RAN1 agreed information A-IoT Msg2: the reader may echo some information from Msg1. FFS what some information is. “Msg2” usage/presence can be further discussed

Firstly, we would like to clarify that the description “The device sends Device ID and/or any other upper layer data” means for the A-IoT Msg1 of “2-step” CBRA, at least the device ID is included so that the reader knows which device the received UL data belongs to.

Observation 3: At least the device ID is included in the A-IoT Msg1 of “2-step” CBRA in order to let the reader know which device the received UL data belongs to.

- Whether to need an additional random ID in A-IoT Msg1

If the random ID is introduced, it needs to be transmitted within the same TB as the one for device ID, because only PDRCH is supported by RAN1 for UL transmission. This differs with NR 2-step RACH, as the MsgA of NR 2-step RACH is carried over PRACH and PUSCH. Therefore, for A-IoT, introducing random ID is not friendly to the Msg1 decoding.

From our perspective, it is feasible to use the device ID for contention resolution. Even SA3 requires the encryption, the device ID can be included in the higher layer, and the A-IoT MAC of the reader does not need to understand it but only responses with the same content in the container of the A-IoT MAC PDU.

So, we suggest that,

Proposal 4: The device ID is used for contention resolution in “2-step” CBRA. Do not introduce additional random ID in A-IoT Msg1.

- The content of A-IoT Msg2 for “2-step” CBRA

On the content of A-IoT Msg2 for “2-step” CBRA, it is used for the ACK of the data transmission in A-IoT Msg1. According to Observation 3, the device ID is always included in the A-IoT Msg1, so the reader can confirm the reception of A-IoT Msg1 by including the device ID in A-IoT Msg2. It may have the concern that the device ID may be too long, so partial device ID may be workable if we design reasonable truncation.

Proposal 5: The reader confirms the reception of data transmission in A-IoT Msg1 by including the device ID information in A-IoT Msg2. FFS including the whole device ID or partial device ID.

  - Further consideration on CFRA

- Clarification on the scenario for CFRA

In last meeting, we gave an initial discussion on the CFRA for A-IoT, and confirmed that we will support single device CFRA; for multiple device CFRA, it further depends on other groups such as RAN1.

From reader perspective, contention-free access procedure we will study single and multi-device case (depending on RAN1 discussion).

For multiple device case, it can be triggered by the initial trigger message with a group ID, not containing an ID, or containing multiple IDs,

RAN2 will study the following cases for AIoT paging message: • a message containing an ID of a single A-IoT device. • a message containing a group ID that maps to multiple A-IoT devices. • a message that does not contain an ID, i.e., addressed for all devices that can receive the AIoT message. • a message containing multiple IDs of A-IoT devices. Need to confirm the need for this use case based on SA2 discussion. What device ID and group ID and scenarios is depending on SA2 discussion.

For the initial trigger message containing multiple IDs, further SA2 discussion is needed to final confirm whether to support this case. Then for the other two cases, some further concern is foreseen.

In NR, the CFRA only applies to RRC_CONNECTED UE since it can be configured/indicated with the CFRA resource by dedicate signaling or PDCCH order. When considering these two cases, since no explicated ID is included in the initial trigger message, how does the device verify the dedicated resource allocated to itself.

Besides, the A-IoT service has no critical requirement on the access latency. Anyway we will consider re-access in the CBRA, which ensures the device to access to the NW, so we do not need much overdesign on the CFRA.

Therefore, we suggest firstly focusing on the single device CFRA.

Proposal 6: RAN2 to firstly study procedure for single device CFRA.

- Procedure for single-device CFRA

For the procedure of single device CFRA, it can be implemented by the following,

Step 0: The device receives the initial trigger message, and only a single device ID is contained within the initial trigger message;

Step 1: The device that matches the device ID sends the device ID and/or upper layer data to the reader.

Since only one device responses the initial trigger message, there is no need on the ACK from the reader after step 1. Even though the reader does not successfully decode the content of Step 1 due to the poor PDRCH channel quality, the reader can directly re-send the initial trigger message for this single device, without waiting for the CN indication to re-trigger the inventory or command service.

Proposal 7a: RAN2 to adopt the following procedure for single device CFRA:

Step 0: The devices receive the initial trigger message, and only a single device ID is contained within the initial trigger message;

Step 1: The device that matches the device ID sends its device ID and/or upper layer data to the reader.

Proposal 7b: The device will perform CFRA if the initial trigger message only contains a single device ID.

Proposal 7c: If the reader fails to decode the device ID and/or upper layer data for CFRA, it’s up to reader implementation to re-send the initial trigger message without waiting for the CN indication to re-trigger the inventory or command service.

  - Additional aspects

Baseline framework

In this section, we present the baseline framework with consideration of the RACH initialization as well as the slot ALOHA-based initial access and re-access, which is summarized as shown in Figure 1.

Figure 1: General framework with slot-ALOHA

According to Figure 1, the devices should perform slot-ALOHA with the following general framework,

1. The devices determine whether to join the current frame started by the Frame Start-like Command from the reader for initial access or re-access;

2. The devices determine whether to use the current access occasion started by the Access Occasion Start-like Command from the reader for initial access or re-access;

Each frame started by the Frame Start-like Command from the reader may include multiple access occasions. The reader may start new frame(s) for devices to perform initial access or re-access.

Each access occasion defines the time-domain resource (from the slot-ALOHA perspective) used by device(s) to perform initial access or re-access.

- How to trigger RACH

Until now we only confirm to use the initial trigger message to let devices know whether to make response. Then how to initiate the RACH procedure needs further discussion. In RFID, the frame start command was introduced to initiate the access procedure, which indicates the device that the access frame has been started and devices can perform the access procedure based on slot-ALOHA. Since we had agreed to take slot-ALOHA as baseline, it’s better to use a new command for the same purpose, instead of initial trigger message.

Since each frame may include multiple access occasions, similar as RFID, the occasion start-like command should be further designed to indicate the start of a new occasion.

Proposal 8a: Introduce frame start-like command to initiate the access procedure based on slot-ALOHA.

Proposal 8b: Introduce occasion start-like command to indicate the start of a new access occasion within the current frame.

- Failure handling on initial trigger message

In NR, the paging message is transmitted in PO(s) based on UE ID to enhance the reception performance. For the reception of initial trigger message in A-IoT, we had confirmed to not use PO-like concept. Besides, RAN1 only agreed to support the repetition for PDRCH, but not PRDCH. So there may be the case where the device fails to receive the initial trigger message.

One way is to allow the PRDCH repetition for the initial trigger message, but it further depends on RAN1 work. From RAN2 perspective, the key point is whether to introduce legacy PO-like repetition for the initial trigger message. In our view, there is no need to do so. Similar as the reception failure of Msg3 in CBRA and Msg1 in CFRA, the reception failure of the initial trigger message can also be left to CN implementation, i.e. the CN re-triggers the inventory or command service if it finds the desired service does not make response.

Proposal 9: It’s up to CN implementation to re-trigger the inventory or command service to handle the situation where the device may fail to receive the initial trigger message.

Additional aspects related to CBRA

- How to select “2-step” or “3-step” CBRA

Upon initialization of the CBRA, how to determine the “2-step” or “3-step” CBRA by the device need to be further confirmed. A most straightforward way is to indicate the “2-step” or “3-step” CBRA by the reader. This indication can be included in the initial trigger message or the frame/access occasion start-like command.

Proposal 10: The reader indicates the device to perform “2-step” or “3-step” CBRA. FFS included in the initial trigger message or the frame/access occasion start-like command.

- How to detect access failure by the device

The device needs to be aware of the failure of contention resolution so that it can perform the re-access. According to the discussion in Section 2.1 and Section 2.2, the detection of access failure by the device can be summarized as follows,

TABLE I: The detection of access failure

| RACH type | Content for contention | The detection of access failure |
| --- | --- | --- |
| “3-step” CBRA | Random ID for contention resolution | The device receives the A-IoT Msg2 from the reader, and the A-IoT Msg2 does not include the random ID sent in the A-IoT Msg1 by the device for the contention resolution |
| “2-step” CBRA | Device ID for data transmission | The device receives the A-IoT Msg2 from the reader, and the A-IoT Msg2 from the reader does not include the device ID information sent in the A-IoT Msg1 by the device for the data transmission |

Based on TABLE I, it is proposed that,

Proposal 11a: For “3-step” CBRA, the device considers the access failure if the received A-IoT Msg2 from the reader does not include the random number sent in the A-IoT Msg1 by the device for contention resolution.

Proposal 11b: For “2-step” CBRA, the device considers the access failure if the received A-IoT Msg2 from the reader does not include the device ID information sent in the A-IoT Msg1 by the device for data transmission.

- How to perform re-access

According to Figure 1, multiple access occasions are distributed within a frame. Generally speaking, there are two way forwards on the design of re-access.

TABLE II: Way forward for re-access

| Potential way forward | Pros | Cons |
| --- | --- | --- |
| WF1: Perform re-access in the subsequent access occasions, including the ones within the current frame or in the subsequent frames | Only perform access procedure one time within a certain access occasion, which simplifies the access design; Not introduce additional access latency on the other devices distributed in the subsequent access occasions; | Aggravate the burden for contention resolution in the subsequent access occasions; |
| WF2: Complete the access procedure within one access occasion, i.e. perform re-access in the same access occasion as the one used for initial access | Not aggravate the burden for contention resolution in the subsequent access occasions; | May have further impact on RAN1 design for the format of frame/slot, i.e., the access occasion needs to be variable in time-domain to allow multiple access attempt; Introduce additional access latency on the other devices distributed in the subsequent access occasions; |

Note that the potential WF2 takes the similar mechanism as that used for RFID, so it may need further check with RAN1 if we adopt the WF2 as baseline for the design of re-access.

Anyway the general procedure for re-access is under RAN2 scope, thus it is suggested RAN2 firstly down-select the baseline for the design of re-access.

Proposal 12: RAN2 to discuss the following options as the baseline of re-access,

Option 1 – Perform re-access in the subsequent access occasions, including the ones within the current frame or in the subsequent frames;

Option 2 – Complete the access procedure within one access occasion, i.e. perform re-access in the same access occasion as the one used for initial access.

- Conclusion

In this paper, we further discuss the RACH-related issues for A-IoT. The observations and proposals of this paper are summarized as follows,

Observation 1: With a large enough ID size and slot-ALOHA, it is feasible to use a fixed ID size to generate the random ID for contention resolution.

Observation 2: The failure of Msg3 transmission is mainly caused by the bad channel quality of PDRCH.

Observation 3: At least the device ID is included in the A-IoT Msg1 of “2-step” CBRA in order to let the reader know which device the received UL data belongs to.

Further consideration on CBRA

Proposal 1: The random ID used in A-IoT Msg1 for “3-step” CBRA is randomly generated by the device within the ID size.

Proposal 2: With a large enough ID size and slot-ALOHA, it is feasible to use a fixed ID size to generate the random ID for contention resolution. RAN2 to discuss whether the ID size in A-IoT Msg1 can be indicated by the reader.

Proposal 3a: Do not introduce Msg4 to handle the Msg3 transmission failure.

Proposal 3b: It’s up to CN implementation to re-trigger the inventory or command service for the situation where the reader fails to decode Msg3.

Proposal 4: The device ID is used for contention resolution in “2-step” CBRA. Do not introduce additional random ID in A-IoT Msg1.

Proposal 5: The reader confirms the reception of data transmission in A-IoT Msg1 by including the device ID information in A-IoT Msg2. FFS including the whole device ID or partial device ID.

Further consideration on CFRA

Proposal 6: RAN2 to firstly study procedure for single device CFRA.

Proposal 7a: RAN2 to adopt the following procedure for single device CFRA:

Step 0: The devices receive the initial trigger message, and only a single device ID is contained within the initial trigger message;

Step 1: The device that matches the device ID sends its device ID and/or upper layer data to the reader.

Proposal 7b: The device will perform CFRA if the initial trigger message only contains a single device ID.

Proposal 7c: If the reader fails to decode the device ID and/or upper layer data for CFRA, it’s up to reader implementation to re-send the initial trigger message without waiting for the CN indication to re-trigger the inventory or command service.

Additional aspects

Proposal 8a: Introduce frame start-like command to initiate the access procedure based on slot-ALOHA.

Proposal 8b: Introduce occasion start-like command to indicate the start of a new access occasion within the current frame.

Proposal 9: It’s up to CN implementation to re-trigger the inventory or command service to handle the situation where the device may fail to receive the initial trigger message.

Proposal 10: The reader indicates the device to perform “2-step” or “3-step” CBRA. FFS included in the initial trigger message or the frame/access occasion start-like command.

Proposal 11a: For “3-step” CBRA, the device considers the access failure if the received A-IoT Msg2 from the reader does not include the random number sent in the A-IoT Msg1 by the device for contention resolution.

Proposal 11b: For “2-step” CBRA, the device considers the access failure if the received A-IoT Msg2 from the reader does not include the device ID information sent in the A-IoT Msg1 by the device for data transmission.

Proposal 12: RAN2 to discuss the following options as the baseline of re-access,

Option 1 – Perform re-access in the subsequent access occasions, including the ones within the current frame or in the subsequent frames;

Option 2 – Complete the access procedure within one access occasion, i.e. perform re-access in the same access occasion as the one used for initial access.

- References

[1] Report of 3GPP TSG RAN WG2 meeting #126, Fukuoka, Japan