3GPP TSG-RAN WG2 Meeting #127	R2-2406850

Maastricht, Netherlands, 19 – 23 August 2024

Agenda item:	8.8.5

Source:	Nokia, Nokia Shanghai Bell

Title:	Addressing Potential Issues for NTN over Regenerative Architecture

WID/SID:	NR_NTN_Ph3 - Release 19

Document for:	Discussion and Decision

# 1	Introduction

In December 2023, RAN approved a Rel-19 work item (WI) aimed at defining Phase 3 enhancements to Non-Terrestrial Networks (NTN). There was a minor revision of the WID at RAN#103. Among the objectives captured in the WID [1] the following can be found:

Support of regenerative payload [RAN3, RAN2, RAN4] - Specify the support of gNB on board in TS 38.300 - Specify, if needed, any necessary enhancements related to the intra and inter-gNB mobility, especially for Xn interface over feeder link or over ISL. [RAN3] Note: if any additional necessary stage-3 specifications impact for e.g. NGAP is identified, RAN3 will handle it.

In this paper we present our view on the impact of existing mobility features over regenerative architecture.

# 2	Discussion

## 2.1	Satellite switching with re-synchronization

As part of Rel-18, RAN2 led the specification of the satellite switching with resynchronization feature. This feature, also known as “unchanged PCI” mobility, allows a UE to switch from one satellite to another one without carrying out an L3 handover (i.e., no dedicated RRC signalling is involved) under the conditions that gNB (which does not change) is on the ground and satellites will work as bent-pipe network nodes (i.e., transparent payload). Since the gNB does not change, the cell configuration can be maintained (i.e. SSB, SIB, paging and UE’s RRC configuration) after the satellite switching and no path switch is conducted at the AMF/UPF. The result is that RAN and CN require less signalling at the expense of a more complex NW implementation (i.e., the NW has less control over when UEs will be available in the target satellite and therefore resource allocation becomes more cumbersome).

Observation 1: Satellite switching with resynchronization was specified in Rel-18 for transparent payloads to reduce radio signalling load.

In a regenerative architecture, the satellite movement will constantly trigger inter-gNB mobility events (gNB on board of the satellite). In this contribution, we raise the question whether the satellite switching with resynchronization is yet feasible under these assumptions. Given that the gNB is no longer on the ground, the single (physical) gNB assumption is no longer valid, which means that the UE will frequently switch from a source gNB to a target gNB with the corresponding increase of signalling load derived from, at least, the patch switching at the AMF/UPF. Note that this is expected regardless of the mobility feature (e.g., during the CHO).

Observation 2: In a regenerative architecture, satellite switching under the assumption of single gNB is no longer valid since gNBs are on board of the satellites.

Observation 3: In a regenerative architecture and regardless of the mobility procedure, the UE will frequently switch between gNBs on board of different satellites, triggering additional AMF/UPF signalling (e.g., for path switching).

This leads to the following questions:

- Can the satellite switching feature with regenerative payload provide similar benefits as in the transparent architecture?

- In a regenerative payload, what would be the benefits of using the satellite switching instead of the existing RRC mechanisms?

- What changes (if any) are required to implement the satellite switching in a regenerative architecture?

One may argue that with a regenerative payload, connected mode mobility is already possible with basic mobility mechanisms such as the handover (HO) or the conditional HO (CHO). The signalling flow is the same as in terrestrial networks; upon HO decision by the source gNB, source and target gNBs exchange information for resource allocation and handover configuration. Then, the source gNB transmit the reconfiguration information to the UE which will use it to access the target gNB later (via RACH-based or RACH-less access). Once the RAN HO is successfully confirmed by the target gNB, RAN and CN coordinate to establish a new data route for the user’s data via the target gNB.

Observation 4: Connected mobility with regenerative payloads is possible with existing mobility mechanisms, i.e. CHO and HO procedures.

Figure 1 provides an overview of the regenerative architecture (including the interfaces connecting the different network components), while Figure 2 shows a possible implementation of the satellite switching with regenerative payload. The first step for the satellite switching could start with the switching decision by the Source gNB. In Step 2 and 3, Source and Target gNBs ‘agree’ on the satellite switch, which may be enhanced with exchange the information beneficial for the switching. As shown in Figure 1, this communication takes place over the Xn interface (either via inter-satellite link or routed through the NTN GW). Once the Source gNB is ready, it broadcasts the target satellite assistance information as part of SIB19 (same procedure as in Rel-18). Note that the preparation of the Target gNB could be done in advance and per cell instead of per UE to avoid a signalling storm.

Figure 1 NTN architecture with regenerative payload

On the UE side, the procedure may remain unchanged as compared with the transparent payload. The UE acquires SIB19 before the switching and initiates the resynchronization towards the target satellite upon t-Service (see Step 5 and 6). In this case, the switching occurs between gNBs, however, nothing prevents both gNBs to use the same cell configuration (i.e., virtually there could be only one cell).  This may require further orchestration and coordination between RAN nodes and OAM entities. Note that part of this coordination is anyway required for the L3 handover procedure (i.e., signalling exchange between source gNB and target gNB), while the UE benefits from skipping the reception of the HO command.

Observation 5: The source and target gNBs can use the same cell configuration.

Observation 6:The coordination between source and target gNB may be similar as compared to the conventional L3 mobility, while the UE benefits from a signalling reduction.

Another aspect to consider is the security aspect. If the target gNB has different security parameters than the serving gNB, inter-gNB mobility procedures often involve a change in security keys, which are used to ensure data integrity and confidentiality. After receiving the handover command from the serving gNB and initiating the handover process, the UE updates its security context with new security keys derived based on the information received from the target gNB. As part of this process, the UE resets its PDCP entity, flushes data buffers, and establishes a new security context with the target gNB.

In order to accommodate the satellite switching feature with a regenerative payload, RAN2 should consider asking SA3 about its implications and feasibility regarding the security aspects.

Observation 7: During the satellite switching, the UE behaviour may remain unchanged regardless of the NTN architecture (apart for the security config).

Once the UE has completed the satellite switching and the NW has acknowledged it, the satellite switching completion can be notified to the Source gNB by the target (Step 7). In order to finish the mobility procedure, the user’s data must be re-routed. As in Step 9 of Figure 2, Target gNB and AMF/UPF convey a path switch so Source gNB can stop forwarding user’s data to the target. Finally, Target gNB sends a UE context release to the Source gNB.

As one may observe, the satellite switching with regenerative payload may re-use large part of the handover procedure, while UEs can benefit from avoiding RRC reconfiguration at the expense of an increased NW complexity in comparison to the baseline HO procedure.

Observation 8: For satellite switching with regenerative payload, Rel-19 UE may benefit from similar gains as in transparent architecture while NW complexity may increase compared with regular L3 procedures (to be checked with RAN3 and SA3 for security issues).

Nonetheless, we propose the following:

Proposal 1: RAN2 to kindly ask RAN3 and SA3 on the feasibility of the satellite switching with resync with regenerative payload.

Proposal 2: If RAN3 and SA3 found the satellite switching with resync to be feasible, RAN2 to discuss the potential benefits with respect to legacy mobility procedures. This might be also subject to additional approval at the RAN Plenary level.

Proposal 3: If satellite switching with resync with regenerative payload is supported, RAN2 to endorse Figure 2 as baseline for the satellite switching with regenerative payload.

Figure 2 Satellite switching with resynchronization in a regenerative architecture.

## 2.2	RACH-less handover

RACH-less handover was introduced in Rel-18 for NTN, among other features. One of the advantages of this procedure is that the UE can access the target cell without exchanging Msg1 and Msg2 of the RACH procedure and therefore reducing the signalling overhead. The main challenge of this procedure is the acquisition of the UL grant for the first UL transmission in the target node.

The UE may acquire the UL grant via a configured grant included in the HO command or dynamically transmitted in the target cell. As indicated in Figure 3, for the latter the UE accesses the target cell and monitors the PDCCH channel waiting for the target cell to transmit an UL assignment. If the timing of when the UE will access the cell is unclear for the target cell this may lead to waste of resources. If the target cell transmits the UL grant too early (UE is not ready yet to acquire it), the cell may have to transmit the UL assignment for longer time wasting NW resources and increasing the risk of the HO failure. If the target cell transmits the UL grant too late, that may result in the UE monitoring the PDCCH for longer than required.

Observation 9: Target cell may underestimate/overestimate the appropriate time to transmit the UL assignment over PDCCH, which may lead to a waste of resources and increased power consumption at the UE’s side.

Figure 3 RACH-less HO with dynamic grant

In order to transmit the UL grant timely, the target cell has to estimate when the most appropriate time is. In Rel-18, only the transparent architecture was considered. In that case, assuming that serving and target cells belong to the same gNB located on the ground, the gNB can know when the source cell will stop serving the area and when the UE will access the cell. In Rel-19, with the specification of the regenerative payload, some issues may arise. Using the RACH-less HO procedure in a regenerative architecture means that serving gNB and target gNB are on board of separate satellites. In this case, the target gNB has no means to know when the UE may attempt cell access and its only time reference is when the HO Request ACK message was transmitted.

Observation 10: In regenerative architecture, target gNB has no means to know when the source gNB will stop serving the area and when the UE will access the cell.

One potential approach to reduce this uncertainty is to combine RACH-less HO with time-based CHO procedure. According to TS 38.423, as part of the HO Request, the source NG-RAN node can send to the target NG-RAN node the T1-T2 parameters (part of the condEventT1). In this case, the target gNB can make a better estimation given that it knows the time window when the UE will execute the HO (i.e., between T1 and T2).

Observation 11: In case of combining RACH-less HO and time-based CHO, the target gNB can better estimate when to transmit the UL assignment since it is aware of the T1-T2 time window.

However, we cannot assume that this is always possible (i.e., according to the specifications a RACH-less access may be combined with other mobility procedures such as the Release 16 CHO or baseline Release 15 HO), and therefore, RAN2 should provide means for all the configurations supported in NTN. For this reason, we propose the following:

Observation 12: The RACH less handover feature works for regenerative architecture, while few optimisations are possible, like informing the target gNB when the UE is supposed to be present in the target cell.

Proposal 4: RAN2 to discuss means to reduce target gNB’s uncertainty during RACH-less handover for regenerative architecture.

# 3	Conclusion

This document has made the following observations:

Observation 1: Satellite switching with resynchronization was specified in Rel-18 for transparent payloads to reduce radio signalling load.

Observation 2: In a regenerative architecture, satellite switching under the assumption of single gNB is no longer valid since gNBs are on board of the satellites.

Observation 3: In a regenerative architecture and regardless of the mobility procedure, the UE will frequently switch between gNBs on board of different satellites, triggering additional AMF/UPF signalling (e.g., for path switching).

Observation 4: Connected mobility with regenerative payloads is possible with existing mobility mechanisms, i.e. CHO and HO procedures.

Observation 5: The source and target gNBs can use the same cell configuration.

Observation 6:The coordination between source and target gNB may be similar as compared to the conventional L3 mobility, while the UE benefits from a signalling reduction.

Observation 7: During the satellite switching, the UE behaviour may remain unchanged regardless of the NTN architecture (apart for the security config).

Observation 8: For satellite switching with regenerative payload, Rel-19 UE may benefit from similar gains as in transparent architecture while NW complexity may increase compared with regular L3 procedures (to be checked with RAN3 and SA3 for security issues).

Observation 9: Target cell may underestimate/overestimate the appropriate time to transmit the UL assignment over PDCCH, which may lead to a waste of resources and increased power consumption at the UE’s side.

Observation 10: In regenerative architecture, target gNB has no means to know when the source gNB will stop serving the area and when the UE will access the cell.

Observation 11: In case of combining RACH-less HO and time-based CHO, the target gNB can better estimate when to transmit the UL assignment since it is aware of the T1-T2 time window.

Observation 12: The RACH less handover feature works for regenerative architecture, while few optimisations are possible, like informing the target gNB when the UE is supposed to be present in the target cell.

And proposed the following:

Proposal 1: RAN2 to kindly ask RAN3 and SA3 on the feasibility of the satellite switching with resync with regenerative payload.

Proposal 2: If RAN3 and SA3 found the satellite switching with resync to be feasible, RAN2 to discuss the potential benefits with respect to legacy mobility procedures. This might be also subject to additional approval at the RAN Plenary level.

Proposal 3: If satellite switching with resync with regenerative payload is supported, RAN2 to endorse Figure 2 as

Proposal 4: RAN2 to discuss means to reduce target gNB’s uncertainty during RACH-less handover for regenerative architecture. baseline for the satellite switching with regenerative payload.

# References

- RP-240775 Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3 3GPP TSG RAN Meeting #103 Maastricht, The Netherlands, March 18-22, 2024

- 3GPP TS 38.331 NR RRC, v18.2.0, July 2024.

- R2-2402355, Discussion on support of broadcast service via NR NTN, CATT, China Broadnet, 3GPP TSG RAN2#125bis meeting, Rel-19, April 2024.