3GPP TSG-RAN WG2 #127	R2-2407543

Maastricht, Netherland, 19th – 23rd Aug, 2024

Source:	NTT DOCOMO, INC.

Title:	Discussion on Procedure and configuration in RRC_IDLE-INACTIVE

Document for:	Discussion

Agenda Item:	8.4.2

- Introduction

A new WID on LP-WUS/WUR has been approved at the RAN#102 meeting [1] and revised at the RAN#103[2]. In the WID, following objectives are included.

- For IDLE/INACTIVE modes - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4)

In RAN2#125-bis and RAN2#126 meeting, following agreements has been made

RAN2 #125bis Agreement The LP-WUS related configuration for IDLE/INACTIVE state is provided via system information. FFS if dedicated configuration is needed. Working assumption: the LP-WUS configuration in SIB at least includes the following information: - - LP-SS configuration - - LP-WUS configuration - - FFS on Entry/exit condition for LP-WUS monitoring The PEI subgrouping method is taken as baseline for LP-WUS subgrouping, i.e. CN assigned and UE_ID based subgrouping. FFS the maximum number of subgroups.

RAN2#126 Agreement RAN2 will further discuss the details about LP-WUS monitoring entry/exit conditions based on RAN1’s existing working assumptions. The LP-WUS related configuration in SIB at least include the following information for IDLE/INACTIVE: - - LP-SS configuration - - LP-WUS configuration - - Entry/exit condition for LP-WUS monitoring (FFS if it is always configured) Baseline for entry condition definition: If the serving cell quality, e.g. RSRP, RSRQ from MR, is above threshold(s) (if configured), UE may start to monitor LP-WUS, if UE monitors LP-WUS, it may stop monitoring the legacy PO. FFS if any measurement from LR is needed. Baseline for exit condition definition: If the serving cell measurement result based on LR is below a threshold (if configured), UE monitors PO as in legacy and it may stop monitoring the LP-WUS. RAN2 understand that if UE is configured with CN-based LP-WUS subgrouping, it is up to CN to assign the LP-WUS subgroup ID to the UE. RAN2 assume the maximum number of subgroups that can be configured for LP-WUS subgrouping is no less than 8. From RAN2 perspective, no new procedure is introduced for SI reception/updates.

In this contribution, we describe our view for procedure and configuration of LP-WUS/WUR for RRC_IDLE/INACTIVE state.

- Discussion

  - Sub-grouping

In RAN2#125bis meeting, it was agreed that the PEI subgrouping method is taken as baseline for LP-WUS subgrouping, i.e. CN assigned and UE_ID based subgrouping. The number of subgroups depends on the LP-WUS structure, and it is still under discussion in RAN1, so RAN2 should wait for the progress of RAN1.

It is proposed in [3] that when UE_ID based subgrouping is used and PEI is used simultaneously, the false paging rate can be reduced by using a different allocation method than subgrouping with PEI. In this case, it is possible to use “5G-S-TMSI” as the UEID for subgrouping of LP-WUS, and change the reference method slightly (e.g. UEID for LPWUS = 5G-S-TMSI / subgroupsNumForUEID) to ensure that the subgrouping pattern will be different from that of PEI. Therefore, RAN2 should discuss how to determine the LP-WUS subgroup using “5G-S-TMSI” as the baseline for calculating the UE_ID based subgrouping.

Proposal 1: 	In UE_ID based subgrouping, a mechanism should be applied that allocates PEI and LP-WUS subgroups differently in order to reduce the false paging rate.

Proposal 2: 	For the LP-WUS subgroup, "5G-S-TMSI" is also used, but a different calculation formula is applied to PEI.

  - Entry/exit condition for LP-WUS monitoring

In RAN2#126 meeting, following agreement has been made.

The LP-WUS related configuration in SIB at least include the following information for IDLE/INACTIVE: - - LP-SS configuration - - LP-WUS configuration - - Entry/exit condition for LP-WUS monitoring (FFS if it is always configured) Baseline for entry condition definition: If the serving cell quality, e.g. RSRP, RSRQ from MR, is above threshold(s) (if configured), UE may start to monitor LP-WUS, if UE monitors LP-WUS, it may stop monitoring the legacy PO. FFS if any measurement from LR is needed. Baseline for exit condition definition: If the serving cell measurement result based on LR is below a threshold (if configured), UE monitors PO as in legacy and it may stop monitoring the LP-WUS.

In the discussions so far on RAN2, it has been FFS if it is always configured the entry/exit condition for LP-WUS monitoring. The coverage of the LP-WUS and the coverage of the serving cell are different due to the characteristics of the signal. In general, the coverage of the LP-WUS is considered to be smaller than coverage of the serving cell. In such a case, if the UE leaves the coverage of the LP-WUS during LP-WUS monitoring, it will be no longer possible from the network side to control the UE. Also, from the UE's point of view, it is difficult to know whether LP-WUS is not being transmitted or it is outside the coverage area, so it is also difficult to exit LP-WUS monitoring by itself. Therefore, in order to clarify UE behavior, the "Entry/exit condition for LP-WUS monitoring" should always be configured, or the UE behavior when it is not configured should be specified.

Proposal 3: 	The "Entry/exit condition for LP-WUS monitoring" in SIB should always be configured, or the UE behavior when it is not configured should be specified.

- Conclusion

Proposal 1: 	In UE_ID based subgrouping, a mechanism should be applied that allocates PEI and LP-WUS subgroups differently in order to reduce the false paging rate.

Proposal 2: 	For the LP-WUS subgroup, "5G-S-TMSI" is also used, but a different calculation formula is applied to PEI.

Proposal 3: 	The "Entry/exit condition for LP-WUS monitoring" in SIB should always be configured, or the UE behavior when it is not configured should be specified.

- References

[1] RP-234056, New WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)

[2] RP-240801, Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)

[3] R2-2402624, Discussion on LP-WUS WUR in RRC_IDLE INACTIVE, vivo